SPI Data Frame Anatomy, Packet Structures & Protocol Analysis

This document provides a comprehensive technical breakdown of SPI Data Frames: physical vs logical framing, command-response architectures, dummy cycle mechanics, device-specific packet archetypes (Flash, IMUs, ADCs, Automotive Safe SPI), CRC verification, and logic analyzer frame decoding.

1. Physical vs. Logical SPI Framing

Unlike UART (which specifies an exact 10-bit physical hardware frame) or I2C (which defines explicit START, ADDRESS, ACK/NACK, and STOP conditions on the wire), raw physical SPI is pure bit streaming.

To build reliable embedded systems, engineers distinguish between Physical Framing and Logical Framing:

PHYSICAL LAYER: CS# Assert ──> [Clock Pulses & Dual Shift Registers] ──> CS# Deassert │ ▼ LOGICAL LAYER: [Opcode / CMD] [Address / Reg] [Turnaround / Dummy] [Data Payload] [CRC]

1.1 Physical Framing Boundaries

Frame Initiation: The Controller drives Chip Select (\overline{CS) from HIGH to LOW. This resets the peripheral's internal bit counter and state machine, and enables its MISO output driver.

Clocking & Shifting: SCLK pulses transmit data bidirectionally between the Controller and Peripheral shift registers.

Frame Termination: The Controller drives \overline{CS from LOW to HIGH. This:

Latches received commands or written data into internal device registers.

Returns the peripheral's MISO pin to High-Z (tri-state).

Resets state logic to prepare for the next transaction.

[IMPORTANT] Chip Select is the Word & Packet Boundary: If \overline{CS does not toggle between independent commands, many SPI peripherals (such as EEPROMs and ADCs) will fail to execute commands, misinterpret opcodes, or permanently ignore subsequent clock cycles.

1.2 Word Size vs. Packet Size

Word Size (Bits per Word): The atomic unit clocked in one continuous chunk. Standard microcontrollers support 8-bit, 16-bit, or 32-bit word lengths. Many precision instrumentation converters use non-standard sizes (10-bit, 12-bit, 14-bit, or 24-bit).

Packet Size (Transaction Length): The total number of bytes transferred between \overline{CS falling and rising. A single packet can range from 1 byte (e.g. Write Enable opcode 0x06) to 256+ bytes (e.g. Flash page programming) or several kilobytes in DMA burst transfers.

2. Universal Anatomy of a Logical SPI Frame

A generalized SPI command-response data frame contains up to five distinct functional phases:

CS# ──────┐ ┌────── └─────────────────────────────────────────────────────────────────────────────────────────────────────────┘ MOSI: ¦ Phase 1: Command ¦ Phase 2: Address ¦ Phase 3: Turnaround ¦ Phase 4: Data Payload ¦ Phase 5: CRC ¦ ¦ (Opcode / R/W) ¦ (Register / Mem) ¦ (Dummy Wait Clocks) ¦ (Write Bytes / 0x00) ¦ (Optional) ¦ MISO: ¦ Don't Care / Echo ¦ Don't Care / Echo ¦ High-Z / Pipelining ¦ (Read Response Bytes) ¦ (Optional) ¦

Phase 1: Instruction / Command Phase (Opcode)

Always driven by the Controller on MOSI.

Identifies the operation to perform: Read, Write, Erase, Read Status, Reset, etc.

In many sensor ICs (accelerometers, gyroscopes), the command byte integrates the R/W direction bit and Auto-Increment / Burst bit directly into the upper bits.

Phase 2: Address Phase

Identifies the destination register address (sensors) or 16-bit / 24-bit / 32-bit memory address (Flash/EEPROM).

Transmitted MSB-first.

For devices exceeding 16 MB (128 Mb) capacity, modern SPI Flash utilizes a 4-byte (32-bit) addressing mode instead of legacy 3-byte (24-bit) addressing.

Phase 3: Turnaround & Dummy Wait States

The Turnaround Problem: In high-speed read transactions (> 25\,MHz), internal memory arrays and ADCs cannot fetch internal data cells fast enough to output valid data on the very next clock edge following the address.

The Solution: The Controller transmits a fixed number of Dummy Cycles (typically 1 to 8 clock cycles / 1 byte) where MOSI outputs 0x00 or 0xFF.

During these dummy cycles, the peripheral retrieves data from its internal array and prepares its output shift register. Data appears on MISO immediately following the dummy phase.

SCLK: ... ──┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ... └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ MOSI: ... ────< Address Bit 0 >───< DUMMY BYTE (Host clocks wire) >─── ... MISO: ... ────< High-Z >───< Array Access >───< Data Bit 7 (Valid Output) > ... ¦<-- t_access ->¦ ▲ First valid read bit

Phase 4: Data Payload Phase

For Write Transactions: MOSI carries user data bytes; MISO echoes dummy bytes, device status flags, or prior register contents.

For Read Transactions: MOSI supplies clocking dummy bytes (0x00 or 0xFF); MISO drives the requested data bytes.

Phase 5: Frame Integrity / Checksum Phase (Optional)

Used in safety-critical, automotive, and industrial peripherals (AEC-Q100, ISO 26262 ASIL).

Carries an 8-bit or 16-bit CRC calculated over all preceding bytes in the frame.

If the calculated CRC does not match, the peripheral rejects the write command and asserts an error flag in its status register.

3. Real-World SPI Frame Archetypes & Case Studies

3.1 Archetype A: SPI NOR Flash Memory (e.g. Winbond W25Q128 / Macronix MX25)

Serial NOR Flash chips are the most common SPI devices in embedded systems, serving as boot media for Linux SoCs and firmware storage for microcontrollers.

1. JEDEC Identification Read Frame (0x9F)

Reads manufacturer code, memory type, and capacity. Total packet length: 4 bytes.

CS# ──────┐ ┌────── └─────────────────────────────────────────────────────────────────────────────┘ SCLK: [ 8 Clocks: Byte 0 ] [ 8 Clocks: Byte 1 ] [ 8 Clocks: Byte 2 ] [ 8 Clocks: Byte 3 ] MOSI: ──[ 0x9F (Command) ]──[ 0x00 (Dummy) ]──[ 0x00 (Dummy) ]──[ 0x00 (Dummy) ]── MISO: ──[ Stale / High-Z ]──[ 0xEF (Mfg ID) ]──[ 0x40 (Mem Type) ]──[ 0x18 (Capacity) ]── (0xEF = Winbond) (SPI / QSPI) (0x18 = 128 Mbit)

2. Standard Read (0x03) vs. High-Speed Fast Read (0x0B)

Standard Read (`0x03`): Maximum clock speed limited to \approx 33 - 50\,MHz. Frame: [0x03] + [24-bit Address] + [Data Bytes...]. No dummy byte.

Fast Read (`0x0B`): Allows operation up to 104 - 133\,MHz. Requires 1 Dummy Byte (8 dummy clocks) after the address to accommodate array access latency:

MOSI: ──[ 0x0B ]──[ Addr[23:16] ]──[ Addr[15:8] ]──[ Addr[7:0] ]──[ DUMMY (0x00) ]──[ 0x00 ]──[ 0x00 ]... MISO: ──[ XX ]──[ XX ]──[ XX ]──[ XX ]──[ WAIT ]──[ D0 ]──[ D1 ]...

3. Page Program (Write) Frame (0x02)

Flash writes require a two-step sequence:

First Frame: Issue Write Enable (`0x06`) (1-byte frame, \overline{CS toggled).

Second Frame: Issue Page Program (`0x02`) with 24-bit address and 1 to 256 payload bytes:

Frame 1: CS# ──┐ ┌── MOSI: └──[0x06]──┘ (WEL bit set in Status Register) Frame 2: CS# ──┐ ┌── MOSI: └──[ 0x02 ]──[ Addr MSB ]──[ Addr MID ]──[ Addr LSB ]──[ Data 0 ]...[ Data 255 ]──┘ MISO: ───[ XX ]──[ XX ]──[ XX ]──[ XX ]──[ XX ]...[ XX ]───

3.2 Archetype B: Digital Sensors & IMUs (e.g. ADXL345, BMI160, ICM-42688)

Digital motion and environmental sensors use compact register-mapped framing. To optimize bandwidth, the command byte packs addressing and control flags into a single 8-bit word.

Frame Format Breakdown (e.g. ADXL345 3-Axis Accelerometer):

Bit 7 ($\text{R}/\overline{\text{W}}$): 1 = Read operation; 0 = Write operation.

Bit 6 ($\text{MB}$): Multiple-Byte / Auto-Increment bit (1 = Stream multiple sequential registers; 0 = Single byte).

Bits [5:0] ($\text{Reg Addr}$): 6-bit register address (0x00 to 0x3F).

Command Byte Bit Layout: ┌───────┬───────┬───────┬───────┬───────┬───────┬───────┬───────┐ │ Bit 7 │ Bit 6 │ Bit 5 │ Bit 4 │ Bit 3 │ Bit 2 │ Bit 1 │ Bit 0 │ ├───────┼───────┼───────┼───────┼───────┼───────┼───────┼───────┤ │ R/W# │ MB │ Register Address [5:0] │ └───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┘

Multi-Byte Burst Read Example (Reading X, Y, Z Acceleration Atomically):

To prevent axis skew where X is sampled at time T_0 and Z at time T_1, all 6 data registers (DATAX0 to DATAZ1, registers 0x32 to 0x37) must be read in a single continuous $\overline{\text{CS}}$ window:

MOSI Byte 0: 0xC0 ¦ 0x32 = 0xF2 (Bit 7=1 for Read, Bit 6=1 for Multi-Byte, Bits 5:0 = 0x32) CS# ──────┐ ┌────── └─────────────────────────────────────────────────────────────────────────────────────────────────────────┘ MOSI: ──[ 0xF2 (CMD) ]──[ 0x00 (Dummy) ]──[ 0x00 (Dummy) ]──[ 0x00 ]──[ 0x00 ]──[ 0x00 ]──[ 0x00 (Dummy) ]── MISO: ──[ Stale / XX ]──[ DATAX0 (0x32)]──[ DATAX1 (0x33)]──[ DATAY0]──[ DATAY1]──[ DATAZ0]──[ DATAZ1 (0x37)]──

During this single frame, the sensor's internal address pointer auto-increments after each byte without requiring repeated command headers.

3.3 Archetype C: Precision Analog-to-Digital Converters (ADCs)

Many high-speed precision ADCs (e.g. Microchip MCP3008, Texas Instruments ADS7886, Analog Devices AD7928) use arbitrary bit-length framing that does not align with standard 8-bit byte boundaries.

Case Study: MCP3008 10-Bit 8-Channel ADC Frame

The MCP3008 requires a 24-clock-cycle (3-byte) transfer to configure the channel multiplexer and clock out a 10-bit analog conversion:

MOSI (Host -> ADC): Byte 0: 0 0 0 0 0 0 0 1 (Start Bit: 0x01) Byte 1: S D2 D1 D0 0 0 0 0 (S=Single/Diff, D2:D0=Channel Select, e.g. 0x80 for CH0 Single) Byte 2: 0 0 0 0 0 0 0 0 (Don't Care / Dummy: 0x00) MISO (ADC -> Host): Byte 0: X X X X X X X X (High-Z / Undefined) Byte 1: X X X X X 0 B9 B8 (Bits [7:3] High-Z; Bit 2 = Null Bit; Bits [1:0] = MSB Bits 9 & 8) Byte 2: B7 B6 B5 B4 B3 B2 B1 B0 (Data Bits 7 down to 0)

Timing Waveform: CS# ──────┐ ┌────── └─────────────────────────────────────────────────────────────────────────────┘ SCLK: ¦ 1 ¦ 2 ¦ 3 ¦ 4 ¦ 5 ¦ 6 ¦ 7 ¦ 8 ¦ 9 ¦10 ¦11 ¦12 ¦13 ¦14 ¦15 ¦16 ¦17 ... 23 ¦24 ¦ MOSI: ──[ 0 0 0 0 0 0 0 1 ] [ S D2 D1 D0 0 0 0 0 ] [ 0 ... 0 ]── MISO: ────────────────────────────[High-Z]──────[Sample] [Null¦ B9 B8 ] [ B7 ... B0 ]── ▲ Null Bit = Always 0

Decoding Formula:

\text{Raw ADC Value (10-bit)} = ((\text{Byte1} \ \&\ 0x03) \ll 8) \ | \ \text{Byte2}

\text{Measured Voltage} = \frac{\text{Raw Value}}{1024} \times V_{REF}

3.4 Archetype D: Safe SPI with Hardware CRC (Automotive & Industrial)

In automotive powertrain, braking, and airbag systems (AEC-Q100 / ISO 26262), corrupted SPI frames caused by EMI, electric motor switching, or wiring faults can have catastrophic consequences. Peripherals implement Safe SPI where every frame contains a hardware-calculated CRC-8 field.

Standard 32-Bit Safe SPI Command Frame: ┌───────────────┬───────────────┬───────────────────────────────┬───────────────┐ │ Bits [31:30] │ Bits [29:24] │ Bits [23:8] │ Bits [7:0] │ ├───────────────┼───────────────┼───────────────────────────────┼───────────────┤ │ Opcode (R/W) │ Register Addr │ Data Payload (16-bit) │ CRC-8 Field │ └───────────────┴───────────────┴───────────────────────────────┴───────────────┘ Matching 32-Bit Safe SPI Response Frame: ┌───────────────┬───────────────┬───────────────────────────────┬───────────────┐ │ Bits [31:30] │ Bits [29:24] │ Bits [23:8] │ Bits [7:0] │ ├───────────────┼───────────────┼───────────────────────────────┼───────────────┤ │ Global Status │ Echoed Addr │ Read Data / Echo │ CRC-8 Field │ └───────────────┴───────────────┴───────────────────────────────┴───────────────┘

CRC-8 Mathematical Specification:

Standard Polynomial: P(x) = x^8 + x^2 + x^1 + 1 (0x07) or SAE J1850 (x^8 + x^4 + x^3 + x^2 + 1 / 0x1D).

Initial Seed: Typically 0xFF or 0x00.

Validation Rule: The peripheral computes the CRC across bits [31:8]. If the calculated CRC does not match bits [7:0], the hardware drops the write, asserts a CRC_ERR flag in the Global Status bits of the response frame, and triggers an interrupt to the host MCU.

4. Multi-Segment Transactions in Software (struct spi_ioc_transfer)

In real-world embedded Linux, complex frames (e.g. Command + Address at 10 MHz followed by streaming data payload at 40 MHz) are executed atomically using an array of struct spi_ioc_transfer structs passed to ioctl(SPI_IOC_MESSAGE(N)).

4.1 Linux Atomic Multi-Transfer Architecture

#include <linux/spi/spidev.h> #include <sys/ioctl.h> int spi_read_flash_fast(int fd, uint32_t addr, uint8_t *rx_buf, size_t len) { uint8_t cmd_hdr[5]; cmd_hdr[0] = 0x0B; // Fast Read Opcode cmd_hdr[1] = (addr >> 16) & 0xFF; // Address MSB cmd_hdr[2] = (addr >> 8) & 0xFF; // Address MID cmd_hdr[3] = addr & 0xFF; // Address LSB cmd_hdr[4] = 0x00; // Dummy Byte struct spi_ioc_transfer xfer[2]; memset(xfer, 0, sizeof(xfer)); // Segment 1: Header (Command + 24-bit Addr + Dummy Byte) xfer[0].tx_buf = (unsigned long)cmd_hdr; xfer[0].rx_buf = 0; // Don't care about incoming data during header xfer[0].len = 5; xfer[0].speed_hz = 25000000; // 25 MHz xfer[0].cs_change = 0; // CRITICAL: Keep CS LOW between segments! // Segment 2: Data Payload Burst Read xfer[1].tx_buf = 0; // Kernel clocks out 0x00 automatically xfer[1].rx_buf = (unsigned long)rx_buf; xfer[1].len = len; xfer[1].speed_hz = 50000000; // Run data phase at 50 MHz xfer[1].cs_change = 0; // Raise CS at conclusion of transaction // Atomic execution: Kernel holds CS continuously low across both segments return ioctl(fd, SPI_IOC_MESSAGE(2), xfer); }

[CAUTION] The `cs_change` Trap: - If cs_change == 0: \overline{CS remains asserted (LOW) until the entire array of transfers completes. - If cs_change == 1: \overline{CS is deasserted (toggled HIGH) momentarily between transfer segments. Setting cs_change = 1 before the data phase will prematurely terminate the memory read command and corrupt the payload!

5. Software Implementation: Safe SPI CRC-8 Verification

The following production Python module demonstrates packet construction, CRC-8 generation, and frame verification for an industrial sensor:

#!/usr/bin/env python3 """ Industrial SPI Frame Formatter and CRC-8 Validator Implements SAE J1850 CRC-8 (Polynomial: 0x1D, Initial: 0xFF) """ def compute_crc8(data: bytes, poly: int = 0x1D, init_val: int = 0xFF) -> int: """Calculates an 8-bit CRC over arbitrary byte sequences.""" crc = init_val for byte in data: crc ^= byte for _ in range(8): if crc & 0x80: crc = ((crc << 1) ^ poly) & 0xFF else: crc = (crc << 1) & 0xFF return crc def pack_safe_spi_frame(rw: int, addr: int, payload: int) -> bytes: """ Constructs a 32-bit Safe SPI Command Frame: [Bit 31: R/W] [Bits 30-24: Addr] [Bits 23-8: Payload] [Bits 7-0: CRC8] """ header = ((rw & 0x01) << 7) ¦ (addr & 0x7F) payload_msb = (payload >> 8) & 0xFF payload_lsb = payload & 0xFF body = bytes([header, payload_msb, payload_lsb]) crc = compute_crc8(body) frame = body + bytes([crc]) return frame def parse_safe_spi_response(raw_bytes: bytes) -> dict: """Parses and verifies a received 4-byte SPI response frame.""" if len(raw_bytes) != 4: raise ValueError(f"Invalid frame length: expected 4 bytes, got {len(raw_bytes)}") body = raw_bytes[:3] received_crc = raw_bytes[3] expected_crc = compute_crc8(body) is_valid = (received_crc == expected_crc) status_byte = raw_bytes[0] payload = (raw_bytes[1] << 8) ¦ raw_bytes[2] return { "valid": is_valid, "status_flags": hex(status_byte), "payload": hex(payload), "crc_expected": hex(expected_crc), "crc_received": hex(received_crc) } if __name__ == "__main__": # Test frame assembly: Write 0xABCD to Register 0x14 tx_frame = pack_safe_spi_frame(rw=0, addr=0x14, payload=0xABCD) print("Transmitted Safe SPI Frame :", [hex(b) for b in tx_frame]) print(f"Computed CRC : 0x{tx_frame[3]:02X}") # Test frame validation: Simulated response simulated_rx = bytes([0x00, 0x12, 0x34, 0x00]) valid_rx = simulated_rx[:3] + bytes([compute_crc8(simulated_rx[:3])]) result = parse_safe_spi_response(valid_rx) print("Parsed Response Status :", result)

6. Decoding & Analyzing Frames on Logic Analyzers

When capturing SPI communication with a logic analyzer (e.g. Saleae Logic, Sigrok PulseView, or Rigol/Keysight DSO):

Protocol Analyzer Trace View: Time [ms] Channel Decoded Value Annotation ──────────────────────────────────────────────────────────────────────────────────────── 0.000000 CS# Falling Edge Frame Start (Active LOW) 0.000010 MOSI 0x03 Command: Read Array (3-byte Addr) 0.000090 MOSI 0x10, 0x00, 0x00 Target Address: 0x100000 0.000330 MISO 0x48 Payload Byte 0: 'H' 0.000410 MISO 0x65 Payload Byte 1: 'e' 0.000490 MISO 0x6C Payload Byte 2: 'l' 0.000570 MISO 0x6C Payload Byte 3: 'l' 0.000650 MISO 0x6F Payload Byte 4: 'o' 0.000730 CS# Rising Edge Frame End (Deselect / Terminate)

6.1 Diagnostic Rules for Frame Decode Anomalies

| Analyzer Visual Symptom | Root Cause | Underlying Hardware Mechanism |
| --- | --- | --- |
| All decoded bytes shifted left by 1 bit (e.g. 0x9F decoded as 0x3E) | Analyzer CPHA configuration inverted. | The analyzer is sampling on the leading edge instead of trailing edge; it captures the bus while the line is still transitioning. |
| MISO outputs `0x00` during byte 0, real data during byte 1 | Normal SPI Shift Register behavior. | During byte 0 (command phase), the peripheral is receiving the opcode. It cannot output valid response data until byte 1. |
| Data bytes read correctly, but device does not respond to subsequent writes | \overline{CS not toggled HIGH between frames. | Many flash/EEPROM devices latch data and start internal write cycles only upon the rising edge of $\overline{\text{CS}}$. |
| Decoded hex displays `0xFF` or `0x00` across all bytes | 1. Slave unpowered.<br>2. Wrong CS routed.<br>3. MISO floating. | When a slave is unselected, its MISO output buffer is in High-Z. A floating line typically drifts to logic HIGH (0xFF) or LOW (0x00). |
| Intermittent corrupt bytes during high-speed burst | Inter-word timing violation (t_{inter\_byte) or lack of dummy clocks. | High-speed memory arrays require dummy wait states to fetch subsequent blocks. |

7. Frame Analysis Quick-Reference Cheatsheet

+-----------------------+---------------------+-------------------+-------------------------------+ ¦ Device Class ¦ Typical Word Size ¦ Typical Frame Len ¦ Key Framing Distinctives ¦ +-----------------------+---------------------+-------------------+-------------------------------+ ¦ SPI NOR Flash ¦ 8-bit ¦ 1 to 260+ bytes ¦ Fast Read needs dummy byte; ¦ ¦ (W25Q, MX25, AT25) ¦ ¦ ¦ Page Program capped at 256 B ¦ +-----------------------+---------------------+-------------------+-------------------------------+ ¦ Motion Sensors / IMUs ¦ 8-bit ¦ 2 to 14 bytes ¦ Bit 7 = R/W#; Bit 6 = AutoInc ¦ ¦ (ADXL345, BMI160) ¦ ¦ ¦ Multi-byte reads atomic ¦ +-----------------------+---------------------+-------------------+-------------------------------+ ¦ Precision ADCs ¦ 10, 12, 16, 24-bit ¦ 2 to 4 bytes ¦ Arbitrary bit alignment; ¦ ¦ (MCP3008, ADS7886) ¦ ¦ ¦ Null bit preceding MSB data ¦ +-----------------------+---------------------+-------------------+-------------------------------+ ¦ Automotive Safe SPI ¦ 32-bit (fixed) ¦ Exactly 4 bytes ¦ 8-bit CRC suffix; ¦ ¦ (Motor/Braking ICs) ¦ ¦ ¦ Write dropped on CRC mismatch ¦ +-----------------------+---------------------+-------------------+-------------------------------+