UART Frame & Protocol Analysis: Framing, Oversampling, Errors & Buffering

This technical guide provides a deep-dive analysis of the UART character frame, 16x oversampling clock synchronization, hardware error detection flags, 9-bit multi-drop addressing, hardware auto-flow control, and high-throughput DMA circular buffer architectures.

1. Bit-Level Frame Anatomy & Signal Transitions

Unlike packetized network buses that include destination headers and CRCs in every packet, UART transmits data as independent character frames framed by start and stop transitions:

Idle (Mark: 1) ──────┐ ┌────── Idle (Mark: 1) │ Start │ D0 │ D1 │ D2 │ D3 │ D4 │ D5 │ D6 │ D7 │Parity │ Stop │ │ Bit(0)│ (LSB) │ │ │ │ │ │ │ (MSB) │ (Opt) │ Bit(1)│ └───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┘ ¦<--------------------------- 1 Frame (10 to 12 Bit Times) ----------------------------->¦

Frame Component Breakdown:

Idle State (Mark / Logic 1):

When no transmission is active, the line is continuously held at positive supply voltage (3.3\,V or 5\,V). This confirms physical cable continuity. A disconnected or broken wire floating to 0\,V is immediately detectable as a fault condition.

Start Bit (Space / Logic 0, exactly 1 bit period):

A high-to-low transition (1 \rightarrow 0) signals the beginning of a new frame.

It resynchronizes the receiver's internal baud rate counter for that character.

Data Bits (Typically 8 bits, configurable 5 to 9 bits):

Transmitted LSB (Least Significant Bit, D0) first, terminating with MSB (D7).

In 9-bit mode, the 9th bit (D8) acts as an address/data discriminator in multi-drop networks.

Parity Bit (Optional, 1 bit):

Evaluated as the exclusive-OR (XOR) sum of all transmitted data bits.

Even Parity: Parity bit is set to 1 if the count of 1s in the data byte is odd, making the total count of 1s even.

Odd Parity: Parity bit is set to 1 if the count of 1s in the data byte is even, making the total count of 1s odd.

Mark / Space Parity: Fixed at 1 or 0 regardless of data (used in legacy protocols).

Stop Bit (Logic 1 / Mark, configurable 1, 1.5, or 2 bits):

Forces the physical line back to the idle HIGH state.

Provides a mandatory processing recovery window for the receiver's shift register and FIFO before the next high-to-low start bit edge arrives.

2. Receiver Clock Recovery & 16x Oversampling Architecture

Because no shared clock wire connects transmitter and receiver, UART receivers utilize an internal $16\times$ (or $8\times$) oversampling clock derived from the local peripheral bus clock to sample incoming bits precisely at their midpoints:

16x Oversampling Clock Cycles across One Bit Period ┌───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┬───┐ │ 1 │ 2 │ 3 │ 4 │ 5 │ 6 │ 7 │ 8 │ 9 │ 10│ 11│ 12│ 13│ 14│ 15│ 16│ └───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┴───┘ ▲ ▲ ▲ └───┼───┘ Majority Voting Window: Samples taken at ticks 7, 8, 9

The Synchronization & Majority Voting Algorithm:

Falling Edge Detection: The receiver continuously polls the RX line at the 16× clock rate. When a high-to-low transition is detected, a counter starts.

Start Bit Verification: At clock ticks 7, 8, and 9 (the mathematical center of the expected Start bit), the line state is sampled three times. If at least 2 of the 3 samples are Logic 0, the Start bit is validated. If not, the transition is rejected as a transient noise spike, and the receiver resets to the idle polling state.

Data Bit Sampling: For each subsequent bit (D0 through Stop), the receiver counts 16 ticks from the center of the previous bit and takes three consecutive samples at ticks 7, 8, and 9.

Majority Voting Logic:

Bit Value = (Sample_7 \land Sample_8) \lor (Sample_7 \land Sample_9) \lor (Sample_8 \land Sample_9)

If all three samples agree (0-0-0 or 1-1-1), the bit is clean.

If samples disagree (2:1 split, e.g., 1-0-1), the majority value is accepted, but the hardware sets the Noise Error (NE) flag in the status register.

3. Hardware Error Detection Flags & Diagnosis

Modern UART controllers (e.g., STM32 USART, TI eUSCI, Microchip PIC) monitor signal integrity continuously and report faults via dedicated status register bits:

Typical UART Status Register (USART_SR / USART_ISR) ┌─────────┬─────────┬─────────┬─────────┬─────────┬─────────┬─────────┬─────────┐ │ Bit 7 │ Bit 6 │ Bit 5 │ Bit 4 │ Bit 3 │ Bit 2 │ Bit 1 │ Bit 0 │ ├─────────┼─────────┼─────────┼─────────┼─────────┼─────────┼─────────┼─────────┤ │ TXE │ TC │ RXNE │ IDLE │ ORE │ NF / NE │ FE │ PE │ │ Tx Empty│ Complete│ Rx Ready│ IdleLine│ Overrun │ NoiseErr│ Framing │ Parity │ └─────────┴─────────┴─────────┴─────────┴─────────┴─────────┴─────────┴─────────┘

| Error Flag | Name | Physical Root Cause | Bench Troubleshooting Action |
| --- | --- | --- | --- |
| FE | Framing Error | Stop bit sampled as Logic 0 instead of Logic 1 | Baud rate mismatch > 3\%, clock jitter, or baud divisor miscalculation |
| ORE | Overrun Error | New byte arrived before CPU/DMA read previous byte | CPU blocked in long ISR; switch to hardware FIFO or DMA circular buffer |
| PE | Parity Error | XOR sum of data bits disagrees with received parity bit | Line electrical noise, EMI bursts, or mismatch in parity config (Even vs Odd) |
| NE / NF | Noise Flag | Majority voting samples (ticks 7, 8, 9) were not unanimous | Ground loop bounce, long unshielded cables, missing pull-up resistors |

4. BREAK Signaling & Multi-Processor 9-Bit Addressing

1. BREAK Signaling (Physical Space Override):

A BREAK condition occurs when the TX line is forcefully driven to Logic 0 (Space) for longer than an entire frame duration (typically ≥ 11 \dots 13 bit times):

Normal 8-N-1 Character BREAK Condition (Continuous Space) ───┐ ┌───┬───┬───┬───┬───┐ ┌────── ─────────────────────────────────────── │ S │ D0│ D1│ D2│...│ D7│ P │ Stop │ Space line held LOW >= 11 to 13 bit periods! └───┴───┴───┴───┴───┴───┘ └────── └──────────────────────────────────────

Primary Uses:

LIN Bus Synchronization: In automotive LIN (Local Interconnect Network), the master initiates every frame with a 13-bit minimum BREAK field to wake up slave nodes and reset slave state machines.

DMX512 Stage Lighting: Lighting consoles precede 512-byte DMX universe frames with an 88\,µs BREAK to synchronize LED fixtures.

Modem / Debug Console Reset: Terminal software sends a BREAK signal (Serial SysRq) to drop Linux into the kernel debugger or reboot a frozen microcontroller.

2. 9-Bit Addressable Multi-Drop Mode (RS-485 Multi-Processor):

In multi-drop RS-485 networks where multiple slave microcontrollers listen to a single master transmitter, software packet parsing creates excessive CPU interrupt overhead. Hardware 9-bit addressing eliminates this:

Master Transmission: 1. Address Frame: [ 9th Bit = 1 ] -> Data Byte = Target Slave ID (e.g. 0x05) ==> All slaves wake up and compare byte to internal Node ID. 2. Data Frame: [ 9th Bit = 0 ] -> Payload Byte #1, #2, #3... ==> Only Slave 0x05 stays awake; all other slaves re-enter hardware mute!

// Example STM32 Hardware Multi-Processor Configuration void UART_MultiProcessor_Init(uint8_t my_node_address) { USART1->CR1 ¦= USART_CR1_M0; // Configure 9-bit word length USART1->CR2 ¦= (my_node_address << 24); // Set hardware address USART1->CR2 ¦= USART_CR2_ADDM7; // 7-bit address detection USART1->CR1 ¦= USART_CR1_WAKE; // Wakeup method: Address Mark (9th bit = 1) USART1->CR1 ¦= USART_CR1_MMM; // Enter Mute Mode (silent until addressed) }

5. Hardware Flow Control: RTS / CTS Handshaking

When a high-speed transmitter sends data faster than the receiving microcontroller can process, buffers overflow, causing catastrophic Overrun Errors (ORE). Hardware flow control (RTS/CTS) provides real-time backpressure:

DTE (Microcontroller) DCE (Cellular Modem / ESP32) ┌──────────────────┐ ┌──────────────────┐ │ TX ├───────────────────────────────────►│ RX │ │ RX │◄───────────────────────────────────┤ TX │ │ RTS ├───────────────────────────────────►│ CTS │ │ CTS │◄───────────────────────────────────┤ RTS │ └──────────────────┘ └──────────────────┘

Signal Definitions & Active-LOW Polarity:

$\overline{\text{RTS}}$ (Request to Send / Ready to Receive): Driven by the receiver. When asserted LOW, it signals: "My internal FIFO has space; you are authorized to send data." When the receiver FIFO fills to its high-watermark threshold (e.g., 75\%), it deasserts RTS (pulls it HIGH), forcing the transmitter to immediately pause.

$\overline{\text{CTS}}$ (Clear to Send): Driven by the transmitter. The transmitter monitors CTS before shifting out each byte. If CTS is HIGH, the transmitter halts immediately between characters.

6. High-Throughput Buffering: DMA Circular Ring Buffer with IDLE Line Detection

In production firmware, handling high-baud UART (e.g., 921.6\,kbps or 1\,Mbps) via byte-by-byte interrupts (RXNE) will saturate CPU interrupt bandwidth (100,000\,ISRs/sec). The industry standard architecture combines Direct Memory Access (DMA) in Circular Mode with IDLE Line Interrupts:

Incoming Serial Bytes (Streamed by DMA directly into SRAM without CPU intervention!) ───────────────────────────────────────────────────────────────────────────────────► Circular Ring Buffer in RAM ┌───────┬───────┬───────┬───────┬───────┐ │ Byte 0│ Byte 1│ Byte 2│ Byte 3│ Byte 4│ └───────┴───────┴───────┴───────┴───────┘ ▲ ▲ │ │ Tail (CPU Read) Head (DMA Write)

How IDLE Line Detection Eliminates Fixed Packet Size Limits:

DMA Autonomous Transfer: DMA channel is mapped directly to USART_RDR. Incoming bytes are automatically written into a circular ring buffer in SRAM without generating a single CPU interrupt.

The IDLE Line Event (`USART_IT_IDLE`):

The hardware UART controller monitors the RX line.

When the remote transmitter finishes sending a burst of data, the line remains in the idle HIGH state for at least 1 full character frame duration (10 to 11 bit times).

The UART hardware instantly triggers a single IDLE Line Interrupt.

Variable-Length Packet Processing:

In the IDLE ISR, the CPU reads the remaining DMA counter register (CNDTR), calculates exactly how many bytes arrived in the burst:

\text{Bytes Received} = \text{Buffer Size} - \text{DMA\_CNDTR}

The CPU dispatches the packet to application processing and updates buffer pointers in mere microseconds!