NFC Data Exchange Format (NDEF) & Card Emulation Architecture

1. High-Level Data Architecture

The NFC Forum standardizes how high-level application data (URLs, contact vCards, Wi-Fi credentials, payment tokens) is packaged so that any NFC-enabled smartphone (Android / iOS) can seamlessly read and execute actions regardless of the underlying silicon tag manufacturer:

+-------------------------------------------------------+ ¦ Application Layer ¦ ¦ (Web Browser, Wi-Fi Connect, Apple Wallet) ¦ +-------------------------------------------------------+ ¦ +-------------------------------------------------------+ ¦ NDEF (NFC Data Exchange Format) ¦ ¦ (URI, Text, MIME, Smart Poster) ¦ +-------------------------------------------------------+ ¦ +-------------------------------------------------------+ ¦ NFC Forum Tag Types & Capability Container ¦ ¦ (Type 1, Type 2, Type 3, Type 4, Type 5) ¦ +-------------------------------------------------------+ ¦ +-------------------------------------------------------+ ¦ Lower Transport Protocols ¦ ¦ (ISO 14443-3/4 Type A & B, JIS X 6319-4, ISO 15693) ¦ +-------------------------------------------------------+

2. NFC Forum Tag Types Comparison Matrix

The NFC Forum classifies physical tags into five distinct memory and protocol types:

| Metric / Parameter | Type 1 Tag | Type 2 Tag | Type 3 Tag | Type 4 Tag | Type 5 Tag |
| --- | --- | --- | --- | --- | --- |
| Reference Silicon | Innovision Topaz | NXP NTAG21x, MIFARE Ultralight | Sony FeliCa Lite-S | NXP DESFire, Smart MX | NXP ICODE SLIX, ST25TV |
| Air Standard | ISO 14443A | ISO 14443A | JIS X 6319-4 | ISO 14443-4 (T=CL) | ISO/IEC 15693 (Vicinity) |
| Over-the-Air Baud | 106 kbps | 106 kbps | 212 / 424 kbps | 106, 212, 424, 848 kbps | 26.48 kbps |
| Memory Size | 96 B – 2 KB | 48 B – 2 KB | Up to 1 MB | Up to 32 KB | 64 B – 8 KB |
| Memory Addressing | Byte / Block | 4-byte Pages | 16-byte Blocks | ISO 7816-4 File System | 4-byte / 8-byte Blocks |
| Read Range | Up to 4 cm | Up to 4 cm | Up to 4 cm | Up to 4 cm | Up to 1 meter (Vicinity) |
| Crypto / Security | None | Lock bytes, optional PWD | Hardware MAC | Hardware 3DES, AES, ECC | Lock blocks, AFI / DSFID |
| Host Card Emulation | No | No | Rare | Universal Standard (HCE) | No |

3. Type 2 Tag Memory Structure (NTAG213 / 215 / 216)

Type 2 is the most prevalent NFC tag silicon deployed worldwide (smart posters, product labels, NFC stickers). Memory is organized strictly into 4-byte pages:

Page Byte 0 Byte 1 Byte 2 Byte 3 Description ----------------------------------------------------------------------------------------- 0x00 SN0 SN1 SN2 BCC0 UID Segment 1 (Manufacturer) 0x01 SN3 SN4 SN5 SN6 UID Segment 2 0x02 BCC1 Internal Lock0 Lock1 BCC1 + Static Lock Bytes 0x03 E1h 10h 12h 00h Capability Container (CC) ----------------------------------------------------------------------------------------- 0x04 03h LEN NDEF Data Byte 0 ... NDEF Message TLV Block 0x05 ... ... ... ... Payload ... ... ... ... ... Payload 0x0F ... ... ... FEh Terminator TLV (0xFE) ----------------------------------------------------------------------------------------- Last Dynamic Lock Configuration, Password, & PACK

3.1 The Capability Container (CC) — Page 0x03

The 4-byte CC page informs the reader whether NDEF data is present and how much memory is available:

Byte 0 (`Magic Number = 0xE1`): Identifies compliance with NFC Forum Type 2 Tag specification.

Byte 1 (`Version Number`): High nibble = Major version (0x1), Low nibble = Minor version (0x0) \rightarrow 0x10.

Byte 2 (`Memory Size Multiplier, T2T_SIZE`): Defines total user data memory:

\text{User Memory (Bytes)} = \text{T2T\_SIZE} \times 8

Example: 0x12 (18 decimal) \implies 18 × 8 = 144 bytes of user space (NTAG213).

Example: 0x3E (62 decimal) \implies 62 × 8 = 496 bytes (NTAG215).

Example: 0x6D (109 decimal) \implies 109 × 8 = 872 bytes (NTAG216).

Byte 3 (`Read/Write Access`): 0x00 indicates read and write allowed without authentication; 0x0F indicates read-only.

3.2 Type-Length-Value (TLV) Data Structure

User memory is wrapped in TLV blocks:

`T = 0x00` (Null TLV): 1-byte padding; ignored by reader.

`T = 0x03` (NDEF Message TLV):

If L ≤ 254: Length is 1 byte (L).

If L ≥ 255: Length is encoded as 3 bytes: 0xFF [LEN_HIGH] [LEN_LOW].

Value contains the raw binary NDEF message.

`T = 0xFE` (Terminator TLV): 1 byte (0xFE); marks the end of valid data in tag memory.

4. Type 4 Tag Architecture & ISO 7816-4 File System

Type 4 tags use smart-card operating systems running over ISO 14443-4 (T=CL). The reader accesses NDEF data by issuing ISO 7816-4 APDU commands to select dedicated file identifiers:

Host Reader (SPI Controller) Type 4 Tag / HCE Phone ¦ ¦ ¦--- Select NDEF Application: [00 A4 04 00 07 D2760000850101 00] ---->¦ ¦<-- Success Response: [90 00] ---------------------------------------¦ ¦ ¦ ¦--- Select CC File: [00 A4 00 0C 02 E1 03] ------------------------->¦ ¦<-- Success Response: [90 00] ---------------------------------------¦ ¦ ¦ ¦--- Read CC File: [00 B0 00 00 0F] --------------------------------->¦ ¦<-- CC Data: [00 0F 20 00 3B 00 34 04 06 E1 04 08 00 00 00] [90 00] -¦ ¦ ¦ ¦--- Select NDEF Data File: [00 A4 00 0C 02 E1 04] ------------------->¦ ¦<-- Success Response: [90 00] ---------------------------------------¦ ¦ ¦ ¦--- Read NDEF File Length (First 2 Bytes): [00 B0 00 00 02] --------->¦ ¦<-- Length Bytes: [00 1E] [90 00] (Payload is 30 bytes) -------------¦ ¦ ¦ ¦--- Read NDEF Data Payload: [00 B0 00 02 1E] ------------------------>¦ ¦<-- Binary NDEF Message Bytes [90 00] -------------------------------¦

NDEF Application ID (AID): D2 76 00 00 85 01 01 (NFC Forum standard).

Capability Container File ID: E1 03.

NDEF Data File ID: E1 04 (typically, as pointed to by the CC file).

5. NDEF (NFC Data Exchange Format) Specification

An NDEF Message consists of one or more NDEF Records. Each record encapsulates a typed payload.

NDEF RECORD BYTE-LEVEL LAYOUT +-------------------------------------------------------------------+ ¦ Byte 0: Header Flags (MB, ME, CF, SR, IL, TNF) ¦ +-------------------------------------------------------------------+ ¦ Byte 1: Type Length (1 Byte) ¦ +-------------------------------------------------------------------+ ¦ Byte 2 .. [2 or 5]: Payload Length (1 Byte if SR=1, 4 B if SR=0) ¦ +-------------------------------------------------------------------+ ¦ [Optional]: ID Length (1 Byte, present only if IL=1) ¦ +-------------------------------------------------------------------+ ¦ Type Field: Identifier (e.g. 'U', 'T', 'text/vcard') ¦ +-------------------------------------------------------------------+ ¦ [Optional] ID Field: Payload Identifier ¦ +-------------------------------------------------------------------+ ¦ Payload Data: Raw application data ¦ +-------------------------------------------------------------------+

5.1 NDEF Record Header Byte (Byte 0) Breakdown

Bit 7 Bit 6 Bit 5 Bit 4 Bit 3 Bit 2 Bit 1 Bit 0 +-------+-------+-------+-------+-------+-------+-------+-------+ ¦ MB ¦ ME ¦ CF ¦ SR ¦ IL ¦ TNF ¦ +-------+-------+-------+-------+-------+-------+-------+-------+

`MB` (Message Begin, Bit 7): Set to 1 on the very first record of an NDEF message.

`ME` (Message End, Bit 6): Set to 1 on the final record of an NDEF message. (For single-record messages, both MB = 1 and ME = 1).

`CF` (Chunk Flag, Bit 5): Set to 1 if the payload is fragmented across multiple chunks.

`SR` (Short Record, Bit 4):

SR = 1: Short Record. Payload length is encoded as a single byte (0 to 255 bytes).

SR = 0: Normal Record. Payload length is encoded as a 4-byte big-endian integer (up to 4.29 GB).

`IL` (ID Length Present, Bit 3):

IL = 1: The 1-byte ID Length field and the ID string are present.

IL = 0: ID Length and ID fields are omitted.

`TNF` (Type Name Format, Bits 2:0):

| TNF Value | Meaning | Description / Use Case |
| --- | --- | --- |
| `0x00` | Empty | Record contains no type and no payload. |
| `0x01` | NFC Forum Well-Known | Standardized RTDs: URI ('U'), Text ('T'), Smart Poster ('Sp'). |
| `0x02` | MIME Media-Type | Standard RFC 2046 MIME types (e.g. text/vcard, application/json). |
| `0x03` | Absolute URI | Full URI defined in RFC 3986 (e.g. https://vvdntech.com). |
| `0x04` | External Type | Custom domain namespaced types (e.g. android.com:pkg). |
| `0x05` | Unknown | Undefined payload format. |
| `0x06` | Unchanged | Used in chunked records for intermediate segments. |

6. Well-Known Record Type Definitions (RTD)

6.1 URI Record Type (Type = 'U', 0x55)

The URI record compresses popular URL prefixes into a single URI Identifier Code byte:

Record Payload: +----------------------------+-------------------------------------+ ¦ Byte 0: URI Identifier Code¦ Byte 1 .. N: URI String Suffix ¦ +----------------------------+-------------------------------------+

Standard URI Identifier Prefix Table:

| Code | Prefix String | Code | Prefix String |
| --- | --- | --- | --- |
| 0x00 | No prefix (raw URI) | 0x07 | ftp://anonymous:anonymous@ |
| 0x01 | http://www. | 0x08 | ftp://ftp. |
| 0x02 | https://www. | 0x09 | ftps:// |
| 0x03 | http:// | 0x0A | sftp:// |
| 0x04 | https:// | 0x0D | tel: |
| 0x05 | telnet:// | 0x0E | mailto: |
| 0x06 | mailto: | 0x1D | file:// |

Concrete Byte Dump Example: Encoding https://vvdntech.com

D1 01 0D 55 04 76 76 64 6E 74 65 63 68 2E 63 6F 6D

D1: Header (MB=1, ME=1, CF=0, SR=1, IL=0, TNF=0x01 Well-Known).

01: Type Length = 1 byte.

0D: Payload Length = 13 bytes (0x0D).

55: Type Field = ASCII 'U' (0x55 for URI).

04: Prefix Code = 0x04 (https://).

76 76 64 6E ...: ASCII string "vvdntech.com".

6.2 Text Record Type (Type = 'T', 0x54)

Record Payload: +---------------+--------------------+-----------------------------+ ¦ Status Byte ¦ Language Code ¦ UTF-8 / UTF-16 Text String ¦ ¦ (1 Byte) ¦ (k Bytes, ASCII) ¦ (N - 1 - k Bytes) ¦ +---------------+--------------------+-----------------------------+

Status Byte (Bit 7): 0 = UTF-8 encoding; 1 = UTF-16 encoding.

Status Byte (Bits 5:0): Length of IANA language code string (k bytes, e.g. 'en' = 2, 'fr' = 2).

7. Card Emulation Modes (PICC) via SPI

In Card Emulation mode, the Host MCU and SPI transceiver behave as a passive NFC tag or contactless smart card responding to an external reader (e.g. transit turnstile or smartphone).

============================================================================================= MODE A: Embedded Secure Element (eSE) / SIM via Single Wire Protocol (SWP) ============================================================================================= +------------------+ SWP (1-Wire) +--------------------+ ¦ Transceiver IC ¦<============================>¦ Hardware SE / SIM ¦ (EMVCo Certified) +------------------+ +--------------------+ ============================================================================================= MODE B: Host Card Emulation (HCE) via High-Speed SPI ============================================================================================= +------------------+ SPI Bus +--------------------+ ¦ Transceiver IC ¦<============================>¦ Host MCU ¦ ¦ (RF Frontend in ¦ IRQ: Field Detected ¦ - ISO 14443-4 T=CL ¦ ¦ PICC / Target ¦ IRQ: APDU Received ¦ - Virtual Type 4 Tag ¦ Mode) ¦ SPI Burst: APDU Response ¦ - Dynamic NDEF Gen ¦ +------------------+ +--------------------+

7.1 Implementing Host Card Emulation (HCE) over SPI

When using an intelligent controller like the PN532 in Target / Card Emulation mode:

Initialize as Target:

Host sends command TgInitAsTarget (0x8C) with target configuration bytes (Mifare parameters, 7-byte UID, ATS parameters).

RF Field Detection:

When an external reader approaches, the transceiver detects the external 13.56 MHz carrier, answers the reader's REQA/WUPA, resolves anti-collision, and responds to RATS autonomously using hardware state machines.

APDU Forwarding over SPI:

The transceiver asserts the IRQ pin.

Host MCU reads the incoming ISO 7816-4 APDU command from the transceiver FIFO over SPI.

Host Application Processing:

The host parses the APDU (e.g. SELECT AID, READ BINARY).

If the AID matches the host's virtual smart card, the host generates the response payload and appends status word 90 00.

Transmit Response:

Host sends TgSetData (0x8E) over SPI to load the APDU response into the transceiver's transmit FIFO.

The transceiver modulates the RF carrier back to the external reader using 848 kHz subcarrier load modulation.

8. Peer-to-Peer (P2P) Communication (ISO/IEC 18092)

NFC P2P enables two active devices (e.g. two smartphones, or an embedded terminal and a smartphone) to exchange data symmetrically:

Initiator: Generates the initial RF field and starts the transaction.

Target: Responds to the initiator.

Communication Modes:

Passive Mode: Initiator generates the carrier continuously; Target uses load modulation (saves target battery power).

Active Mode: Both devices generate their own RF field alternately during their respective transmission phases.

Protocol Stack:

LLCP (Logical Link Control Protocol): Provides connectionless (unreliable datagram) and connection-oriented (reliable flow-controlled) data links.

SNEP (Simple NDEF Exchange Protocol): Transports NDEF messages over LLCP using standard HTTP-like commands (PUT, GET).