How SPI and RFID Interact to Create NFC: The Comprehensive Mentor Briefing

1. The Core Concept: The "Big Picture" Mental Model

When explaining an SPI-based NFC implementation to a mentor or senior engineer, you must clearly frame it as a three-part collaborative system:

+-------------------+ +-----------------------+ +--------------------------+ ¦ HOST MCU ¦ ¦ RFID TRANSCEIVER IC ¦ ¦ ANTENNA & TAG ¦ ¦ (The "Brain") ¦ ¦ (The "Modem") ¦ ¦ (The "Air Link") ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ - Decides WHAT to ¦ SPI ¦ - Converts digital ¦ 13.56M ¦ - Transfers energy via ¦ ¦ transmit/read ¦=========> ¦ bytes to RF pulses ¦=========> ¦ magnetic field ¦ ¦ - Runs protocol ¦ (Digital) ¦ - Demodulates minute ¦ (Analog) ¦ - Tag replies by ¦ ¦ state machines ¦ ¦ tag load-modulation ¦ ¦ wiggling coil load ¦ ¦ - Parses NDEF ¦ ¦ - Manages FIFO & CRC ¦ ¦ (Load Modulation) ¦ +-------------------+ +-----------------------+ +--------------------------+

The Three Entities and Their Exact Roles:

The Host Microcontroller (The Brain):

Contains the high-level application (e.g., read a URL, unlock a door, process a payment).

Speaks purely digital logic (0s and 1s) at standard MCU voltage levels (3.3V or 1.8V).

Why it cannot do this alone: A microcontroller's GPIO pins cannot oscillate at 13.56 MHz with 15V peak-to-peak differential power, cannot provide RF impedance matching to an antenna, and cannot detect a 10 millivolt signal riding on top of a 20V carrier.

The RFID Transceiver IC (The Physical RF Modem):

Silicon chips such as the NXP PN532, NXP MFRC522, or STMicroelectronics ST25R3916.

Contains analog oscillators, RF push-pull power amplifiers (TX1, TX2), envelope detectors, phase decoders, and a hardware FIFO buffer.

Converts the digital bytes received from the host into electromagnetic pulses, and converts tiny analog fluctuations from the antenna back into digital bytes.

The SPI Bus (The High-Speed Conveyor Belt):

The synchronous serial link connecting the Brain to the Modem.

Enables fast, low-latency transfer of commands, register configurations, and data packets so the transceiver's FIFO never starves or overflows.

2. The Fundamental Distinction: RFID vs. NFC

A common mentor question is: "Is this an RFID system or an NFC system?"

Here is the exact technical distinction you should give:

+-------------------------------------------------------------------------------+ ¦ NFC ¦ ¦ (Bidirectional, NDEF data format, 13.56 MHz, d <= 4 cm, smart devices & tags) ¦ ¦ - Operating Modes: Reader/Writer, Host Card Emulation (HCE), Peer-to-Peer ¦ ¦ - Standardized ISO 14443-4 T=CL & ISO 7816-4 APDU transport ¦ +-------------------------------------------------------------------------------+ ¦ Built on top of: ¦ +-------------------------------------------------------------------------------+ ¦ 13.56 MHz High-Frequency (HF) RFID ¦ ¦ (Physical air interface: Inductive magnetic coupling, 100% ASK, Load Mod) ¦ ¦ - Silicon: MFRC522, PN532, ST25R3916, TRF7970A ¦ ¦ - Standards: ISO/IEC 14443-3 (Anti-collision), ISO/IEC 15693, FeliCa ¦ +-------------------------------------------------------------------------------+

RFID (Radio Frequency Identification) is the underlying physical radio technology. It defines how an RF reader powers a coil at 13.56 MHz and isolates a tag's serial number (UID) using magnetic induction.

NFC (Near Field Communication) is the standardized application and protocol ecosystem built on top of 13.56 MHz RFID. It adds:

Standardized Data Packaging (NDEF): Any phone or reader can read URLs, text, and MIME data without needing proprietary software.

Three Operating Modes:

Reader/Writer Mode (reading passive stickers and smart cards).

Card Emulation Mode (emulating a contactless credit card or transit ticket).

Peer-to-Peer Mode (two powered devices exchanging data directly).

High-Level Transport (ISO 14443-4 T=CL): Error handling, block chaining, and smart card APDU exchanges.

[NOTE] Summary for your mentor: "Our hardware uses an RFID transceiver IC over an SPI bus to physically generate the 13.56 MHz magnetic field and handle the low-level RF modulation. By implementing the ISO 14443-3/4 state machine, anti-collision, and NDEF data structures in firmware, we create a full NFC implementation."

3. How SPI and the RFID Transceiver Interact: Step-by-Step

Let's trace what actually happens at the hardware and bit level when your host MCU wants to read an NFC tag.

HOST MCU SPI BUS RFID TRANSCEIVER ANTENNA / AIR NFC TAG ======== ======= ================ ============= ======= [Step 1: RF Field ON] Host writes 0x03 to --------> MOSI: [0x28 0x03] ----> TX1/TX2 drivers turn ON ---> 13.56 MHz unmodulated ----> Tag harvests power; TxControlReg (0x14) CS# asserted & released carrier generated charges reservoir cap (Mandatory 5 ms delay) [Step 2: Polling (REQA)] Host sets BitFraming -------> MOSI: [0x1A 0x07] ----> Configures 7-bit mode Host writes 0x26 to FIFO ---> MOSI: [0x12 0x26] ----> Shifts 7 bits to RF --------> 100% ASK Pause (2.5 us) -> Tag recognizes REQA; replies with ATQA [Step 3: ATQA Reception] Transceiver demodulates <----------------------------------------------------------- Tag load-modulates -------- Tag transmits 848 kHz subcarrier into FIFO 848 kHz subcarrier ATQA: 0x0004 Transceiver asserts IRQ ----> IRQ Pin goes LOW Host reads FIFO <------------ MISO: [0x00 0x04] <---- Clocks out 2 bytes [Step 4: Cascade Anti-Collision Walk] Host sends SEL (0x93) ------> MOSI: [0x93 0x20] ----> Shifts 16 bits to RF -------> Tag modulates UID --------> Multiple tags may collide; Transceiver detects collision; Transceiver phase detector Transceiver flags bit reports colliding bit position pinpoints collision bit position to Host Host resolves collision bits, selects specific tag (NVB=0x70), and receives SAK (0x00 = Type 2, 0x20 = ISO 14443-4) [Step 5: High-Level NDEF Read] Host issues READ Block 4 ---> MOSI: [0x30 0x04] ----> RF transmission -----------> Reads EEPROM Pages 4-7 ----> Tag load-modulates data Host clocks in 16 bytes <---- MISO: [NDEF Data] <---- Transceiver validates CRC <------------------------------- Tag CRC verified! Host parses NDEF payload: "https://vvdntech.com"

Detailed Breakdown of the Steps:

Step 1: Awakening the RF Field & The Mandatory Guard Delay

SPI Action: The MCU asserts \overline{CS, sends the register address for TxControlReg with the write bit, and writes 0x03 to turn on the differential antenna pins TX1 and TX2.

RFID Transceiver Action: The chip's internal RF oscillator connects to the push-pull output drivers, pumping a sinusoidal 13.56 MHz current through the EMC filter and PCB antenna coil.

Physical Result: A magnetic H-field expands outward from the coil.

The Critical Rule: The host MCU must wait at least $5.0\text{ ms}$ before sending any data. The tag has no battery; it must harvest energy from this unmodulated carrier, rectify it, charge its reservoir capacitor, and release its internal Power-On Reset (POR).

Step 2: The Polling Mystery — Why 7 Bits Matter

The Problem: ISO 14443-A mandates that the initial request (REQA) must be exactly 7 bits (0x26 = 0100110b), not 8 bits!

How SPI Solves It: Microcontroller SPI peripherals only transfer whole bytes (multiples of 8 bits). You cannot tell an MCU's SPI controller to "send 7 bits".

The Solution: The host writes to the transceiver's BitFramingReg over SPI to tell its internal bit-engine: "The next byte in the FIFO only has 7 valid bits".

The Transmission: The transceiver's digital state machine takes the 0x26 byte from its FIFO, shifts out only 7 bits to the RF modulator, creating a 2.5\ µs blanking pause (100% ASK) on the 13.56 MHz carrier.

Step 3: Tag Response via Inductive Load Modulation

The tag has no transmitter. How does it reply?

It turns an internal MOSFET on and off, switching an extra resistor across its own coil.

Because the tag's coil and the reader's coil are magnetically coupled like an air-core transformer, changing the tag's load changes the reflected impedance seen at the reader's coil.

This creates tiny amplitude variations (10 to 50 mV) on the reader's 15V carrier at an $848\text{ kHz}$ subcarrier offset.

The RFID transceiver's analog envelope detector and bandpass filter isolate this 848 kHz signal, decode the Manchester bits, assemble them into bytes, and push them into the internal FIFO buffer.

The transceiver pulls its `IRQ` pin LOW to tell the Host MCU: "I have received data!"

The Host MCU initiates an SPI burst read to pull the ATQA (0x0004) out of the transceiver FIFO.

Step 4: Resolving Collisions (Multi-Card Detection)

If two NFC cards are placed on the reader simultaneously, they both respond to REQA at the exact same instant.

Where their serial numbers differ (e.g. Card 1 has bit 12 = 0, Card 2 has bit 12 = 1), their load modulation signals conflict, violating the Manchester timing rules.

The RFID transceiver's phase comparator flags a Bit Collision, identifies the exact bit position (e.g., bit 12), and asserts the CollErr flag.

The Host MCU reads this collision position over SPI, chooses a path (e.g. chooses bit 12 = 0), and re-transmits. Card 2 goes silent, and Card 1 is successfully isolated and selected!

Step 5: High-Level NDEF Reading

Once the tag is uniquely selected, the Host MCU sends a Type 2 Tag READ command (0x30 0x04) over SPI.

The tag returns 16 bytes of memory (Pages 4, 5, 6, and 7).

The Host MCU clocks this data in over SPI and parses the NDEF record:

Validates the header (0xD1), identifies the type as 'U' (URI), expands the prefix code (0x04 = https://), and extracts "vvdntech.com".

The result: A complete, fully functional NFC interaction achieved via an SPI-controlled RFID hardware frontend.

4. Why SPI? (Comparing Against I2C and UART)

When your mentor asks: "Why did you use SPI rather than I2C or UART to talk to the RFID chip?", use this comparison:

| Metric | UART | I2C | SPI (Our Choice) |
| --- | --- | --- | --- |
| Max Bus Clock | 115.2 kbps – 1 Mbps | 100 kHz – 400 kHz | 5 MHz – 10 MHz |
| Throughput | ~10 KB/s – 100 KB/s | ~10 KB/s – 40 KB/s | 600 KB/s – 1200 KB/s |
| Duplex | Full Duplex | Half Duplex | Full Duplex |
| Protocol Overhead | Start/Stop bits (~20%) | 7-bit address, ACK/NACK | Zero overhead (direct register shifting) |
| FIFO Drain Time | Several milliseconds | \sim 1.5 ms | $< 60\ \mu\text{s}$ (instant) |
| Suitability for High-Speed NFC | Poor (Causes FIFO overrun) | Marginal (Bottlenecks 848k) | Optimal (Handles 848 kbps seamlessly) |

The "FIFO Overflow" Engineering Argument:

At high NFC bitrates (848 kbps), the over-the-air RF interface delivers 106 bytes per millisecond.

A standard RFID chip FIFO is only 64 bytes. That means the FIFO will overflow and permanently corrupt the data in $603\ \mu\text{s}$!

On I2C at 400 kHz, draining 64 bytes takes over 1500\ µs \rightarrow FIFO overflow occurs!

On SPI at 10 MHz, draining 64 bytes takes only $51.2\ \mu\text{s}$ \rightarrow Safe with massive margin!

5. Mentor Defense & Interview Q&A Cheat Sheet

Use these structured answers during technical reviews with your mentor:

Q1: "What is the difference between MFRC522 and PN532?"

[NOTE] "MFRC522 is a direct register-driven Analog Front-End (AFE)*. It has no internal CPU. Our microcontroller must run the full ISO 14443-3 protocol state machine, anti-collision loop, and timing in software. In contrast, the PN532 is an intelligent controller with an embedded 8051 microcontroller and ROM firmware. It handles anti-collision, card emulation, and P2P internally, communicating with our host MCU via high-level SPI command packets."*

Q2: "What is the most notorious firmware pitfall when interfacing the PN532 over SPI?"

[NOTE] "The LSB-first bit order trap. In 99% of embedded SPI peripherals, data is transferred MSB-first. However, the PN532's hardware SPI interface shifts data LSB-first (least significant bit first). If the host MCU's SPI controller is not reconfigured for LSB-first, or if bytes are not reversed in software, the PN532 receives garbled commands and appears completely dead."

Q3: "How does an unpowered NFC tag communicate back to the reader?"

[NOTE] "Through Inductive Load Modulation. The tag has no battery or RF transmitter. It switches an internal load resistor or capacitor across its own antenna coil. Because of mutual inductance ($M = k\sqrt{L_1 L_2}$), this load change reflects back onto the reader's primary coil, creating minute voltage variations at an $848\text{ kHz}$ subcarrier frequency, which the transceiver's analog receiver demodulates."

Q4: "Why can't we design the antenna with an extremely high Q-factor to get maximum range?"

[NOTE] "Because of the Quality Factor vs. Bandwidth trade-off ($BW = f_c / Q$). While a high $Q$ ($> 40$) increases magnetic field strength, it narrows the RF bandwidth below $340\text{ kHz}$, which severely attenuates the $848\text{ kHz}$ load-modulation sidebands and causes excessive pause ringing during 100% ASK modulation. For 106 kbps Type A, $Q$ must be damped to $20\text{ to }30$, and for high-speed 848 kbps, down to $10\text{ to }15$."

Q5: "What is the 5 ms Guard Time and why is it mandatory?"

[NOTE] "Under ISO/IEC 14443-3 Section 5.1, after the reader powers on the 13.56 MHz carrier, it must wait at least $5.0\text{ ms}$ before transmitting the first command (REQA). A passive tag needs this time to harvest energy from the magnetic field, charge its internal reservoir capacitor, and release its Power-On Reset circuit. Without this delay, the tag fails to wake up and the reader reports 'No Card Found'."