SPI-Based RFID/NFC Transceiver Interface Architecture

1. Why SPI is the Interface of Choice for NFC Frontends

Modern NFC transceivers must bridge high-speed over-the-air RF bitstreams to a host microcontroller. While some chips offer auxiliary I2C or UART interfaces, SPI (Serial Peripheral Interface) is the industry standard for production embedded systems for the following engineering reasons:

Bandwidth & Latency:

Contactless smart card protocols (ISO/IEC 14443-4) support data rates of 106, 212, 424, and 848 kbps.

At 848 kbps, payload data arrives at \sim 106 KB/s. An I2C bus at standard 100 kHz (effective \sim 10 KB/s) or Fast Mode 400 kHz (effective \sim 36 KB/s) cannot keep pace without massive on-chip buffering, causing internal FIFO overflows.

An SPI bus running at 5 MHz to 10 MHz transfers data at \sim 600 to 1200 KB/s, draining transceiver FIFOs in microseconds.

Deterministic Full-Duplex Streaming:

SPI has zero bus arbitration overhead, zero in-band 7-bit addressing, and zero clock-stretching delays.

Interrupt Latency Tolerance:

Small hardware FIFOs (typically 64 bytes in MFRC522 or PN532) fill in less than 600\ µs at 848 kbps. High-speed SPI allows burst reads during interrupt service routines (ISRs) before buffer overrun occurs.

2. Hardware Pinout & Electrical Interconnect

A robust SPI-based NFC subsystem requires the standard 4-wire SPI bus plus two to three dedicated hardware sideband GPIO lines:

+-------------------+ +-------------------------+ ¦ Host MCU ¦ ¦ NFC Transceiver IC ¦ ¦ ¦----------- SCLK (Max 10 MHz) -------->¦ SCK / SCLK ¦ ¦ ¦----------- MOSI / COPI -------------->¦ MOSI / SDI ¦ ¦ ¦<---------- MISO / CIPO ---------------¦ MISO / SDO ¦ ¦ ¦----------- CS# / NSS ---------------->¦ NSS / CS# ¦ ¦ ¦ ¦ ¦ ¦ ¦<---------- IRQ (Interrupt Line) ------¦ IRQ (TxDone, RxDone, WL)¦ ¦ ¦----------- RSTPD# / RESET ----------->¦ RSTPD / RST (Hard Reset)¦ ¦ ¦<---------- BUSY / READY (Optional) ---¦ BUSY (Handshake Line) ¦ +-------------------+ +-------------------------+

2.1 Signal Definitions & Design Rules

$\overline{\text{CS}}$ / NSS (Slave Select / Chip Select):

Driven active-LOW by the Host MCU.

Crucial Rule: The host must explicitly deassert CS (drive HIGH) between commands or burst transfers to reset the transceiver's internal SPI bit counters.

SCLK (Serial Clock):

Driven by Host MCU. Transceivers typically support f_{SCLK up to 10 MHz (MFRC522, ST25R3916) or 5 MHz (PN532).

MOSI / SDI (Master Out, Slave In):

Serial command/data stream from Host to Transceiver.

MISO / SDO (Master In, Slave Out):

Serial status/data stream from Transceiver to Host. Enters High-Z (tri-state) when \overline{CS is HIGH.

IRQ (Interrupt Request):

Essential for Non-Blocking Firmware: Driven by the transceiver to notify the host of critical events (TxDone, RxDone, FIFOLevel watermark, Idle, Err, Timer, RFFieldDetect).

Can be configured as active-LOW open-drain or active-HIGH push-pull depending on register settings.

$\overline{\text{RSTPD}}$ / RST (Reset / Power-Down):

Hard reset and low-power sleep control. Driving this pin LOW shuts down internal analog oscillators, PLLs, and RF drivers, cutting quiescent current from \sim 30 mA down to < 5\ µA.

BUSY / READY (PN532 Handshake):

Used in intelligent controllers to indicate internal processor execution state.

2.2 Level Shifting Considerations

Most modern Host MCUs operate at $3.3\text{V}$ or $1.8\text{V}$ I/O logic.

NFC transceivers frequently power their RF transmitter stage (TVDD or AVDD) from $5.0\text{V}$ to maximize magnetic field strength (H), but supply their digital logic (DVDD, VDD_IO) from $3.3\text{V}$.

Always verify that VDD_IO is tied to the Host MCU supply rail to eliminate the need for external level shifters.

3. Archetype 1: Direct Register-Driven Transceivers (MFRC522, ST25R3916, CLRC663)

In register-driven transceivers, the chip has no internal general-purpose microprocessor. The host MCU directly manipulates hardware state machines, analog registers, and a hardware FIFO over SPI.

REGISTER-DRIVEN SPI FRAME FORMAT (MFRC522 EXAMPLE) Byte 0: Address & Control Byte Byte 1 .. N: Data Bytes +---+---+---+---+---+---+---+---+ +---+---+---+---+---+---+---+---+ ¦ 7 ¦ 6 ¦ 5 ¦ 4 ¦ 3 ¦ 2 ¦ 1 ¦ 0 ¦ ¦ 7 ¦ 6 ¦ 5 ¦ 4 ¦ 3 ¦ 2 ¦ 1 ¦ 0 ¦ +---+---+---+---+---+---+---+---+ +---+---+---+---+---+---+---+---+ ¦ <------- 6 Bits -------> ¦ <------ Data Payload --------> ¦ Reg Address ¦ ¦ +---> Fixed '0' +---> Read/Write Bit: 0 = Write to Register 1 = Read from Register

3.1 SPI Bus Configuration

SPI Mode: Mode 0 (CPOL = 0, CPHA = 0) — Clock idles LOW, data sampled on the rising edge, shifted on the falling edge.

Bit Order: MSB-First (Most Significant Bit transmitted first).

Max Clock Speed: Typically 10 MHz.

3.2 Register Addressing Protocol

Single Register Write:

Assert \overline{CS LOW.

Send Byte 0: (Address << 1) & 0x7E (Bit 7 = 0 for Write; Bit 0 = 0).

Send Byte 1: Data value to write.

Deassert \overline{CS HIGH.

Single Register Read:

Assert \overline{CS LOW.

Send Byte 0: ((Address << 1) & 0x7E) | 0x80 (Bit 7 = 1 for Read; Bit 0 = 0).

Send Dummy Byte (0x00), simultaneously read Data value on MISO.

Deassert \overline{CS HIGH.

Continuous Burst FIFO Read / Write:

To read/write the 64-byte FIFO without repeating address bytes:

Assert \overline{CS LOW.

Send Byte 0: FIFO address (0x09 on MFRC522 \rightarrow Read Byte: (0x09 << 1) | 0x80 = 0x92).

Continue sending dummy bytes (read) or payload bytes (write) while holding \overline{CS LOW. The transceiver automatically streams bytes into or out of the internal FIFO ring buffer.

Deassert \overline{CS HIGH.

3.3 The Bit Framing Engine (BitFramingReg / TxLastBits)

Standard SPI hardware is strictly byte-oriented (transfers in multiples of 8 bits). However, NFC air interfaces require sending non-byte-aligned frames:

REQA / WUPA frames are exactly 7 bits long (0x26 or 0x52).

Anti-Collision frames can end on arbitrary bit boundaries (e.g. 23 bits, 31 bits).

To handle this, register-driven chips contain a Bit Framing Engine:

`BitFramingReg` (Address `0x0D` on MFRC522):

Bits 2:0 (TxLastBits): Defines the number of valid bits to transmit for the final byte in the FIFO (0 = all 8 bits, 1 = 1 bit, ..., 7 = 7 bits).

Bits 6:4 (RxAlign): Defines the bit position for the first received bit.

Bit 7 (StartSend): When set, immediately starts RF transmission from FIFO.

4. Archetype 2: Packet-Oriented Intelligent NFC Controllers (NXP PN532, PN7160)

The NXP PN532 contains an internal 8051 microcontroller running proprietary ROM firmware that executes the ISO 14443-3 and ISO 14443-4 state machines internally. Communication over SPI occurs via a Handshake Packet Protocol.

PN532 NORMAL SPI DATA FRAME STRUCTURE +----------+-------------+-----+-----+-----+-------------+-----+----------+ ¦ Preamble ¦ Start Code ¦ LEN ¦ LCS ¦ TFI ¦ Packet Data ¦ DCS ¦ Postamble¦ ¦ 0x00 ¦ 0x00 0xFF ¦ 1 B ¦ 1 B ¦ 1 B ¦ N Bytes ¦ 1 B ¦ 0x00 ¦ +----------+-------------+-----+-----+-----+-------------+-----+----------+ ¦<--------->¦ ¦<----------->¦ LEN + LCS=0 Sum(TFI..Data) + DCS = 0

4.1 THE INFAMOUS CRITICAL TRAP: LSB-First Bit Order

[WARNING] CRITICAL EMBEDDED PITFALL: In 99% of embedded SPI peripherals (Flash, IMUs, ADCs, MFRC522, ST25R), SPI operates MSB-First. However, the PN532 hardware SPI controller operates LSB-FIRST (Least Significant Bit transmitted first, Bit 0 before Bit 7)! If your microcontroller's SPI hardware does not support runtime hardware LSB-first switching, you must perform bit-reversal in software on every single byte transmitted and received, or all commands will be completely rejected by the PN532.

// Software bit-reversal lookup function for MSB-only SPI controllers static inline uint8_t nfc_reverse_bits(uint8_t b) { b = (b & 0xF0) >> 4 ¦ (b & 0x0F) << 4; b = (b & 0xCC) >> 2 ¦ (b & 0x33) << 2; b = (b & 0xAA) >> 1 ¦ (b & 0x55) << 1; return b; }

4.2 SPI Handshake Protocol & Flow Control

The PN532 uses three single-byte SPI command opcodes to manage bus handshaking:

`0x01` — Read Status Byte:

Host sends 0x01, then clocks in 1 byte.

Bit 0 is the `Ready` bit:

Status & 0x01 == 0x01: PN532 has finished processing and response data is ready to read.

Status & 0x01 == 0x00: PN532 is busy; host must poll again or wait for the IRQ pin.

`0x02` — Write Data:

Host sends 0x02, followed immediately by the command packet frame.

`0x03` — Read Data:

Host sends 0x03, followed by clocking in the response packet frame.

Host MCU PN532 ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x02 (Write Data Opcode) + Command Packet -------------->¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦ (PN532 processes RF / ISO commands...) ¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x01 (Read Status Opcode) ------------------------------>¦ ¦<-- Status Byte: 0x00 (Not Ready, Still Busy) ---------------¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦ [Optionally: Wait for PN532 IRQ line to go LOW] ¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x01 (Read Status Opcode) ------------------------------>¦ ¦<-- Status Byte: 0x01 (READY!) ------------------------------¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x03 (Read Data Opcode) -------------------------------->¦ ¦<-- Response Packet Frame (TFI=0xD5, Data...) ---------------¦ ¦--- CS# HIGH ----------------------------------------------->¦

4.3 PN532 Packet Frame Fields

Preamble (`0x00`): Single synchronizing null byte.

Start Code (`0x00 0xFF`): Identifies start of packet.

Length (`LEN`): Total count of bytes in the TFI + Packet Data payload (1 ≤ LEN ≤ 255).

Length Checksum (`LCS`):

(\text{LEN} + \text{LCS}) \ \& \ 0\text{xFF} = 0\text{x}00 \implies \text{LCS} = (\sim\text{LEN} + 1) \ \& \ 0\text{xFF}

Target Frame Identifier (`TFI`):

0xD4: Direction = Host MCU \rightarrow PN532.

0xD5: Direction = PN532 \rightarrow Host MCU.

Data Checksum (`DCS`):

\left( \text{TFI} + \sum \text{Data Bytes} + \text{DCS} \right) \ \& \ 0\text{xFF} = 0\text{x}00

Postamble (`0x00`): Concluding byte.

4.4 Standard Handshake Packets

Acknowledge (ACK) Packet: Sent by PN532 immediately after receiving a valid command:

00 00 FF 00 FF 00

Negative Acknowledge (NACK) Packet: Indicates checksum or framing error:

00 00 FF FF 00 00

5. FIFO Buffer Architecture & Interrupt Watermark Mechanics

NFC transceivers rely on internal hardware FIFO buffers to decouple the real-time RF air interface from host SPI processing:

TRANSCEIVER HARDWARE FIFO MANAGEMENT +-------------------------------------------------------------------+ ¦ 64-Byte FIFO Ring Buffer ¦ ¦ [B0][B1][B2][B3] ... [B31] ... [B62][B63] ¦ +-------------------------------------------------------------------+ ^ ^ ¦ Write Pointer ¦ Read Pointer ¦ ¦ Air Interface RX SPI Burst Read to MCU (106 - 848 kbps) (5 - 10 MHz)

5.1 FIFO Watermark Latency Budget Calculation

Let us calculate the critical timing budget to prevent FIFO overflow during an ISO 14443-4 transmission at 848 kbps:

RF Arrival Rate:

\text{Bit Rate} = 848\text{ kbps} \implies \text{Byte Rate} = \frac{848,000}{8} = 106,000\text{ bytes/sec} = 106\text{ bytes/ms}

Buffer Fill Time:

A 64-byte FIFO will completely fill in:

t_{fill} = \frac{64\text{ bytes}}{106\text{ bytes/ms}} \approx 0.603\text{ ms} = 603\ \mu\text{s}

Watermark Threshold Setting:

If the WaterLevel interrupt is programmed to fire at half-full (32 bytes):

t_{margin} = \frac{32\text{ bytes}}{106\text{ bytes/ms}} \approx 301\ \mu\text{s}

Hard Real-Time Constraint: The Host MCU must service the external GPIO interrupt and execute an SPI burst read within $301\ \mu\text{s}$, or data will be permanently lost due to FIFO overrun!

SPI Read Drain Time:

At f_{SCLK = 5 MHz, transferring 32 bytes over SPI takes:

t_{spi} = \frac{32\text{ bytes} \times 8\text{ bits}}{5\text{ MHz}} = 51.2\ \mu\text{s}

Because 51.2\ µs \ll 301\ µs, the SPI bus easily drains the FIFO before overflow occurs, provided the host firmware does not block inside higher-priority interrupt handlers.

6. AC Timing Specifications & Bus Constraints

To ensure data integrity, host firmware and hardware design must strictly adhere to the transceiver's AC switching characteristics:

CS# \___________________________________________________________/-----\______ ¦<-- t_lead -->¦ ¦<-- t_lag -->¦ t_CS_high ¦ SCLK ________________/-\_/-\_/-\ ... /-\_/-\_/-\_______________________/-\_/-\ MOSI -------< Bit 7 >< Bit 6 > ... < Bit 0 >-----------------------< Bit 7 > ¦<-t_su->¦<-t_h->¦ MISO -------< Bit 7 >< Bit 6 > ... < Bit 0 >--------------------------------

| Timing Parameter | Symbol | Min Value | Max Value | Description |
| --- | --- | --- | --- | --- |
| SCLK Clock Frequency | f_{SCLK | DC | 10 MHz | Maximum SPI clock rate |
| SCLK Pulse Width High / Low | t_{CLKH, t_{CLKL | 45 ns | — | SCLK duty cycle must stay within 40%–60% |
| Chip Select Lead Time | t_{lead | 50 ns | — | Delay from \overline{CS falling edge to first SCLK edge |
| Chip Select Lag Time | t_{lag | 50 ns | — | Delay from last SCLK edge to \overline{CS rising edge |
| Chip Select Deselection Time | t_{CS\_high | 100 ns | — | Mandatory pause between back-to-back SPI transfers |
| Data Setup Time | t_{su | 15 ns | — | Input data valid before SCLK sampling edge |
| Data Hold Time | t_h | 15 ns | — | Input data held valid after SCLK sampling edge |
| Output Valid Delay | t_{v | — | 25 ns | Delay from SCLK shift edge until MISO data is stable |