Hardware Compliance, Quality, and Reliability Standards

Welcome to the Hardware Compliance, Quality, and Reliability Knowledge Base at VVDN. This directory provides comprehensive, production-grade engineering documentation on mandatory regulatory compliance frameworks, environmental directives, automotive qualification standards, and component handling protocols.

1. Quick Navigation & Topic Index

| Topic | Standard / Directive | Governing Body | Domain | Primary Focus | Guides Available |
| --- | --- | --- | --- | --- | --- |
| RoHS | Directive 2011/65/EU + (EU) 2015/863 (RoHS 3) | European Union (CE Mark) | Environmental & Chemical Compliance | Restriction of 10 hazardous substances (Pb, Cd, Hg, Cr6+, phthalates) at homogeneous material level | Markdown Guide \ |
| REACH | Regulation (EC) No 1907/2006 | ECHA (European Chemicals Agency) | Chemical Safety & Material Declaration | Registration, evaluation, authorization of chemicals; SVHC declaration (>0.1% w/w) & SCIP reporting | Markdown Guide \ |
| AEC | AEC-Q100, Q101, Q102, Q103, Q104, Q200 | Automotive Electronics Council | Component Reliability & Stress Qualification | Failure-mechanism-based qualification testing, temperature grades (-40°C to +150°C), zero-defect criteria | Markdown Guide \ |
| MSL | IPC/JEDEC J-STD-020 & J-STD-033 | IPC / JEDEC | SMT Manufacturing & Packaging Reliability | Classification of moisture sensitivity (Levels 1–6), floor life tracking, desiccant packaging, and baking | Markdown Guide \ |

2. Hardware Lifecycle Mapping

Hardware compliance and reliability are not afterthoughts; they are enforced at distinct stages of the product development lifecycle:

┌─────────────────────────────────────────────────────────────────────────────────────────────┐ │ HARDWARE PRODUCT LIFECYCLE │ └─────────────────────────────────────────────────────────────────────────────────────────────┘ 1. Component Selection & Architecture ├── AEC-Q100/Q101/Q200: Select automotive-grade silicon & passives based on operating temp ├── RoHS: Verify lead-free package terminations, verify RoHS 3 (10 substances) compliance └── REACH: Screen BOM against latest ECHA SVHC Candidate List (avoid SVHC > 0.1% w/w) │ ▼ 2. Sourcing, Procurement & Supplier Quality ├── Collect Full Material Disclosures (FMD) / IPC-1752A XML declarations ├── Collect Supplier Certificates of Compliance (CoC) and test reports (IEC 62321) └── Review supplier Part Submission Warrant (PSW) & PPAP Level 3 documentation │ ▼ 3. Warehouse, Storage & Inventory ├── MSL: Inspect Moisture Barrier Bags (MBB), verify Desiccant & Humidity Indicator Card (HIC) └── Track floor life consumption upon unsealing; maintain dry cabinets (<5% / <10% RH) │ ▼ 4. SMT Manufacturing & Assembly ├── MSL: Bake components exceeding floor life according to J-STD-033 before reflow ├── RoHS: Implement Lead-Free solder alloys (SAC305) with 245°C–260°C peak reflow profiles └── Assembly Quality: Inspect solder joints (IPC-A-610 Class 2/3) & prevent tin whiskering │ ▼ 5. Regulatory Certification & Market Delivery ├── Compile Technical Documentation File under EN IEC 63000 (CE DoC for RoHS) ├── Submit SCIP notifications to ECHA for articles with SVHC > 0.1% w/w └── Automotive OEM: Deliver PPAP packages and ISO 26262 functional safety audit trails

3. High-Level Comparison Matrix

| Parameter | RoHS | REACH | AEC Qualifications | MSL (IPC/JEDEC) |
| --- | --- | --- | --- | --- |
| Type of Rule | Mandatory EU Directive (statutory law, CE mark) | Mandatory EU Regulation (statutory law) | Industry standard (automotive OEM customer requirement) | Industry standard (IPC/JEDEC assembly guideline) |
| Target Object | Finished Electrical & Electronic Equipment (EEE) | Chemical substances, mixtures, and articles (all goods) | Individual electronic components (ICs, discretes, passives) | Surface Mount Devices (SMD plastic ICs, LEDs, connectors) |
| Key Threshold | Strict concentration limits (e.g., 0.1% / 1000 ppm) per homogeneous material | 0.1% w/w per individual article (SVHC notification threshold) | Zero defects allowed (c=0) across stress test sample lots | Floor life duration (e.g., 168 hours for MSL 3 at ≤ 30^{\circC / 60\% RH) |
| Failure Mode Addressed | Environmental toxicity, landfill leaching, heavy metal poisoning | Human health hazards (carcinogens, mutagens, endocrine disruptors) | Mechanical/thermal/electrical degradation under automotive stress | Solder reflow steam explosion ("popcorning"), internal delamination |
| Key Documents | CE Declaration of Conformity (DoC), IEC 62321 test lab reports | Full Material Declaration (FMD), SCIP submission number | AEC qualification report, PPAP package, Part Submission Warrant (PSW) | Moisture Caution Label (MBB label), HIC card, factory bake records |

4. Directory Structure

standards/ ├── README.md (.docx) # Master index and engineering lifecycle mapping ├── rohs/ │ ├── rohs-guide.md # Comprehensive Markdown documentation │ └── rohs-guide.docx # Formatted Word document for mentor / review ├── reach/ │ ├── reach-guide.md # Comprehensive Markdown documentation │ └── reach-guide.docx # Formatted Word document for mentor / review ├── aec/ │ ├── aec-guide.md # Comprehensive Markdown documentation │ └── aec-guide.docx # Formatted Word document for mentor / review └── msl/ ├── msl-guide.md # Comprehensive Markdown documentation └── msl-guide.docx # Formatted Word document for mentor / review