Smart Programmable Power Supply Module: 85–265 V AC Input, 5–20 V / 3 A DC Output

Welcome to the VVDN Engineering Hub Smart Programmable Power Supply (SPPS) Knowledge Base. This repository serves as the definitive engineering design, mathematical formulation, hardware schematic, firmware implementation, and testing reference for a high-efficiency Universal AC-DC Flyback Converter with MCU-Controlled Synchronous Buck-Boost Post-Regulator, Real-Time Dual-Domain Efficiency Monitoring, and MicroSD Data Logging.

1. Project Executive Summary & System Specifications

The Smart Programmable Power Supply Module is an intelligent, digitally controlled bench/industrial power supply engineered to provide a tightly regulated, programmable DC voltage output from 5.0\,V to 20.0\,V at currents up to 3.0\,A (60\,W maximum continuous output power) from a universal mains AC input (85\,V \dots 265\,V AC RMS, 47\,Hz \dots 63\,Hz).

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Smart Programmable Power Supply Key Engineering Specs │ ├────────────────────────────┬─────────────────────────────┬────────────────────────┤ │ Parameter / Specification │ Value Range / Unit │ Design Condition │ ├────────────────────────────┼─────────────────────────────┼────────────────────────┤ │ **AC Input Voltage Range** │ 85 V – 265 V AC (Nom. 230V) │ Universal Mains (RMS) │ │ **AC Input Frequency** │ 47 Hz – 63 Hz │ Global Grid Compatible │ │ **Inrush Current Limit** │ < 30 A Peak │ Cold start via NTC │ │ **Primary Topology** │ Quasi-Resonant (QR) Flyback │ Galvanic Isolation 3kV │ │ **Intermediate DC Bus** │ +24.0 V DC (Fixed, 72W max) │ Low ripple (< 50mV pk) │ │ **Post-Regulator Topology**│ 4-Switch Synchronous BB │ High efficiency > 95% │ │ **Programmable Output (V)**│ 5.0 V – 20.0 V DC │ 10 mV digital step size│ │ **Programmable Output (I)**│ 0.1 A – 3.0 A Continuous │ 10 mA step (CC mode) │ │ **Maximum Output Power** │ 60.0 W Continuous │ Convection / fanless │ │ **Output Voltage Ripple** │ < 25 mV pk-pk @ 20V/3A │ Full load 20MHz BW │ │ **Transient Recovery** │ < 150 µs to 1% │ 50% to 100% load step │ │ **Overall System Eff (η)** │ 84% – 89% (End-to-End) │ Across 115V/230V mains │ │ **Real-Time Metering** │ Dual AC (RMS) + DC (True V,I│ Efficiency update 10Hz │ │ **Data Logging Engine** │ MicroSD FAT32 (CSV format) │ Timestamp, V, I, P, η │ │ **Isolation & Safety** │ Reinforced (IEC 62368-1) │ Creepage >= 6.4mm │ │ **EMC Compliance** │ EN 55032 / CISPR 32 Class B │ Conducted/Radiated EMI │ └────────────────────────────┴─────────────────────────────┴────────────────────────┘

2. High-Level Architectural Block Diagram

The module adopts a two-stage cascaded conversion architecture with isolated primary control and intelligent secondary digital regulation:

85-265V AC Mains In │ ├───[ Fuse 2A ]───[ NTC Inrush ]───[ EMI Filter (X2, CMC) ]───[ Bridge Rectifier ] │ │ │ +V_RECT (120V - 375V DC) │ │ │ ┌──────────────────────────────────────────────────────────────────┴──────────────────┐ │ │ STAGE 1: AC-DC FLYBACK CONVERTER │ │ │ │ │ │ ┌──────────────┐ PQ26/20 Transformer ┌──────────────────┐ │ │ │ │ QR Flyback ├─────[ Np ] [ Ns ]─────┤ Synchronous ├── +24V Intermediate │ │ │ Controller │ ││ ││ │ Rectifier │ Bus (72W) │ │ │ (UCC28740) │ [ Aux ] ││ │ (MP6908 + MOSFET)│ │ │ │ └──────┬───────┘ ││ ││ └────────┬─────────┘ │ │ │ │ │ │ │ │ │ │ Primary FET │ │ TL431 + PC817 │ │ │ (Superjunction) │ │ Opto Feedback │ │ └────────────────────────────┼───────┼───────────────────┬───────────────────┘ │ │ │ │ │ ===========================│=======│===================│======================== │ REINFORCED SAFETY ISOLATION│BARRIER│ (>= 6.4mm Creepage│ / 3000V RMS Hipot) │ ===========================│=======│===================│======================== │ │ │ │ │ │ │ │ │ ┌────────────────────────────┴───────┴───────────────────┴───────────────────┐ │ │ STAGE 2: SYNCHRONOUS BUCK-BOOST POST-REGULATOR │ │ │ │ │ │ +24V In ────[ 4-Switch Sync Buck-Boost H-Bridge ]────┬────> +V_OUT (5V-20V @ 3A) │ │ (Buck-Boost Inductor + 4x MOSFETs) │ Programmable DC │ │ │ │ │ │ High-Resolution PWM / │ │ │ DAC Feedback Injection │ │ └────────────────────────────────┼──────────────────────┼────────────────────┘ │ │ │ │ ┌────────────────────────────────┴──────────────────────┴────────────────────┐ │ │ STAGE 3: MCU CONTROL, SENSING & TELEMETRY │ │ │ │ │ │ ┌──────────────────┐ ┌──────────────────┐ ┌─────────────────────────┐ │ │ │ │ AC Power Metering│ │ DC Power Metering│ │ STM32G4 / ESP32-S3 MCU │ │ │ │ │ (ADE7953 / │ │ (INA226 I2C │ │ - 32-bit Cortex Core │ │ │ │ │ AMC1311 Isolated│ │ 0.1% Shunt) │ │ - Real-time η = Pdc/Pac │ │ │ │ │ RMS V, I, P_ac) │ │ True V, I, P_dc │ │ - CV / CC State Machine │ │ │ │ └────────┬─────────┘ └────────┬─────────┘ └────────────┬────────────┘ │ │ │ │ │ │ │ │ │ └──────────────────────┴─────────────┬────────────┘ │ │ │ │ │ │ │ ┌────────────────────┐ ┌────────────────────┴───┐ ┌─────────────────┐ │ │ │ │ MicroSD Logger │ │ 1.3" OLED Display │ │ USB-C / SCPI │ │ │ │ │ (FAT32 CSV Log) │ │ (V, I, P, η, Temp) │ │ Telemetry & CLI │ │ │ └──┴────────────────────┴───┴────────────────────────┴───┴─────────────────┴─┘

3. Two-Stage Power Architecture Rationale

Why is a two-stage cascaded architecture (Universal AC-DC Flyback + Synchronous Buck-Boost) vastly superior to a single-stage variable flyback for a precision programmable bench supply?

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Single-Stage Variable Flyback vs. Cascaded Two-Stage Topology │ ├───────────────────────┬───────────────────────────┬───────────────────────────────┤ │ Design Dimension │ Single-Stage Var. Flyback │ Cascaded Two-Stage (Chosen) │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ **Dynamic Range** │ Very difficult to maintain│ Flyback is fixed at +24V DC. │ │ │ stability and auxiliary │ Auxiliary bias winding stays │ │ │ VCC over a 4:1 (5V-20V) │ rock-solid across all loads! │ │ │ output voltage range. │ │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ **Transformer Design**│ Extreme primary turns vs. │ Optimized for a single 24V │ │ │ saturation trade-off; poor│ operating point; maximum core │ │ │ winding utilization. │ utilization and peak eff. │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ **Output Ripple & BW**│ 100 Hz/120 Hz mains ripple│ Buck-Boost post-regulator │ │ │ bleeds into output; slow │ switches at 250 kHz, yielding │ │ │ optocoupler control loop. │ sub-25mV ripple & fast step. │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ **Secondary Sensing** │ High-voltage isolation │ Secondary MCU handles CV/CC │ │ │ crossing required for DAC │ locally on secondary ground. │ │ │ voltage adjustment. │ No isolated DAC required! │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ **Transient Response**│ Sluggish (> 5 ms) due to │ Ultra-fast (< 150 µs) dynamic │ │ │ optocoupler low bandwidth.│ loop response under 3A steps. │ └───────────────────────┴───────────────────────────┴───────────────────────────────┘

4. Key Subsystems & Functional Blocks

AC-DC Primary Flyback Converter Stage:

Universal mains rectification (85\,V \dots 265\,V AC) with EMI Pi-filtering (common-mode choke + differential film caps) compliant with CISPR 32 Class B.

Quasi-Resonant (QR) zero-voltage valley switching controller (TI UCC28740 / ON Semi NCP1342) driving an 800\,V Superjunction MOSFET.

Precision transformer design on PQ26/20 ferrite core with triple-insulated secondary wire (≥ 3\,kV_{RMS reinforced barrier).

Synchronous rectification on the secondary using an MP6908 fast SR controller and 40\,V low-R_{DS(on) N-channel MOSFET, achieving > 91\% stage efficiency.

Regulated +24.0\,V intermediate DC bus output with optocoupler isolation (PC817) and TL431 reference.

MCU-Controlled Synchronous Buck-Boost Post-Regulator:

4-switch non-inverting synchronous Buck-Boost topology seamlessly stepping down (24\,V \rightarrow 5\,V) or boosting (24\,V \rightarrow 20\,V) with up to 97\% efficiency.

Controlled by high-frequency PWM or dedicated multi-mode controller (e.g., LM5176 / LTC3789) driven by an embedded 32-bit microcontroller (STM32G474 / ESP32-S3).

Digital feedback summing node: An on-chip 12-bit DAC injects offset current into the voltage divider node, steering the output voltage from 5.0\,V to 20.0\,V with 10\,mV precision.

Hardware Constant Current (CC) control loop: Fast low-side/high-side shunt sense amplifier clamps the error amplifier when load current reaches the programmable current limit (0.1\,A \dots 3.0\,A).

Dual-Domain Real-Time Power Metering & Efficiency Engine:

Primary AC Metering: Galvanically isolated AC voltage divider + current transformer (or isolated \Delta\Sigma modulator AMC1311 + AMC1300) measuring true RMS voltage (V_{AC,rms), true RMS current (I_{AC,rms), real power (P_{AC in Watts), and power factor (\cos\phi).

Secondary DC Metering: High-precision I2C digital power monitor (INA226 / INA228) measuring true output voltage (V_{DC), load current (I_{DC), and output power (P_{DC).

Real-Time Efficiency ($\eta$): Microcontroller computes true end-to-end efficiency at 10\,Hz:

\eta(t) = \frac{P_{DC}(t)}{P_{AC}(t)} \times 100\%

Telemetry, Data Logging & User Interface:

MicroSD Card Data Logger: Automatic continuous CSV logging of timestamped records (timestamp_ms, Vac_rms, Iac_rms, Pac_w, Vdc_out, Idc_out, Pdc_w, eff_pct, temp_primary_c, temp_secondary_c).

Local Display: 1.3-inch 128× 64 OLED displaying real-time V/I meters, power, efficiency bar-graph, and fault status.

Digital Control & SCPI Protocol: USB-C virtual serial interface accepting industry-standard SCPI commands (:VOLT 12.0, :CURR 2.5, :OUTP ON, :MEAS:EFF?).

5. Documentation Suite Roadmap

This project is organized into four specialized, comprehensive technical dossiers accompanied by Microsoft Word (.docx) deliverables:

smart_programmable_power_supply/ ├── README.md (.docx) # Master Hub, system specs, block diagrams & roadmap ├── ac-dc-flyback-stage-design.md (.docx) # Universal AC frontend, QR Flyback, transformer & SR ├── mcu-buck-boost-post-regulator.md (.docx) # 4-switch Buck-Boost, digital CV/CC loops & DAC control ├── efficiency-monitoring-and-telemetry.md (.docx) # Dual AC/DC metering, real-time η, MicroSD & SCPI └── system-schematics-and-pcb-design.md (.docx) # Complete system schematics, BOM, isolation & bring-up

1. AC-DC Flyback Stage Design & Magnetic Engineering

Universal AC input frontend (85\,V \dots 265\,V AC): Fuse, NTC thermistor, MOV surge clamp (14D471K), and 2-stage EMI filter.

Bridge rectification and bulk capacitor calculation (C_{bulk = 100\,µF / 450\,V) for 10\,ms holdup time.

Quasi-Resonant (QR) Flyback operation, valley-switching physics, and primary peak current formulations.

Complete Transformer Design (PQ26/20 core, 3C95 ferrite): Primary turns (N_p), secondary turns (N_s), auxiliary turns (N_{aux), air gap (l_g), and primary inductance (L_p).

Primary 800\,V Superjunction MOSFET selection and RCD clamp snubber calculation (R_{snub, C_{snub, D_{snub).

Secondary synchronous rectification (MP6908 + 40\,V MOSFET) and TL431/PC817 optocoupler feedback loop design for stable +24\,V DC bus.

Complete component-level ASCII schematic of the AC-DC Flyback stage.

2. MCU-Controlled Synchronous Buck-Boost Post-Regulator

4-switch non-inverting synchronous Buck-Boost power stage topology (24\,V \rightarrow 5\,V \dots 20\,V @ 3\,A).

Inductor sizing (L = 10\,µH) and input/output ceramic MLCC filter sizing for < 25\,mV ripple.

High-side and low-side gate driver architecture with bootstrap capacitors.

Digital closed-loop regulation: 12-bit DAC current-injection network into the feedback node for 5\,V \dots 20\,V programming (10\,mV step).

Fast analog Constant Current (CC) control loop using high-side current sense amplifier (INA240 / INA282) and precision reference clamp.

Complete component-level ASCII schematic of the Synchronous Buck-Boost post-regulator.

3. Real-Time Efficiency Monitoring, Data Logging & Telemetry

Primary AC side isolated power metering: Isolated \Delta\Sigma modulator (AMC1311/AMC1300) or dedicated metering IC (ADE7953) for true RMS AC voltage, current, active power, and power factor.

Secondary DC side digital power monitor: INA226 with 10\,m\Omega 0.1\% Kelvin shunt for millivolt/milliamp measurement.

Real-time digital efficiency algorithm (\eta = P_{DC / P_{AC) with low-pass filtering and rolling average buffers.

MicroSD SPI/SDIO data logging architecture: FAT32 filesystem, ring buffers, non-blocking FreeRTOS tasks, and CSV data schema.

Embedded firmware architecture, SCPI command parser over USB-CDC, and OLED GUI display state machine.

4. Complete System Schematics, Safety & PCB Design

Complete end-to-end system interconnection schematic uniting AC input, Flyback, Buck-Boost, MCU, and Telemetry.

Creepage and clearance rules: IEC 62368-1 / UL 60950 compliance (≥ 6.4\,mm creepage, ≥ 5.0\,mm clearance, Y1 safety caps).

High-voltage 4-layer PCB stackup and layout guidelines (Primary high di/dt loop, Secondary power ground vs. sensitive analog ground separation).

Comprehensive Bill of Materials (BOM) with manufacturer part numbers and footprints.

Bench bring-up sequence, calibration procedure, and Root Cause Analysis (RCA) troubleshooting matrix.

6. CLI Engineering Utility: tools/psu_calc.py

A dedicated Python CLI calculation utility is provided in **`tools/psu_calc.py`** to automate power stage sizing:

# 1. Size QR Flyback transformer (universal 85-265V AC in, 24V/3A out, 65kHz, PQ26/20 core): python3 tools/psu_calc.py --flyback --vin-min 85 --vin-max 265 --vout 24 --iout 3.0 --fsw 65000 # 2. Size Flyback primary RCD clamp snubber: python3 tools/psu_calc.py --snubber --vin-max 265 --vout 24 --turns-ratio 3.5 --leakage-ind 15e-6 --ipeak 1.8 --fsw 65000 # 3. Size 4-switch Buck-Boost inductor and capacitors (24V in, 5V-20V out, 3A, 250kHz): python3 tools/psu_calc.py --buckboost --vin 24 --vout-min 5.0 --vout-max 20.0 --iout 3.0 --fsw 250000 # 4. Calculate DAC feedback summing resistor network for 5V-20V programming: python3 tools/psu_calc.py --feedback --vout-min 5.0 --vout-max 20.0 --vdac-max 3.3 --vref 1.2