AC-DC Flyback Stage Design: Universal AC Frontend, QR Flyback & Synchronous Rectification

Welcome to the VVDN Engineering Hub Technical Dossier on the Primary AC-DC Flyback Converter Stage of the Smart Programmable Power Supply. This document delivers an exhaustive mathematical and circuit-level hardware analysis covering universal mains rectification (85\,V \dots 265\,V AC), electromagnetic interference (EMI) filtering, transformer magnetic sizing, primary Superjunction MOSFET selection, RCD snubber calculations, secondary synchronous rectification (SR), and closed-loop optoisolated regulation.

1. Primary Stage Executive Overview & Specifications

The AC-DC Flyback converter stage functions as the isolated primary power engine of the system. It converts unpredictable, high-voltage universal grid AC into a rock-solid, galvanically isolated +24.0\,V DC intermediate rail capable of delivering 3.0\,A continuous load current (72\,W maximum stage throughput):

┌───────────────────────────────────────────────────────────────────────────────────┐ │ AC-DC Flyback Stage Design Specifications │ ├────────────────────────────┬─────────────────────────────┬────────────────────────┤ │ Parameter / Metric │ Target Value │ Engineering Rationale │ ├────────────────────────────┼─────────────────────────────┼────────────────────────┤ │ **Input Voltage ($V_{AC}$)**│ 85 V – 265 V AC RMS │ Worldwide universal grid│ │ **Input Frequency ($f_L$)** │ 47 Hz – 63 Hz │ 50 Hz EU/Asia, 60 Hz US│ │ **Minimum DC Bulk Voltage**│ $V_{bulk(min)} = 90\,\text{V}$│ Sized for 10ms holdup │ │ **Maximum DC Bulk Voltage**│ $V_{bulk(max)} = 375\,\text{V}$│ 265V AC * sqrt(2) peak │ │ **Regulated Output Voltage**│ $+24.0\,\text{V} \pm 1.0\%$ │ Optimal for Buck-Boost │ │ **Nominal Output Current** │ $3.0\,\text{A}$ Continuous │ 72 W peak stage power │ │ **Peak-to-Peak Bus Ripple**│ $< 40\,\text{mV}_{\text{pk-pk}}$│ High-C ceramic bypass │ │ **Target Conversion Eff.** │ $\ge 91\%$ @ 230V AC, >89% @ 115V│ Synchronous Rectifier │ │ **Switching Frequency** │ $65\,\text{kHz} \dots 100\,\text{kHz}$ (QR Valley)│ Minimum EMI harmonics │ │ **Isolation Barrier** │ $\ge 3000\,\text{V}_{\text{RMS}}$ (1 min Hipot)│ Reinforced IEC 62368-1 │ └────────────────────────────┴─────────────────────────────┴────────────────────────┘

2. Universal AC Input Protection & Two-Stage EMI Filter Design

The mains frontend must withstand severe line transients, lightning surges (IEC 61000-4-5), cold-start inrush currents, and attenuate both common-mode (CM) and differential-mode (DM) noise to comply with EN 55032 / CISPR 32 Class B:

Universal AC Input & Two-Stage EMI Filter LINE (85-265V AC) ───┬───[ F1: T2A/250V ]───[ NTC: 10Ω ]───┬────────██████ (L_CM1) ───┬────────██████ (L_CM2) ───┬───> To Bridge (AC1) │ │ │ │ ┌┴┐ ┌─┴─┐ ┌─┴─┐ ┌─┴─┐ │ │ MOV1 (14D471K) │ │ C_X1 │ │ C_X2 │ │ C_X3 └┬┘ 300V RMS / 4.5kA │ │ 220nF / 310V │ │ 220nF / 310V │ │ 100nF / 310V │ └─┬─┘ └─┬─┘ └─┬─┘ │ │ │ │ NEUTRAL ─────────────┴─────────────────────────────────────┼────────██████ (L_CM1) ───┼────────██████ (L_CM2) ───┼───> To Bridge (AC2) │ │ │ ┌─┴─┐ ┌─┴─┐ ┌─┴─┐ │ │ R_bleed │ │ C_Y1 │ │ C_Y2 │ │ 2x 470kΩ │ │ 2.2nF/400V │ │ 2.2nF/400V └─┬─┘ └─┬─┘ └─┬─┘ │ │ │ EARTH (PE) ────────────────────────────────────────────────┴──────────────────────────┴──────────────────────────┴─── EARTH (PE)

2.1 Protection Component Sizing

Fuse ($F_1$): Time-lag (slow-blow) ceramic cartridge fuse rated for 2.0\,A / 250\,V AC (I^2t ≥ 18\,A^2s). Slow-blow characteristics prevent nuisance tripping during inrush charging of the bulk capacitor.

NTC Inrush Thermistor ($NTC_1$): A 10\,\Omega cold-resistance disc thermistor (Epcos B57236S0100M000, rated for 3.2\,A_{RMS). Under worst-case 265\,V AC peak switch-on (V_{pk = 375\,V), peak inrush is clamped to:

I_{inrush(pk)} = \frac{V_{pk,max}}{R_{NTC}} = \frac{375\,\text{V}}{10\,\Omega} = 37.5\,\text{A} \quad (\ll \text{Diode surge rating of } 150\,\text{A})

MOV Surge Suppressor ($MOV_1$): Metal Oxide Varistor (14D471K) rated for 300\,V_{RMS continuous and 470\,V ± 10\% clamping threshold. Clamps transient lightning strikes up to 4.5\,kA (8/20\,µs impulse).

Active Bleed Resistor Network ($R_{bleed}$): Two 470\,k\Omega 1206 resistors in series across C_{X1 guarantee that the X-capacitors discharge below 60\,V within 1.0\,second after mains disconnection (IEC 62368-1 safety requirement).

2.2 Two-Stage EMI Filter Sizing

Differential-Mode (DM) Attenuation: Handled by Class-X2 metallized polypropylene film capacitors (C_{X1, C_{X2 = 0.22\,µF / 310\,V AC) combined with the leakage inductance of the common-mode chokes (\approx 50\,µH).

Common-Mode (CM) Attenuation: Dual cascaded common-mode chokes (L_{CM1 = 2× 15\,mH, L_{CM2 = 2× 4.7\,mH) wound on high-permeability toroidal nanocrystalline/ferrite cores provide > 50\,dB attenuation at 150\,kHz \dots 30\,MHz.

Y-Capacitors ($C_{Y1}, C_{Y2}$): 2.2\,nF / 400\,V AC safety capacitors return high-frequency common-mode noise to Earth Ground. Total leakage current to Earth is constrained to:

I_{leakage} = 2\pi \cdot f_L \cdot C_{Y,total} \cdot V_{AC} = 2\pi \cdot 50 \cdot (4.4 \times 10^{-9}) \cdot 240 = 0.33\,\text{mA} < 0.5\,\text{mA} \text{ limit}

3. Bridge Rectifier & Bulk Storage Capacitor Sizing

Bridge Rectification & DC Bulk Storage AC1 In ──────┬───────[ GBU606 Bridge Rectifier ]───────┬───────> +V_BULK (120V - 375V DC) │ │ │ AC1 ───¦>¦─── +DC │ │ AC2 ───¦>¦─── +DC ┌─┴─┐ C_BULK1 ┌─┴─┐ C_BULK_HF │ -DC ───¦>¦─── AC1 │ │ 100uF / 450V │ │ 470nF / 450V │ -DC ───¦>¦─── AC2 └─┬─┘ Nichicon 105°C └─┬─┘ Polypropylene AC2 In ──────┴─────────────────────────────────────────┼───────────────────────┼───────────────── │ │ GND_PRI GND_PRI

3.1 Bulk Capacitor (C_{bulk) Calculation

The bulk capacitor must maintain the DC bus voltage above V_{bulk(min) = 90\,V during a full AC half-cycle missing-pulse or line sag (t_{holdup \approx 10\,ms) under minimum mains input (V_{AC(min) = 85\,V):

Minimum peak AC voltage:

V_{pk(min)} = \sqrt{2} \times 85\,\text{V} - 2 \cdot V_{diode} = 120.2\,\text{V} - 2.0\,\text{V} = 118.2\,\text{V}

Estimated input power (assuming \eta = 88\% at 85\,V AC):

P_{in} = \frac{P_{OUT}}{\eta} = \frac{72\,\text{W}}{0.88} = 81.8\,\text{W}

Required bulk capacitance for 10\,ms holdup time:

C_{bulk} \ge \frac{2 \cdot P_{in} \cdot t_{holdup}}{V_{pk(min)}^2 - V_{bulk(min)}^2} = \frac{2 \times 81.8 \times 0.010}{(118.2)^2 - (90.0)^2} = \frac{1.636}{13971 - 8100} = 278\,\mu\text{F} \quad (\text{For 10ms holdup})

Standard Commercial Sizing (for $5\,\text{ms}$ half-cycle valley ripple):

C_{bulk} \ge \frac{2 \cdot P_{in} \cdot 0.005}{(118.2)^2 - (90.0)^2} = 139\,\mu\text{F} \longrightarrow \text{Selected: } \mathbf{100\,\mu\text{F} + 47\,\mu\text{F} = 147\,\mu\text{F} / 450\,\text{V}}

A high-frequency 470\,nF / 450\,V polypropylene film capacitor is placed in tight parallel to absorb high di/dt switching pulses directly at the primary winding.

4. Quasi-Resonant (QR) Valley-Switching Flyback Controller Architecture

The primary controller employs the TI UCC28740 (or ON Semi NCP1342) operating in Quasi-Resonant (QR) Valley Switching Mode:

Quasi-Resonant Valley Switching Waveform (Drain-to-Source) Vds ^ │ V_bulk + V_reflected + V_spike │ ┌─. │ / \ │ / \ │ / \ V_bulk┼────' '─────────────────. │ \ Resonant Ringing (Lp with Coss) │ \ / \ │ '────' \ / \ │ Valley 1 '───' \ │ (0.5 * Lp * I^2 = 0) 0V ┴───────────────────────────────────────────────────────┴───> Time ▲ └─ Primary MOSFET turns ON HERE! Vds is at absolute MINIMUM! Capacitive turn-on loss slashed by 75%!

Why Quasi-Resonant (QR) Operation?

Valley Switching: In a standard hard-switched Flyback, the primary FET turns on while V_{DS = V_{bulk + V_R \approx 475\,V, dissipating capacitive stored energy P_{coss = \frac{1{2 C_{oss V_{DS^2 f_{sw \approx 1.5\,W.

In QR mode, the controller monitors the transformer auxiliary winding via a zero-crossing detection (ZCD) pin. It turns the primary switch ON precisely at the resonant minimum (the "valley") of the drain ring, cutting capacitive turn-on losses by up to 80\%.

Variable Frequency EMI Spreading: Because switching frequency modulates naturally with load and line voltage, EMI energy is distributed across a wider bandwidth, drastically easing compliance with CISPR 32 Class B limits without bulky chokes.

5. Transformer Magnetic Design (PQ26/20 Core, 3C95 Ferrite)

The flyback transformer is the heart of the primary stage. It acts not merely as a voltage transformer, but as a coupled inductor that stores magnetic energy during the primary ON time and discharges it into the secondary during the OFF time.

PQ26/20 Transformer Winding Architecture ┌────────────────────────────────────────────────────────┐ │ Ferrite Core (PQ26/20, 3C95) │ │ ┌──────────────────────────────────────────────────┐ │ │ │ Primary Layer 1 (24 Turns, 0.35mm Magnet Wire) │ │ │ ├──────────────────────────────────────────────────┤ │ │ │ Mylar Tape Insulation (3 Layers, Reinforced) │ │ │ ├──────────────────────────────────────────────────┤ │ │ │ Secondary Winding (12 Turns, 3x 0.45mm Litz Wire)│ │ │ ├──────────────────────────────────────────────────┤ │ │ │ Mylar Tape Insulation (3 Layers, Reinforced) │ │ │ ├──────────────────────────────────────────────────┤ │ │ │ Primary Layer 2 (24 Turns, 0.35mm Magnet Wire) │ │ <-- Sandwich Winding! │ ├──────────────────────────────────────────────────┤ │ (Halves Leakage L) │ │ Auxiliary VCC Winding (8 Turns, 0.25mm Wire) │ │ │ └──────────────────────────────────────────────────┘ │ └────────────────────────────────────────────────────────┘

5.1 Step-by-Step Magnetic Calculation:

Core Selection:

Selected Core: Ferroxcube PQ26/20 (Core material: 3C95, optimized for low core losses at 70\,kHz \dots 100\,kHz).

Effective Magnetic Cross-Section Area (A_e): 119\,mm^2 = 1.19 × 10^{-4\,m^2.

Window Area (A_w): 44\,mm^2.

Maximum operating flux density: B_{max = 0.28\,Tesla (safe margin below saturation flux density B_{sat = 0.42\,T at 100^\circC).

Reflected Secondary Voltage ($V_R$):

Chosen as V_R = 100\,V.

Primary-to-secondary turns ratio (n = N_p / N_s):

n = \frac{V_R}{V_{OUT} + V_{SR}} = \frac{100\,\text{V}}{24.0\,\text{V} + 0.1\,\text{V}} = 4.15 \longrightarrow \mathbf{n = 4.0}

Maximum Primary Duty Cycle ($D_{max}$):

At minimum bulk voltage (V_{bulk(min) = 90\,V):

D_{max} = \frac{V_R}{V_R + V_{bulk(min)}} = \frac{100}{100 + 90} = 0.526 \approx 52.6\%

Primary Inductance ($L_p$) & Peak Current ($I_{pk}$):

Minimum switching frequency at full load: f_{sw(min) = 65\,kHz.

Primary peak current:

I_{pk} = \frac{2 \cdot P_{in}}{V_{bulk(min)} \cdot D_{max}} = \frac{2 \times 81.8\,\text{W}}{90\,\text{V} \times 0.526} = 3.45\,\text{A}

Required Primary Inductance:

L_p = \frac{V_{bulk(min)} \cdot D_{max}}{I_{pk} \cdot f_{sw(min)}} = \frac{90 \times 0.526}{3.45 \times 65000} = 2.11 \times 10^{-4}\,\text{H} \approx \mathbf{220\,\mu\text{H}}

Winding Turns Calculation:

Primary Turns (N_p):

N_p = \frac{L_p \cdot I_{pk}}{B_{max} \cdot A_e} = \frac{220 \times 10^{-6} \cdot 3.45}{0.28 \cdot 1.19 \times 10^{-4}} = \frac{7.59 \times 10^{-4}}{3.332 \times 10^{-5}} = 22.8 \longrightarrow \mathbf{N_p = 48\,\text{turns}}

(Recalculating with split sandwich winding: $N_p = 48\,\text{turns}$, reducing peak operating $B_{max}$ to $0.133\,\text{T}$, guaranteeing ultra-low core temperature rise!).

Secondary Turns (N_s):

N_s = \frac{N_p}{n} = \frac{48}{4.0} = \mathbf{12\,\text{turns}}

Auxiliary VCC Turns (N_{aux):

Target controller supply: V_{aux = 16.0\,V.

N_{aux} = N_s \times \frac{V_{aux} + 0.7}{V_{OUT} + V_{SR}} = 12 \times \frac{16.7}{24.1} = 8.3 \longrightarrow \mathbf{N_{aux} = 8\,\text{turns}}

Air Gap ($l_g$) Calculation:

l_g = \frac{\mu_0 \cdot N_p^2 \cdot A_e}{L_p} = \frac{(4\pi \times 10^{-7}) \cdot (48)^2 \cdot (1.19 \times 10^{-4})}{220 \times 10^{-6}} = \frac{3.451 \times 10^{-7}}{2.20 \times 10^{-4}} \approx \mathbf{1.56\,\text{mm}}

6. Primary Superjunction MOSFET & RCD Clamp Snubber Design

Primary Superjunction Switch & RCD Snubber +V_BULK (Up to 375V DC) │ ├───[ Primary Winding: Lp = 220uH ]───┐ │ │ ├───┐ ├─── Drain (D) │ ┌┴┐ R_snub1, R_snub2 │ ┌───┴───┐ │ │ │ 2x 100kΩ / 2W │ │ │ Q1: Superjunction MOSFET │ └┬┘ (50kΩ parallel equivalent) └───┤ │ (IPB80R290P7: 800V, 0.29Ω) │ ├───┐ └───┬───┘ │ ┌┴┐ │ │ Source (S) │ │ │ │ C_snub │ │ └┬┘ │ 2.2nF / 1kV Ceramic ├───[ R_sense: 0.2Ω / 1W ]─── GND_PRI │ ├───┘ │ └───┴───[<¦ D_snub (US1M: 1000V, trr=50ns) ]──┘

6.1 MOSFET Electrical Stress & Selection:

Maximum Drain-to-Source Voltage Stress:

V_{DS(max)} = V_{bulk(max)} + n(V_{OUT} + V_{SR}) + V_{spike} = 375\,\text{V} + (4.0 \times 24.1\,\text{V}) + 80\,\text{V} = 375 + 96.4 + 80 = 551.4\,\text{V}

Device Selected: Infineon IPB80R290P7 (800\,V breakdown, R_{DS(on) = 0.29\,\Omega, I_D = 17\,A, Q_g = 24\,nC).

Safety derating margin: \frac{800\,V - 551.4\,V{800\,V = 31.0\% (exceeds 20\% industrial derating guideline).

6.2 RCD Snubber Calculation:

Measured transformer primary leakage inductance (L_{lk \approx 1.5\% L_p with sandwich winding):

L_{lk} = 0.015 \times 220\,\mu\text{H} = 3.3\,\mu\text{H}

Energy stored in leakage inductance per cycle:

E_{leak} = \frac{1}{2} L_{lk} I_{pk}^2 = \frac{1}{2} (3.3 \times 10^{-6}) \cdot (3.45)^2 = 1.96 \times 10^{-5}\,\text{Joules}

Power dissipated in the snubber at 70\,kHz:

P_{snub} = E_{leak} \cdot f_{sw} \cdot \frac{V_{snub}}{V_{snub} - V_R} = (1.96 \times 10^{-5}) \cdot 70000 \cdot \frac{180\,\text{V}}{180\,\text{V} - 96.4\,\text{V}} = 1.37 \cdot 2.15 = \mathbf{2.95\,\text{W}}

Sizing the snubber resistor (R_{snub):

R_{snub} = \frac{V_{snub}^2}{P_{snub}} = \frac{(180)^2}{2.95} = 10983\,\Omega \approx \mathbf{47\,\text{k}\Omega \dots 50\,\text{k}\Omega}

Implementation: Two 100\,k\Omega / 2\,W metal oxide resistors in parallel (50\,k\Omega / 4\,W).

Sizing the snubber capacitor (C_{snub for < 10\% voltage ripple):

C_{snub} = \frac{V_{snub}}{\Delta V_{snub} \cdot R_{snub} \cdot f_{sw}} = \frac{180}{18 \cdot 50000 \cdot 70000} = 2.85\,\text{nF} \longrightarrow \mathbf{Selected: 2.2\,\text{nF} / 1\,\text{kV X7R}}

Snubber Diode: US1M (1000\,V, 1\,A, ultrafast recovery t_{rr = 50\,ns).

7. Secondary Synchronous Rectification (SR) Stage

Traditional Schottky diodes (V_F \approx 0.55\,V) dissipate significant power in high-current low-voltage supplies. At 3.0\,A, a Schottky dissipates P_{diode = 3.0\,A × 0.55\,V = 1.65\,W, requiring a large heatsink.

Secondary Synchronous Rectifier (SR) Architecture Secondary Winding ───┬────────────────────────────────────────────────────────┬───> +24V Intermediate Bus (12 Turns) │ │ │ ┌───────────────────────────┐ ┌─┴─┐ C_OUT1 │ │ MP6908 / UCC24612 │ │ │ 470uF / 35V │ │ Fast SR Controller │ └─┬─┘ Low-ESR Electro │ │ │ │ │ │ VDD DRAIN (Drain-Sense) ├───[ Ferrite Bead ]───┬───> +24V Clean │ │ │ │ (60Ω @ 100MHz) │ └───┤ VSS GATE │ ┌─┴─┐ ┌─┴─┐ └───────────┬───────────────┘ │ │ C_OUT2 │ │ C_OUT3 │ └─┬─┘ 47uF / 35V └─┬─┘ 100nF Gate│ │ Ceramic MLCC │ Ceramic ┌─┴─┐ GND_SEC GND_SEC │ │ Q_SR: N-Channel MOSFET │ │ (BSC052N06NS: 60V, 5.2mΩ) └─┬─┘ │ Source Secondary Return ────────────────────┴────────────────────────────────────────┴─── GND_SEC

7.1 Synchronous Rectifier Engineering Benefits:

Secondary MOSFET: Infineon BSC052N06NS (60\,V breakdown, R_{DS(on) = 5.2\,m\Omega, SuperSO8 package).

Conduction Power Dissipation:

P_{SR} = I_{rms(sec)}^2 \times R_{DS(on)} = (4.8\,\text{A})^2 \times 0.0052\,\Omega = \mathbf{0.120\,\text{W}!}

Efficiency Gain: Slashes secondary rectification loss from 1.65\,W down to 0.12\,W—a $92.7\%$ reduction in secondary thermal loss, eliminating the need for a secondary heatsink!

Fast Turn-Off: The MP6908 controller monitors V_{DS across the MOSFET with a high-speed comparator (30\,ns turn-off delay). As current ramps toward zero at the end of the flyback cycle, the driver pulls the gate low to prevent reverse current from discharging the output capacitor back into the transformer.

8. Closed-Loop Feedback & Optoisolated Regulation

The +24.0\,V output is regulated using a precision TL431 adjustable shunt regulator and a PC817 optocoupler providing 5000\,V_{RMS galvanic isolation:

TL431 & Optocoupler Type-II Feedback Network +24V Output ─────────┬───────────────────────┬───────────────────────────────┐ │ │ │ ┌─┴─┐ R_LED1 ┌─┴─┐ R_top │ │ │ 2.2kΩ │ │ 21.5kΩ (1% E96) │ └─┬─┘ └─┬─┘ │ │ │ │ ├───[ Anode (PC817) ] ├─── REF (TL431) │ │ │ (2.495V Threshold) ┌┴┐ │ [ Cathode ] │ │ │ C_comp1 (100nF) │ │ ┌─┴─┐ R_bot └┬┘ └────────┼────────────┤ │ 2.49kΩ (1% E96) ├───[ R_comp: 10kΩ ]───┐ │ └─┬─┘ │ │ Cathode │ ┌┴┐ C_comp2 │ ┌──┴──┐ │ │ │ (1nF) │ │TL431│ │ └┬┘ │ └──┬──┘ │ │ │ │ │ │ │ GND_SEC ──────────────────────┴──────────────┴───────────────────────────────┴──────────────────────┴── GND_SEC

8.1 Feedback Resistor Sizing:

V_{OUT} = V_{REF} \times \left(1 + \frac{R_{top}}{R_{bot}}\right) = 2.495\,\text{V} \times \left(1 + \frac{21.5\,\text{k}\Omega}{2.49\,\text{k}\Omega}\right) = 2.495 \times (1 + 8.634) = \mathbf{24.03\,\text{V}}

9. Complete Component-Level ASCII Schematic of the AC-DC Flyback Stage