Real-Time Efficiency Monitoring, Data Logging & SCPI Telemetry Subsystem

Welcome to the VVDN Engineering Hub Technical Dossier on the Efficiency Monitoring, MicroSD Data Logging, and Digital Telemetry Subsystem for the Smart Programmable Power Supply. This document delivers an exhaustive architectural and implementation analysis covering dual-domain AC/DC power metering, real-time efficiency computation (\eta = P_{DC / P_{AC), non-blocking FAT32 circular data logging, and industry-standard SCPI communication protocols.

1. Subsystem Executive Overview & Specifications

A core innovation of the Smart Programmable Power Supply is its ability to continuously measure true grid AC input power and DC load output power simultaneously across a reinforced isolation boundary, computing real-time conversion efficiency at 10\,Hz rates and streaming data to a MicroSD logger, local OLED display, and USB-C SCPI interface:

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Dual-Domain Metering & Telemetry Specifications │ ├────────────────────────────┬─────────────────────────────┬────────────────────────┤ │ Parameter / Specification │ Value / Unit │ Implementation Detail │ ├────────────────────────────┼─────────────────────────────┼────────────────────────┤ │ **AC Voltage Sensing** │ $85\,\text{V} \dots 265\,\text{V}$ AC RMS│ Isolated $\Delta\Sigma$ AMC1311│ │ **AC Current Sensing** │ $0.05\,\text{A} \dots 2.0\,\text{A}$ RMS │ High-accuracy CT / Shunt│ │ **AC Power Measurement** │ Real Active Power ($P_{AC}$ in W)│ Instantaneous $v(t) \cdot i(t)$│ │ **AC Power Factor ($\cos\phi$)│ $0.00 \dots 1.00$ True PF │ Displacement & harmonic│ │ **DC Voltage Sensing** │ $0.0\,\text{V} \dots 24.0\,\text{V}$ DC │ INA226 16-bit ADC │ │ **DC Current Sensing** │ $0.00\,\text{A} \dots 3.50\,\text{A}$ DC│ $10\,\text{m}\Omega$ Kelvin Shunt│ │ **DC Power Measurement** │ $P_{DC} = V_{OUT} \times I_{OUT}$│ Hardware INA226 Mult. │ │ **Real-Time Efficiency (η)**│ $0.0\% \dots 100.0\%$ ($\pm 0.5\%$)│ Computed at $10\,\text{Hz}$ rate│ │ **Data Logging Storage** │ MicroSD Card (FAT32) │ Timestamped CSV schema │ │ **Logging Sample Interval**│ $100\,\text{ms} \dots 10\,\text{s}$ Configurable│ Non-blocking FreeRTOS │ │ **Local Display** │ 1.3" $128\times 64$ Monochrome OLED│ I2C SSD1306/SH1106 │ │ **Remote Interface** │ USB-C Virtual COM Port │ Full SCPI Command Set │ └────────────────────────────┴─────────────────────────────┴────────────────────────┘

2. Dual-Domain Power Metering Architecture

================================ PRIMARY HIGH-VOLTAGE AC DOMAIN ================================ 85-265V AC Line │ ├───[ 4x 1MΩ Resistor Divider ]───┐ │ │ │ Current Transformer (CT) ├─── VINP (AMC1311 Isolated Amp) └───[ 1000:1 / Talema AC1005 ]───┐│ │├─── VINN ││ ┌─┴┴──────────────────────────────┐ │ TI AMC1311 Isolated Modulator │ │ Reinforced Isolation (5kV RMS) │ └─┬───────────────────────────────┘ │ Digital Differential Bitstream ================================ SAFETY ISOLATION BARRIER (>= 6.4mm) =========================== │ ┌─┴───────────────────────────────┐ │ Sinc3 Decimation Filter │ │ (Hardware peripheral inside │ │ STM32G474 Microcontroller) │ └─┬───────────────────────────────┘ │ True V_AC_RMS, I_AC_RMS, P_AC (Watts) │ ================================ SECONDARY LOW-VOLTAGE DC DOMAIN =============================== +V_OUT (5V - 20V) ───────────────────┬────────────────────────────────────────────────────────┐ │ │ ┌─┴─┐ R_shunt ┌─┴─┐ Load │ │ 10mΩ / 0.1% Kelvin │ │ Terminal └─┬─┘ └─┬─┘ ├─── IN+ (INA226) │ │ GND_SEC └─── IN- (INA226) ──── VBUS Sense │ ┌─┴───────────────────────────────┐ │ TI INA226 16-Bit Power Monitor │ │ 10µV Offset / 0.1% Gain Error │ └─┬───────────────────────────────┘ │ I2C Bus (400 kHz) ▼ ┌──────────────────────────────────┐ │ STM32G474 / ESP32-S3 Central MCU │ │ - Real-Time Efficiency Engine │ │ - CV/CC State Machine & DAC │ │ - MicroSD FAT32 Data Logger │ └──────────────────────────────────┘

3. AC Input Power Measurement & Calculations

Real active AC power (P_{AC) cannot be calculated by simply multiplying RMS voltage by RMS current (V_{RMS × I_{RMS), because non-linear rectifiers draw pulsed, distorted currents with a power factor (PF) significantly below unity (PF \approx 0.55 \dots 0.65 without active PFC).

3.1 Mathematical Active Power Formulation:

True active power is the average of instantaneous voltage and current products sampled at high frequency (f_s ≥ 10\,kHz):

V_{RMS} = \sqrt{\frac{1}{N} \sum_{k=0}^{N-1} v_{AC}^2[k]}

I_{RMS} = \sqrt{\frac{1}{N} \sum_{k=0}^{N-1} i_{AC}^2[k]}

P_{AC(real)} = \frac{1}{N} \sum_{k=0}^{N-1} v_{AC}[k] \cdot i_{AC}[k]

\text{Apparent Power } S = V_{RMS} \times I_{RMS} \quad [\text{VA}]

\text{Power Factor } \text{PF} = \frac{P_{AC(real)}}{S} = \frac{P_{AC}}{V_{RMS} \times I_{RMS}}

3.2 Primary Sensing Hardware Design:

Isolated Voltage Sensing: An ultra-high-impedance resistor chain (4× 1.0\,M\Omega 1206 resistors in series with a 2.0\,k\Omega precision 0.1\% shunt) scales 375\,V peak down to 187.5\,mV, matching the linear ± 2.0\,V input range of the TI AMC1311 reinforced isolation amplifier.

Current Sensing: A 1000:1 toroidal current transformer (Talema AC1005) placed in series with the AC Line conductor steps down the 2\,A mains current to 2\,mA. A 100\,\Omega burden resistor generates a clean 200\,mV_{RMS secondary signal, routed into the secondary ADC with zero DC offset.

4. DC Output Power Measurement via INA226

The secondary load voltage and current are monitored using the TI INA226, an industry-standard 16-bit I2C power monitoring IC:

INA226 Kelvin Current Shunt Interface From Inductor L ───┬────────────────────────────────────────────────────────┬───> +V_OUT Terminal │ │ ┌─┴─┐ R_shunt (10mΩ, 0.1%, 3W, 15ppm/°C) │ │ │ Bourns CSS2H-2512R-L010F │ └─┬─┘ │ │ Kelvin Force 1 Kelvin Force 2 │ ├─── Sense+ Sense- ────┤ │ │ ┌─┴────────────────────────────────────────────────────────┴─┐ │ INA226 (16-Bit I2C Power Monitor) │ │ │ │ IN+ IN- VBUS SDA SCL │ └─┬─────────────┬─────────────┬────────────┬───────────┬─────┘ │ │ │ │ │ └─────────────┴─────────────┘ └─────┬─────┘ I2C to MCU

4.1 Calibration & Resolution Register Configuration:

Shunt Resistor: R_{shunt = 10.0\,m\Omega = 0.010\,\Omega.

Maximum Expected Current: I_{max = 3.50\,A.

Current LSB Calculation:

\text{Current\_LSB} = \frac{I_{max}}{2^{15}} = \frac{3.50}{32768} = 106.8\,\mu\text{A/bit} \longrightarrow \text{Selected: } \mathbf{100\,\mu\text{A/LSB} = 0.1\,\text{mA/LSB}}

Calibration Register Value ($CAL$):

CAL = \text{trunc}\left(\frac{0.00512}{\text{Current\_LSB} \times R_{shunt}}\right) = \text{trunc}\left(\frac{0.00512}{0.000100 \times 0.010}\right) = \mathbf{5120} \quad (\text{0x1400})

Power LSB:

\text{Power\_LSB} = 25 \times \text{Current\_LSB} = 25 \times 0.000100\,\text{A} = \mathbf{2.5\,\text{mW/LSB}}

5. Real-Time Efficiency Engine (\eta) & Digital Filtering

At a steady 10\,Hz rate (100\,ms timer interrupt), the microcontroller fetches the latest P_{AC and P_{DC values to calculate efficiency:

\eta[n] = \frac{P_{DC}[n]}{P_{AC}[n]} \times 100\%

Efficiency Engine Signal Processing Chain Raw P_AC (from AC Metering) ───┐ ▼ [ Sanity Check ] ──> If P_AC < 0.5W: Clamp η = 0.0% (Standby State) │ Raw P_DC (from INA226) ────────┤ ▼ [ Division Unit ] ──> η_raw = (P_DC / P_AC) * 100% │ ▼ [ Low-Pass EMA Filter ] ──> η_filt[n] = 0.2 * η_raw + 0.8 * η_filt[n-1] │ ├───> 1.3" OLED Display (Efficiency Gauge) ├───> MicroSD Data Logger Task (CSV Record) └───> SCPI Telemetry Parser (:MEAS:EFF?)

Edge-Case Handling:

Zero-Load / Standby Mode: When the output is enabled but no load is connected (I_{OUT = 0\,A), the primary stage draws standby quiescent power (P_{AC \approx 180\,mW). Because P_{DC = 0\,W, the efficiency is mathematically 0.0\%. The algorithm clamps efficiency to 0.0\% to avoid spurious division or noisy fluctuating readings.

Exponential Moving Average (EMA): Mitigates visual jitter on the OLED display caused by line-cycle ripple:

\eta_{filt}[n] = \alpha \cdot \eta_{raw}[n] + (1 - \alpha) \cdot \eta_{filt}[n-1] \quad (\alpha = 0.20)

6. MicroSD Data Logging Engine & FAT32 File Format

Data logging is implemented as an independent, low-priority FreeRTOS worker task. A lock-free circular ring buffer decouples the high-speed metering engine from flash memory write latencies:

Non-Blocking MicroSD Data Logging Architecture 10Hz Metering ISR / Task │ ├─── Creates Telemetry Record struct │ { timestamp, Vac, Iac, Pac, PF, Vdc, Idc, Pdc, eff, T_pri, T_sec } ▼ ┌────────────────────────────────────────────────────────┐ │ Lock-Free Circular Ring Buffer (RAM: 64 Records) │ │ [Rec 0] [Rec 1] [Rec 2] ... [Rec 63] │ └──────────────────────────┬─────────────────────────────┘ │ FreeRTOS Queue Notification ▼ ┌────────────────────────────────────────────────────────┐ │ MicroSD Logging Task (Low Priority FreeRTOS Task) │ │ - Batches 10 records into a 512-byte sector buffer │ │ - Writes via FatFs `f_write()` in single sector bursts │ │ - Flushes to physical Flash via `f_sync()` every 5 sec │ └──────────────────────────┬─────────────────────────────┘ │ SPI / SDIO Bus (25 MHz) ▼ ┌────────────────────────────────────────────────────────┐ │ MicroSD Flash Memory Card (FAT32 File System) │ │ File: `/LOGS/SPPS_20260929_001.CSV` │ └────────────────────────────────────────────────────────┘

6.1 CSV File Schema Specification:

Every session creates a new file numbered sequentially (SPPS_YYYYMMDD_NNN.CSV):

timestamp_ms,vac_rms,iac_rms,pac_w,pf,vdc_out,idc_out,pdc_w,eff_pct,temp_pri_c,temp_sec_c,mode 1000,230.4,0.125,18.42,0.64,12.012,1.250,15.015,81.51,42.3,38.7,CV 1100,230.3,0.126,18.45,0.64,12.011,1.251,15.026,81.44,42.4,38.7,CV 1200,230.5,0.125,18.40,0.64,12.012,1.250,15.015,81.60,42.5,38.8,CV

7. SCPI Command Parser & Telemetry Protocol

The module supports the IEEE 488.2 Standard Commands for Programmable Instruments (SCPI) protocol via USB-CDC (Virtual COM Port, 115200\,baud, 8N1):

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Supported SCPI Command Dictionary │ ├───────────────────────┬───────────────────────────┬───────────────────────────────┤ │ SCPI Command │ Parameter / Type │ Description / Response │ ├───────────────────────┼───────────────────────────┼───────────────────────────────┤ │ `*IDN?` │ None │ `VVDN,SPPS-60W,SN2026-09,V1.0`│ │ `*RST` │ None │ Resets supply to 5.0V / 0.5A │ │ `:VOLT <value>` │ Float ($5.0 \dots 20.0$) │ Sets output voltage setpoint │ │ `:VOLT?` │ None │ Queries voltage setpoint │ │ `:CURR <value>` │ Float ($0.1 \dots 3.0$) │ Sets current limit threshold │ │ `:CURR?` │ None │ Queries current limit │ │ `:OUTP <ON¦OFF>` │ Boolean (`1` or `0`) │ Enables/disables power stage │ │ `:OUTP?` │ None │ Queries output state (1 or 0) │ │ `:MEAS:VOLT?` │ None │ Measures DC output voltage │ │ `:MEAS:CURR?` │ None │ Measures DC output current │ │ `:MEAS:POW?` │ None │ Measures DC output power (W) │ │ `:MEAS:AC:VOLT?` │ None │ Measures RMS AC mains voltage │ │ `:MEAS:AC:POW?` │ None │ Measures real active AC power │ │ `:MEAS:EFF?` │ None │ Queries real-time efficiency │ │ `:LOG:START` │ None │ Starts MicroSD CSV data log │ │ `:LOG:STOP` │ None │ Stops logging and closes file │ │ `:SYST:ERR?` │ None │ `0,"No error"` or error code │ └───────────────────────┴───────────────────────────┴───────────────────────────────┘

8. Embedded C Firmware Architecture: Core Telemetry Loop

Below is the production-grade C implementation of the 10Hz dual-domain power metering and efficiency logging task:

/** * @file spps_telemetry.c * @brief Real-Time Dual-Domain Power & Efficiency Metering Task * @company VVDN Technologies */ #include <stdint.h> #include <stdbool.h> #include <stdio.h> #include "spps_telemetry.h" #include "ina226.h" #include "ac_metering.h" #include "fatfs.h" #define EMA_ALPHA 0.20f /* Filter smoothing coefficient */ #define MIN_STANDBY_POWER 0.50f /* Power threshold below which eff = 0 */ typedef struct { uint32_t timestamp_ms; float vac_rms; float iac_rms; float pac_watts; float power_factor; float vdc_out; float idc_out; float pdc_watts; float eff_percent; float temp_primary; float temp_secondary; char mode[4]; } TelemetryData_t; static TelemetryData_t current_data; static float filtered_efficiency = 0.0f; static FIL log_file; static bool logging_active = false; void SPPS_Telemetry_Task(void *pvParameters) { TickType_t xLastWakeTime = xTaskGetTickCount(); const TickType_t xFrequency = pdMS_TO_TICKS(100); /* 100ms = 10Hz */ for (;;) { vTaskDelayUntil(&xLastWakeTime, xFrequency); /* 1. Acquire Primary AC Measurements */ AC_Metering_Read(&current_data.vac_rms, &current_data.iac_rms, &current_data.pac_watts, &current_data.power_factor); /* 2. Acquire Secondary DC Measurements via INA226 */ INA226_Read_VBUS(&current_data.vdc_out); INA226_Read_Current(&current_data.idc_out); current_data.pdc_watts = current_data.vdc_out * current_data.idc_out; /* 3. Compute Real-Time Conversion Efficiency */ if (current_data.pac_watts > MIN_STANDBY_POWER && current_data.pdc_watts > 0.05f) { float raw_eff = (current_data.pdc_watts / current_data.pac_watts) * 100.0f; if (raw_eff > 100.0f) raw_eff = 100.0f; /* Physical sanity clamp */ /* Apply Exponential Moving Average */ filtered_efficiency = (EMA_ALPHA * raw_eff) + ((1.0f - EMA_ALPHA) * filtered_efficiency); } else { filtered_efficiency = 0.0f; /* Zero-load or standby state */ } current_data.eff_percent = filtered_efficiency; /* 4. Write CSV Record to MicroSD if Logging is Active */ if (logging_active) { char csv_buffer[128]; int len = snprintf(csv_buffer, sizeof(csv_buffer), "%lu,%.1f,%.3f,%.2f,%.2f,%.3f,%.3f,%.3f,%.2f,%.1f,%.1f,%s\n", xTaskGetTickCount() * portTICK_PERIOD_MS, current_data.vac_rms, current_data.iac_rms, current_data.pac_watts, current_data.power_factor, current_data.vdc_out, current_data.idc_out, current_data.pdc_watts, current_data.eff_percent, current_data.temp_primary, current_data.temp_secondary, current_data.mode); UINT bytes_written; f_write(&log_file, csv_buffer, len, &bytes_written); } } }