System Schematics, Safety Compliance & PCB Design: Smart Programmable Power Supply

Welcome to the VVDN Engineering Hub Technical Dossier on System Schematics, Safety Compliance, PCB Layout, and Hardware Bring-Up for the Smart Programmable Power Supply. This document provides the complete end-to-end interconnection schematic, IEC 62368-1 safety isolation rules, 4-layer PCB layout guidelines, Bill of Materials (BOM), bring-up checklist, and bench troubleshooting matrix.

1. Complete End-to-End System Interconnect Schematic

Below is the master system interconnection schematic uniting the universal AC frontend, QR Flyback converter, +24.0\,V intermediate DC bus, 4-switch synchronous Buck-Boost post-regulator, dual-domain power metering, central MCU, and user interface:

==================================================================================================================================================== COMPLETE SYSTEM INTERCONNECT SCHEMATIC: SMART PROGRAMMABLE POWER SUPPLY (85-265V AC -> 5-20V / 3A DC) ==================================================================================================================================================== [ MAINS INPUT ] 85-265V AC ──[ FUSE 2A ]──[ NTC 10Ω ]──┬──[ 2-STAGE EMI FILTER ]──[ BRIDGE RECTIFIER ]──┬──> +V_BULK (120-375V DC) │ (C_X, L_CM1, L_CM2) (GBU606: 600V/6A) │ ┌┴┐ ┌─┴─┐ C_BULK (100uF/450V) │ │ MOV1 (14D471K) └─┬─┘ └┬┘ │ │ GND_PRI EARTH (PE) ────────────────────────────┴────────────────────────────────────────────────┴─── EARTH (PE) [ STAGE 1: ISOLATED QR FLYBACK CONVERTER ] +V_BULK ───[ Primary Winding (PQ26/20: 220uH) ]───┬─── Drain (IPB80R290P7: 800V Superjunction) │ │ [ RCD Snubber ] (50kΩ / 2.2nF) │ Gate ─── Driven by UCC28740 Controller │ Source ───[ R_sense: 0.2Ω ]─── GND_PRI ================================================================================================================================================== REINFORCED SAFETY ISOLATION BARRIER (>= 6.4mm Creepage / 3000V RMS Hipot / Y1 Safety Cap) ================================================================================================================================================== Secondary Winding (12T) ───[ MP6908 + BSC052N06NS Synchronous Rectifier ]───┬───> +24V Intermediate DC Bus (72W) │ ┌──┴──┐ │TL431│ ───> [ PC817 Optocoupler ] ───> (Feedback to UCC28740) └──┬──┘ │ GND_SEC [ STAGE 2: 4-SWITCH SYNCHRONOUS BUCK-BOOST POST-REGULATOR ] +24V Intermediate Bus ───[ Q_A: Buck HS ]───┬───[ Inductor: 10uH / 8.5A ]───┬───[ Q_D: Boost HS ]───┬───[ R_shunt: 10mΩ ]───> +V_OUT (5-20V/3A) │ │ │ [ Q_B: Buck LS ] [ Q_C: Boost LS ] ┌─┴─┐ C_OUT (4x 22uF MLCC + 100uF Poly) │ │ └─┬─┘ GND_SEC GND_SEC │ GND_SEC [ STAGE 3: DIGITAL CONTROL & DUAL-DOMAIN POWER METERING ] AC Input ──[ AMC1311 Isolated Modulator ]──┐ ▼ +V_OUT ──[ INA226 16-Bit I2C Monitor ]──┼───> [ STM32G474 / ESP32-S3 Central MCU ] │ - Dynamic Real-Time η = P_DC / P_AC (10Hz) DAC1 Out ──[ 11kΩ Summing Resistor ]───────┼───> - Controls V_OUT (5.0V - 20.0V in 10mV steps) DAC2 Out ──[ CC Analog Clamp Comparator ]──┘ - Controls I_LIMIT (0.1A - 3.0A in 10mA steps) │ ├─── [ 1.3" OLED Display: V, I, P, η, Temp ] ├─── [ MicroSD Logger: CSV Timestamped Log ] └─── [ USB-C Interface: SCPI Protocol & CLI] ====================================================================================================================================================

2. Safety Compliance & Reinforced Isolation Design (IEC 62368-1)

Because this equipment operates directly from hazardous mains AC (85\,V \dots 265\,V AC) and delivers a touchable, user-accessible low-voltage DC bench output (5\,V \dots 20\,V), strict adherence to IEC 62368-1 (Audio/video, information and communication technology equipment safety) is mandatory:

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Safety Isolation Spacing Rules (IEC 62368-1) │ ├────────────────────────────┬─────────────────────────────┬────────────────────────┤ │ Parameter / Barrier │ Minimum Requirement │ SPPS Design Value │ ├────────────────────────────┼─────────────────────────────┼────────────────────────┤ │ **Clearance (Air Spacing)**│ $\ge 4.0\,\text{mm}$ │ **$\ge 5.2\,\text{mm}$**│ │ **Creepage (Surface Track)│ $\ge 5.0\,\text{mm}$ (IIb) │ **$\ge 6.8\,\text{mm}$**│ │ **Isolation Slot in PCB** │ Recommended under barrier │ **$2.0\,\text{mm}$ Air Gap Slot**│ │ **Dielectric Withstand** │ $3000\,\text{V}_{\text{RMS}}$ (1 minute)│ Tested to $3750\,\text{V}_{\text{RMS}}$│ │ **Y-Capacitor Class** │ Class Y1 or Dual Y2 │ **Class Y1 (2.2nF/400VAC)**│ │ **Optocoupler Rating** │ V_IOTV $\ge 5000\,\text{V}_{\text{RMS}}$│ **PC817 / EL817 (5kV RMS)**│ │ **Transformer Wire** │ Triple Insulated Wire (TIW) │ **Furukawa TEX-E (Reinforced)**│ └────────────────────────────┴─────────────────────────────┴────────────────────────┘

PCB Isolation Slot Milling Across Primary-Secondary Boundary PRIMARY AC DOMAIN (HOT!) SECONDARY DC DOMAIN (SELV SAFE!) +V_BULK, Q1, Primary Winding +24V, Buck-Boost, MCU, User Terminals ┌───────────────────────────┐ ┌───────────────────────────┐ │ │ │ │ │ High Voltage Track │ 2.0mm Milled│ Low Voltage Track │ │ ─────────────────── │ Air Gap Slot│ ─────────────────── │ │ │ (Air: infinite│ │ │ Optocoupler Primary │ creepage!)│ Optocoupler Secondary │ │ ┌─────────┐ │ ========= │ ┌─────────┐ │ │ │ PC817 │ ───────┼── │ SLOT │ ┼─────── │ PC817 │ │ │ └─────────┘ │ ========= │ └─────────┘ │ │ │ │ │ │ Transformer Primary │ │ Transformer Secondary │ │ ┌─────────┐ │ ========= │ ┌─────────┐ │ │ │ PQ26/20 │ ───────┼── │ SLOT │ ┼─────── │ PQ26/20 │ │ │ └─────────┘ │ ========= │ └─────────┘ │ │ │ │ │ └───────────────────────────┘ └───────────────────────────┘ ◄────── Creepage Distance >= 8.0mm across milled slot ────────────────►

3. High-Voltage 4-Layer PCB Stackup & Layout Guidelines

The PCB is architected as a high-density 4-layer board ($1.6\,\text{mm}$ FR-4, $2\,\text{oz}$ copper on outer layers, $1\,\text{oz}$ copper on inner layers):

┌───────────────────────────────────────────────────────────────────────────────────┐ │ 4-Layer PCB Layer Stackup │ ├───────┬──────────────────────────┬────────┬───────────────────────────────────────┤ │ Layer │ Name │ Copper │ Primary Signal Allocation │ ├───────┼──────────────────────────┼────────┼───────────────────────────────────────┤ │ **L1**│ Top Layer (High Power) │ 2.0 oz │ Power switches, inductors, AC traces │ │ **L2**│ Inner Layer 1 (Ground) │ 1.0 oz │ Partitioned GND_PRI and GND_SEC │ │ **L3**│ Inner Layer 2 (Power) │ 1.0 oz │ +16V Aux, +3.3V Logic, +5V Aux planes │ │ **L4**│ Bottom Layer (Signals) │ 2.0 oz │ Feedback, I2C/SPI buses, Kelvin traces│ └───────┴──────────────────────────┴────────┴───────────────────────────────────────┘

Essential PCB Layout Guidelines:

High $di/dt$ Primary Loop Area Minimization:

The loop formed by C_{bulk \rightarrow Transformer Primary \rightarrow Q_1 \rightarrow R_{sense \rightarrow C_{bulk ground return carries pulsed currents of 3.5\,A at 70\,kHz. This loop must be routed with minimum physical enclosure area to suppress magnetic dipole radiation.

Switch Node (SW1, SW2) Copper Island Sizing:

The switch nodes experience extreme dV/dt transitions (> 20\,V/ns). The copper polygon must be sized adequately to carry 3.5\,A continuous without overheating, but kept physically compact to avoid forming an RF radiating patch antenna.

Kelvin-Sense Routing for Shunt Resistors:

Both the primary current sense resistor (R_{sense = 0.2\,\Omega) and the secondary output shunt (R_{shunt = 10\,m\Omega) must use true 4-wire differential Kelvin connections routed directly from the inner component pads to the sensing ICs, running parallel on Layer 4 shielded by the Layer 2 ground plane.

Separation of Grounds:

GND_PRI: Primary rectified ground (never connect to Earth or Secondary!).

GND_SEC_PWR: Secondary high-current power ground carrying Buck-Boost switched currents.

GND_SEC_ANA: Sensitive analog ground for MCU ADC references and TL431. Connected to GND_SEC_PWR at exactly one single star point directly beneath the output capacitor bank.

4. Comprehensive Bill of Materials (BOM)

┌───────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ Bill of Materials (BOM): Key Active & Passive Parts │ ├──────┬─────┬───────────────────────────┬─────────────────────────┬──────────────┬────────────────────────────────────────┤ │ Item │ Qty │ Reference Designator │ Description │ Package │ Manufacturer & Part Number │ ├──────┼─────┼───────────────────────────┼─────────────────────────┼──────────────┼────────────────────────────────────────┤ │ 1 │ 1 │ F1 │ Fuse Time-Lag 2A / 250V │ 5x20mm Cart. │ Littelfuse 0218002.MXP │ │ 2 │ 1 │ NTC1 │ Inrush Thermistor 10Ω │ Disc 13mm │ TDK/Epcos B57236S0100M000 │ │ 3 │ 1 │ MOV1 │ Varistor 300V RMS 4.5kA │ Radial 14mm │ Bourns MOV-14D471K │ │ 4 │ 2 │ L_CM1, L_CM2 │ Common Mode Choke 15mH │ Toroidal │ Wurth Elektronik 744823215 │ │ 5 │ 1 │ BR1 │ Bridge Rectifier 600V 6A│ GBU-4 │ Diodes Inc. GBU606 │ │ 6 │ 1 │ C_BULK1 │ Electrolytic 100uF/450V │ Radial 18x25 │ Nichicon LGW2W101MELZ25 │ │ 7 │ 1 │ TX1 │ Transformer PQ26/20 220uH│ PQ26/20 │ Custom VVDN / Ferroxcube 3C95 │ │ 8 │ 1 │ Q1 │ Superjunction FET 800V │ D2PAK / TO263│ Infineon IPB80R290P7 │ │ 9 │ 1 │ U1 (Primary Controller) │ QR Flyback Controller │ SOIC-8 │ Texas Instruments UCC28740DR │ │ 10 │ 1 │ U2 (Synchronous Rectifier)│ Smart SR Controller │ SOIC-8 │ Monolithic Power MP6908GJ │ │ 11 │ 1 │ Q_SR │ Secondary MOSFET 60V │ SuperSO8 │ Infineon BSC052N06NS │ │ 12 │ 1 │ OPTO1 │ Optocoupler 5000V RMS │ DIP-4 / SMD │ Everlight EL817(S)(TA) │ │ 13 │ 1 │ U3 (Buck-Boost Controller)│ 4-Switch Sync BB IC │ HTSSOP-28 │ Texas Instruments LM5176PWPR │ │ 14 │ 4 │ Q_A, Q_B, Q_C, Q_D │ Power MOSFET 40V 3.4mΩ │ SuperSO8 │ Infineon BSC034N04LS │ │ 15 │ 1 │ L1 │ Flat-Wire Inductor 10uH │ 10x10mm SMD │ Wurth Elektronik 7443321000 │ │ 16 │ 1 │ R_shunt │ Kelvin Shunt 10mΩ 0.1% │ 2512 SMD │ Bourns CSS2H-2512R-L010F │ │ 17 │ 1 │ U4 (DC Power Monitor) │ 16-Bit I2C Power IC │ VSSOP-10 │ Texas Instruments INA226AIDGSR │ │ 18 │ 1 │ U5 (Isolated AC Modulator)│ Isolated Delta-Sigma IC │ SOIC-8 Wide │ Texas Instruments AMC1311DWVR │ │ 19 │ 1 │ U6 (Microcontroller) │ 32-Bit Cortex-M4F MCU │ LQFP-64 │ STMicroelectronics STM32G474RET6 │ │ 20 │ 1 │ DISP1 │ 1.3" OLED 128x64 I2C │ Module │ Waveshare 1.3inch OLED (SH1106) │ └───────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┘

5. Production Bring-Up Procedure & Testing Checklist

Follow this rigorous multi-step procedure when assembling and verifying the first prototype board:

┌───────────────────────────────────────────────────────────────────────────────────┐ │ Hardware Bring-Up Checklist │ ├──────┬──────────────────────┬─────────────────────────────────────────────────────┤ │ Step │ Verification Phase │ Test Description & Expected Result │ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **1**│ Cold Resistance & Hipot│ Verify impedance with DMM: │ │ │ │ - Across L-N: $> 1\,\text{M}\Omega$ (Bleed resistor) │ │ │ │ - Primary to Secondary: $> 100\,\text{M}\Omega$ │ │ │ │ - Apply $3000\,\text{V}_{\text{RMS}}$ Hipot tester: │ │ │ │ Leakage current must be $< 1.0\,\text{mA}$. │ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **2**│ Low-Voltage Primary │ Apply current-limited $+30\,\text{V}$ DC from bench │ │ │ Functional Check │ supply to $C_{bulk}$. Check UCC28740 startup pulses │ │ │ │ on gate using oscilloscope. Verify VCC charges. │ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **3**│ AC Mains Variac Ramp │ Connect through an Isolation Transformer + Variac. │ │ │ │ Slowly ramp AC voltage from $0\,\text{V} \rightarrow 85\,\text{V} \dots 230\,\text{V}$.│ │ │ │ Verify $+24.0\,\text{V}$ intermediate bus stabilizes │ │ │ │ with $< 50\,\text{mV}$ ripple. Check QR valley ring. │ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **4**│ Buck-Boost & DAC Test│ Flash initial MCU firmware via ST-LINK. Command DAC │ │ │ │ to output voltages: $5.0\,\text{V}, 12.0\,\text{V}, 20.0\,\text{V}$.│ │ │ │ Verify output with 6.5-digit DMM matches within 10mV.│ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **5**│ CC Mode Clamping │ Connect programmable electronic load in CC mode. │ │ │ │ Set current limit to $1.50\,\text{A}$. Increase load │ │ │ │ to $2.0\,\text{A}$. Verify voltage folds back and │ │ │ │ current remains strictly clamped at $1.50\,\text{A}$.│ ├──────┼──────────────────────┼─────────────────────────────────────────────────────┤ │ **6**│ Efficiency & Logger │ Run $60\,\text{W}$ full-load test ($20\,\text{V} / 3\,\text{A}$) for│ │ │ Thermal Soak │ 1 hour. Verify end-to-end efficiency $\ge 85\%$. │ │ │ │ Confirm MicroSD writes CSV records cleanly at 10Hz. │ │ │ │ FLIR thermal camera: Max heatsink $T \le 75^\circ\text{C}$.│ └──────┴──────────────────────┴─────────────────────────────────────────────────────┘

6. Root Cause Analysis (RCA) Troubleshooting Matrix

┌───────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ Bench Root Cause Analysis (RCA) Matrix │ ├────────────────────────────┬───────────────────────────────────────────┬──────────────────────────────────────────────────┤ │ Observed Symptom │ Probable Root Cause │ Diagnostic & Corrective Action │ ├────────────────────────────┼───────────────────────────────────────────┼──────────────────────────────────────────────────┤ │ **Flyback does not start** │ 1. Auxiliary VCC not reaching UVLO turn-on│ 1. Check startup resistor from bulk rail; verify │ │ (0V on +24V bus) │ threshold ($V_{CC(on)} \approx 14.5\text{V}$).│ $V_{CC}$ capacitor is not leaking. │ │ │ 2. Optocoupler phototransistor shorted. │ 2. Measure FB pin voltage; if grounded, controller│ │ │ 3. CS current sense resistor open/blown. │ inhibits switching. Check TL431 reference pin.│ ├────────────────────────────┼───────────────────────────────────────────┼──────────────────────────────────────────────────┤ │ **Excessive primary switch │ 1. RCD snubber diode ($D_{snub}$) too slow│ 1. Ensure $D_{snub}$ is an ultrafast diode ($t_{rr}│ │ voltage spike (>650V)** │ ($t_{rr} > 100\,\text{ns}$). │ \le 50\,\text{ns}$ e.g. US1M, ES1J). │ │ │ 2. Transformer leakage inductance ($L_{lk}$)│ 2. Verify sandwich primary winding construction; │ │ │ exceeds $3\%$ of $L_p$. │ lower $R_{snub}$ from $50\,\text{k}\Omega$ to │ │ │ │ $39\,\text{k}\Omega$ to dissipate more clamp. │ ├────────────────────────────┼───────────────────────────────────────────┼──────────────────────────────────────────────────┤ │ **Buck-Boost output hunts │ 1. Inadequate phase margin in error amp │ 1. Increase feedforward capacitor $C_{FF}$ across│ │ or oscillates at 5-10 kHz**│ compensation network. │ $R_{top}$ from $47\,\text{pF}$ to $100\,\text{pF}$.│ │ │ 2. Excessive ESR on output capacitor bank.│ 2. Add ceramic MLCCs in parallel with polymer cap│ │ │ 3. Digital DAC noise coupling into FB pin.│ 3. Add $100\,\text{pF}$ bypass directly at DAC pin│ ├────────────────────────────┼───────────────────────────────────────────┼──────────────────────────────────────────────────┤ │ **MicroSD card write error │ 1. Flash block erase latency ($>100\text{ms}$)│ 1. Use FreeRTOS queue ring buffer; never write │ │ or MCU watchdog reset** │ blocking main execution thread. │ to SD card directly inside the timer ISR! │ │ │ 2. 3.3V SD supply rail dips during write. │ 2. Place dedicated $47\,\mu\text{F}$ capacitor at│ │ │ │ MicroSD socket VDD pin to buffer burst load. │ ├────────────────────────────┼───────────────────────────────────────────┼──────────────────────────────────────────────────┤ │ **Efficiency displays 0.0% │ 1. Load current beneath minimum sensing │ 1. Verify INA226 calibration register ($CAL=5120$)│ │ or fluctuates wildly** │ threshold ($< 50\,\text{mA}$). │ is loaded properly over I2C. │ │ │ 2. Standby power clamp logic active. │ 2. Verify current transformer burden resistor has│ │ │ 3. Phase shift mismatch between V and I. │ zero DC offset and scales correctly in C code.│ └────────────────────────────┴───────────────────────────────────────────┴──────────────────────────────────────────────────┘