Dual Active Bridge (DAB) DC-DC Converter: Bidirectional Physics, Modulation & ZVS

Welcome to the VVDN Engineering Hub Technical Dossier on the Dual Active Bridge (DAB) Converter. This guide provides an advanced mathematical and hardware analysis of the DAB topology—the premier isolated bidirectional DC-DC architecture for Electric Vehicle (EV) onboard chargers, Vehicle-to-Grid (V2G) systems, battery energy storage systems (BESS), and solid-state transformers (SST).

1. Operating Principle & Bidirectional Architecture

The Dual Active Bridge (DAB) converter comprises two active full-bridge (H-bridge) switching stages coupled across a high-frequency isolation transformer with an energy-transfer power inductance (L):

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: BIDIRECTIONAL DUAL ACTIVE BRIDGE (DAB) CONVERTER (400V <-> 48V, 3.3kW EV/ESS) ======================================================================================================================== +================ PORT 1: HIGH-VOLTAGE DC BUS (+400V) ================+ +================ PORT 2: LOW-VOLTAGE DC BUS (+48V) =================+ +V1 (+400V DC) ─────────┬───────────────────────┬─────────────────────┐ +V2 (+48V DC) ──────────┬───────────────────────┬─────────────────────┐ │ │ │ │ │ │ │ │ ┌┴─────────────────┐ │ │ ┌┴┐ ┌┴─────────────────┐ │ │ ┌┴┐ │ C1_BULK │ │ │ │ │ C1_CER│ C2_BULK │ │ │ │ │ C2_CER │ 470µF / 450V │ │ │ └┬┘ 4x 1µF│ 1200µF / 63V │ │ │ └┬┘ 6x 10µF │ Poly Film │ │ │ │ 630V │ Low-ESR Poly │ │ │ │ 100V └┬─────────────────┘ │ │ │ └┬─────────────────┘ │ │ │ │ Drain(D) Drain(D) │ │ Drain(D) Drain(D) │ │ ┌────┴──────────┐ ┌────┴──────────┐ │ │ ┌────┴──────────┐ ┌────┴──────────┐ │ │ │ S1: HS SiC-FET│ │ S3: HS SiC-FET│ │ │ │ S5: HS N-MOS │ │ S7: HS N-MOS │ │ │ +----->│ C3M0065090D │+----->│ C3M0065090D │ │ │ +----->│ IPT012N08N5 │+----->│ IPT012N08N5 │ │ │ ¦ Gate │ (900V, 65mΩ) ¦¦ Gate │ (900V, 65mΩ) │ │ │ ¦ Gate │ (80V, 1.2mΩ) ¦¦ Gate │ (80V, 1.2mΩ) │ │ │ ¦ └────┬──────────┘¦ └────┬──────────┘ │ │ ¦ └────┬──────────┘¦ └────┬──────────┘ │ │ ¦ Source│ ¦ Source│ │ │ ¦ Source│ ¦ Source│ │ │ ¦ │ ¦ │ │ │ ¦ │ ¦ │ │ │ ¦ +=== NODE A ¦ +=== NODE B │ │ ¦ +=== NODE C ¦ +=== NODE D │ │ ¦ ¦ (Leg A) ¦ ¦ (Leg B) │ │ ¦ ¦ (Leg C) ¦ ¦ (Leg D) │ │ ¦ ¦ ¦ ¦ │ │ ¦ ¦ ¦ ¦ │ │ ¦ Drain(D) ¦ Drain(D) │ │ ¦ Drain(D) ¦ Drain(D) │ │ ¦ ┌────┴──────────┐¦ ┌────┴──────────┐ │ │ ¦ ┌────┴──────────┐¦ ┌────┴──────────┐ │ │ ¦ │ S2: LS SiC-FET│¦ │ S4: LS SiC-FET│ │ │ ¦ │ S6: LS N-MOS │¦ │ S8: LS N-MOS │ │ │ +---+----->│ C3M0065090D │+----->│ C3M0065090D │ │ │ +---+----->│ IPT012N08N5 │+----->│ IPT012N08N5 │ │ │ ¦ ¦ Gate └────┬──────────┘¦ Gate └────┬──────────┘ │ │ ¦ ¦ Gate └────┬──────────┘¦ Gate └────┬──────────┘ │ │ ¦ ¦ Source│ ¦ Source│ │ │ ¦ ¦ Source│ ¦ Source│ │ GND_1 ──┴───┼───────────┴───────────┼───────────┴─────────────────────┴── GND_1│GND_2 ──┴───┼───────────┴───────────┼───────────┴─────────────────────┴── GND_2 │ │ │ │ │ │ │ +---[ ISOLATED GATE DRIVERS: UCC21520 ]<---+ │ +---[ ISOLATED GATE DRIVERS: UCC21520 ]<---+ │ (Reinforced 5.7kV Isolation) │ │ (High-Current 4A Source / 6A Sink) │ │ │ │ │ │ +=== ENERGY TRANSFER INDUCTOR & TRANSFORMER========+ │ +=== DIGITAL CONTROL SYSTEM (TMS320F28379D) ========================+ │ ¦ ¦ │ ¦ - Phase-Shift Modulator: Generates 8 PWM outputs with resolution ¦ └───+---[ L_ext: 18µH / 20A ]───[ Primary N1: 16T ]────+ ¦ of 150 picoseconds (High-Resolution PWM: HRPWM) ¦ Wurth 7443641800 * (Dot at Node A) ¦ ¦ - Closed-loop digital phase angle shift: -90° <= φ <= +90° ¦ ¦ ¦ - Seamless bidirectional power flow reversal in < 1 millisecond ¦ ==============================================================│====================+==================================================================== ¦ ¦ [ Secondary N2: 2T ]───┴────────────────────┘ * (Dot at Node C)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +V1 (Port 1 Rail) | High-Voltage Battery / DC Bus | C_1 bank (+), S_1 Drain, S_3 Drain | Stiff 400V DC input/output rail | Polypropylene film capacitors handle high continuous RMS switching current (I_rms ≈ 10 A). |
| NODE_A / NODE_B | Primary Half-Bridge Legs A & B | Series Inductor L_ext, Transformer Primary N_1 | High-frequency AC excitation port (v_ac1) | 50% square wave at switching frequency f_s = 100 kHz; amplitude is ± 400 V. |
| TANK_AC_LOOP | Node A -> Inductor L_ext Pin 1 | Transformer N_1 Pin 1 -> Node B | High-frequency inductive power transfer branch | Power is transferred by driving current through L_ext; current is quasi-trapezoidal/sinusoidal. |
| NODE_C / NODE_D | Secondary Half-Bridge Legs C & D | Transformer Secondary N_2 | Low-voltage AC excitation port (v_ac2) | 50% square wave phase-shifted by angle φ relative to Port 1; amplitude is ± 48 V. |
| +V2 (Port 2 Rail) | Low-Voltage Battery / DC Bus | C_2 bank (+), S_5 Drain, S_7 Drain | Stiff 48V DC input/output rail | Very low ESR polymer capacitor bank absorbs high circulating RMS currents (I_rms ≈ 70 A). |
| GND_1 / GND_2 | Port 1 Ground / Port 2 Ground | Galvanic isolation barrier | Fully isolated system grounds | Galvanic isolation barrier rated for > 3000 V_RMS withstand; creepage >= 8.0 mm. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| S_1 ... S_4 | Primary SiC Power MOSFETs | Wolfspeed C3M0065090D | V_DS = 900 V, I_D = 36 A, R_DS(on) = 65 mΩ, Q_g = 30 nC | Silicon carbide (SiC) provides zero reverse-recovery and ultra-low switching loss at 100 kHz. |
| S_5 ... S_8 | Secondary LV N-MOSFETs | Infineon IPT012N08N5 | V_DS = 80 V, I_D = 300 A, R_DS(on) = 1.2 mΩ, Q_g = 178 nC | OptiMOS 5 in 8-pin PowerBlock package handles high secondary RMS current (I_rms ≈ 55 A). |
| L_ext | Energy Transfer Inductor | Würth Elektronik 7443641800 | L = 18 µH, I_sat = 28 A, I_rms = 22 A, DCR = 4.8 mΩ | Determines maximum bidirectional power transfer: P_max = (n V_1 V_2) / (8 f_s L_ext). |
| T_1 | Planar Isolation Transformer | Custom E64 Planar Core (3C95) | Turns: 16:2 (8:1), L_m = 1.2 mH, L_lk < 0.8 µH | Multi-layer heavy copper PCB planar windings achieve > 99\% transformer efficiency and low leakage. |
| C_1 Bank | Port 1 DC Link Film Caps | KEMET C4AEGBW5300A3FJ | 470 µF, 450 V_DC, Metallized Polypropylene Film | Handles continuous high-frequency triangular ripple current without dielectric breakdown. |
| C_2 Bank | Port 2 DC Link Polymer Caps | Panasonic 63SVPF220M | 6 * 220 µF, 63 V, OS-CON Polymer, ESR = 12 mΩ | Paralleled array yields < 2 mΩ net ESR to prevent low-voltage bus voltage ripple. |
| U_drv1-4 | Isolated Dual Gate Drivers | TI UCC21520DW | 5.7 kV_RMS reinforced isolation, 4 A / 6 A sink/source | High CMTI (> 100 V/ns) prevents false gate triggering during fast SiC switching edges. |
| U_ctrl | Central Digital Controller | TI TMS320F28379D | Dual-Core 200MHz C28x DSP, 150ps HRPWM resolution | Controls phase-shift angle φ in real-time based on bus current and voltage demand. |

1.1 Core Energy Transfer Mechanism:

Both primary and secondary bridges generate high-frequency AC square-wave voltages (v_ac1 and v_ac2) with 50\% duty cycle at frequency f_s (50 kHz ... 200 kHz).

Power transfer is not controlled by varying duty cycle or frequency; instead, it is governed by the Phase-Shift Angle (φ) between the primary AC square wave and the secondary AC square wave across the energy-transfer inductor (L):

Forward Power Flow (P_1 -> 2): Primary leads Secondary (φ > 0).

Reverse Power Flow (P_2 -> 1): Secondary leads Primary (φ < 0).

Zero Power Flow (P = 0): Bridges are in exact phase synchronization (φ = 0).

2. Mathematical Formulations & Phase-Shift Modulation

Single Phase Shift (SPS) Key Waveforms v_ac1 ^ +V1 │ ┌──────────────┐ ┌──────────────┐ └───┘ └──────────────┘ └───────> Time -V1 v_ac2 ^ +V2' (Reflected) │ ┌──────────────┐ ┌──────────────┐ └─────────┘ └──────────────┘ ──> Time │◄─φ─►│ Phase Shift Angle i_L ^ I_pk │ / \ / \ │ / \ / \ 0A ┼────────/─────\───────────────────────/─────\───────────> Time │ / \ / \ │ / \-I_pk / \-I_pk

2.1 Transferred Power Equation (Single Phase Shift - SPS):

Defining the normalized phase-shift ratio d = φ / π (where -1 <= d <= 1):

P = (V_1 * V_2') / (2π f_s L) * φ (1 - ¦φ¦ / π) = (V_1 * (n V_2)) / (2 f_s L) * d (1 - ¦d¦)

where:

V_1: DC voltage of Port 1

V_2' = n * V_2: DC voltage of Port 2 reflected to primary (n = N_1 / N_2)

L = L_ext + L_leakage: Total series energy-transfer inductance

f_s: Switching frequency (Hz)

2.2 Maximum Transferred Power:

The power parabolic curve peaks at a phase shift of 90° (φ = π/2, or d = 0.5):

P_max = (V_1 * V_2') / (8 f_s L)

Engineering Design Guideline: Normal operating power is sized at d_nom ≈ 0.2 ... 0.35 (φ ≈ 36° ... 63°) to reserve dynamic margin and maintain high efficiency.

3. Zero-Voltage Switching (ZVS) Soft-Switching Boundaries

The DAB topology achieves ultra-high efficiency (> 97.5\%) by operating with Zero-Voltage Switching (ZVS) on all 8 switches:

Before each MOSFET turns on, the circulating inductive current i_L(t) discharges the output capacitance (C_oss) of the incoming switch and charges C_oss of the outgoing switch.

When V_DS drops to zero, the anti-parallel body diode clamps the switch to zero volts, allowing lossless turn-on (P_sw(on) = 0).

ZVS Operating Range as a Function of Voltage Ratio (M) Phase Shift (d) ^ 0.5 ┼─────────────────────────────────────────────┐ │ FULL ZVS REGION │ │ (All 8 switches achieve │ │ Zero-Voltage Switching) │ │ .─────────. │ │ .──' '──. │ │ .──' '──. │ │ .──' '──. │ 0.0 ┴────'───────────────────────────────────'────┴───> Voltage Ratio M 0.0 1.0 2.0 (M = n*V2 / V1)

ZVS Condition Formulations:

Defining the voltage conversion ratio M = (n V_2) / V_1:

Primary Bridge ZVS Condition:

d >= (M - 1) / (2 M) (For M > 1)

Secondary Bridge ZVS Condition:

d >= (1 - M) / 2 (For M < 1)

When M = 1.0 (perfectly matched transformer ratio), ZVS is maintained down to virtually zero load!

When M !=q 1.0 at light loads, inductive energy is insufficient to completely discharge C_oss, causing loss of ZVS and increased switching dissipation.

4. Advanced Modulation Strategies

To extend the soft-switching ZVS boundary and eliminate reactive circulating currents during light-load operation, advanced multi-variable modulation techniques are employed:

┌───────────────────────────────────────────────────────────────────────────────────┐ │ DAB Modulation Strategies Comparison │ ├─────────────────────┬───────────────────┬─────────────────────────────────────────┤ │ Modulation Strategy │ Control Degrees │ Engineering Advantages │ ├─────────────────────┼───────────────────┼─────────────────────────────────────────┤ │ **Single Phase Shift│ 1 Degree of │ Simplest control implementation; │ │ (SPS)** │ Freedom (φ) │ high circulating current at light load. │ ├─────────────────────┼───────────────────┼─────────────────────────────────────────┤ │ **Extended Phase │ 2 Degrees │ Introduces inner duty cycle on primary │ │ Shift (EPS)** │ (φ, D_1) │ bridge; expands ZVS down to 20% load. │ ├─────────────────────┼───────────────────┼─────────────────────────────────────────┤ │ **Dual Phase Shift │ 2 Degrees │ Synchronized inner duty cycles │ │ (DPS)** │ (φ, D_1=D_2) │ (D_1 = D_2); slashes reactive power. │ ├─────────────────────┼───────────────────┼─────────────────────────────────────────┤ │ **Triple Phase Shift│ 3 Degrees │ Independent control of φ, D_1, D_2;│ │ (TPS)** │ (φ, D_1, D_2)│ Global minimum RMS current optimization.│ └─────────────────────┴───────────────────┴─────────────────────────────────────────┘

5. Industrial Design Example: 10 kW EV Fast-Charging DAB Stage

Port 1 (DC Bus): V_1 = 400 V DC

Port 2 (EV Battery): V_2 = 300 V ... 450 V DC (Nominal 400 V)

Target Power: P = 10.0 kW

Switching Frequency: f_s = 100 kHz

Transformer Ratio: n = N_1 / N_2 = 1.0

Nominal Phase Shift: d = 0.25 (φ = 45°)

Energy-Transfer Inductor Sizing (L):

L = (V_1 * (n V_2)) / (2 f_s * P) * d(1 - d) = (400 * 400) / (2 * 100000 * 10000) * 0.25(1 - 0.25) = 160000 / (2 * 10^9) * 0.1875 = 15.0 µH

Inductor Realization: High-frequency low-loss Sendust / Nanocrystalline distributed-gap toroidal inductor carrying 35 A_RMS AC current.