Isolated Flyback DC-DC Converter: Theory, CCM/DCM Modes & Snubber Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Flyback DC-DC Converter. This document delivers an exhaustive hardware engineering, magnetic analysis, and mathematical breakdown of the isolated flyback topology, covering coupled-inductor energy storage, Continuous Conduction Mode (CCM) vs. Discontinuous Conduction Mode (DCM), Quasi-Resonant (QR) valley switching, RCD clamp snubber design, and multi-output cross-regulation.

1. Operating Principle & Working Mechanics

The Flyback Converter is an isolated, buck-boost-derived switched-mode power supply topology. The core magnetic element is not an ideal transformer, but a coupled inductor with a gapped magnetic core designed specifically to store energy during the switch ON time and release it during the OFF time:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: ISOLATED QUASI-RESONANT FLYBACK CONVERTER (100V-375V DC -> +12V/3A SELV OUT) ======================================================================================================================== +VBULK (100V-375V DC) ─────────────────┬─────────────────────────────────────────────────────────────┐ │ │ │ ┌┴─────────────────┐ │ PRIMARY RCD CLAMP SNUBBER │ │ C_BULK │ │ +-----[ D_snub: US1M (1000V / 1A) ]<-----+ │ │ 100µF / 450V │ │ ¦ (Ultra-Fast Recovery: trr < 75ns) ¦ │ │ Aluminum Elec. │ │ ¦ ¦ │ └┬─────────────────┘ │ +-----[ R_snub: 47kΩ / 3W ]--------------+ │ │ │ ¦ [ C_snub: 2.2nF / 630V Poly ]------+ │ │ │ ¦ │ │ +----+=== TRANSFORMER PRIMARY (Np: 48T) │ │ ¦* (Dot at +VBULK) │ │ ¦ │ │ ¦ │ │ ¦ (Dot inverted) │ │ +===================================+ │ │ │ │ │ │ Drain (D) │ │ ┌┴──────────────┐│ │ │ Q1: N-MOSFET ││ │ │ IPB80R290P7 ││ │ +----->│ (800V, 290mΩ) ││ │ ¦ Gate └┬──────────────┘│ │ ¦ │ Source (S) │ │ +---[ PWM / QR CONTROLLER (e.g. UCC28740) ]---+ ¦ │ │ │ ¦ ¦ ¦ +---[ CS Filter ] │ ¦ GATE (Pin 6) -------[ R_gate: 10Ω ]---------+ ¦ │ R_cs: 1kΩ │ │ ¦ ¦ ┌┴┐ C_cs: 220pF│ │ ¦ CS (Pin 3) <--------------------------------------------------+ │ │ R_sense │ │ ¦ └┬┘ 0.25Ω / 2W 1%│ │ ¦ VCC (Pin 5) <----+ │ │ │ ¦ │ GND_PRI │ │ ¦ FB (Pin 1) <--+ │ │ │ ¦ │ │ │ │ ¦ GND (Pin 4) │ │ │ │ +--------+--------+-+---------------------------------------------------------------------+ │ │ │ │ │ GND_PRI │ └──[ D_aux: 1N4148 ]<--+ AUXILIARY WINDING (Naux: 7T) │ │ │* (Provides controller bias VCC = +14V) │ │ ┌┴┐ C_aux: 10µF/25V │ │ └┬┘ │ │ │ │ │ GND_PRI │ │ │ +---[ Collector (Pin 4) ] OPTOCOUPLER: PC817A │ [ Emitter (Pin 3) ] ---> GND_PRI │ GND_PRI ───────────────────────────────────────────────────────────────────────────────────────────── │ ┌┴┐ CY1: Safety Y-Capacitor (2.2nF / 400VAC Class Y1: Reinforced Isolation Across Barrier) └┬┘ │ ============================================= ISOLATION BARRIER (>= 6.4mm Creepage) =================== │ GND_SEC ───────────────────────────────────────────────────────────────────────────────────────────── │ │ │ +===================================+ │ ¦ │ ¦ TRANSFORMER SECONDARY (Ns: 6T) │ ¦ (Dot inverted relative to Np) │ ¦* │ +----[ D_sec: V30100P Schottky ]----+ │ ¦ (100V / 30A Trench) │ │ ¦ [ Anode to Ns, Cathode to Out] │ │ ¦ │ │ +---[ R_snub_sec: 4.7Ω / 1W ] │ │ ¦ │ │ │ ¦ [ C_snub_sec: 1nF / 100V ] │ │ ¦ │ │ │ ¦ GND_SEC │ │ ¦ │ │ ¦ +=== +VOUT (+12V/3A) │ ¦ │ │ ¦ ┌┴──────────────────┐ │ ¦ │ COUT_BULK │ │ ¦ ┌┴┐ 2x 470µF/25V ┌┴┐ COUT_CER │ ¦ │ │ Poly (ESR=10mΩ) │ │ 2x 22µF/25V │ ¦ └┬┘ └┬┘ X7R 1210 │ ¦ │ │ │ ¦ +VOUT │ +VOUT │ │ ¦ │ │ │ │ │ ¦ [ 1kΩ ] │ [ R_fb1: 38.3kΩ ] │ ¦ │ │ │ │ ¦ [ Anode: Pin 1 ] │ +---> REF (Pin 1) │ ¦ PC817 OPTOCOUPLER │ │ (V_ref=2.50V) │ ¦ [ Cathode: Pin 2 ] │ [ R_fb2: 10.0kΩ ] │ ¦ │ │ │ │ ¦ +----[ CATHODE ]----+ GND_SEC │ ¦ ¦ TL431 PRECISION¦ │ ¦ ¦ SHUNT REGULATOR¦ │ ¦ +----[ ANODE: GND ]─+ │ │ │ GND_SEC ─────────────────────────────────────────┴──────────────────────────────────┴─── GND_SEC (Return)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +VBULK | Input Bridge Rectifier (+) | C_bulk (+), Transformer Primary Pin 1, RCD Snubber | High-voltage rectified DC input rail | Trace clearance must satisfy IEC 62368-1 high-voltage spacing (>= 2.5 mm). |
| DRAIN_PRI | Transformer Primary Pin 2 | Q_1 Drain, Diode D_snub Cathode | Primary switching node (0 V ... 650 V) | Keep copper loop from Primary Pin 2 through D_snub and C_snub extremely short to suppress leakage ringing. |
| CS_PRI | Q_1 Source | Sense Resistor R_sense Top Pad, R_cs Filter | Primary peak current sensing | Low-inductance resistor layout; Kelvin sense trace to controller CS comparator pin. |
| VCC_AUX | Aux Winding N_aux Pin 2 | Diode D_aux Anode -> C_aux (+), Controller VCC | Controller primary bootstrap bias rail | Delivers steady +14 V DC to controller after initial startup resistor charges C_aux. |
| OPTO_COL | PC817 Optocoupler Pin 4 | Controller FB Pin | Isolated closed-loop feedback signal | Modulates controller internal current setpoint; optocoupler emitter connects to GND_PRI. |
| +VOUT (SELV) | Diode D_sec Cathode | C_out bank (+), Opto Pullup, Feedback R_fb1 | Regulated +12V SELV isolated DC output rail | Complies with Safety Extra Low Voltage limits (< 60 V DC touchable). |
| TL431_REF | Divider R_fb1/R_fb2 Node | TL431 Shunt Regulator Reference (Pin 1) | Output voltage error sense | Precision reference node (2.500 V); TL431 sinks cathode current to drive Opto LED. |
| GND_PRI / GND_SEC | Primary GND / Secondary GND | Safety Capacitor C_Y1 bridging barrier | Galvanically isolated ground planes | Minimum 6.4 mm creepage slot milled through PCB laminate beneath optocoupler and transformer. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| Q_1 | Primary High-Voltage FET | Infineon IPB80R290P7 | V_DS = 800 V, I_D = 17 A, R_DS(on) = 290 mΩ, Q_g = 23 nC | 800 V rating absorbs V_bulk,max + V_reflect + V_spike = 375 V + 96 V + 150 V = 621 V. |
| T_1 | Flyback Coupled Inductor | Custom PQ26/20 Core (3C95) | L_p = 220 µH, N_p:N_s:N_aux = 48:6:7, L_lk < 4.0 µH | Gapped core prevents magnetic saturation at I_pk = 2.4 A; triple-insulated wire (TIW) for secondary. |
| D_snub | RCD Snubber Diode | Diodes Inc. US1M | V_RRM = 1000 V, I_F = 1 A, t_rr < 75 ns, C_j = 15 pF | Fast recovery prevents reverse charge dumping into C_snub. |
| R_snub, C_snub | RCD Snubber Resistor/Cap | Vishay AC03 / Vishay MKP385 | R = 47 kΩ / 3 W, C = 2.2 nF / 630 V Film | Clamps leakage spike below 700 V; dissipates P_snub ≈ 1.8 W at full load. |
| D_sec | Secondary Output Rectifier | Vishay V30100P | V_RRM = 100 V, I_F = 30 A, V_F = 0.52 V | Trench MOS barrier Schottky handles peak inverse voltage V_PIV = V_o + V_bulk * (N_s/N_p) = 60 V. |
| C_out,bulk | Output Bulk Capacitor | Panasonic 25SVPF470M | 2 * 470 µF, 25 V, OS-CON Polymer, ESR = 10 mΩ | Absorbs large secondary discontinuous triangular current pulses (I_sec,rms ≈ 6.2 A). |
| U_1 (Opto) | Safety Optocoupler | Everlight EL817(B) | V_IOTV = 5000 V_RMS, CTR = 130\% ... 260\% | Connects isolated output error amplifier to primary PWM controller across safety barrier. |
| U_2 (Ref) | Precision Shunt Reference | TI TL431AQDBZR | V_ref = 2.495 V ± 0.5\%, I_k = 1 mA ... 100 mA | Closes voltage loop with high gain; includes Type-II compensation across Cathode and Ref. |
| C_Y1 | Safety Y1 Barrier Cap | Murata DE1E3KX222MA4BP01F | 2.2 nF, 400 V_AC, Class Y1 (Reinforced) | Provides low-impedance return path for common-mode displacement currents; minimizes EMI emissions. |

1.1 Conduction Cycle Breakdown:

Interval 1: Switch ON (0 < t <= D * T_s):

Primary switch Q1 turns ON. Input DC voltage V_IN is applied across primary winding N_p.

Due to the inverted dot polarity, secondary winding N_s generates a negative potential at the anode of D_sec.

Secondary diode D_sec is reverse-biased; no current flows to the secondary.

Energy is stored in the transformer's core air gap as magnetic flux:

E_stored = 0.5 * Lp * I_pk^2

Where:

E_stored: Magnetic energy stored in coupled inductor core per switching cycle (J)

Lp: Primary magnetizing inductance (H)

I_pk: Peak primary drain current reached at end of switch on-time (A)

Primary current ramps up linearly: di_p / dt = V_IN / L_p.

The output load current is supplied entirely by the output capacitor bank C_out.

Interval 2: Switch OFF (D * T_s < t <= T_s):

Q1 turns OFF. Primary current ceases.

By Faraday's and Lenz's laws, the collapsing magnetic field reverses the polarity of all windings.

Secondary winding potential jumps positive, forward-biasing D_sec.

The stored magnetic energy discharges into output capacitor C_out and the load:

di_s / dt = -(V_OUT + V_F) / L_s

Primary voltage rings up to V_IN + n(V_OUT + V_F) + V_spike, clamped by the RCD snubber.

2. Continuous (CCM) vs. Discontinuous (DCM) Conduction Modes

Flyback Magnetizing Current: CCM vs. DCM Waveforms CCM (Continuous Conduction) DCM (Discontinuous Conduction) Ip,Is ^ Ip,Is ^ │ Secondary Current (Is/n) │ Secondary Current (Is/n) │ ┌──. │ ┌─. │ / \ │ / \ │ / \ │ / \ │ / \ │ / \ I_ped ┼───. '──. I_ped │ / '──. 0A │ / Primary \ │ / Primary │ Dead-time (Id = 0) │ / Current (Ip) \ │ / Current (Ip) │ Resonant Ringing 0A ┴'───────────────────'───────────> Time 0A ┴'─────────────────'───────.────.─> Time │◄── D*Ts ──►◄─(1-D)*Ts─►│ │◄── D*Ts ──►◄─ D2*Ts ─►◄── D3*Ts ──►│

2.1 Comparative Analysis:

Continuous Conduction Mode (CCM):

Magnetizing energy does not fall to zero before the next cycle (I_ped > 0).

Advantages: Lower peak current (I_pk), lower RMS conduction losses in MOSFET and capacitor, lower output voltage ripple.

Drawbacks: Slower dynamic response due to Right-Half-Plane (RHP) Zero in the control loop; secondary diode experiences hard-switching reverse recovery (Q_rr).

Discontinuous Conduction Mode (DCM):

Stored energy completely discharges to zero during every cycle (I_ped = 0).

Advantages: Diode turns off at zero current (Zero Reverse Recovery); no RHP zero, enabling wide loop bandwidth and ultra-fast load step response.

Drawbacks: High peak currents (2 to 3 higher than CCM), higher I_rms resistive losses, larger bulk capacitors needed.

Quasi-Resonant (QR) / Critical Conduction Mode (CrCM):

Detects the zero-current point and waits for the drain voltage to resonate down to its lowest valley before triggering turn-on, combining DCM advantages with low switching losses.

3. Mathematical Design Formulations

3.1 Voltage Conversion Ratio (CCM):

V_OUT = V_IN * (N_s / N_p) * (D / (1 - D))

3.2 Primary Inductance (L_p) Sizing:

For boundary conduction mode (BCM) between CCM and DCM at minimum input voltage:

L_p = (V_IN(min)^2 * D_max^2) / (2 * P_in * f_sw)

Primary peak current:

I_pk = (2 * P_in) / (V_IN(min) * D_max)

3.3 Semiconductor Voltage Stress Calculations:

Primary Switch Maximum Voltage Stress (V_DS(max)):

V_DS(max) = V_IN(max) + n(V_OUT + V_F) + V_spike

where n = N_p / N_s is the primary-to-secondary turns ratio, and V_spike is the unabsorbed leakage spike (50 V ... 100 V).

Secondary Diode Reverse Voltage Stress (V_rev(sec)):

V_rev(sec) = V_OUT + V_IN(max) / n

4. Primary RCD Clamp Snubber Design

During primary switch turn-off, the energy trapped in the transformer's primary leakage inductance (L_lk) cannot couple to the secondary. It rings violently with the MOSFET output capacitance (C_oss), threatening overvoltage breakdown:

RCD Snubber Clamping Action Leakage Inductance (L_lk) ───██████────────┬─────────────────────────────┐ │ │ ├───[>¦] D_snub (UF4007) │ │ │ │ │ ┌─┴─┐ Drain (D) │ │ │ C_snub ┌───┴───┐ │ └─┬─┘ │ │ Q1 MOSFET │ ├───[ R_snub ]──┐ └───┬───┘ │ │ │ │ Source (S) +Vin DC ─────────┴────────┴───────────────┴──────┴─── GND_PRI

4.1 Step-by-Step Sizing Equations:

Leakage Energy per Cycle:

E_leak = 1 / 2 L_lk * I_pk^2

Power Dissipated in Snubber Resistor:

P_snub = E_leak * f_sw * (V_clamp / (V_clamp - n(V_OUT + V_F)))

Snubber Resistor (R_snub):

R_snub = V_clamp^2 / P_snub

Snubber Capacitor (C_snub) (constraining ripple to Δ V_clamp <= 10\% V_clamp):

C_snub = V_clamp / (Δ V_clamp * R_snub * f_sw)

Snubber Diode Selection: Must be an ultrafast recovery diode (t_rr <= 50 ns, e.g., US1M, ES1J) rated for > 1.2 * V_DS(max).

5. Multi-Output Flyback Generation & Cross-Regulation

The Flyback topology is uniquely suited for multi-rail auxiliary supplies because adding an isolated DC rail requires merely one secondary winding, one diode, and one capacitor:

Multi-Output Auxiliary Flyback Configuration Secondary 1 ┌───[>¦]───┬───> +5V Main (Regulated, Opto Feedback) │ ┌─┴─┐ ┌─────┤ │ │ C1 │ │ └─┬─┘ Primary Winding │ Secondary 2│ +Vin DC ───[ Np ]───┤ │ ┌───[>¦]───┼───> +15V Gate Drive Rail (Cross-Regulated) │ ├─────┤ ┌─┴─┐ Q1 │ │ │ │ C2 │ │ │ └─┬─┘ GND_PRI │ Secondary 3│ │ ┌───[>¦]───┼───> -15V Negative Gate Rail └─────┤ ┌─┴─┐ │ │ │ C3 └────────┴─┬─┘ GND_SEC

The Cross-Regulation Challenge:

Only the main output (+5 V) is enclosed within the optocoupler feedback loop. Auxiliary rails (+15 V, -15 V) rely on magnetic coupling. Imperfect winding coupling (leakage flux) causes auxiliary rails to sag under load or spike during light loads.

Solution: Sandwich winding geometry, bifilar auxiliary winding, or post-regulation using linear LDOs.