Isolated Forward DC-DC Converter: Working Mechanism, Core Reset & Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Forward DC-DC Converter. This guide provides an exhaustive hardware engineering and mathematical analysis of the isolated forward topology, exploring its continuous energy-transfer mechanics, magnetic transformer core reset methods (tertiary demagnetizing winding vs. two-switch forward), secondary LC output filtering, and component stress derivations.

1. Operating Principle & Working Mechanisms

The Forward Converter is an isolated, buck-derived switched-mode power supply topology. Unlike the Flyback converter—where energy is stored in the transformer's magnetic field during the switch ON time and discharged to the secondary during the OFF time—the Forward converter transfers energy instantaneously from primary to secondary while the primary switch is ON:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: SINGLE-SWITCH FORWARD CONVERTER WITH TERTIARY RESET (48V -> 12V/10A) ======================================================================================================================== +VIN (+48V DC Telecom Bus) ─────────┬───────────────────────────────┬───────────────────────────────┐ │ │ │ │ ┌┴─────────────────┐ │* (Dot at +VIN) │ [ Cathode ] │ │ CIN_BULK │ │ │ D_reset: ES1J (600V / 1A) │ │ 220µF / 100V │ │ TRANSFORMER PRIMARY │ (Ultra-Fast trr < 35ns) │ │ Electrolytic │ │ WINDING (Np: 14T) │ [ Anode ] │ └┬─────────────────┘ │ │ │ │ │ │* (Dot at Cathode / D_reset) │ │ │ │ │ │ +=== SWITCHING NODE (SW) │ TERTIARY RESET │ │ ¦ (Swings 0V to +96V) │ WINDING (Nter: 14T) │ │ ¦ │ │ │ ├──[ R_snub_pri: 10Ω / 2W ] │ │ │ │ │ │ │ │ │ [ C_snub_pri: 470pF/200V ] │ │ │ │ │ │ │ │ │ GND_PRI │ │ │ │ │ │ │ │ Drain (D) │ │ │ ┌┴──────────────┐ │ │ │ │ Q1: N-MOSFET │ │ │ │ │ BSC093N15NS5 │ │ │ │ +----->│ (150V, 9.3mΩ) │ │ │ │ ¦ Gate └┬──────────────┘ │ │ │ ¦ │ Source (S) │ │ │ ¦ │ │ │ │ ¦ +---[ Kelvin CS+ ] │ │ │ ¦ │ │ │ │ GATE DRIVER IC ¦ ┌┴┐ R_shunt: 15mΩ / 2W 1% │ │ │ UCC27517 (Low-Side) ¦ │ │ Metal Alloy │ │ │ OUT ----[ R_g: 4.7Ω ]+ └┬┘ (Current Sense) │ │ │ │ │ │ GND_PRI ────────────────────────────┴───────────────────────────────┴───────────────────────────────┴─── GND_PRI │ ┌┴┐ CY1: Safety Y-Capacitor (1.0nF / 250VAC Class Y2 Across Barrier) └┬┘ │ ============================================= ISOLATION BARRIER (>= 4.0mm Clearance) =================== │ GND_SEC ────────────────────────────────────────────────────────────┬───────────────────────────────┬─── GND_SEC │ │ │ │ TRANSFORMER SECONDARY │ │ │ WINDING (Ns: 7T) │ │ │ * (Dot aligned with Np) │ │ │ │ │ │ │ +---[ D1: Forward Diode ]-------+=== NODE SW_SEC │ │ V40100P (100V / 40A) ¦ (0V to +24V Rectified) │ │ [ Anode to Ns, Cathode Out] │ │ │ │ │ │ +---[ R_snub1: 4.7Ω / 1W ]--+ │ │ ¦ │ │ │ │ ¦ [ C_snub1: 1nF / 100V ] │ │ │ ¦ │ │ │ │ ¦ GND_SEC │ │ │ ¦ │ │ │ ¦ [ Cathode ] │ │ │ +---[ D2: Freewheel Diode ]-+ │ │ V40100P (100V / 40A) │ │ │ [ Anode to GND_SEC ] │ │ │ │ │ │ │ GND_SEC │ │ │ │ │ │ +---[ Lo: 15µH / 14A Choke ]----+ │ Coilcraft AGP4233-153 │ │ (DCR = 4.2mΩ, Isat = 16A) │ │ │ │ +=== +VOUT (+12V/10A) │ │ │ ┌┴──────────────────┐ │ │ COUT_BULK │ │ ┌┴┐ 3x 330µF/25V ┌┴┐ COUT_CER │ │ │ Poly (ESR=8mΩ) │ │ 4x 22µF/25V │ └┬┘ └┬┘ X7R 1210 │ │ │ │ │ +VOUT │ │ │ │ │ │ │ [ R_fb1: 38.3kΩ ] │ │ │ │ │ +---> TL431 REF │ │ │ (V_ref=2.50V) │ │ [ R_fb2: 10.0kΩ ] │ │ │ │ │ GND_SEC │ │ │ GND_SEC ───────────────────────────────────────────────────────────────────────────────────────────┴────────+───────────> GND_SEC (Return)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +VIN_RAW | Input Connector Pin 1 | C_in,bulk (+), Primary N_p Pin 1, Reset Diode D_reset Cathode | +48V DC input bus | Primary winding pin 1 and reset diode cathode tie directly to supply rail. |
| DRAIN_PRI | Transformer Primary N_p Pin 2 | Q_1 Drain, Primary Snubber R_snub,pri | Primary switching node (0 V ... 2 V_IN) | With N_p = N_ter, peak drain voltage is clamped precisely to 2 V_IN = 96 V. |
| RESET_NODE | Tertiary Winding N_ter Pin 1 | Reset Diode D_reset Anode | Magnetic core demagnetization rail | Winding dot is inverted relative to N_p; returns magnetizing energy back to V_IN during OFF interval. |
| SW_SEC | Forward Diode D_1 Cathode, Freewheel D_2 Cathode | Output Inductor L_o Pin 1, Secondary Snubber | Secondary pulsating rectangular voltage node | Swings between V_IN * (N_s/N_p) = 24 V and -V_F ≈ -0.5 V. |
| +VOUT | Output Inductor L_o Pin 2 | C_out bank (+), Feedback R_fb1, Load (+) | Regulated +12V DC output rail | Continuous current output stage delivers low ripple and tight dynamic regulation. |
| GND_PRI / GND_SEC | Primary Ground / Secondary Ground | Safety Y2 Capacitor (C_Y1) | Isolated return planes | Minimum 4.0 mm clearance maintained across PCB boundary. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| Q_1 | Primary N-MOSFET | Infineon BSC093N15NS5 | V_DS = 150 V, I_D = 75 A, R_DS(on) = 9.3 mΩ, Q_g = 25 nC | Rated for 150 V to provide 50\% margin above 2 V_IN = 96 V clamp level. |
| T_1 | Forward Transformer | Custom ETD34 Core (3C90) | Turns: N_p:N_s:N_ter = 14:7:14, L_m = 450 µH, L_lk < 2.0 µH | Ungapped high-permeability core; bifilar winding of N_p and N_ter minimizes leakage inductance. |
| D_reset | Core Demagnetizing Diode | Vishay ES1J | V_RRM = 600 V, I_F = 1 A, t_rr < 35 ns | High-voltage ultra-fast diode conducts magnetizing current I_m back into input rail during OFF time. |
| D_1, D_2 | Secondary Rectifier Diodes | Vishay V40100P | V_RRM = 100 V, I_F = 40 A, V_F = 0.58 V, t_rr < 25 ns | Dual Schottky diode in TO-247 package; provides forward conduction (D_1) and freewheeling loop (D_2). |
| L_o | Output Filter Inductor | Coilcraft AGP4233-153ME | L = 15 µH, I_sat = 16 A, I_rms = 14 A, DCR = 4.2 mΩ | High DC current power choke with flat-wire winding; maintains continuous conduction down to 1 A. |
| C_out,bulk | Output Bulk Capacitor | Panasonic 25SVPF330M | 3 * 330 µF, 25 V, Polymer, ESR = 8 mΩ | Accommodates output ripple current Δ I_Lo = 2.5 A with < 20 mV peak-to-peak ripple. |

Core Operating Phases:

Phase 1: Switch ON (0 < t <= D * T_s):

Primary switch Q1 turns ON. Full DC input voltage V_IN is applied across primary winding N_p.

Transformer dot convention is aligned: Secondary winding N_s induces a positive voltage V_s = V_IN * (N_s / N_p).

Forward diode D_1 is forward-biased, conducting current into the output inductor L_o and charging capacitor C_o. Freewheeling diode D_2 is reverse-biased.

Concurrently, magnetizing current I_m builds up linearly in the primary magnetizing inductance (L_m):

i_m(t) = V_IN / L_m * t

Phase 2: Switch OFF (D * T_s < t <= T_s):

Q1 turns OFF. Transformer primary current abruptly terminates.

The stored magnetizing energy induces a reverse polarity across all windings.

Forward diode D_1 turns OFF. The output inductor current freewheels continuously through diode D_2.

Crucial Core Reset: Diode D_reset conducts, clamping the voltage across tertiary winding N_ter to -V_IN and returning the trapped magnetizing energy back to the input bulk capacitor.

2. The Core Saturation Hazard & Demagnetization Techniques

Because energy is transferred directly during the ON phase, the transformer core must operate purely as an AC transformer without storing DC energy. If the magnetizing flux Φ_m is not completely returned to zero during each switching period, the core enters flux walking / progressive magnetic saturation, resulting in catastrophic switch overcurrent.

Transformer B-H Loop & Core Reset Trajectory B (Flux Density) ^ │ Operating Point during ON-time B_max ┼───────────────. (Delta B = Vin * D * Ts / (Np * Ae)) │ / │ / │ / Magnetization during Switch ON │ / B_rem ┼──────────. (Remanence Flux) │ / │ / Demagnetization during Reset (OFF-time) │ / 0 ┴──────'───────────────────────────────> H (Magnetic Field) │ Reset must return B to B_rem before next cycle!

2.1 Sizing the Tertiary Demagnetizing Winding:

To prevent core saturation, the volt-second integral across the magnetizing inductance over one switching period must equal zero:

int_0^T_s v_L(t) dt = 0 => V_IN * t_on = V_reset * t_reset

V_IN * (D * T_s) = (V_IN * N_p / N_ter) * t_reset

Solving for the required reset time (t_reset):

t_reset = D * T_s * (N_ter / N_p)

To ensure complete demagnetization before the next cycle begins, the reset time must not exceed the available OFF time (t_reset <= (1 - D) * T_s):

D * (N_ter / N_p) <= 1 - D => D_max <= 1 / (1 + N_ter / N_p)

[IMPORTANT] The 50% Duty Cycle Limit: In standard practice, the tertiary winding is wound with identical turns to the primary (N_ter = N_p). Consequently:

D_max <= 1 / (1 + 1) = 50\%

> Exceeding 50\% duty cycle with N_ter = N_p causes incomplete magnetic reset and rapid transformer saturation! ### 2.2 Switch Voltage Stress: During demagnetization, the reflected voltage from the tertiary winding adds directly to the input rail:

V_DS(max) = V_IN  (1 + N_p / N_ter) = 2  V_IN (For N_ter = N_p)

*Constraint*: For a 400 V DC bus, the primary switch must be rated for at least 2 * 400 V + V_spike >= 900 V ... 1000 V. --- ## 3. The Two-Switch Forward Converter (Industry Standard) To eliminate the expensive tertiary winding and reduce switch voltage stress to exactly V_IN, the **Two-Switch Forward Converter** is universally preferred for powers from 100 W to 500 W:

Two-Switch Forward Converter Topology

+Vin DC ───┬─────────────────────────────────────────────────────────┐

│                                                         │

Drain (D)                                                   ┌─┴─┐ D_clamp1

┌───┴───┐                                                   │   │ (Clamps SW2 to Vin)

│       │ Q1 (High-Side Switch)                             └─┬─┘

PWM_HS ──┤       │                                                     │

└───┬───┘                                                     │

│ Source (S)                                              │

├──────── SW1 Node ──[ Primary Winding: Np ]── SW2 Node ──┤

│                                                         │

┌─┴─┐ D_clamp2                                          Drain (D)

│   │ (Clamps SW1 to GND)                               ┌───┴───┐

└─┬─┘                                                   │       │ Q2 (Low-Side Switch)

│                                            PWM_LS ──┤       │

│                                                     └───┬───┘

│                                                         │ Source (S)

GND_PRI ─────────────────────────────────────────────────────┴─── GND_PRI

### Advantages of the Two-Switch Configuration: 1. **Voltage Clamping to V_IN**: When Q1 and Q2 turn off simultaneously, the magnetizing current forces diodes D_clamp1 and D_clamp2 into forward conduction. The primary winding is clamped directly across +V_IN and GND with inverted polarity. 2. **Maximum Switch Stress**:

V_DS1(max) = V_DS2(max) = V_IN + V_diode_drop

In a 400 V system, standard cost-effective 500 V - 600 V MOSFETs can be safely employed instead of fragile 1000 V devices! 3. **Automatic Leakage Energy Recovery**: Transformer primary leakage inductance energy is returned non-dissipatively to the input supply via the clamp diodes, eliminating the need for an RCD snubber. --- ## 4. Mathematical Design Equations ### 4.1 Voltage Conversion Ratio (Continuous Conduction Mode):

V_OUT = V_IN  (N_s / N_p)  D

### 4.2 Output Inductor Sizing (L_o): To maintain continuous conduction mode with peak-to-peak inductor ripple Δ I_L = r * I_OUT (typically r = 20\% ... 40\%):

L_o = ((V_s - V_OUT)  D) / (Δ I_L  f_sw) = (V_OUT  (1 - D)) / (Δ I_L  f_sw)

### 4.3 Output Capacitor Sizing (C_o): To constrain output voltage ripple to Δ V_OUT:

C_o >= (Δ I_L) / (8  f_sw  Δ V_OUT)

ESR_max <= (Δ V_OUT,ESR) / (Δ I_L)