Zeta DC-DC Converter: Inverse SEPIC Mechanics & Continuous Output Current

Welcome to the VVDN Engineering Hub Technical Dossier on the Zeta (Inverse SEPIC) DC-DC Converter. While the SEPIC converter offers continuous input current and a ground-referenced switch, its output current is discontinuous. The Zeta Converter flips this architecture: it features an output inductor in series with the load (like a buck converter), delivering ultra-low output voltage ripple and continuous output current while maintaining non-inverting step-up/step-down capability.

This guide explores the Zeta topology, derives its voltage and current transfer functions, compares it against the SEPIC, and provides component sizing formulas.

1. Operating Principle & Zeta Topology

The Zeta Converter utilizes a high-side active switch (Q_1), a parallel inductor (L_1), a series flying coupling capacitor (C_z), a freewheeling diode (D_1), and an output filter inductor (L_2) connected directly to the output:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: NON-INVERTING ZETA DC-DC CONVERTER (9V-18V IN -> 12V/4A OUT) ======================================================================================================================== +-----------------[ D_boot: DFLS1100 ]<------+ +V_DRV (+12V) ¦ [ 100V / 1A Schottky] ¦ ¦ [R_boot: 2.2Ω] ¦ ¦ ¦ +-----------[ C_boot: 0.1µF/50V X7R ]--+ ¦ ¦ ¦ ¦ ¦ +---[ HIGH-SIDE GATE DRIVER (e.g. UCC27282 / LM5113) ]---+ ¦ ¦ ¦ ¦ +-------->¦ BOOT [HO] Pin 8 ----[ R_g: 2.2Ω ]----+ ¦ ¦ ¦ ¦ ¦ +-->¦ PHASE/SW1 [VCC] Pin 2 <--- +V_DRV ¦ ¦ ¦ ¦ ¦ PWM_IN ---------->¦ IN_HS [COM] Pin 4 ---> PGND ¦ ¦ +-------------------------------------------¦------------+ ¦ +VIN (9V-18V DC) ───────────────────────────────────────────┐ ¦ │ │ ¦ [F1: 8A Fuse] │ ¦ │ │ ¦ ┌┴─────────────────┐ │ ¦ │ CIN_BULK │ │ Drain ¦ ┌┴┐ 220µF/35V Poly ┌┴┐ CIN_CER ┌┴───────┴──────┐ │ │ (Low-ESR 12mΩ) │ │ 2x 10µF/35V X7R │ Q1: N-MOSFET │<----+ └┬┘ └┬┘ │ BSC035N10NS5 │ (Gate) │ │ │ (100V, 3.5mΩ) │ │ │ └┬──────────────┘ │ │ │ Source (S) │ │ │ │ │ +=== NODE SW1 (Switch Node 1) │ │ ¦ (0V to +18V pulse) │ │ ¦ │ │ FLYING CAPACITOR ├──[ R_snub: 3.3Ω / 1W ] │ │ +------[ Cz: 10µF / 50V ]-------+ │ │ │ ¦ TDK C3225X7R1H106M ¦ [ C_snub: 470pF ] │ │ ¦ (Precharged to Vout) ¦ │ │ │ ¦ ¦ PGND │ │ ¦ PRIMARY INDUCTOR ¦ │ │ ¦ +--[ L1: 22µH / 6.0A ]------+ │ │ ¦ ¦ Coilcraft MSD1278-223 ¦ │ │ ¦ ¦ (DCR = 32mΩ) ¦ │ │ ¦ +-------------+-------------+ │ │ ¦ │ │ │ ¦ PGND (Ground Return) │ │ ¦ │ │ +=== NODE SW2 (Diode / Inductor Node) │ │ ¦ (0V to +30V pulse) │ │ ¦ │ │ +---[ D1: Schottky Diode (Cathode) ] │ │ ¦ V30100P (100V / 30A Trench) │ │ ¦ [ Anode to PGND ] │ │ ¦ │ │ │ ¦ PGND │ │ ¦ │ │ ¦ OUTPUT FILTER INDUCTOR │ │ +---[ L2: 22µH / 6.0A Shielded ]---+ │ │ Coilcraft MSD1278-223 ¦ │ │ (DCR = 32mΩ, Isat = 6.8A) ¦ │ │ ¦ │ │ +=== +VOUT (+12V/4A Regulated) │ │ │ │ │ ┌┴──────────────────┐ │ │ │ COUT_BULK │ │ │ ┌┴┐ 2x 270µF/25V ┌┴┐ COUT_CER │ │ │ │ Poly (ESR=10mΩ) │ │ 3x 22µF/25V │ │ └┬┘ └┬┘ X7R 1210 │ │ │ │ │ │ │ +VOUT │ │ │ │ │ │ │ │ │ [ R_fb1: 90.9kΩ, 0.1% ] │ │ │ │ │ │ │ +---> V_FB (To Controller) │ │ │ │ (V_ref = 1.200V) │ │ │ [ R_fb2: 10.0kΩ, 0.1% ] │ │ │ │ │ │ │ AGND (Quiet Ground) │ │ │ │ │ │ │ (Single Star Point) PGND┴──────────────────┴───────────────────────────────────────┴────────+───────────────────┴─── PGND (0V Rail)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +VIN_FILT | Fuse F_1 Output | C_in bank (+), Q_1 Drain (Pins 5-8) | Filtered positive DC input bus | High-side N-channel MOSFET drain connected directly to input rail. |
| SW1 (Switch Node 1) | Q_1 Source (Pins 1-3) | Inductor L_1 Pin 1, Flying Cap C_z Pin 1, Driver PHASE | High-side pulsating switching node | Requires bootstrap gate driver referenced to SW1; swings between 0 V and +V_IN. |
| SW2 (Switch Node 2) | Flying Cap C_z Pin 2 | Diode D_1 Cathode, Output Inductor L_2 Pin 1 | Secondary pulsating node | Swings between 0 V (when D_1 conducts) and V_IN + V_o (when Q_1 conducts). |
| +VOUT | Inductor L_2 Pin 2 | C_out bank (+), Feedback R_fb1, Load (+) | Non-inverting continuous-current output | Output inductor L_2 provides continuous DC current, resulting in very low output voltage ripple. |
| PGND | C_in (-), Inductor L_1 Pin 2, Diode D_1 Anode, C_out (-) | System power ground return | Common zero-volt power ground | Continuous ground plane; carries circulating inductor and diode currents. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| Q_1 | High-Side N-MOSFET | Infineon BSC035N10NS5 | V_DS = 100 V, I_D = 100 A, R_DS(on) = 3.5 mΩ, Q_g = 28 nC | Requires floating bootstrap driver circuit (D_boot, C_boot) to bias gate above V_IN. |
| D_1 | Freewheeling Diode | Vishay V30100P | V_RRM = 100 V, I_F = 30 A, V_F = 0.52 V, t_rr < 20 ns | Anode to ground, cathode to SW2. Sees peak reverse voltage V_R = V_IN + V_o. |
| C_z | Flying Energy Capacitor | TDK C3225X7R1H106M | 10 µF, 50 V, X7R Ceramic, 1210 package | Steady-state DC voltage across C_z equals V_o; handles high AC ripple current. |
| L_1, L_2 | Coupled / Dual Inductors | Coilcraft MSD1278-223MLD | 2 * 22 µH, I_sat = 6.8 A, DCR = 32 mΩ | L_2 placed at output creates low-noise output spectrum identical to a standard Buck converter. |
| C_out,bulk | Output Bulk Capacitor | Panasonic 25SVPF270M | 2 * 270 µF, 25 V, OS-CON Polymer, ESR = 10 mΩ | Smooths residual inductor ripple current; low ESR ensures < 20 mV ripple. |

1.1 Conduction Intervals:

Interval 1: Switch ON (0 < t <= D * T_s):

High-side switch Q_1 conducts. Node SW_1 is pulled to V_IN.

Inductor L_1 is energized directly across V_IN to GND:

di_L1 / dt = V_IN / L_1

The flying capacitor C_z (precharged to V_out) pulls node SW_2 positive (V_SW2 = V_IN + V_out), reverse-biasing diode D_1.

Inductor L_2 current ramps up, energized by V_IN + V_out - V_out = V_IN:

di_L2 / dt = V_IN / L_2

Interval 2: Switch OFF (D * T_s < t <= T_s):

Q_1 turns OFF. The collapsing fields of L_1 and L_2 pull nodes SW_1 and SW_2 negative.

Diode D_1 turns ON, clamping node SW_2 to GND (0 V).

Inductor L_1 discharges its energy into flying capacitor C_z through diode D_1.

Inductor L_2 freewheels through diode D_1 into the output capacitor C_out and the load:

di_L2 / dt = (-V_out) / L_2

2. Voltage and Current Waveforms

Zeta Key Operating Waveforms i_Cin ^ Pulsating Input Current │ ┌──────┐ ┌──────┐ │ │ │ │ │ 0A ┼───┴──────┴──────────────────────┴──────┴────────────────────> Time │◄─ D*Ts ─►│ i_L2 ^ Continuous Output Current (Buck-Like!) │ / \ / \ I_out ┼─ ─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ │ / \ / \ 0A ┼────────/─────────\────────────/─────────\───────────────────> Time

3. Mathematical Formulations & Component Sizing

3.1 Voltage Conversion Ratio:

Applying volt-second balance to output inductor L_2:

(V_IN) * D * T_s + (-V_out) * (1 - D) * T_s = 0

V_out = V_IN * D / (1 - D)

The conversion ratio is identical to the SEPIC and classic buck-boost, delivering non-inverting positive output voltage.

3.2 Switch & Diode Voltage Stresses:

V_DS,max = V_diode,rev = V_IN,max + V_out

3.3 Continuous Output Filter Sizing:

Because output inductor L_2 feeds C_out directly, the output ripple voltage formula is identical to a Buck converter:

Δ V_out = (Δ I_L2) / (8 * f_s * C_out)

Huge Engineering Advantage: Output voltage ripple in a Zeta converter is orders of magnitude lower than in a SEPIC or Boost converter for the same capacitor value!

3.4 Flying Capacitor Sizing (C_z):

The steady-state DC voltage across C_z is equal to V_out:

V_Cz = V_out

To maintain capacitor voltage ripple Δ V_Cz <= 5\% * V_out:

C_z >= (I_out * (1 - D)) / (f_s * Δ V_Cz)

4. Head-to-Head Comparison: SEPIC vs. Zeta

| Engineering Parameter | SEPIC Converter | Zeta Converter (Inverse SEPIC) |
| --- | --- | --- |
| Output Polarity | Positive (Non-inverting) | Positive (Non-inverting) |
| Switch Gate Drive | Low-Side: Referenced to GND (Simple standard driver) | High-Side: Floating switch (Requires bootstrap or P-FET) |
| Input Current Waveform | Continuous: Low input ripple, small input filter | Pulsating: High input ripple, requires larger C_in |
| Output Current Waveform | Pulsating: High output ripple, requires large C_out | Continuous: Ultra-low output ripple, small C_out |
| Shutdown Disconnect | Yes (Series C_sep blocks DC) | No (Diode and L_2 path) |
| Optimal Application | Battery-powered devices, automotive inputs | Low-noise analog sensor rails, instrumentation |