LDO: Calculations, Feedback Sizing & Thermal Engineering

1. Power Dissipation, Efficiency & Thermal Mechanics

Unlike switching power supplies that store and transfer energy reactively via inductors and capacitors, a linear regulator drops excess voltage entirely through resistive dissipation across the active channel of its pass transistor.

1.1 Power Dissipation Equations

The total power dissipated as heat inside an LDO package (P_D) consists of two distinct components:

P_D = P_pass + P_Q

Where:

P_D: Total power dissipated as heat inside the regulator package (W)

P_pass: Power dissipated in the series pass transistor channel (W)

P_Q: Quiescent control power consumed internally by bandgap and bias networks (W)

A. Pass Transistor Dissipation (P_pass):

The power burned dropping the unregulated input voltage (V_IN) down to the regulated output voltage (V_OUT) while supplying load current (I_LOAD):

P_pass = (V_IN - V_OUT) * I_LOAD = V_drop * I_LOAD

B. Quiescent Control Dissipation (P_Q):

The power consumed internally by the bandgap reference, error amplifier, and internal bias circuits:

P_Q = V_IN * I_Q

Total Power Dissipation (P_D):

P_D = (V_IN - V_OUT) * I_LOAD + (V_IN * I_Q)

Where:

P_D: Total power dissipated as heat in the regulator package (W)

V_IN: Unregulated DC input voltage (V)

V_OUT: Regulated DC output voltage (V)

I_LOAD: Output load current (A)

I_Q: Internal quiescent current flowing to ground (A)

[NOTE] In normal full-load operation where I_LOAD >> I_Q (e.g., I_LOAD = 1.0 A and I_Q = 2.0 mA), the quiescent power contributes less than 0.2\% of total heat dissipation and is often negligible:

P_D ≈ (V_IN - V_OUT) * I_LOAD

--- ### 1.2 Electrical Efficiency (η) The conversion efficiency of a linear regulator is fundamentally bounded by the ratio of output voltage to input voltage:

η = P_OUT / P_IN  100\% = (V_OUT  I_LOAD) / (V_IN  (I_LOAD + I_Q))  100\%

When I_LOAD >> I_Q:

η ≈ V_OUT / V_IN * 100\%

*Efficiency Benchmark Comparisons*: - Stepping 5.0 V down to 3.3 V: η ≈ 3.3 / 5.0 = 66.0\% (34\% lost as pure heat). - Stepping 3.3 V down to 1.8 V: η ≈ 1.8 / 3.3 = 54.5\% (45.5\% lost as heat). - Stepping 1.2 V down to 1.0 V (using an ultra-low dropout NMOS LDO):

η ≈ 1.0 / 1.2 = 83.3\%

*(Notice: At low headroom, linear regulators rival the efficiency of switching buck converters while providing zero ripple!).* --- ## 2. Thermal Resistance Network & Junction Temperature (T_J) All heat generated inside the silicon die must travel through the IC packaging into the surrounding ambient air. The thermal behavior is modeled using **Thermal Ohm's Law**:

Δ T = P_D * θ_JA

T_J = T_A + (P_D * θ_JA)

Where: - T_J: Silicon Junction Temperature (Maximum safe limit typically **+125°C** for commercial/industrial and **+150°C** for automotive). - T_A: Ambient Air Operating Temperature (e.g., +25°C laboratory, +50°C industrial enclosure, +85°C automotive engine compartment). - θ_JA: Junction-to-Ambient Thermal Resistance (°C/W). - P_D: Total Power Dissipated in Watts.

Silicon Junction (T_J)

│

[ ] Thermal Resistance Junction-to-Case (θ_JC)

│

IC Package Case (T_C)

│

[ ] Thermal Resistance Case-to-Heat-Sink / PCB (θ_CS)

│

Heat Sink / Copper Plane (T_S)

│

[ ] Thermal Resistance Sink-to-Ambient (θ_SA)

│

Ambient Air (T_A)

θ_JA = θ_JC + θ_CS + θ_SA

--- ### 2.1 Worked Numerical Example: Thermal Verification **Operating Conditions**: - V_IN = 5.0 V - V_OUT = 3.3 V - I_LOAD = 800 mA (0.8 A) - Ambient Temperature: T_A = 50°C - LDO Package: **SOT-223** with standard PCB layout (θ_JA = 65°C/W). 1. Calculate Power Dissipation:

P_D ≈ (5.0 V - 3.3 V)  0.8 A = 1.7 V  0.8 A = 1.36 W

2. Calculate Temperature Rise (Δ T):

Δ T = 1.36 W * 65°C/W = 88.4°C

3. Calculate Junction Temperature (T_J):

T_J = T_A + Δ T = 50°C + 88.4°C = 138.4°C

4. **Thermal Verdict**: **FAIL!** T_J exceeds the maximum safe limit (125°C) by 13.4°C. The LDO will enter thermal shutdown and cycle ON/OFF. #### Sizing Maximum Permissible θ_JA with a Heat Sink / Thermal Vias:

θ_JA(max) <= (T_J(max) - T_A) / P_D = (125°C - 50°C) / 1.36 W = 75°C / 1.36 W ≈ 55.1°C/W

*Corrective Action*: Expand PCB thermal copper area or migrate to a **TO-263 (D2PAK)** package (θ_JA ≈ 30°C/W on 1 in^2 copper). --- ### 2.2 PCB Thermal Copper Sizing Rules 1. **Thermal Via Array**: Place a dense matrix of thermal vias (0.3 mm drill hole diameter, 1.0 mm ... 1.2 mm grid pitch) directly beneath the IC's exposed power thermal pad (PowerPAD / ePad). 2. **Internal Plane Stitching**: Connect thermal vias directly to internal and bottom-layer Ground/Power copper flood planes (1 oz or 2 oz copper) to distribute heat across the entire board surface. --- ## 3. Feedback Resistor Divider Network Sizing For adjustable LDOs, the output voltage is configured using an external resistive voltage divider (R_1, R_2):

Vout ────────┬────────

│

[R1] (Top Resistor)

│

├───────────> FB Pin (V_FB = V_REF)

│

[R2] (Bottom Resistor)

│

GND

### 3.1 Voltage Transfer Function The feedback pin draws a minuscule input bias current (I_FB ≈ 10 nA ... 100 nA). The exact equation is:

V_OUT = V_REF  (1 + R1 / R2) + (I_FB  R1)

Where: - V_OUT: Regulated DC output voltage (V) - V_REF: Internal bandgap reference voltage (V) - R1: Upper feedback divider resistor connected between V_OUT and FB pin (Ω) - R2: Lower feedback divider resistor connected between FB pin and GND (Ω) - I_FB: Input bias leakage current entering the error amplifier FB pin (A) Because modern CMOS error amplifiers have I_FB < 50 nA, the second term is negligible when R_1 is sized reasonably (R_1 <= 100 kΩ):

V_OUT ≈ V_REF * (1 + R_1 / R_2)

Solving for the resistor ratio:

R_1 / R_2 = V_OUT / V_REF - 1

### 3.2 Sizing Guidelines & Current Trade-Offs - **Divider Current (I_div)**: Sized to be at least 100* higher than I_FB to ensure thermal leakage currents do not cause output voltage drift:

I_div = V_REF / R_2 >= 100 * I_FB

- **Low-Power vs. High-PSRR Trade-Off**: - *Battery-operated micropower designs*: Select R_2 = 100 kΩ ... 500 kΩ (I_div ≈ 2 µA ... 10 µA) to maximize battery runtime. - *High-speed RF / Precision Analog designs*: Select R_2 = 1 kΩ ... 20 kΩ (I_div ≈ 60 µA ... 1.2 mA) to minimize Johnson-Nyquist thermal noise (e_n = sqrt(4k_B T R)) and prevent capacitive noise pick-up on the high-impedance FB node. --- ## 4. Feedforward Capacitor (C_FF) Phase-Lead Compensation In adjustable LDO regulators, adding a small **Feedforward Capacitor (C_FF)** (10 pF ... 100 pF) in parallel with top resistor R_1 drastically improves dynamic performance:

Vout ────────┬──────────┐

│          │

[R1]      ─┴─ C_FF (Feedforward Capacitor)

│        ─┬─ (10pF - 100pF)

├──────────┘

│

[R2]

│

GND

### 4.1 Pole-Zero Derivation of the Feedback Network With C_FF installed, the feedback impedance Z_1(s) = R_1 ¦¦ 1 / (s C_FF). The transfer function from V_OUT to V_FB becomes:

V_FB(s) / V_OUT(s) = ( R_2 / (R_1 + R_2) ) * (1 + s R_1 C_FF) / (1 + s (R_1 || R_2) C_FF)

This introduces a **Phase-Lead Zero (f_z)** and a **High-Frequency Pole (f_p)**:

f_z_ff = 1 / (2π  R_1  C_FF)

f_p_ff = 1 / (2π  (R_1 || R_2)  C_FF) = f_z_ff * (1 + R_1 / R_2)

### 4.2 Engineering Benefits of C_FF 1. **Phase Margin Boost**: The zero is placed near the unity-gain crossover frequency (10 kHz ... 100 kHz), injecting +20° ... +45° of phase boost to suppress transient ringing. 2. **Improved High-Frequency PSRR**: At higher frequencies, C_FF bypasses R_1, coupling output ripple directly to the inverting input of the error amplifier with unity gain rather than attenuated by R_2 / (R_1+R_2). 3. **Faster Load Step Recovery**: High-speed transient edges on V_OUT are fed directly to the error amplifier without experiencing the R_1 C_parasitic low-pass delay. --- ## 5. Output Capacitor Dynamic Sizing & ESR Zero ### 5.1 Sizing C_OUT for Load Transient Undershoot When a digital load (such as a microcontroller awakening from sleep) suddenly steps its current demand from 0 to I_step in time t_edge ≈ 10 ns, the LDO error amplifier requires finite response time (t_response ≈ 2 µs ... 10 µs) to adjust the pass transistor gate. During this brief window, **100\% of the step current must be supplied by the output capacitor (C_OUT)**:

Δ V_OUT ≈ (I_step  t_response) / C_OUT + (I_step  R_ESR)

Solving for the minimum required output capacitance:

C_OUT(min) >= (I_step  t_response) / (Δ V_allowed - (I_step  R_ESR))

*Worked Example*: - Load step: I_step = 1.0 A - Regulator response time: t_response = 5 µs - Allowable voltage droop: Δ V_allowed = 100 mV - Output capacitor ESR: R_ESR = 15 mΩ (I_step * R_ESR = 15 mV)

C_OUT(min) >= (1.0 A  5  10^-6 s) / (0.100 V - 0.015 V) = (5.0 * 10^-6) / 0.085 V ≈ 58.8 µF