PMOS LDO: Working Mechanism, Loop Dynamics & Architecture

1. Architectural Topology & Circuit Schematic

In a PMOS-based Low-Dropout Regulator, the series pass transistor is a P-channel enhancement-mode MOSFET configured in a Common-Source (CS) amplifier topology.

Unregulated Input (Vin) │ ├─── Source (S) │ ┌───────────────┐ │ │ PMOS Pass │ └───┤ Transistor │ │ (W/L Large) │ Gate (G) <───┤ │ └───────┬───────┘ │ Drain (D) ├──────────────────────────────┬───────────────> Regulated Vout │ │ │ [R1] Feedback │ │ Resistors │ ├─── (V_FB) ┌─────────────┴─────────────┐ │ │ Error Amplifier (EA) │ [R2] │ │ │ V_REF ────>│ (+) Non-inverting Input │ GND V_FB ────>│ (-) Inverting Input │ │ │ │ Output sinks gate current ┼─── Gate Drive (V_GATE) └───────────────────────────┘

1.1 Pin and Terminal Polarities

Source (S): Connected directly to the raw input supply voltage rail (V_IN).

Drain (D): Connected directly to the output voltage rail (V_OUT).

Gate (G): Driven by the output of the high-gain Error Amplifier.

Substrate / N-Well Body (B): Tied internally to the Source (V_IN) to prevent forward-biasing the source-body junction.

2. Operating Mechanism & Gate Drive Physics

2.1 Negative Gate-to-Source Drive (V_GS)

A PMOS transistor conducts when its Gate voltage is pulled lower than its Source voltage by at least its threshold voltage magnitude (|V_th|):

V_GS = V_GATE - V_IN < -¦V_th¦ <=> V_GATE < V_IN - ¦V_th¦

Because the source is connected to the highest positive voltage in the system (V_IN), the Error Amplifier simply has to pull the gate voltage down towards Ground (0V) to turn the transistor ON:

Max Available Gate Overdrive ¦V_GS(max)¦ = V_IN - V_GATE(min) ≈ V_IN - 0.2 V

[IMPORTANT] Single-Supply Simplicity: Unlike NMOS LDOs, the PMOS pass element requires no gate voltage higher than V_IN. This allows the PMOS LDO to run entirely from a single input rail without an auxiliary supply pin (V_BIAS) or an on-chip charge pump.

2.2 Dropout Voltage Mechanics in Triode (Linear) Region

When V_IN approaches V_OUT (at low headroom), the PMOS transistor is driven fully ON and enters its linear / triode ohmic region. The regulator behaves as a small resistor:

R_DS(on) = 1 / (µ_p C_ox (W / L) (¦V_GS¦ - ¦V_th¦))

The minimum dropout voltage (V_DO) is given directly by Ohm's Law:

V_DO = I_LOAD * R_DS(on)

Where:

V_DO: Dropout voltage of the PMOS regulator (V)

I_LOAD: DC output load current (A)

R_DS(on): On-resistance of PMOS pass element (Ω)

Numerical Example:

For a target dropout of V_DO <= 100 mV at I_LOAD = 1.0 A:

R_DS(on) <= 100 mV / 1.0 A = 0.10 Ω = 100 mΩ

2.3 The Silicon Area & Gate Capacitance Penalty

Because holes have significantly lower mobility than electrons (µ_p ≈ 1 / 2.5 µ_n to 1 / 3 µ_n in silicon):

A PMOS pass transistor requires 2.5* to 3* larger channel width (W) than an equivalent NMOS transistor to achieve the same R_DS(on).

This physical enlargement dramatically increases silicon die area, cost, and parasitic gate capacitance (C_gate = C_GS + C_GD). A large PMOS pass element can easily present 500 pF ... 2000 pF of gate capacitance to the error amplifier output!

3. Small-Signal Loop Dynamics & The Two-Pole Stability Challenge

The primary engineering challenge in PMOS LDO design is loop stability across varying load currents.

┌───────────┐ ┌───────────┐ ┌───────────────┐ │ Error │ V_gate │ PMOS │ V_out │ Feedback │ V_fb ──(+)─>│ Amplifier ├────────>│ Pass FET ├────────>│ Divider ├───(-) │ A_EA(s) │ │ -gm_p*ro │ │ β = R2/(R1+R2)│ └─────┬─────┘ └─────┬─────┘ └───────────────┘ │ │ Pole 1: Pole 2: Internal Load-Dependent Gate Pole Output Pole (p_gate) (p_out)

3.1 High Common-Source Output Impedance (r_o)

Because the PMOS operates in Common-Source, its small-signal drain output impedance is naturally high:

R_OUT = r_ds ¦¦ (R1 + R2) ¦¦ R_LOAD ≈ r_ds ¦¦ R_LOAD

Where:

R_OUT: Small-signal open-loop output impedance of the Common-Source stage (Ω)

r_ds: Small-signal channel resistance of PMOS transistor (Ω)

R1, R2: Upper and lower feedback divider resistors (Ω)

R_LOAD: Effective DC load resistance, R_LOAD = V_OUT / I_LOAD (Ω)

3.2 The Two Competing Low-Frequency Dominant Poles

The open-loop transfer function contains two distinct low-frequency poles:

1. The Output Pole (p_out):

Formed by the high output resistance (R_OUT) and the output filter capacitor (C_OUT):

p_out = 1 / (2π * R_OUT * C_OUT) ≈ 1 / (2π * (V_OUT / I_LOAD) * C_OUT)

Critical Observation: Because R_LOAD = V_OUT / I_LOAD, the output pole frequency moves drastically with load current!

At full load (I_LOAD = 1 A, V_OUT = 3.3 V, C_OUT = 10 µF):

R_LOAD = 3.3 Ω => p_out = 1 / (2π * 3.3 Ω * 10 µF) ≈ 4.8 kHz

At light load (I_LOAD = 1 mA, R_LOAD = 3300 Ω):

p_out = 1 / (2π * 3300 Ω * 10 µF) ≈ 4.8 Hz

The output pole shifts by three orders of magnitude across normal operating currents!

2. The Internal Gate Pole (p_gate):

Formed by the high output impedance of the error amplifier stage (R_EA) and the massive gate capacitance of the PMOS pass transistor (C_gate):

p_gate = 1 / (2π * R_EA * C_gate)

Because both R_EA and C_gate are large, p_gate typically falls in the range of 100 Hz ... 50 kHz.

The Stability Threat:

With two dominant poles located closely in frequency, each contributing -90° of phase lag, the total open-loop phase shift reaches -180° before the loop gain drops to unity (0 dB). According to the Barkhausen stability criterion, this produces a negative phase margin, causing spontaneous, violent oscillation on the V_OUT rail!

4. Frequency Compensation & The ESR Stability Tunnel

To stabilize a PMOS LDO, a phase-lead zero (z_esr) must be inserted into the loop to cancel one of the dominant poles and boost phase margin back above +45° ... +60°.

4.1 The Output Capacitor ESR Zero

Every real-world capacitor exhibits internal Equivalent Series Resistance (ESR) (R_ESR). In series with C_OUT, it creates a transfer function zero:

z_esr = 1 / (2π * R_ESR * C_OUT)

Loop Gain (dB) 60 ¦──────┐ (p_out) ¦ \ -20 dB/dec 40 ¦ \────────┐ (p_gate) ¦ \ -40 dB/dec 20 ¦ \ ┌────── (z_esr restores slope to -20 dB/dec!) ¦ \ / 0 ¦─────────────────────\───/────────────── 0 dB Crossover (UGBW) +────────+─────────────+─/───+───────> Frequency p_out p_gate z_esr

4.2 The ESR Stability "Tunnel" (Horseshoe Curve)

Because the stability of early PMOS LDOs depended entirely on the ESR zero, the output capacitor had to fall within a strictly bounded "tunnel":

Capacitor ESR (Ω) 10.0 ¦───────────────────────────────────────────── ¦ UNSTABLE REGION 5.0 ¦───────[ Upper ESR Limit (p_esr) ]─────────── 1.0 ¦ ¦ STABLE TUNNEL 0.1 ¦ (Phase Margin > 45°) 0.05 ¦───────[ Lower ESR Limit (z_esr too high) ]── ¦ UNSTABLE REGION (Pure Ceramic MLCC) 0.001+───────────────────────────────────────────── 0.1 mA Load Current 1.0 A

Why Too Low ESR Causes Instability (R_ESR < 50 mΩ):

Modern multi-layer ceramic capacitors (MLCCs) have ultra-low ESR (5 mΩ ... 15 mΩ). With such tiny resistance, the zero is pushed to megahertz frequencies (z_esr > 5 MHz), well past the unity-gain crossover. The zero arrives too late to rescue the phase margin, and the regulator oscillates!

Why Too High ESR Causes Instability (R_ESR > 3 Ω):

Excessive ESR (common in aged electrolytic capacitors) introduces a secondary high-frequency pole (p_esr) and produces unacceptable transient output voltage spikes (Δ V = I_step * R_ESR).

4.3 Modern MLCC Ceramic-Stable PMOS LDOs

To allow engineers to use modern low-ESR ceramic capacitors without oscillation, modern PMOS LDOs integrate internal frequency compensation networks:

Internal Damping Zero Generation: The IC internally senses output current and synthesizes an internal resistor-capacitor zero in the feedback path, providing phase lead regardless of external capacitor ESR.

Feedforward Capacitor (C_FF): A small capacitor (10 pF ... 100 pF) placed across top feedback resistor R_1:

z_ff = 1 / (2π * R_1 * C_FF)

This injects phase lead directly into the error amplifier's inverting terminal.

5. Reverse Current Protection & Parasitic Body Diode

A critical vulnerability of standard PMOS LDOs is the intrinsic parasitic P-N body diode formed between the Drain (V_OUT, P-type diffusion) and the N-Well Substrate (V_IN, N-type diffusion).

Vin (Source) │ ├───┐ │ │ │ ▼ [PMOS Channel] │ │ │ ├───┤◀├───┐ (Parasitic Body Diode) │ │ Diode │ │ ▼ │ └───┴─────────┼───> Vout (Drain) │ (When Vout > Vin: Diode conducts massive reverse current!)

5.1 The Reverse-Biased Failure Mode

If V_OUT ever exceeds V_IN by more than 0.6 V (such as when the input power is abruptly switched off while a large, charged output capacitor holds V_OUT high, or if an active battery is connected to V_OUT), the parasitic body diode conducts.

Uncontrolled current surges backwards from V_OUT into the input supply rail, frequently melting the internal silicon bond wires or destroying the input supply.

5.2 Silicon Protection Techniques

External Reverse Schottky Diode: Connecting an external low-voltage drop Schottky diode from V_OUT (Anode) to V_IN (Cathode) clamps the reverse voltage to < 0.3 V, preventing the internal silicon body diode from turning on.

True Reverse-Current Blocking ICs (Back-to-Back PMOS / Body-Switching):

Modern ICs incorporate a second internal PMOS transistor in series back-to-back, so their body diodes point in opposite directions.

Alternatively, an internal comparator continuously compares V_IN and V_OUT. When V_OUT > V_IN, the comparator dynamically switches the N-Well substrate connection from V_IN to V_OUT, keeping the parasitic diode reverse-biased under all conditions.

6. Real-World PMOS LDO ICs & Application Schematics

Texas Instruments TPS7A4700 (PMOS LDO) Ultra-Low Noise, 1A ┌───────────────────────────┐ Vin (3.8V - 5.5V) ──┤ IN OUT ├─────┬───────> Clean Vout (3.3V, 1A) │ │ │ ─┴─ │ SENSE├─────┤ ─┴─ Output MLCC C_IN (10uF) ─┬─ │ │ │ ─┬─ (10uF, X7R, 16V) GND │ NR/FB GND ├─┐ │ GND └────┬───────────────────┬──┘ │ │ │ │ │ │ ─┴─ C_NR (10nF) GND GND │ ─┬─ (Noise Reduct) │ GND │ ┌─────────────────────────────────┴┐ │ ANY-OUT Pin Strapping Network │ │ (Programs output voltage without │ │ external feedback resistors) │ └──────────────────────────────────┘

Representative Production PMOS LDOs:

TI `TPS7A4700`: High-performance 1.0 A PMOS LDO featuring 4.17 µV_RMS ultra-low noise, 82 dB PSRR at 100 Hz, and internal pin-programmable voltage dividers.

ADI / Linear Tech `LT1763`: Industry-standard 500 mA micropower PMOS LDO with 20 µV_RMS noise, ideal for battery-operated RF transceivers.

TI `TPS795`: 500 mA PMOS regulator with high bandwidth and low dropout (110 mV at 500 mA).