Dual-Gate MOSFETs (Tetrode FET): RF Mixers & Automatic Gain Control

A Dual-Gate MOSFET (often referred to as a MOSFET Tetrode) is a multi-electrode field-effect transistor featuring two separate, independently biased control gates (Gate 1 and Gate 2) arranged in series over a single shared conduction channel between Source and Drain. By monolithically integrating a cascode amplifier topology onto a single silicon die, dual-gate MOSFETs achieve an ultra-low reverse transfer (Miller) capacitance (C_rss < 0.02 pF), wide dynamic Automatic Gain Control (AGC) range, and superior intermodulation performance across VHF, UHF, and microwave communications.

1. Semiconductor Physics & Monolithic Cascode Equivalence

Source (S) Gate 1 (G1) Gate 2 (G2) Drain (D) ┌────────┐ ┌────────┐ ┌────────┐ ┌────────┐ │ Al │ │ Poly-Si│ │ Poly-Si│ │ Al │ ├───┬────┴────────────┴───┬────┴───────────┴───┬────┴───────────┴───┬────┤ │n+ │ SiO2 Gate 1 Oxide │ n-Island (Virtual) │ SiO2 Gate 2 Oxide │ n+ │ └───┴─────────────────────┴────────────────────┴────────────────────┴────┘ p-Substrate (Internally connected to Source)

Monolithic Cascode Equivalence:

A dual-gate MOSFET behaves electrically as two distinct transistors connected in series:

Lower Transistor (M_1): A Common-Source (CS) amplifier whose gate is Gate 1.

Upper Transistor (M_2): A Common-Gate (CG) amplifier whose gate is Gate 2.

Drain (D) │ ┌──┴──┐ Gate 2 (G2) ──────┤ M2 │ (Common-Gate Stage: Low input impedance, high output Z) (AGC / LO) └──┬──┘ │ (Virtual Internal Node) ┌──┴──┐ Gate 1 (G1) ──────┤ M1 │ (Common-Source Stage: High input impedance) (RF Input) └──┬──┘ │ Source (S) (Tied to RF Ground)

Elimination of Miller Capacitance & Unconditional Stability:

In conventional single-gate RF amplifiers, the Drain-to-Gate capacitance (C_gd ≈ 1 - 5 pF) feeds inverted high-voltage RF output back to the input gate. At high frequencies, this negative conductance causes self-oscillation unless complex, band-limited neutralization transformers are added.

In a Dual-Gate MOSFET, Gate 2 is bypassed to AC ground via a decoupling capacitor (10 nF).

Gate 2 acts as a grounded electrostatic Faraday shield interposed between the drain and Gate 1.

Reverse feedback capacitance is slashed to C_rss <= 0.02 pF, ensuring unconditional high-frequency stability up to 1 GHz without neutralization!

2. RF Circuit Topologies: AGC Amplifiers & Active Mixers

1. Low-Noise RF Amplifier with Automatic Gain Control (AGC):

In communication receivers (FM, VHF maritime, SDR front-ends), incoming signal strength varies over orders of magnitude (1 µV ... 100 mV). A dual-gate MOSFET provides clean dynamic gain adjustment:

+V_DD Supply (+9V / +12V) │ [ L1 ] RF Choke / LC Tuned Tank │ ├───[ Cout ]───> RF Output │ Drain (D) │ ┌──┴──┐ AGC Bias ─────┤ G2 │ Dual-Gate FET (e.g. BF998) Control │ │ (0V to +4V) └──┬──┘ │ ┌──┴──┐ RF Signal ────┤ G1 │ Input (50Ω) └──┬──┘ │ Source (S) ──┬──[ Rs: 270Ω ]──┬── GND │ │ └──[ Cs: 10nF ]──┘

How AGC Operates Without Detuning the Front-End:

Varying the DC voltage on Gate 2 (0 V ... +4 V) modulates the drain-to-source voltage of the lower transistor M_1, dynamically adjusting overall transconductance g_m over a > 40 dB attenuation range.

Crucially, changing V_G2 has virtually zero influence on the input capacitance at Gate 1 (C_iss(G1)). The tuned RF input filter remains precisely centered on the desired RF channel without frequency pulling or bandwidth distortion!

2. Active Dual-Gate RF Mixer:

By injecting two distinct frequencies into Gate 1 and Gate 2, the non-linear cross-multiplication across the shared channel acts as a high-performance active mixer:

Gate 1 (G_1): Driven by weak received RF antenna signal (f_RF).

Gate 2 (G_2): Driven by local oscillator (f_LO).

Drain (D): Connected to an intermediate frequency (IF) tuned tank (f_IF = |f_RF - f_LO|).

High isolation between G_1 and G_2 (> 30 dB) prevents local oscillator radiation from leaking back into the antenna.

3. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | V_DS(max) | I_D(max) | Forward Transfer g_fs | C_rss (Typ) | Target Application |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BF998 | NXP / Vishay | SOT-143 | 12 V | 30 mA | 24 mS | 0.015 pF | VHF/UHF TV tuners, wideband SDR front-ends |
| BF992 | NXP Semiconductor | SOT-143 | 20 V | 30 mA | 20 mS | 0.020 pF | Low-noise RF preamplifiers, 433/868 MHz ISM |
| 3SK294 | Toshiba | S-Mini (4-pin) | 10 V | 20 mA | 28 mS | 0.012 pF | UHF wireless microphones, cellular LNA |
| CF750 | Siemens / Infineon | SOT-143 | 12 V | 25 mA | 22 mS | 0.018 pF | Low-noise FM broadcast front-ends |