LIN Protocol: Architecture, Standards & Engineering Guide

Welcome to the VVDN Engineering Hub LIN Protocol Knowledge Base. This documentation suite provides an exhaustive, industry-grade reference covering the single-wire physical layer, battery-level signaling, deterministic schedule tables, auto-baud clock synchronization, Protected Identifier (PID) parity, Classic vs. Enhanced checksum algorithms, LIN 2.1 / ISO 17987 specifications, and bench bring-up troubleshooting for automotive mechatronics and body electronics.

1. Executive Summary & Bus Overview

LIN (Local Interconnect Network) is a standardized, single-wire, low-cost serial communications sub-bus developed by the LIN Consortium (Motorola, Audi, BMW, DaimlerChrysler, Volvo, Volkswagen, and Volcano Automotive) in 1999 and internationally codified under ISO 17987.

Designed specifically as a cost-effective complementary sub-network beneath high-speed CAN networks, LIN eliminates unnecessary silicon and wiring costs for simple mechatronic actuators and sensors where CAN transceivers and quartz crystals are economically prohibitive.

Unlike multi-master arbitration buses (CAN) or multi-line point-to-point buses (SPI), LIN operates on a strictly deterministic Single-Master / Multi-Slave architecture:

Single-Wire Simplicity: Uses a single physical copper wire referenced to the vehicle chassis Ground (GND), drastically reducing automotive harness weight and connector pin counts.

Battery-Level Signaling: Operates directly at vehicle battery potential (V_BAT = 12 V nominal, 9 V ... 18 V dynamic operating range), eliminating the need for step-down voltage regulators on basic sensor modules.

Deterministic Scheduling: The Master Node commands 100% of bus communication via fixed Schedule Tables. Bus collisions cannot occur during normal operation, ensuring guaranteed message latency and zero jitter.

Ultra-Low Cost Slaves (No Quartz Crystal Required): LIN slaves can run on inexpensive internal RC oscillators (± 14\% drift). Every frame header begins with a known calibration byte (0x55), allowing slaves to dynamically calibrate their bit clocks to the Master's high-precision crystal clock on every single frame!

+12V Battery (VBAT) │ │ [1kΩ] [30kΩ] │─¦<─ │─¦<─ (Diode) │ │ LIN Bus ──────┼─────────┼─────────────────────────────── │ │ ┌──────┴──────┐ ┌┴────────────┐ │ Master Node │ │ Slave Node │ (Up to 15 Slaves) │ (ECU/BCM) │ │ (Door/Seat) │ └─────────────┘ └─────────────┘

2. LIN Standards & Generational Evolution

| Standard Version | Release Year | Governing Body | Key Enhancements & Milestones |
| --- | --- | --- | --- |
| LIN 1.3 | 2002 | LIN Consortium | Initial mature specification. 8-N-1 UART framing, Classic Checksum (data bytes only), fixed baud rates up to 19.2 kbps. |
| LIN 2.0 | 2003 | LIN Consortium | Major architectural upgrade: Introduced Enhanced Checksum (includes PID), diagnostic frames (0x3C, 0x3D), node capability files (NCF), and schedule table switching. |
| LIN 2.1 | 2006 | LIN Consortium | Node configuration services (Assign NAD, Conditional Change NAD), standardized Transport Layer (LIN TP), and event-triggered collision resolving schedule tables. |
| LIN 2.2A | 2010 | LIN Consortium | Clarifications on physical layer conformance, wake-up timeout handling, and node configuration responses. |
| ISO 17987 (1 to 7) | 2016 | ISO International | Transitioned LIN into formal ISO standard. Added 12V and 24V commercial vehicle electrical conformance, LIN TP testing, and formal physical layer test procedures. |

3. Protocol Comparison Matrix: LIN vs. Vehicle & Serial Buses

| Metric / Parameter | LIN (ISO 17987) | CAN 2.0 (ISO 11898) | UART (RS-232 / TTL) | I2C | K-Line (ISO 9141) |
| --- | --- | --- | --- | --- | --- |
| Signal Wires | 1 wire + GND | 2 wires (CAN_H, CAN_L) | 2 wires (TX, RX) + GND | 2 wires (SDA, SCL) + GND | 1 wire + GND (Bidirectional) |
| Operating Voltage | 12V nominal (9-18V) | 2.5V ... 3.5V diff | 3.3V / 5V or ± 12V | 1.8V ... 5.0V | 12V nominal |
| Max Bitrate | 19.2 kbps (Max 20 kbps) | 1.0 Mbps (FD: 5 Mbps) | 115.2 kbps – 1.5 Mbps | 100k, 400k, 1 Mbps | 10.4 kbps |
| Bus Topology | Single-Master / Multi-Drop | Multi-Master / Multi-Drop | Point-to-Point | Multi-Master / Multi-Drop | Point-to-Point diagnostic |
| Max Node Count | 16 nodes (1 M + 15 S) | 30 to 100+ nodes | 2 nodes | Up to 128 (7-bit addr) | 1 ECU + 1 Tester |
| Max Bus Length | 40 meters | 40m @ 1M, 1km @ 50k | < 15 meters | < 2 to 3 meters | < 10 meters |
| Clock Requirements | Master: Quartz (± 0.5\%),<br>Slave: RC osc (± 14\%) | Crystal required<br>(± 0.1\% to ± 50ppm) | Quartz / Resonator<br>(± 2\%) | Synchronous clock line<br>(Master driven) | Quartz required (± 1\%) |
| Arbitration | None (Deterministic Slot) | Non-destructive CSMA/CR | None | Open-drain Wired-AND | None |
| Typical Cost Ratio | 0.3x – 0.5x of CAN | 1.0x (Baseline) | 0.2x of CAN | 0.1x of CAN | 0.3x of CAN |
| Primary Domain | Door mirrors, seats, wipers | Powertrain, ADAS, BCM | Debug consoles, GPS | On-board sensors, PMIC | Legacy OBD-II diagnostics |

4. Documentation Suite Roadmap

The LIN documentation suite is partitioned into four deep-dive engineering modules:

protocols/lin/ ├── README.md (.docx) # Master Hub, standard comparison, roadmap & quick links ├── lin-working-and-architecture.md (.docx) # Single-wire physical layer, 12V signaling, master-slave scheduling ├── lin-frame-and-protocol-analysis.md (.docx) # Frame format (Break, Sync, PID, Data, Checksum), Classic vs Enhanced ├── lin-circuit-and-hardware-connections.md (.docx)# Schematics: Master/Slave nodes, ISO 7637-2 surge protection, harness wiring └── lin-timing-synchronization-and-hardware.md (.docx) # Auto-baud math, RC sync, slot sizing, circuitry & RCA matrix

1. Working Mechanism & Electrical Architecture

Single-wire physical layer (ISO 17987-4): Recessive (>= 0.8  V_BAT) vs Dominant (<= 0.2  V_BAT) voltage thresholds.

Termination asymmetry: Master pull-up (1 kΩ + diode) vs Slave pull-up (30 kΩ + diode).

Transceiver architectures (TJA1021, MCP2003, TLIN1029): open-drain NMOS, TXD dominant clamp timeout, and EMC slew-rate control.

Deterministic schedule table execution, cycle times, and collision-resolving tables for event-triggered frames.

Low-power Sleep Mode (<10 µA) and dominant Wake-Up pulse mechanics.

LIN 2.1 Node Configuration and Identification services (NAD, Supplier ID, Function ID).

2. Frame Data & Protocol Analysis

Master Header anatomy: Synch Break Field (>= 13 bits low), Break Delimiter (>= 1 bit high), Synch Byte (0x55), and Protected Identifier (PID).

PID parity equations (P0, P1) and the 64-frame ID allocation map.

Slave Response anatomy: 1 to 8 payload bytes in 8-N-1 UART format and 8-bit checksum.

Classic Checksum (LIN 1.3 / Diagnostic) vs Enhanced Checksum (LIN 2.x) algorithms with carry handling.

LIN Frame Types: Unconditional, Event-Triggered, Sporadic, and Diagnostic (0x3C, 0x3D).

LIN Transport Layer (LIN TP / ISO 17987-2): Single Frame (SF) and Multi-Frame (FF, CF) message flows.

Logic analyzer and oscilloscope decoded trace walk-through.

3. Hardware Connections & Circuit Schematics

Complete schematic for Automotive Master Node with 1 kΩ pull-up, reverse blocking diode, and EMC filter.

Automotive Slave Node schematic with internal 30 kΩ pull-up and 220 pF filter.

Automotive transient surge protection network (ISO 7637-2 pulse 1, 2a, 3a/3b) using TVS diodes (PESD1LIN).

Full vehicle body harness cluster wiring topology for up to 16 nodes.

4. Timing Calculations, Synchronization & Hardware Engineering

Bit timing theory (T_bit) across standard baud rates (19200, 9600, 2400 bps).

Nominal frame duration and maximum slot sizing formula (T_frame_max = 1.4 * T_frame_nom).

Slave auto-baud clock synchronization mathematics: 5 falling edges of 0x55 spanning 8 bit times (8 * T_bit).

Microcontroller timer input capture implementation and RC oscillator frequency drift compensation (± 14\% down to ± 1.5\%).

Total bus line capacitance budget (C_bus <= 10 nF) and passive rise time (t_r).

Automotive protection circuitry: Load dump (ISO 7637-2), reverse polarity diode, and TVS clamp layout.

Root Cause Analysis (RCA) troubleshooting matrix for bench and assembly bring-up.

5. Engineering CLI Utility: tools/lin_calc.py

An automated hardware calculator is provided in the repository to calculate frame slot timing, generate/verify PID parity bits, calculate Classic and Enhanced checksums, and evaluate auto-baud timer capture parameters:

# 1. Calculate frame duration, max slot time, and bus RC parameters for 19.2 kbps with 8 bytes payload python3 tools/lin_calc.py --baud 19200 --data-bytes 8 --slaves 8 # 2. Compute PID and parity bits (P0, P1) for LIN ID 0x23 python3 tools/lin_calc.py --pid 0x23 # 3. Calculate Enhanced Checksum for PID 0x23 with 4 payload bytes python3 tools/lin_calc.py --checksum --pid 0x23 --data 0x01 0x02 0x03 0x04 # 4. Calculate auto-baud timer ticks and clock resolution for 48 MHz timer clock at 19.2 kbps python3 tools/lin_calc.py --autobaud --timer-clk 48MHz --baud 19200

6. Document Synchronization

All Markdown guides in this directory are synchronized with Microsoft Word (.docx) companion files for mentor and customer deliverables using the repository generator:

python3 tools/md_to_docx.py protocols/lin/README.md python3 tools/md_to_docx.py protocols/lin/lin-working-and-architecture.md python3 tools/lin/lin-frame-and-protocol-analysis.md python3 tools/lin/lin-timing-synchronization-and-hardware.md