LIN Hardware Connection & Circuit Schematic Guide

This engineering guide provides detailed, practical hardware schematics and connection blueprints for the LIN (Local Interconnect Network, ISO 17987) single-wire automotive bus across Master and Slave nodes, automotive surge protection networks (ISO 7637-2), and vehicle body harness topologies.

1. Automotive Master Node Circuit Architecture (TJA1021 / TLIN1029-Q1)

In a LIN cluster, the Master Node coordinates all scheduling and transmits frame headers. It requires an explicit external 1 kΩ master pull-up termination path:

+12V Battery Power (VBAT / KL30) │ ├───[ Diode: BAS21 / 1N4148 ] (Prevents reverse current if VBAT drops) │ [R_master] 1.0kΩ 1% (Master Pull-Up) │ ├───┬─────────────────────────────────────────────────────────────> Single-Wire LIN Bus │ │ (To All Slaves) │ [C_master] 1.0nF Ceramic (EMC Slew-Rate Filter) │ │ │ GND │ │ LIN (Pin 6) ┌─────┴───────────────────────────────────┐ │ │ │ NXP TJA1021 / TI TLIN1029 │ │ Automotive LIN Transceiver │ │ │ MCU │ TXD (Pin 1) VBAT (Pin 7)├───[ Reverse Protection Diode ]─── +12V VBAT ────┤ (From UART TX) │ │ RXD (Pin 4) WAKE (Pin 8)├─── Open / Tie to Ground ◄───┤ (To UART RX) │ │ NSLP (Pin 2) GND (Pin 3) ├─── GND (Chassis Return) ────┤ (Sleep Control GPIO) │ └─────────────────────────────────────────┘

Critical Component Roles:

1.0 kΩ Master Termination Resistor:

The master pull-up drops the total bus impedance to ≈ 1 kΩ || (30 kΩ / N_slaves) ≈ 800 Ω ... 950 Ω.

This low impedance guarantees fast bus rise times (t_r < 5 µs) even with 10 nF total harness wiring capacitance.

Reverse Blocking Diode (BAS21 / 1N4148):

Placed in series with the 1 kΩ pull-up resistor.

If the vehicle battery voltage (V_BAT) fluctuates or drops during engine crank (6 V cold crank), this diode prevents current from back-feeding from the bus capacitance into the battery rail.

Master EMC Filter Capacitor (C_master = 1.0 nF):

Placed directly between the LIN bus pin and Ground on the master board to shape transition edges and suppress RF radiated emissions.

2. Automotive Slave Node Circuit Architecture

Slave nodes (door lock motors, seat heaters, rain/light sensors) contain an integrated internal 30 kΩ pull-up and require only minimal external passive components:

+12V Local VBAT │ ┌───────┴───────┐ │ [ 0.1µF MLCC]│ Decoupling └───────┬───────┘ │ ┌───────┴─────────────────────────────────────────┐ │ VBAT (Pin 7) │ │ Microchip MCP2003A / TI TLIN1029 │ │ Automotive Slave Transceiver │ │ │ MCU UART TX ─────────────►│ TXD (Pin 1) LIN (Pin 6) ├──────► Single-Wire LIN Bus │ │ (From Master) MCU UART RX ◄─────────────┤ RXD (Pin 4) │ │ │ │ (Integrated │ MCU Sleep GPIO ──────────►│ CS / NSLP (Pin 2) │ 30kΩ + Diode)│ │ │ │ │ GND (Pin 3) │ │ └───────┬─────────────────────────┼───────────────┘ │ │ GND [220pF] Slave EMC Filter Cap │ GND

Slave Design Guidelines:

No External 1 kΩ Resistor: Adding an external 1 kΩ pull-up to a slave node will violate LIN specification pull-up requirements and overload transceiver output transistors.

Slave Capacitance (C_slave ≈ 220 pF): The LIN specification restricts each slave node to <= 250 pF of total pin and filter capacitance to ensure a cluster of 16 slave nodes does not exceed the total maximum network capacitance limit (10 nF).

3. Automotive Transient Surge Protection Network (ISO 7637-2 / ISO 16750)

In automotive wiring harnesses, load dump transients, inductive relay kicks, and electrostatic discharge (ESD) will destroy unshielded transceivers. A dedicated automotive surge suppression network is mandatory:

LIN Transceiver Pin External Wiring Harness Connector ──────────────────┬─────────────────[ Ferrite Bead ]──────────────┬───────────────> LIN Wire to Harness │ (600Ω @ 100MHz) │ [C] 220pF ┌─┴─┐ │ │ │ Automotive TVS Diode GND │ │ (PESD1LIN / SMAJ24CA) │ │ Clamps -27V to +40V pulses └─┬─┘ │ Chassis Earth

Automotive Transient Standards Met:

ISO 7637-2 Pulse 1 (Inductive Disconnect): -100 V negative inductive spike clamped safely by the TVS forward diode drop.

ISO 7637-2 Pulse 2a / 3a / 3b (Switching & Coupling): +75 V / ± 150 V high-frequency transients absorbed by the ferrite bead and TVS.

ESD Ruggedness (IEC 61000-4-2): ± 15 kV air discharge and ± 8 kV contact discharge.

4. Complete Vehicle Body Cluster Wiring Topology

Master Electronic Control Unit (BCM / Gateway) ┌──────────────────────────────────────────────┐ │ Master Transceiver │ │ [1kΩ Pull-Up + BAS21 Diode + 1nF Filter] │ └──────────────────────┬───────────────────────┘ │ │ Single-Wire Automotive Harness (0.35 mm² to 0.5 mm² wire) │ (Total Length: Up to 40 meters, Max 16 Nodes) ├───┬───────────────────────────────┬───────────────────────────────┐ │ │ │ │ ▼ ▼ ▼ ▼ ┌────────┴──────────────┐ ┌────────┴──────────────┐ ┌────────┴──────────────┐ │ Slave Node #1 │ │ Slave Node #2 │ │ Slave Node #3 │ │ (Driver Door Lock) │ │ (Window Lifter Motor) │ │ (Side Mirror Fold) │ │ [Internal 30kΩ, 220pF]│ │ [Internal 30kΩ, 220pF]│ │ [Internal 30kΩ, 220pF]│ └───────────────────────┘ └───────────────────────┘ └───────────────────────┘