LIN Protocol: Timing Calculations, Synchronization & Hardware Engineering

1. Bit Timing & Frame Duration Calculations

LIN operates at standardized serial bitrates capped at a maximum of 20.0 kbps (with standard industry implementations operating at 19,200 bps, 9,600 bps, and 2,400 bps). The 20 kbps ceiling is an intentional electromagnetic compatibility (EMC) design constraint: by limiting the bitrate and enforcing controlled slew rates, LIN eliminates radiated RF emissions without requiring expensive shielded twisted-pair cabling.

1.1 Bit Period & Frame Duration Formulas

Given a configured baud rate (BR):

Step 1: Nominal Bit Time (T_bit)

T_bit = 1 / BR

Where:

T_bit: Nominal bit time duration (s)

BR: LIN communication baud rate (typically 19200 bps or 9600 bps) (bps)

At 19,200 bps: T_bit = 1 / 19200 ≈ 52.08 µs

At 9,600 bps: T_bit = 1 / 9600 ≈ 104.17 µs

Step 2: Nominal Header Duration (T_header_nom)

The Master Header consists of:

Synch Break: 13 dominant bits + 1 recessive delimiter bit = 14 bits

Synch Byte (0x55): 1 Start + 8 Data + 1 Stop = 10 bits

Protected ID (PID): 1 Start + 8 Data + 1 Stop = 10 bits

T_header_nom = 34 * T_bit

Step 3: Nominal Response Duration (T_response_nom)

For an N-byte payload (N in {1 ... 8}) plus 1 Checksum byte:

T_response_nom = 10 * (N + 1) * T_bit

Step 4: Total Nominal Frame Duration (T_frame_nom)

T_frame_nom = T_header_nom + T_response_nom = ( 34 + 10 * (N + 1) ) * T_bit

Step 5: Maximum Allowable Frame Slot Budget (T_frame_max)

Because slaves may have internal processing latency, inter-byte delays, and clock frequency drift, the LIN specification defines an official 1.4* slot multiplier:

T_frame_max = 1.4 * T_frame_nom

1.2 LIN Frame Timing Reference Table (at 19,200 bps)

| Payload (N) | Header Bits | Response Bits | Total Bits | Nominal Time (T_nom) | Max Slot Budget (T_max) | Max Frames / Sec |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Byte | 34 | 20 | 54 bits | 2.81 ms | 3.94 ms | 253.9 fps |
| 2 Bytes | 34 | 30 | 64 bits | 3.33 ms | 4.67 ms | 214.2 fps |
| 4 Bytes | 34 | 50 | 84 bits | 4.38 ms | 6.12 ms | 163.3 fps |
| 8 Bytes | 34 | 90 | 124 bits | 6.46 ms | 9.04 ms | 110.6 fps |

[NOTE] [!TIP] When designing Master Schedule Tables, set the slot duration to at least T_frame_max (e.g., allocate a 10.0 ms schedule slot for an 8-byte frame at 19.2 kbps) to prevent header truncation.

2. Slave Auto-Baud Clock Synchronization Mathematics

A core economic pillar of LIN is allowing high-volume slave microcontrollers to run without external quartz crystals or ceramic resonators. Instead, slaves operate from low-cost internal on-chip RC oscillators that exhibit up to ± 14\% uncalibrated frequency drift across automotive temperature extremes (-40°C ... +125°C).

2.1 The 8-Bit Sync Window Measurement

The Master initiates every frame with a precise crystal-derived Synch Byte (`0x55`).

Because 0x55 equals 01010101b, it generates 5 distinct falling edges on the UART line:

Edge 1 Edge 2 Edge 3 Edge 4 Edge 5 │ │ │ │ │ ──┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌─────── │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ ¦<- 2T ->¦<- 2T ->¦<- 2T ->¦<- 2T ->¦ ¦<--------------------- Sync Window = Exactly 8 T_bit ---------------------->¦

Timer Capture: An internal MCU hardware timer configured for Input Capture records the timestamp of Edge 1 (t_1) and Edge 5 (t_5).

Elapsed Quanta:

Δ T = t_5 - t_1 = 8 * T_bit_master

Derived Bit Period (T_bit):

T_bit = (Δ T) / 8

Hardware UART Baud Rate Register Reload:

Given an internal slave timer clock frequency f_timer:

Baud Divisor (BRR) = (Δ T_timer ticks) / 8

/* lin_autobaud_isr.c - Production Slave Auto-Baud Input Capture ISR */ #include <stdint.h> volatile uint32_t edge_timestamps[5]; volatile uint8_t edge_count = 0; volatile uint16_t measured_bit_ticks = 0; void TIM2_IRQHandler(void) { if (TIM2->SR & TIM_SR_CC1IF) { /* Read captured timer counter */ uint32_t current_tick = TIM2->CCR1; if (edge_count < 5) { edge_timestamps[edge_count++] = current_tick; } if (edge_count == 5) { /* Delta between Edge 1 and Edge 5 spans exactly 8 bit periods */ uint32_t delta_ticks = edge_timestamps[4] - edge_timestamps[0]; measured_bit_ticks = (uint16_t)(delta_ticks / 8); /* Reload hardware UART Baud Rate Register (USART_BRR) */ USART1->BRR = measured_bit_ticks; /* Switch pin from Timer Input Capture mode to UART RX mode */ reconfigure_pin_to_uart(); edge_count = 0; } } }

2.2 Oscillator Clock Tolerance Requirements (ISO 17987)

To ensure reliable data sampling without framing errors across all 8 data bytes:

| Node Type | Oscillator Requirement | Max Permissible Clock Drift |
| --- | --- | --- |
| Master Node | Quartz Crystal or High-Precision Oscillator | <= ± 0.5\% |
| Slave Node (Resonator) | Ceramic Resonator (No auto-baud calibration required) | <= ± 1.5\% |
| Slave Node (On-Chip RC) | Internal RC Oscillator (Prior to Synch Byte capture) | <= ± 14.0\% |
| Slave Node (On-Chip RC) | Internal RC Oscillator (Calibrated post-Synch capture) | <= ± 1.5\% across the entire frame |

3. Electrical Characteristics & Duty Cycle Timing Budgets

To guarantee reliable threshold detection across varying battery voltages, ISO 17987-4 specifies transmitter and receiver duty cycles across standardized voltage thresholds:

12V (VBAT) 0.744 VBAT ─────── / ─────────────┬───────────── \ ─────── TH_rec(max) = 0.744 VBAT / │ \ 0.581 VBAT ───────/─ ─ ─ ─ ─ ─ ─ ─│─ ─ ─ ─ ─ ─ ─ ─\─ ─ ─ TH_dom(max) = 0.581 VBAT / │ \ 0.422 VBAT ─────/─ ─ ─ ─ ─ ─ ─ ─ ─│─ ─ ─ ─ ─ ─ ─ ─ ─\─── TH_rec(min) = 0.422 VBAT / │ \ 0.284 VBAT ───/───────────────────┴───────────────────\─── TH_dom(min) = 0.284 VBAT GND (0V) ¦<------------- t_bus_dom -------------->¦

3.1 Transmitter Duty Cycle Equations

The physical transmitter must ensure that the generated dominant pulse duration (t_bus_dom) satisfies two formal duty cycle limits:

Duty Cycle 1 (D1) = t_bus_rec(min) / (2 * T_bit) >= 0.396

Duty Cycle 2 (D2) = t_bus_rec(max) / (2 * T_bit) <= 0.581

(where t_bus_rec is the recessive interval measured across the standardized receiver threshold levels at 19.2 kbps).

4. Circuit Design, Protection & PCB Layout

+12V Battery Supply (VBAT) │ ─┴─ Reverse Polarity Diode (e.g., BAS21 / 1N4007) ▲─┬ │ [ ] Master Pull-up: 1.0 kΩ (1%) [Slave: 30 kΩ] │ LIN Pin ──────┼──────────────────────────────┬───────────────────> LIN Bus │ │ ─┴─ Bus Capacitor ─┴─ Bidirectional TVS ─┬─ 1.0 nF (Master) / 220 pF ─┬─ (e.g., PESD1LIN) │ (Slave) │ GND GND

4.1 Bus Capacitance & Passive Rise Time Budget

The total lumped bus capacitance (C_bus) comprises the capacitance of all connected nodes plus the physical wiring harness (C_wire ≈ 100 pF/meter):

C_bus = C_master + Sum(i=1 to N) C_slave_i + (L_cable * C_wire) <= 10.0 nF

Passive RC Rise Time (t_r):

The transition from Dominant (0V) back to Recessive (12V) is driven purely by the passive pull-up resistors:

τ = R_bus_eq * C_bus

t_r ≈ 2.2 * τ = 2.2 * R_bus_eq * C_bus

For R_bus_eq = 789.5 Ω and C_bus = 4.7 nF:

t_r = 2.2 * 789.5 Ω * 4.7 * 10^-9 F ≈ 8.16 µs

Since T_bit = 52.08 µs at 19.2 kbps, an 8.16 µs rise time represents 15.6\% of the bit period, comfortably within the 20\% maximum limit.

4.2 Automotive Transient & Surge Protection (ISO 7637-2)

Vehicle 12V electrical rails routinely experience high-energy voltage transients:

Pulse 1 (Inductive load disconnect): -100 V to -150 V negative spike.

Pulse 2a (Harness inductance): +37 V to +55 V positive spike.

Pulse 3a/3b (Switch arcing & relay chatter): ± 150 V high-frequency bursts.

Pulse 5b (Alternator Load Dump): +35 V ... +40 V clamped surge lasting up to 400 ms.

Hardware Mitigation:

Protect the LIN bus pin with an automotive-grade TVS diode (e.g., Nexperia PESD1LIN or Bourns CDSOT23-T24CAN) rated for V_WM >= 24 V and breakdown voltage V_BR >= 27 V.

Insert a series blocking diode (e.g., BAS21) in series with the pull-up resistor to block reverse battery and negative inductive transients from reaching the microcontroller.

5. Root Cause Analysis (RCA) Troubleshooting Matrix

When troubleshooting LIN networks in the laboratory or on production vehicles, use this structured diagnosis table:

| Observed Symptom | Oscilloscope / Logic Analyzer Signature | Probable Root Cause | Verification & Corrective Action |
| --- | --- | --- | --- |
| Slaves ignore all Master headers; no response on the bus. | Oscilloscope shows Break field pulse duration is only 9 ... 10 bit times instead of >= 13 bits. | UART peripheral framing error: Master firmware is transmitting standard 0x00 instead of a genuine Hardware LIN Break. | Configure UART peripheral in true LIN Mode (e.g., STM32 HAL_LIN_SendBreak()) to generate an uncorrupted 13-bit dominant break. |
| Communication works for early data bytes, but 7th and 8th bytes corrupt with framing errors. | Bit edges drift progressively away from sample instants across the frame. | Slave RC oscillator frequency drift exceeding ± 1.5\% across the frame duration. | Verify slave auto-baud timer capture logic. Ensure timer clock resolution is >= 1 MHz. If operating over extreme temperature ranges, consider tuning the RC oscillator trims or selecting a ceramic resonator. |
| Master flags Checksum Error on diagnostic frames (`0x3C`/`0x3D`). | Header and payload bytes appear valid on logic analyzer, but Master rejects response. | Checksum Mode Mismatch: Slave calculated Enhanced Checksum instead of Classic Checksum. | By ISO 17987 standard, diagnostic frames (0x3C and 0x3D) must always use Classic Checksum (data bytes only, excluding PID). |
| Bus line is permanently pinned Dominant (0.8V); no communication possible. | LIN bus remains at 0.8 V constantly; no node can pull the bus high. | Transceiver TXD pin shorted to Ground or slave firmware crash holding TXD LOW on a transceiver lacking dominant timeout. | Disconnect slave nodes one by one until bus releases to 12 V. Replace old transceivers with modern ICs featuring internal TXD Dominant Clamping (e.g., TJA1021). |
| Rising edges are heavily rounded; bits fail to reach 0.8 VBAT at 19.2 kbps. | Exponential RC rise curve with t_r > 15 µs; high bits truncated. | Excessive bus capacitance (C_bus > 10 nF) or missing Master 1 kΩ pull-up resistor. | Check master termination resistor (1 kΩ to V_BAT). If cable harness is excessively long, reduce node filter capacitors from 1 nF to 220 pF. |
| Slaves fail to wake up from Sleep Mode when button pressed. | Dominant wake-up pulse on bus is only 50 µs wide. | Wake-up pulse too short: Transceiver filters require at least 150 µs ... 250 µs dominant pulse. | Increase GPIO pull-down duration in firmware to 500 µs ... 1.0 ms to ensure all sleeping transceivers reliably detect the wake-up event. |