SPI-Based RFID/NFC Transceiver Interface Architecture

1. Why SPI is the Interface of Choice for NFC Frontends

Modern NFC transceivers must bridge high-speed over-the-air RF bitstreams to a host microcontroller. While some chips offer auxiliary I2C or UART interfaces, SPI (Serial Peripheral Interface) is the industry standard for production embedded systems for the following engineering reasons:

Bandwidth & Latency:

Contactless smart card protocols (ISO/IEC 14443-4) support data rates of 106, 212, 424, and 848 kbps.

At 848 kbps, payload data arrives at sim 106 KB/s. An I2C bus at standard 100 kHz (effective sim 10 KB/s) or Fast Mode 400 kHz (effective sim 36 KB/s) cannot keep pace without massive on-chip buffering, causing internal FIFO overflows.

An SPI bus running at 5 MHz to 10 MHz transfers data at sim 600 to 1200 KB/s, draining transceiver FIFOs in microseconds.

Deterministic Full-Duplex Streaming:

SPI has zero bus arbitration overhead, zero in-band 7-bit addressing, and zero clock-stretching delays.

Interrupt Latency Tolerance:

Small hardware FIFOs (typically 64 bytes in MFRC522 or PN532) fill in less than 600\ µs at 848 kbps. High-speed SPI allows burst reads during interrupt service routines (ISRs) before buffer overrun occurs.

2. Hardware Pinout & Electrical Interconnect

A robust SPI-based NFC subsystem requires the standard 4-wire SPI bus plus two to three dedicated hardware sideband GPIO lines:

+-------------------+ +-------------------------+ ¦ Host MCU ¦ ¦ NFC Transceiver IC ¦ ¦ ¦----------- SCLK (Max 10 MHz) -------->¦ SCK / SCLK ¦ ¦ ¦----------- MOSI / COPI -------------->¦ MOSI / SDI ¦ ¦ ¦<---------- MISO / CIPO ---------------¦ MISO / SDO ¦ ¦ ¦----------- CS# / NSS ---------------->¦ NSS / CS# ¦ ¦ ¦ ¦ ¦ ¦ ¦<---------- IRQ (Interrupt Line) ------¦ IRQ (TxDone, RxDone, WL)¦ ¦ ¦----------- RSTPD# / RESET ----------->¦ RSTPD / RST (Hard Reset)¦ ¦ ¦<---------- BUSY / READY (Optional) ---¦ BUSY (Handshake Line) ¦ +-------------------+ +-------------------------+

2.1 Signal Definitions & Design Rules

NOT(CS) / NSS (Slave Select / Chip Select):

Driven active-LOW by the Host MCU.

Crucial Rule: The host must explicitly deassert CS (drive HIGH) between commands or burst transfers to reset the transceiver's internal SPI bit counters.

SCLK (Serial Clock):

Driven by Host MCU. Transceivers typically support f_SCLK up to 10 MHz (MFRC522, ST25R3916) or 5 MHz (PN532).

MOSI / SDI (Master Out, Slave In):

Serial command/data stream from Host to Transceiver.

MISO / SDO (Master In, Slave Out):

Serial status/data stream from Transceiver to Host. Enters High-Z (tri-state) when NOT(CS) is HIGH.

IRQ (Interrupt Request):

Essential for Non-Blocking Firmware: Driven by the transceiver to notify the host of critical events (TxDone, RxDone, FIFOLevel watermark, Idle, Err, Timer, RFFieldDetect).

Can be configured as active-LOW open-drain or active-HIGH push-pull depending on register settings.

NOT(RSTPD) / RST (Reset / Power-Down):

Hard reset and low-power sleep control. Driving this pin LOW shuts down internal analog oscillators, PLLs, and RF drivers, cutting quiescent current from sim 30 mA down to < 5\ µA.

BUSY / READY (PN532 Handshake):

Used in intelligent controllers to indicate internal processor execution state.

2.2 Level Shifting Considerations

Most modern Host MCUs operate at 3.3V or 1.8V I/O logic.

NFC transceivers frequently power their RF transmitter stage (TVDD or AVDD) from 5.0V to maximize magnetic field strength (H), but supply their digital logic (DVDD, VDD_IO) from 3.3V.

Always verify that VDD_IO is tied to the Host MCU supply rail to eliminate the need for external level shifters.

3. Archetype 1: Direct Register-Driven Transceivers (MFRC522, ST25R3916, CLRC663)

In register-driven transceivers, the chip has no internal general-purpose microprocessor. The host MCU directly manipulates hardware state machines, analog registers, and a hardware FIFO over SPI.

REGISTER-DRIVEN SPI FRAME FORMAT (MFRC522 EXAMPLE) Byte 0: Address & Control Byte Byte 1 .. N: Data Bytes +---+---+---+---+---+---+---+---+ +---+---+---+---+---+---+---+---+ ¦ 7 ¦ 6 ¦ 5 ¦ 4 ¦ 3 ¦ 2 ¦ 1 ¦ 0 ¦ ¦ 7 ¦ 6 ¦ 5 ¦ 4 ¦ 3 ¦ 2 ¦ 1 ¦ 0 ¦ +---+---+---+---+---+---+---+---+ +---+---+---+---+---+---+---+---+ ¦ <------- 6 Bits -------> ¦ <------ Data Payload --------> ¦ Reg Address ¦ ¦ +---> Fixed '0' +---> Read/Write Bit: 0 = Write to Register 1 = Read from Register

3.1 SPI Bus Configuration

SPI Mode: Mode 0 (CPOL = 0, CPHA = 0) — Clock idles LOW, data sampled on the rising edge, shifted on the falling edge.

Bit Order: MSB-First (Most Significant Bit transmitted first).

Max Clock Speed: Typically 10 MHz.

3.2 Register Addressing Protocol

Single Register Write:

Assert NOT(CS) LOW.

Send Byte 0: (Address << 1) & 0x7E (Bit 7 = 0 for Write; Bit 0 = 0).

Send Byte 1: Data value to write.

Deassert NOT(CS) HIGH.

Single Register Read:

Assert NOT(CS) LOW.

Send Byte 0: ((Address << 1) & 0x7E) | 0x80 (Bit 7 = 1 for Read; Bit 0 = 0).

Send Dummy Byte (0x00), simultaneously read Data value on MISO.

Deassert NOT(CS) HIGH.

Continuous Burst FIFO Read / Write:

To read/write the 64-byte FIFO without repeating address bytes:

Assert NOT(CS) LOW.

Send Byte 0: FIFO address (0x09 on MFRC522 -> Read Byte: (0x09 << 1) | 0x80 = 0x92).

Continue sending dummy bytes (read) or payload bytes (write) while holding NOT(CS) LOW. The transceiver automatically streams bytes into or out of the internal FIFO ring buffer.

Deassert NOT(CS) HIGH.

3.3 The Bit Framing Engine (BitFramingReg / TxLastBits)

Standard SPI hardware is strictly byte-oriented (transfers in multiples of 8 bits). However, NFC air interfaces require sending non-byte-aligned frames:

REQA / WUPA frames are exactly 7 bits long (0x26 or 0x52).

Anti-Collision frames can end on arbitrary bit boundaries (e.g. 23 bits, 31 bits).

To handle this, register-driven chips contain a Bit Framing Engine:

`BitFramingReg` (Address `0x0D` on MFRC522):

Bits 2:0 (TxLastBits): Defines the number of valid bits to transmit for the final byte in the FIFO (0 = all 8 bits, 1 = 1 bit, ..., 7 = 7 bits).

Bits 6:4 (RxAlign): Defines the bit position for the first received bit.

Bit 7 (StartSend): When set, immediately starts RF transmission from FIFO.

4. Archetype 2: Packet-Oriented Intelligent NFC Controllers (NXP PN532, PN7160)

The NXP PN532 contains an internal 8051 microcontroller running proprietary ROM firmware that executes the ISO 14443-3 and ISO 14443-4 state machines internally. Communication over SPI occurs via a Handshake Packet Protocol.

PN532 NORMAL SPI DATA FRAME STRUCTURE +----------+-------------+-----+-----+-----+-------------+-----+----------+ ¦ Preamble ¦ Start Code ¦ LEN ¦ LCS ¦ TFI ¦ Packet Data ¦ DCS ¦ Postamble¦ ¦ 0x00 ¦ 0x00 0xFF ¦ 1 B ¦ 1 B ¦ 1 B ¦ N Bytes ¦ 1 B ¦ 0x00 ¦ +----------+-------------+-----+-----+-----+-------------+-----+----------+ ¦<--------->¦ ¦<----------->¦ LEN + LCS=0 Sum(TFI..Data) + DCS = 0

4.1 THE INFAMOUS CRITICAL TRAP: LSB-First Bit Order

[WARNING] CRITICAL EMBEDDED PITFALL: In 99% of embedded SPI peripherals (Flash, IMUs, ADCs, MFRC522, ST25R), SPI operates MSB-First. However, the PN532 hardware SPI controller operates LSB-FIRST (Least Significant Bit transmitted first, Bit 0 before Bit 7)! If your microcontroller's SPI hardware does not support runtime hardware LSB-first switching, you must perform bit-reversal in software on every single byte transmitted and received, or all commands will be completely rejected by the PN532.

// Software bit-reversal lookup function for MSB-only SPI controllers static inline uint8_t nfc_reverse_bits(uint8_t b) { b = (b & 0xF0) >> 4 ¦ (b & 0x0F) << 4; b = (b & 0xCC) >> 2 ¦ (b & 0x33) << 2; b = (b & 0xAA) >> 1 ¦ (b & 0x55) << 1; return b; }

4.2 SPI Handshake Protocol & Flow Control

The PN532 uses three single-byte SPI command opcodes to manage bus handshaking:

`0x01` — Read Status Byte:

Host sends 0x01, then clocks in 1 byte.

Bit 0 is the `Ready` bit:

Status & 0x01 == 0x01: PN532 has finished processing and response data is ready to read.

Status & 0x01 == 0x00: PN532 is busy; host must poll again or wait for the IRQ pin.

`0x02` — Write Data:

Host sends 0x02, followed immediately by the command packet frame.

`0x03` — Read Data:

Host sends 0x03, followed by clocking in the response packet frame.

Host MCU PN532 ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x02 (Write Data Opcode) + Command Packet -------------->¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦ (PN532 processes RF / ISO commands...) ¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x01 (Read Status Opcode) ------------------------------>¦ ¦<-- Status Byte: 0x00 (Not Ready, Still Busy) ---------------¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦ [Optionally: Wait for PN532 IRQ line to go LOW] ¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x01 (Read Status Opcode) ------------------------------>¦ ¦<-- Status Byte: 0x01 (READY!) ------------------------------¦ ¦--- CS# HIGH ----------------------------------------------->¦ ¦ ¦ ¦--- CS# LOW ------------------------------------------------>¦ ¦--- 0x03 (Read Data Opcode) -------------------------------->¦ ¦<-- Response Packet Frame (TFI=0xD5, Data...) ---------------¦ ¦--- CS# HIGH ----------------------------------------------->¦

4.3 PN532 Packet Frame Fields

Preamble (`0x00`): Single synchronizing null byte.

Start Code (`0x00 0xFF`): Identifies start of packet.

Length (`LEN`): Total count of bytes in the TFI + Packet Data payload (1 <= LEN <= 255).

Length Checksum (`LCS`):

(LEN + LCS) \ & \ 0xFF = 0x00 => LCS = (simLEN + 1) \ & \ 0xFF

Target Frame Identifier (`TFI`):

0xD4: Direction = Host MCU -> PN532.

0xD5: Direction = PN532 -> Host MCU.

Data Checksum (`DCS`):

( TFI + Sum Data Bytes + DCS ) \ & \ 0xFF = 0x00

Postamble (`0x00`): Concluding byte.

4.4 Standard Handshake Packets

Acknowledge (ACK) Packet: Sent by PN532 immediately after receiving a valid command:

00 00 FF 00 FF 00

Negative Acknowledge (NACK) Packet: Indicates checksum or framing error:

00 00 FF FF 00 00

5. FIFO Buffer Architecture & Interrupt Watermark Mechanics

NFC transceivers rely on internal hardware FIFO buffers to decouple the real-time RF air interface from host SPI processing:

TRANSCEIVER HARDWARE FIFO MANAGEMENT +-------------------------------------------------------------------+ ¦ 64-Byte FIFO Ring Buffer ¦ ¦ [B0][B1][B2][B3] ... [B31] ... [B62][B63] ¦ +-------------------------------------------------------------------+ ^ ^ ¦ Write Pointer ¦ Read Pointer ¦ ¦ Air Interface RX SPI Burst Read to MCU (106 - 848 kbps) (5 - 10 MHz)

5.1 FIFO Watermark Latency Budget Calculation

Let us calculate the critical timing budget to prevent FIFO overflow during an ISO 14443-4 transmission at 848 kbps:

RF Arrival Rate:

Bit Rate = 848 kbps => Byte Rate = 848,000 / 8 = 106,000 bytes/sec = 106 bytes/ms

Buffer Fill Time:

A 64-byte FIFO will completely fill in:

t_fill = (64 bytes) / (106 bytes/ms) ≈ 0.603 ms = 603\ µs

Watermark Threshold Setting:

If the WaterLevel interrupt is programmed to fire at half-full (32 bytes):

t_margin = (32 bytes) / (106 bytes/ms) ≈ 301\ µs

Hard Real-Time Constraint: The Host MCU must service the external GPIO interrupt and execute an SPI burst read within 301\ µs, or data will be permanently lost due to FIFO overrun!

SPI Read Drain Time:

At f_SCLK = 5 MHz, transferring 32 bytes over SPI takes:

t_spi = (32 bytes * 8 bits) / (5 MHz) = 51.2\ µs

Because 51.2\ µs << 301\ µs, the SPI bus easily drains the FIFO before overflow occurs, provided the host firmware does not block inside higher-priority interrupt handlers.

6. AC Timing Specifications & Bus Constraints

To ensure data integrity, host firmware and hardware design must strictly adhere to the transceiver's AC switching characteristics:

CS# \___________________________________________________________/-----\______ ¦<-- t_lead -->¦ ¦<-- t_lag -->¦ t_CS_high ¦ SCLK ________________/-\_/-\_/-\ ... /-\_/-\_/-\_______________________/-\_/-\ MOSI -------< Bit 7 >< Bit 6 > ... < Bit 0 >-----------------------< Bit 7 > ¦<-t_su->¦<-t_h->¦ MISO -------< Bit 7 >< Bit 6 > ... < Bit 0 >--------------------------------

| Timing Parameter | Symbol | Min Value | Max Value | Description |
| --- | --- | --- | --- | --- |
| SCLK Clock Frequency | f_SCLK | DC | 10 MHz | Maximum SPI clock rate |
| SCLK Pulse Width High / Low | t_CLKH, t_CLKL | 45 ns | — | SCLK duty cycle must stay within 40%–60% |
| Chip Select Lead Time | t_lead | 50 ns | — | Delay from NOT(CS) falling edge to first SCLK edge |
| Chip Select Lag Time | t_lag | 50 ns | — | Delay from last SCLK edge to NOT(CS) rising edge |
| Chip Select Deselection Time | t_CS_high | 100 ns | — | Mandatory pause between back-to-back SPI transfers |
| Data Setup Time | t_su | 15 ns | — | Input data valid before SCLK sampling edge |
| Data Hold Time | t_h | 15 ns | — | Input data held valid after SCLK sampling edge |
| Output Valid Delay | t_v | — | 25 ns | Delay from SCLK shift edge until MISO data is stable |