SPI Modes, Timing Budgets & Propagation Delay Calculations

This document details the exact state transitions of SPI clock modes (CPOL & CPHA), shift register physics, AC timing parameters, round-trip delay calculations for high-speed operation, and lab measurement techniques.

1. Clock Polarity (CPOL) & Clock Phase (CPHA) Deep Dive

In SPI, the Serial Clock (SCLK) synchronizes all data transmission. Because SPI lacks a formal IEEE specification, device manufacturers utilize combinations of two fundamental parameters: Clock Polarity (CPOL) and Clock Phase (CPHA).

1.1 Parameter Definitions

A. Clock Polarity (CPOL)

CPOL specifies the idle logic level of the SCLK line when no data is being transferred (NOT(CS) = HIGH):

CPOL = 0: SCLK idles at Logic LOW (0V).

The leading (first) edge is a Rising Edge (0 -> 1).

The trailing (second) edge is a Falling Edge (1 -> 0).

CPOL = 1: SCLK idles at Logic HIGH (V_DD).

The leading (first) edge is a Falling Edge (1 -> 0).

The trailing (second) edge is a Rising Edge (0 -> 1).

B. Clock Phase (CPHA)

CPHA specifies the alignment between data bit transitions (toggling/shifting) and data bit sampling (latching/capturing):

CPHA = 0: Data is sampled on the Leading (first) clock edge.

The first data bit must be driven onto the bus before the first clock edge (immediately upon NOT(CS) assertion).

Subsequent data bits are shifted onto the line on the Trailing (second) clock edge.

CPHA = 1: Data is sampled on the Trailing (second) clock edge.

The first data bit is shifted onto the line on the Leading (first) clock edge.

Data bits are captured on the Trailing (second) clock edge.

1.2 Comprehensive Waveform Comparison Across All 4 Modes

The following ASCII diagram illustrates the transmission of one byte (0b10100101 / 0xA5) under each of the four standard modes:

CS# ──────┐ ┌────── └─────────────────────────────────────────────────────────────────────────────┘ --- MODE 0: CPOL=0, CPHA=0 (Idle Low, Sample on Rising Edge, Shift on Falling Edge) --- SCLK ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ──────────────┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └──────────────────────── ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ DATA ──────────────< 1 >─< 0 >─< 1 >─< 0 >─< 0 >─< 1 >─< 0 >─< 1 >────────────────────────────────────── CS asserts -> ▲ ▲ ▲ ▲ ▲ ▲ ▲ ▲ D7 Driven Sample1 Sample2 Sample3 Sample4 Sample5 Sample6 Sample7 Sample8 --- MODE 1: CPOL=0, CPHA=1 (Idle Low, Shift on Rising Edge, Sample on Falling Edge) --- SCLK ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ──────────────┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └──────────────────────── ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ DATA ──────────────────< 1 >─< 0 >─< 1 >─< 0 >─< 0 >─< 1 >─< 0 >─< 1 >────────────────────────────────── Shift1 ▲ Sample1 --- MODE 2: CPOL=1, CPHA=0 (Idle High, Sample on Falling Edge, Shift on Rising Edge) --- SCLK ──────────────┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌──────────────────────── └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ DATA ──────────────< 1 >─< 0 >─< 1 >─< 0 >─< 0 >─< 1 >─< 0 >─< 1 >────────────────────────────────────── CS asserts -> ▲ ▲ D7 Driven Sample1 Sample2 --- MODE 3: CPOL=1, CPHA=1 (Idle High, Shift on Falling Edge, Sample on Rising Edge) --- SCLK ──────────────┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌───┐ ┌──────────────────────── └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ └───┘ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ ▼ ▲ DATA ──────────────────< 1 >─< 0 >─< 1 >─< 0 >─< 0 >─< 1 >─< 0 >─< 1 >────────────────────────────────── Shift1 ▲ Sample1

1.3 Mode Matrix Summary & Industry Prevalence

| Mode | CPOL | CPHA | Idle SCLK | Data Driven / Shifted | Data Sampled / Latched | Typical Applications |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Low | Falling edge (and on NOT(CS) assert) | Rising edge | Industry Standard: NOR Flash, SD Cards, EEPROMs, IMUs |
| 1 | 0 | 1 | Low | Rising edge | Falling edge | Specialized ADCs, display controllers |
| 2 | 1 | 0 | High | Rising edge (and on NOT(CS) assert) | Falling edge | Legacy DSP peripherals, industrial converters |
| 3 | 1 | 1 | High | Falling edge | Rising edge | Flash Standard: Winbond, Macronix, Microchip memory |

[NOTE] Why Mode 0 and Mode 3 Dominate Memory: Notice that in both Mode 0 and Mode 3, data is sampled on the Rising edge and shifted on the Falling edge. The only physical difference is the idle state of the clock between frames. Consequently, almost all serial NOR Flash memories (e.g. Winbond W25Q series, Macronix MX25 series) specify support for both Mode 0 and Mode 3.

2. Shift Register Operation: Bit-by-Bit Cycle Budget

An SPI interface is physically implemented as a pair of coupled shift registers clocked simultaneously by SCLK:

+------------- CONTROLLER REGISTER (TX: 0x9F) -------------+ ¦ [1] [0] [0] [1] [1] [1] [1] [1] ¦ +---¦-------------------------------------------------------^----+ ¦ MOSI ¦ MISO v ¦ +---¦-------------------------------------------------------^----+ ¦ [0] [1] [0] [1] [0] [0] [1] [1] ¦ +-------------- PERIPHERAL REGISTER (TX: 0x53) ------------+

Trace Table for Mode 0 (8 Clock Cycles):

| Clock Pulse | Controller Shifts Out (MOSI) | Peripheral Shifts Out (MISO) | Controller Shift Reg (After Edge) | Peripheral Shift Reg (After Edge) |
| --- | --- | --- | --- | --- |
| CS LOW | Driven: 1 (Bit 7) | Driven: 0 (Bit 7) | 10011111 | 01010011 |
| Clock 1 | Latched by target (1) | Latched by host (0) | 00111110 | 10100111 |
| Clock 2 | Latched by target (0) | Latched by host (1) | 01111101 | 01001110 |
| Clock 3 | Latched by target (0) | Latched by host (0) | 11111010 | 10011100 |
| Clock 4 | Latched by target (1) | Latched by host (1) | 11110101 | 00111001 |
| Clock 5 | Latched by target (1) | Latched by host (0) | 11101010 | 01110011 |
| Clock 6 | Latched by target (1) | Latched by host (0) | 11010100 | 11100111 |
| Clock 7 | Latched by target (1) | Latched by host (1) | 10101001 | 11001111 |
| Clock 8 | Latched by target (1) | Latched by host (1) | 01010011 (0x53) | 10011111 (0x9F) |

At the completion of cycle 8:

Controller holds 0x53 (received from Peripheral).

Peripheral holds 0x9F (received from Controller).

Exact swap achieved in 8 clock cycles with zero protocol overhead.

3. AC Timing Parameters & Bus Timing Budget

At high clock frequencies (>= 20 MHz), physical propagation delays through PCB traces, digital isolators, and internal semiconductor silicon consume a major fraction of the clock period.

3.1 Key AC Timing Parameters Defined

tCSS tCSH CS# ────────┐ ┌────── └────────────────────────────────────────────────────────────┘ ¦<--- tHIGH --->¦<- tLOW ->¦ SCLK ───────────────────┌───────────────┐ ┌───────────────┐ │ └──────────┘ └─── ¦<------------ TSCLK ------------->¦ ▲ ▼ ¦<-- tSU -->¦<- tH ->¦ ¦<- tV ->¦ DATA (MOSI/MISO) ───────X───────────X────────X─────X────────X────────── ¦ Valid Input Data ¦ ¦ New Output Valid

T_SCLK (Clock Period): Total duration of one SCLK cycle (1 / f_SCLK).

t_HIGH / t_LOW (Clock Pulse Widths): SCLK High and Low durations (typically 45\% - 55\% of T_SCLK).

t_SU (Setup Time): Minimum duration data must remain stable before the active sampling clock edge.

t_H (Hold Time): Minimum duration data must remain stable after the active sampling clock edge.

t_V or t_CO (Clock-to-Output Valid Delay): Propagation time from the active clock edge until the transmitting device drives valid data onto the wire.

t_DIS (Output Disable Time): Time for the peripheral's MISO buffer to return to High-Z after NOT(CS) goes HIGH.

t_CSS (CS Setup Time): Time between NOT(CS) active (LOW) and the first SCLK edge.

t_CSH (CS Hold Time): Time between the last SCLK edge and NOT(CS) deasserting (HIGH).

t_CS_HIGH (CS Deselect / Recovery Time): Minimum inactive HIGH duration required between consecutive transactions.

3.2 Maximum Operating Frequency & Round-Trip Delay Formula

In an SPI transaction, the Controller generates SCLK.

For Write operations (MOSI): SCLK and MOSI travel in the same direction from Controller to Peripheral. Timing margin is relatively generous.

For Read operations (MISO): SCLK travels from Controller to Peripheral, triggers the Peripheral's output buffer, and MISO must travel all the way back to the Controller before the Controller's sampling edge.

This creates a Round-Trip Propagation Constraint:

T_SCLK, min > 2 * (t_prop_out + t_prop_in) + t_CO_peripheral + t_SU_controller + t_skew

For symmetrical forward and return paths:

T_SCLK, min > 2 * t_prop_path + t_CO_peripheral + t_SU_controller + t_margin

Where:

t_prop_path = t_trace + t_cable + t_isolator (one-way propagation delay).

t_CO_peripheral = Peripheral clock-to-output delay (from peripheral datasheet).

t_SU_controller = Controller input data setup time (from SoC / MCU datasheet).

t_margin = Recommended engineering safety margin (≈ 2 - 3 ns).

The maximum safe read frequency is:

f_SCLK, max = 1 / (T_SCLK, min)

3.3 Worked Real-World Engineering Scenarios

Scenario 1: High-Speed On-Board NOR Flash (Winbond W25Q128FV)

Physical Layout: On-board Flash memory located 50 mm (2 inches) from an STM32H7 MCU.

Propagation Parameters:

FR4 PCB Trace Delay: ≈ 6.5 ps/mm => t_trace = 50 mm * 6.5 ps/mm = 0.325 ns.

Isolators / Buffers: None (t_isolator = 0).

One-way path delay: t_prop_path ≈ 0.33 ns.

Silicon Parameters (from Datasheets):

Flash t_CO (Clock to output valid): 7.0 ns.

STM32H7 SPI Controller Setup Time t_SU: 3.0 ns.

Safety Margin t_margin: 2.0 ns.

Calculation:

T_SCLK, min = (2 * 0.33 ns) + 7.0 ns + 3.0 ns + 2.0 ns = 12.66 ns

f_SCLK, max = 1 / (12.66 * 10^-9 s) ≈ 78.9 MHz

Verdict: PASS for 50 MHz operation (T_SCLK = 20 ns > 12.66 ns). The interface runs reliably at 50 MHz.

Scenario 2: Off-Board Industrial Sensor with Cable & Galvanic Isolator

Physical Layout: Precision SPI ADC connected via a 30 cm ribbon cable and an optocoupler / digital isolator (e.g. ADuM1401).

Propagation Parameters:

30 cm Ribbon Cable Delay (≈ 5.0 ns/m): t_cable = 0.3 m * 5.0 ns = 1.5 ns.

Digital Isolator Propagation Delay: t_isolator = 25.0 ns per direction!

Total One-Way Path Delay: t_prop_path = 1.5 ns + 25.0 ns = 26.5 ns.

Silicon Parameters:

ADC t_CO: 18.0 ns.

Controller t_SU: 5.0 ns.

Safety Margin: 3.0 ns.

Calculation:

T_SCLK, min = (2 * 26.5 ns) + 18.0 ns + 5.0 ns + 3.0 ns = 53.0 ns + 26.0 ns = 79.0 ns

f_SCLK, max = 1 / (79.0 * 10^-9 s) ≈ 12.6 MHz

Verdict:

Attempting to run this sensor at standard 20 MHz (T_SCLK = 50 ns) will fail completely, resulting in corrupted MISO reads.

The maximum safe operating clock is 10 MHz or lower (e.g. 5–8 MHz).

4. Gross Clock Rate vs. Effective Payload Throughput

While SPI has zero frame-addressing overhead, actual payload throughput is affected by:

NOT(CS) Assertion and Deselect Overhead: Controller must lower CS before clocking and raise it between transactions.

Command & Address Overhead: In Flash and sensors, 1 to 5 bytes of command and address precede payload data.

Dummy Wait Cycles: High-speed Flash reads require 4 to 8 dummy clock cycles for internal array access.

Inter-Byte Latency: CPU interrupt-driven transfers introduce microsecond delays between bytes compared to hardware DMA streams.

4.1 Throughput Comparison Across Bus Configurations

Assumes a continuous 1 KB (1024-byte) burst read:

| Clock Rate (f_SCLK) | Raw Bitrate | Standard SPI (1-bit) Effective Throughput | Dual SPI (2-bit) Effective Throughput | Quad SPI (QSPI 4-bit) Effective Throughput |
| --- | --- | --- | --- | --- |
| 1 MHz | 1.0 Mbps | 120 KB/s (0.96 Mbps) | 235 KB/s | 450 KB/s |
| 10 MHz | 10.0 Mbps | 1.20 MB/s (9.6 Mbps) | 2.35 MB/s | 4.55 MB/s |
| 25 MHz | 25.0 Mbps | 3.05 MB/s (24.4 Mbps) | 5.95 MB/s | 11.6 MB/s |
| 50 MHz | 50.0 Mbps | 6.10 MB/s (48.8 Mbps) | 11.9 MB/s | 23.3 MB/s |
| 80 MHz | 80.0 Mbps | 9.75 MB/s (78.0 Mbps) | 19.1 MB/s | 37.5 MB/s |
| 104 MHz (QSPI) | 104.0 Mbps | — | 24.8 MB/s | 48.9 MB/s |

5. High-Speed Signal Integrity & PCB Routing Rules

At edge transition speeds under 2 ns, PCB traces behave as transmission lines regardless of the fundamental clock frequency.

5.1 Critical Trace Length Formula

A PCB trace must be treated as a transmission line when the trace length exceeds:

l_crit = t_rise / (2 * t_prop_per_cm)

On standard FR4 PCB material:

t_prop_per_cm ≈ 65 ps/cm (0.065 ns/cm)

If an MCU output has a rise time of t_rise = 1.0 ns:

l_crit = 1.0 ns / (2 * 0.065 ns/cm) ≈ 7.69 cm (≈ 3.0 inches)

Any trace longer than 7.5 cm will experience severe reflections and edge ringing without proper termination.

5.2 Source Series Damping Resistor Calculation

To eliminate ringing and double-clocking on SCLK, match the driver's output impedance to the characteristic trace impedance:

R_series = Z_0 - R_driver

Where:

Z_0 = Characteristic impedance of the PCB microstrip trace (standardly 50 Ω).

R_driver = Internal output impedance of the Controller's CMOS push-pull pin (typically 15 - 25 Ω).

Required Series Resistor: R_series ≈ 50 Ω - 20 Ω = 30 Ω (Standard values: 27 Ω to 33 Ω).

Place this resistor as close as physically possible to the Controller's SCLK and MOSI output pins.

6. Lab Measurement & Logic Analyzer Decoding Guide

6.1 Setting Up a Logic Analyzer (Saleae / Sigrok PulseView)

Sampling Rate Rule of Thumb: Set the logic analyzer sample rate to at least 4* (ideally 8* - 10*) the SPI clock rate.

For a 10 MHz SPI bus, set sample rate >= 80 MSa/s.

Channel Connections:

Channel 0: SCLK

Channel 1: MOSI (COPI)

Channel 2: MISO (CIPO)

Channel 3: NOT(CS)

Ground Lead: Connect directly to the DUT digital ground.

Trigger Configuration:

Set Trigger condition on Channel 3 (NOT(CS)) to Falling Edge.

Configuring Protocol Decoder:

Protocol: SPI

Select correct channels.

Set Polarity and Phase matching the device datasheet.

Set Bit Order: MSB First.

Set Word Length: 8 bits.

6.2 Diagnostic Rules for Scope Screen Inspection

If data byte is shifted left by 1 bit: The analyzer or MCU is sampling one edge too early (CPHA mismatch; switch CPHA from 0 to 1).

If data byte is shifted right by 1 bit: The analyzer or MCU is sampling one edge too late (switch CPHA from 1 to 0).

If data reads all `0xFF` or `0x00`: Check that NOT(CS) is actually falling LOW during the clock burst; if NOT(CS) remains HIGH, the target's MISO output is tri-stated.

7. Using the Workspace SPI Calculator Tool

You can calculate clock periods, round-trip timing budgets, maximum safe read frequencies, and payload throughput using the workspace tool:

# Example 1: Standard on-board Flash (5 cm trace, tCO = 7 ns, tSU = 3 ns) python3 tools/spi_calc.py --clock 50MHz --trace-cm 5 --tco 7 --tsu 3 # Example 2: Off-board sensor with 30 cm ribbon cable and 25 ns digital isolator python3 tools/spi_calc.py --clock 10MHz --trace-cm 5 --cable-cm 30 --isolator-ns 25 --tco 15 --tsu 5 # Example 3: Quad-SPI (QSPI) throughput analysis at 104 MHz python3 tools/spi_calc.py --clock 104MHz --qspi