UART Protocol: Architecture, Hardware, and Debugging Guide

1. What is UART?

UART stands for Universal Asynchronous Receiver-Transmitter.

Universal: Configurable parameters (baud rate, data length, parity, stop bits).

Asynchronous: No shared clock line is transmitted between devices (unlike SPI or I2C). Both devices must agree on transmission speed (baud rate) in advance.

Receiver-Transmitter: Dual independent channels allow simultaneous transmit and receive (Full Duplex).

2. UART Frame Anatomy

In UART, data is transmitted one bit at a time over a single wire. When no data is transmitted, the line is in an Idle state (Logic High / Mark).

Idle (High) ──────┐ ┌───┬───┬───┬───┬───┬───┬───┬───┐ ┌────── Idle (High) │ Start(0) │D0 │D1 │D2 │D3 │D4 │D5 │D6 │D7 │ Parity│Stop(1) └──────────┴───┴───┴───┴───┴───┴───┴───┴───┴───────┘ (Data bits: LSB first to MSB)

Components of a Frame:

Start Bit (1 bit, always Logic 0 / Low):

Transitions the line from High to Low.

Signals the receiver's internal counter to start clocking and sample incoming bits.

Data Bits (typically 8 bits, can be 5 to 9):

Sent LSB (Least Significant Bit) first, ending with MSB.

Parity Bit (Optional, 1 bit):

Used for simple error detection.

None (N): No parity bit sent.

Even (E): Set to 1 or 0 so the total count of 1s (including parity) is even.

Odd (O): Set to 1 or 0 so the total count of 1s (including parity) is odd.

Stop Bit (1, 1.5, or 2 bits, always Logic 1 / High):

Brings the line back to the Idle (High) state.

Gives the receiver time to process the received byte before the next Start bit.

[NOTE] Industry Default: 8-N-1 (8 Data bits, No Parity, 1 Stop bit) = 10 physical bits per transmitted byte.

3. Baud Rate: Significance, Hardware Generation & Calculations

[NOTE] A dedicated deep-dive document with worked examples and error analysis is available at baud-rate-calculations.md.

3.1 What Does Baud Rate Signify?

Baud Rate is named after French engineer Émile Baudot. It measures the number of signal changes (symbol changes or modulations) occurring per second on the communication line.

A. Baud Rate vs. Bit Rate

Bit Rate (bps): The actual number of information bits transmitted per second.

Baud Rate (Bd): The number of physical line-state transitions per second.

In binary UART: Each state change represents exactly one bit (0V = 0, 3.3V = 1). Therefore:

Baud Rate (symbols/sec) = Bit Rate (bits/sec)

(In multi-level modulations like QAM or PAM4, one baud can carry 2, 4, or more bits. In UART, 1 Baud = 1 bps).

B. Physical Significance: Bit Period (T_bit)

The baud rate strictly determines the physical duration in time that every single bit occupies on the wire:

T_bit = 1 / (Baud Rate)

At 115,200 Baud:

T_bit = 1 / 115200 ≈ 8.68 * 10^-6 s = 8.68 µs

Every bit (Start, Data 0-7, Parity, Stop) must be held on the wire for exactly 8.68 µs. Because UART is asynchronous (no clock wire), both sender and receiver rely on internal timers calibrated to this exact interval.

C. Gross Baud Rate vs. Actual Useful Payload Throughput

In an 8-N-1 configuration, transmitting 1 byte (8 bits) of real payload requires:

Total Bits on Wire = 1 Start + 8 Data + 0 Parity + 1 Stop = 10 bits

Protocol efficiency is 8 / 10 = 80\%.

Effective Data Throughput (Bytes per second):

Payload Rate (Bytes/s) = (Baud Rate) / (Total Bits per Frame) = (Baud Rate) / 10

| Baud Rate | T_bit | Frame Time (10 bits) | Theoretical Payload Throughput |
| --- | --- | --- | --- |
| 9600 | 104.17 µs | 1.042 ms | 960 Bytes/s (0.94 KB/s) |
| 19200 | 52.08 µs | 520.8 µs | 1,920 Bytes/s (1.88 KB/s) |
| 38400 | 26.04 µs | 260.4 µs | 3,840 Bytes/s (3.75 KB/s) |
| 57600 | 17.36 µs | 173.6 µs | 5,760 Bytes/s (5.63 KB/s) |
| 115200 | 8.68 µs | 86.8 µs | 11,520 Bytes/s (11.25 KB/s) |
| 921600 | 1.085 µs | 10.85 µs | 92,160 Bytes/s (90.0 KB/s) |
| 1500000 | 0.667 µs | 6.67 µs | 150,000 Bytes/s (146.5 KB/s) |

3.2 How Hardware Calculates & Generates the Baud Rate (Baud Rate Generator / BRG)

Microcontrollers and SoCs (e.g., STM32, ESP32, NXP, Qualcomm, 16550 UART) generate baud rates from an internal system/peripheral clock (f_clk).

A. The Oversampling Mechanism

To reliably sample asynchronous data without a shared clock:

The receiver runs an oversampling clock, standardly at 16* the target baud rate (some controllers allow 8*).

When the line drops from High to Low (Start bit falling edge), the UART counter starts.

At tick 7 or 8 (midpoint of the start bit), the receiver samples again to verify it is a valid start bit (not a noise glitch).

Each subsequent bit is sampled 16 ticks later, guaranteeing sampling at the exact middle of each bit period where the signal is cleanest.

Clock Ticks (16x): 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 ... 23 24 Line Level: HIGH ────────┐ ┌───────────── └──────── START BIT (Low) ─────────┘ DATA D0... Sample Point: ▲ (Tick 7-8: Midpoint) ▲ (+16 Ticks)

B. The Hardware Divisor Formula

The peripheral clock f_clk is divided down using a register called the Divisor (e.g., UBRR in AVR, USART_BRR in STM32, DLL/DLM in 16550):

Baud Rate = f_clk / (OS * Divisor)

Where:

f_clk = Clock frequency supplied to the UART peripheral (in Hz).

OS = Oversampling factor (typically 16).

Divisor = Value loaded into the Baud Rate Generator register.

C. Calculating the Required Divisor:

Divisor = f_clk / (OS * Baud_target)

3.3 Baud Rate Error & Tolerance Calculation

In hardware, the divisor is often an integer register. When f_clk / (OS  Baud) does not yield a whole number, rounding creates a baud rate error*.

A. Actual Baud Rate & Percentage Error Formulas

Baud_actual = f_clk / (OS * Divisor_rounded)

Error (\%) = ( (Baud_actual - Baud_target) / Baud_target ) * 100\%

B. Why Does Error Cause Framing Errors? (Cumulative Phase Drift)

The receiver resynchronizes its clock only at the falling edge of the Start bit.

For all 8 data bits, parity, and the stop bit (up to 10 bit periods), the receiver runs purely open-loop.

With each passing bit, any timing mismatch accumulates:

Δ t_accumulated = N * (T_bit, actual - T_bit, target)

By bit 9 or 10 (the Stop bit), if the sampling point drifts by more than ± 0.5 * T_bit, the receiver will sample during a logic transition or adjacent bit.

Rule of Thumb: Total clock difference between sender and receiver must remain within ± 2.5\% to ± 3\%. Exceeding 3\% results in corrupted bytes and Framing Errors (FE).

3.4 Practical Real-World Examples

Example 1: 16 MHz Clock with Integer Divisor (The "Bad Clock" Problem)

Given: f_clk = 16 MHz = 16,000,000 Hz, Target Baud = 115200, OS = 16.

Compute Divisor:

Divisor = 16,000,000 / (16 * 115200) = 16,000,000 / 1,843,200 ≈ 8.68055

An integer-only register must round to 9 (or 8):

With Divisor = 9:

Baud_actual = 16,000,000 / (16 * 9) = 111,111.1 bps

Error = ( (111111.1 - 115200) / 115200 ) * 100\% = -3.55\%

Verdict: Fails! -3.55\% exceeds the 3\% limit. Communication will suffer frequent framing errors and corrupted packets.

Example 2: Fractional Baud Rate Generator (Modern MCUs / SoCs)

Modern microcontrollers (like STM32) provide fractional dividers (e.g. 12-bit Mantissa + 4-bit Fraction):

Ideal Divisor = 8.68055

Mantissa = 8

Fraction = round(0.68055 * 16) = round(10.88) = 11

Effective Divisor = 8 + 11 / 16 = 8.6875

Recalculate Actual Baud:

Baud_actual = 16,000,000 / (16 * 8.6875) = 115,107.9 bps

Error = ( (115107.9 - 115200) / 115200 ) * 100\% = -0.08\%

Verdict: Passes with flying colors! (< 0.1\% error).

Example 3: Why "Baud Rate Friendly" Crystals Exist (11.0592 MHz, 18.432 MHz)

In older hardware (8051, AVR, legacy industrial controllers), you will see unusual crystal frequencies like 11.0592 MHz:

Divisor for 115,200 baud:

Divisor = 11,059,200 / (16 * 115200) = 11,059,200 / 1,843,200 = 6.0000

Divisor for 9600 baud:

Divisor = 11,059,200 / (16 * 9600) = 72.0000

Because 11.0592 MHz is an exact multiple of all standard baud rates, integer divisors yield 0.00\% baud rate error.

3.5 How to Measure Unknown Baud Rate in the Lab (Oscilloscope / Logic Analyzer)

When connecting to an unknown board or unlabelled test point:

Connect oscilloscope / logic analyzer probe to the board's TX pin and GND.

Trigger the scope on a falling edge while the board boots or transmits data.

Zoom in and find the narrowest single pulse (shortest high or low pulse). In random data, the narrowest pulse corresponds to a single bit (1 * T_bit).

Measure the pulse width with cursor Δ t.

Compute the baud rate:

Baud Rate = 1 / (Δ t)

Measurement Cheatsheet:

If Δ t ≈ 104 µs -> 9600 Baud

If Δ t ≈ 26.0 µs -> 38400 Baud

If Δ t ≈ 8.68 µs -> 115200 Baud

If Δ t ≈ 1.08 µs -> 921600 Baud

If Δ t ≈ 0.67 µs -> 1500000 (1.5M) Baud

4. Electrical Signatures & Standards

UART is the logical protocol (controller/state machine). The physical voltage on the wire depends on the standard:

| Standard | Logic 0 | Logic 1 | Typical Distance | Topology |
| --- | --- | --- | --- | --- |
| TTL / CMOS (3.3V) | 0 V | 3.3 V | < 1 meter (board-level) | Point-to-point |
| TTL / CMOS (1.8V) | 0 V | 1.8 V | < 0.5 meter (modern SoCs) | Point-to-point |
| RS-232 | +3 V... +15 V | -3 V... -15 V | Up to 15 meters | Point-to-point (Inverted logic!) |
| RS-485 | Differential: V_A - V_B < -200 mV | Differential: V_A - V_B > +200 mV | Up to 1200 meters | Multi-drop bus (up to 32+ devices) |

[CAUTION] Never connect an RS-232 cable directly to a TTL UART header (e.g. Raspberry Pi, Qualcomm/NXP/Rockchip SoC header). RS-232 voltages (± 12V) will permanently fry the SoC pins. Always use a level converter/transceiver (e.g., MAX3232).

5. Flow Control

When the receiver's hardware FIFO or software buffer fills up, it must tell the sender to pause.

None:

Assumes the receiver can always keep up. If receiver drops bytes, data is lost (Buffer Overrun).

Hardware Flow Control (RTS/CTS):

Uses dedicated lines:

RTS (Request to Send): Driven by receiver to indicate readiness to accept data.

CTS (Clear to Send): Read by sender before pushing bytes onto TX.

Extremely reliable and has zero protocol overhead.

Software Flow Control (XON / XOFF):

Uses in-band control characters over the data line:

XOFF (0x13, DC3 / Ctrl+S): "Pause transmission"

XON (0x11, DC1 / Ctrl+Q): "Resume transmission"

Not suitable for binary data transfer unless escaped.

6. Linux Serial Subsystem & Debugging Tools

Identifying Ports:

# Check kernel detection of USB serial converters (FTDI, CP210x, CH340, PL2303) dmesg ¦ grep -E "tty¦usb" # List available tty devices ls -l /dev/ttyUSB* /dev/ttyACM* /dev/ttyS*

Checking & Modifying Port Parameters (stty):

# View current configuration stty -F /dev/ttyUSB0 -a # Set baud rate to 115200 with raw mode (no echo, no carriage return translation) stty -F /dev/ttyUSB0 115200 cs8 -cstopb -parenb raw -echo

Common Terminal Emulators:

picocom (Lightweight, recommended for embedded bringup):

picocom -b 115200 /dev/ttyUSB0 # Exit: Ctrl+A then Ctrl+X

minicom:

minicom -D /dev/ttyUSB0 -b 115200 # Configure: Ctrl+A then Z

screen:

screen /dev/ttyUSB0 115200 # Exit: Ctrl+A then k, then y

7. Code Examples

A. Python (pyserial)

import serial import time def run_uart(): # Initialize port ser = serial.Serial( port="/dev/ttyUSB0", baudrate=115200, bytesize=serial.EIGHTBITS, parity=serial.PARITY_NONE, stopbits=serial.STOPBITS_ONE, timeout=1.0 # Read timeout in seconds ) try: # Send data message = b"hello target\n" ser.write(message) print(f"Sent: {message}") # Read response response = ser.readline() print(f"Received: {response.decode(errors='replace')}") finally: ser.close() if __name__ == "__main__": run_uart()

B. POSIX C (termios)

#include <stdio.h> #include <stdlib.h> #include <fcntl.h> #include <unistd.h> #include <termios.h> #include <string.h> int open_uart(const char *portname, speed_t baud) { int fd = open(portname, O_RDWR ¦ O_NOCTTY ¦ O_SYNC); if (fd < 0) { perror("Error opening serial port"); return -1; } struct termios tty; if (tcgetattr(fd, &tty) != 0) { perror("Error from tcgetattr"); close(fd); return -1; } // Set baud rates cfsetospeed(&tty, baud); cfsetispeed(&tty, baud); // 8-N-1 configuration tty.c_cflag = (tty.c_cflag & ~CSIZE) ¦ CS8; // 8-bit chars tty.c_cflag &= ~PARENB; // No parity tty.c_cflag &= ~CSTOPB; // 1 stop bit tty.c_cflag &= ~CRTSCTS; // No hardware flow control tty.c_cflag ¦= (CLOCAL ¦ CREAD); // Enable read & ignore ctrl lines // Raw input mode tty.c_lflag &= ~(ICANON ¦ ECHO ¦ ECHOE ¦ ISIG); tty.c_iflag &= ~(IXON ¦ IXOFF ¦ IXANY); // Turn off s/w flow control tty.c_iflag &= ~(IGNBRK ¦ BRKINT ¦ PARMRK ¦ ISTRIP ¦ INLCR ¦ IGNCR ¦ ICRNL); // Raw output mode tty.c_oflag &= ~OPOST; // Read timeout: 0.5s (VTIME in deciseconds), minimum 0 bytes tty.c_cc[VMIN] = 0; tty.c_cc[VTIME] = 5; if (tcsetattr(fd, TCSANOW, &tty) != 0) { perror("Error from tcsetattr"); close(fd); return -1; } return fd; } int main() { int fd = open_uart("/dev/ttyUSB0", B115200); if (fd < 0) return 1; char buf[128]; int n = read(fd, buf, sizeof(buf) - 1); if (n > 0) { buf[n] = '\0'; printf("Received %d bytes: %s\n", n, buf); } close(fd); return 0; }

8. Common Errors & Troubleshooting Checklist

Framing Error (FE):

Symptom: Receiver detected invalid Stop bit (sampled 0 instead of 1).

Cause: Baud rate mismatch, noisy signal lines, or clock drift exceeding ~3%.

Parity Error (PE):

Symptom: Bit count does not match expected parity bit.

Cause: Corrupted bit over long wire or mismatched parity setting (e.g. Even vs None).

Overrun Error (OE):

Symptom: New character arrived in UART buffer before CPU/driver read previous character.

Cause: High interrupt latency, baud rate too fast for CPU, or lack of hardware flow control.

Break Condition:

Symptom: Continuous logic Low (0) for longer than one full frame.

Cause: Cable disconnected, RX floating, or target board held in reset.

Garbage Characters:

Cause: Mismatched baud rate (e.g., target sending at 115200, host listening at 9600 or 1500000).

No Output At All:

Check:

Are TX and RX swapped? (Target TX must go to Host RX).

Is GND connected? (Without common reference, voltages cannot be measured).

Is the device actually booted / receiving power?