Power Electronics Converters: Complete Architectural, Design & Study Guide

Welcome to the VVDN Engineering Hub Power Electronics Converters Knowledge Base. This repository is an exhaustive, mathematically rigorous, and practical hardware engineering reference covering the four fundamental conversion domains: DC-DC, AC-AC, DC-AC, and AC-DC.

1. Universal Power Electronics Conversion Matrix

Power electronics converters transform electrical energy between direct current (DC) and alternating current (AC) with maximum efficiency using solid-state semiconductor switching devices (MOSFETs, IGBTs, Thyristors/SCRs, Diodes, and Wide-Bandgap SiC/GaN):

┌───────────────────────────────────────────────────────────────────────────────────┐ │ The Four Fundamental Conversion Quadrants │ ├─────────────────────┬─────────────────────────────────┬───────────────────────────┤ │ From \ To │ DC (Direct Current) │ AC (Alternating Current) │ ├─────────────────────┼─────────────────────────────────┼───────────────────────────┤ │ **DC** │ **DC-DC Converters** │ **DC-AC Inverters** │ │ │ - Isolated: Forward, Flyback, │ - Single-Phase: Half/Full │ │ │ DAB, Full/Half-Bridge, Push- │ Bridge, Bipolar/Unipolar│ │ │ Pull, Resonant (LLC/SRC/PRC) │ - Three-Phase: 6-Switch │ │ │ - Non-Isolated: Buck, Boost, │ VSI, 180°/120°, SPWM, │ │ │ Buck-Boost, Cuk, SEPIC, Zeta │ Space Vector PWM (SVPWM)│ ├─────────────────────┼─────────────────────────────────┼───────────────────────────┤ │ **AC** │ **AC-DC Rectifiers** │ **AC-AC Converters** │ │ │ - Uncontrolled: 1-Ph & 3-Ph │ - Cycloconverters: Line- │ │ │ Diode Bridges, Ripple factor │ commutated, frequency │ │ │ - Controlled: Thyristor Phase- │ reduction (P/N banks) │ │ │ Controlled (Semi/Full Conv.) │ - AC Voltage Controllers: │ │ │ - Active PFC / AFE: Boost PFC, │ Phase-angle control, │ │ │ Bridgeless, Vienna Rectifier │ Integral cycle (TRIAC) │ └─────────────────────┴─────────────────────────────────┴───────────────────────────┘

2. Directory Navigation & Modular Architecture

This repository is partitioned into dedicated domain directories, each containing specialized, technical dossiers:

A. DC-DC Converters (`dc-dc/README.md`)

Isolated Topologies:

Forward Converter (DOCX)

Flyback Converter (DOCX)

Dual Active Bridge (DAB) (DOCX)

Full-Bridge Converter (PSFB) (DOCX)

Half-Bridge Converter (DOCX)

Push-Pull Converter (DOCX)

Resonant Converters (SRC, PRC, LLC) [IMP] (DOCX)

Non-Isolated Topologies:

Buck Converter (DOCX)

Boost Converter (DOCX)

Buck-Boost Converter (Inverting & 4-Switch FSBB) (DOCX)

Ćuk Converter (DOCX)

SEPIC Converter (DOCX)

Zeta Converter (DOCX)

B. AC-AC Converters (`ac-ac/README.md`)

Cycloconverters (DOCX)

AC Voltage Controllers (DOCX)

C. DC-AC Inverters (`dc-ac/README.md`)

Single-Phase Inverters (DOCX)

Three-Phase Inverters & Space Vector PWM (SVPWM) (DOCX)

D. AC-DC Rectifiers (`ac-dc/README.md`)

Uncontrolled Diode Rectifiers (DOCX)

Controlled Phase-Rectifiers (Semi/Full & Inversion) (DOCX)

Active PFC Rectifiers (Boost, Totem-Pole & Vienna) (DOCX)

3. High-Level Comparison & Selection Benchmark

| Conversion Class | Typical Topologies | Power Range | Typical Efficiency | Dominant Semiconductor | Target Applications |
| --- | --- | --- | --- | --- | --- |
| DC-DC Isolated | Flyback, Forward, DAB, LLC, PSFB | 5\,W \dots 50\,kW | 88\% \dots 98\% | MOSFET / SiC / GaN | EV On-board Chargers, Server PSUs, Telecom bricks |
| DC-DC Non-Isolated | Buck, Boost, Buck-Boost, Cuk, SEPIC | 1\,W \dots 5\,kW | 92\% \dots 98.5\% | Low-R_{DS(on) Trench FET | Point-of-Load (PoL), Battery chargers, LED drivers |
| AC-AC Direct | Cycloconverter, AC Voltage Controller | 1\,kW \dots 20\,MW | 95\% \dots 99\% | Phase-Control Thyristor / TRIAC | Heavy industrial grinding mills, soft starters, dimmers |
| DC-AC Inverters | 1-Phase & 3-Phase VSI, SVPWM | 500\,W \dots 1\,MW | 95\% \dots 99\% | IGBT / SiC MOSFET | EV Traction Inverters, Solar Grid-Tie, Motor Drives |
| AC-DC Rectifiers | Diode Bridge, Active Boost PFC | 10\,W \dots 500\,kW | 90\% \dots 98.5\% | Diode / Fast Recovery / SiC | Power supplies, EV DC Fast Chargers, Industrial DC bus |

4. Document Synchronization

All technical dossiers in this knowledge base are maintained simultaneously as GitHub-Flavored Markdown (`.md`) for version-controlled study and Microsoft Word (`.docx`) for enterprise sharing and mentor reviews using:

python3 tools/md_to_docx.py <path/to/document.md>