Uncontrolled Diode Rectifiers: 1-Phase & 3-Phase Bridge Physics & Filter Sizing

Welcome to the VVDN Engineering Hub Technical Dossier on Uncontrolled Diode Rectifiers. Uncontrolled rectifiers utilize passive semiconductor PN-junction or Schottky diodes to achieve AC-to-DC conversion without active gating signals. They are ubiquitous as input rectification stages in legacy linear power supplies, low-cost offline switched-mode power supplies (SMPS under 75\,W), and industrial motor drive front-ends.

This guide explores single-phase and three-phase rectifier topologies, capacitor filtering physics, diode conduction angle contraction, crest factor, Peak Inverse Voltage (PIV) ratings, and inrush surge protection.

1. Operating Principle & Rectifier Architectures

1.1 Single-Phase Full-Wave Diode Bridge (Graetz Bridge):

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: SINGLE-PHASE AC-DC BRIDGE RECTIFIER WITH FRONTEND EMI & INRUSH PROTECTION ======================================================================================================================== AC LINE (230V RMS) ──[ F1: 6.3A Time-Lag ]──[ NTC: 10Ω/5A Inrush ]──┬──────────────────┐ (Relay K1 Bypass) │ │ ┌┴┐ MOV1 ┌┴┐ CX1: X2 Cap │ │ 14D471K │ │ 0.47µF / 310VAC └┬┘ (Surge 4.5kA) └┬┘ (Differential EMI) │ │ AC NEUT (0V RMS) ───────────────────────────────────────────────────┼──────────────────┴───+ │ │ │ COMMON-MODE CHOKE │ ├──[ CMC1: 2x 15mH ]───┼──────────┐ │ Wurth 744825315 │ │ │ │ │ ┌┴┐ CY1 ┌┴┐ CY2 │ │ │ 2.2nF/400V Y2 │ │ 2.2nF │ └┬┘ └┬┘ │ EARTH (PE) ─────────────────────────────────────────────────────────┴──────────────────────┴───+ │ │ │ +---[ FULL-WAVE BRIDGE RECTIFIER (e.g. GBU808: 800V/8A) ]------+ ¦ ¦ ¦ [ AC1: Pin 2 ] ------------------------------------------+ ¦ [ AC2: Pin 3 ] <-----------------------------------------+ ¦ ¦ ¦ [ +DC: Pin 1 ] ──────────────┬───────────────────────────+ ¦ [ -DC: Pin 4 ] ──┐ │ +----------------------¦-----------¦---------------------------+ │ │ │ +=== +VDC BUS (+325V Peak DC) │ │ │ ┌┴──────────────────┐ │ │ C_BULK BANK │ │ ┌┴┐ 2x 470µF/450V ┌┴┐ C_HF_CER │ │ │ Aluminum Elec. │ │ 2x 0.47µF/630V │ └┬┘ (Low ESR) └┬┘ Film / X7R │ │ │ │ │ +VDC │ │ │ │ │ │ │ [ R_bleed1: 150kΩ / 1W ] │ │ [ R_bleed2: 150kΩ / 1W ] │ │ │ │ │ │ (Safety Discharge) │ │ │ │ +──────────┴────────+──────────┴───> -VDC (DC Return GND)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| AC_LINE_RAW | AC Input IEC C14 Receptacle (L) | Fuse F_1 Input | 230V AC Live utility mains | Sized for 6.3A time-lag rating to survive startup capacitive charging surges. |
| AC_LINE_PROT | Fuse F_1 Output | NTC Thermistor Pin 1, MOV1 Pin 1, C_{X1 Pin 1 | Protected Live rail | MOV1 clamps line voltage spikes exceeding 385\,V_{RMS / 505\,V_{DC. |
| AC1 (Bridge) | Common Mode Choke Pin 2 | GBU808 Bridge Rectifier Pin 2 (AC1) | Filtered AC Line input | Input to leg formed by top diode D_1 and bottom diode D_4. |
| AC2 (Bridge) | Common Mode Choke Pin 4 | GBU808 Bridge Rectifier Pin 3 (AC2) | Filtered AC Neutral input | Input to leg formed by top diode D_3 and bottom diode D_2. |
| +VDC (DC Bus) | GBU808 Bridge Rectifier Pin 1 (+) | C_{bulk (+), Bleeder Resistors, Load (+) | Full-wave rectified pulsating DC bus | Peak voltage V_{pk = \sqrt{2 \cdot 230\,V \approx 325\,V; double line frequency ripple (100\,Hz). |
| -VDC (GND) | GBU808 Bridge Rectifier Pin 4 (-) | C_{bulk (-), Bleeder Return, Load Return | Rectifier zero-volt DC return | Common cathode-referenced return for downstream converters. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| $BR_1$ | Glass Passivated Bridge | Diodes Inc. GBU808 | V_{RRM = 800\,V, I_{F(AV) = 8.0\,A, I_{FSM = 200\,A, V_F = 1.0\,V | 800\,V rating survives 2.5\,kV ring-wave surges; heatsink mounted for loads > 3\,A. |
| $C_{bulk}$ | Bulk Filter Capacitors | Nichicon LGU2W471MELY | 2 × 470\,µF, 450\,V_{DC, 105^\circC, ESR = 0.22\,\Omega | Sized to ensure DC bus hold-up time t_{hold ≥ 20\,ms during AC utility cycle dropouts. |
| $NTC_1$ | Inrush Current Limiter | TDK / EPCOS B57237S0100M000 | R_{25 = 10.0\,\Omega, I_{max = 5.0\,A, Energy = 55\,J | Limits cold-turn-on inrush current to I_{inrush = \frac{325\,V{10\,\Omega = 32.5\,A. |
| $MOV_1$ | Metal Oxide Varistor | Littelfuse V275LA20CP | V_{RMS = 275\,V, V_{DC = 369\,V, I_{peak = 6500\,A, W_{max = 120\,J | Clamps lightning and grid switching transients per IEC 61000-4-5 Class 3. |
| $CMC_1$ | Common-Mode Choke | Würth Elektronik 744825315 | 2 × 15.0\,mH, I_{rated = 3.5\,A, DCR = 82\,m\Omega | High common-mode impedance (> 5\,k\Omega at 1\,MHz) attenuates conducted EMI. |
| $C_{X1}, C_{Y1/2}$ | Safety EMI Filter Caps | Vishay F1772 (X2) / Murata (Y2) | C_X = 0.47\,µF / 310\,V_{AC, C_Y = 2.2\,nF / 400\,V_{AC | Certified to UL/ENEC 60384-14 for direct across-the-line and line-to-earth suppression. |

1.3 Three-Phase Six-Pulse Diode Bridge Rectifier:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: THREE-PHASE 6-PULSE DIODE BRIDGE RECTIFIER (400V LINE-LINE RMS -> 540V DC) ======================================================================================================================== LINE L1 (R-Phase) ──[ F1: 20A ]──┬─────────────────────────────────────────────────────────────┐ LINE L2 (S-Phase) ──[ F2: 20A ]──┼───────────────┬─────────────────────────────────────────────┼──────────────┐ LINE L3 (T-Phase) ──[ F3: 20A ]──┼───────────────┼───────────────┬─────────────────────────────┼──────────────┼──────────────┐ │ │ │ │ │ │ ┌┴┐ D1 ┌┴┐ D3 ┌┴┐ D5 │ │ │ │>│ Diodes Inc. │>│ Diodes Inc. │>│ Diodes Inc. │ │ │ │ │ 40TPS12 │ │ 40TPS12 │ │ 40TPS12 │ │ │ └┬┘ (1200V/35A) └┬┘ (1200V/35A) └┬┘ (1200V/35A) │ │ │ │ │ │ │ │ │ +---------------+---------------+=== COMMON CATHODE BUS │ │ │ ¦ (Positive DC Output) │ │ │ ¦ │ │ │ ├──[ L_dc: 2.5mH / 30A Choke ]┼──────────────┼──────────────┼───> +VDC (+540V) ¦ (Damps 300Hz Ripple) │ │ │ │ ┌┴┐ D4 ┌┴┐ D6 ┌┴┐ D2 │ ┌┴────────────────┐ │>│ 40TPS12 │>│ 40TPS12 │>│ 40TPS12 │ │ C_DC BANK │ └┬┘ (Anode to DC-) └┬┘ └┬┘ │ ┌┴┐ 2x 1000µF/450V │ │ │ │ │ │ │ Series Paired │ +─────────────────────────────+──────────────+──────────────┴───└┬┘ (ESR = 45mΩ) │ ¦ │ (Bleeders 100k)│ +=================================================================+───> -VDC (GND)

Top diodes (D_1, D_3, D_5) conduct whichever phase has the highest positive potential.

Bottom diodes (D_4, D_6, D_2) conduct whichever phase has the most negative potential.

Commutation occurs naturally every 60^\circ (\frac{\pi{3 radians).

Fundamental DC Output Ripple Frequency:

f_{ripple} = 6 \cdot f_{line} = 6 \cdot 50\,\text{Hz} = 300\,\text{Hz}

Average DC Voltage (without filter cap):

V_{dc,avg} = \frac{3 \sqrt{3}}{\pi} \cdot \hat{V}_{phase} = \frac{3}{\pi} \cdot \hat{V}_{line-line} \approx 1.35 \cdot V_{LL,rms}

(For $400\,\text{V}_{rms}$ three-phase grid, $V_{dc,avg} \approx 540\,\text{V}$ DC).

2. Capacitive Filter Physics & Diode Conduction Angle

When a bulk capacitor C_{bulk is connected across the rectifier output to smooth the DC rail, the diodes do not conduct for the full half-cycle (180^\circ):

Diode Conduction Interval & Pulsed Current Spikes Voltage ^ Vpk ┼───────. .───────. .─────── │ / \ / \ / \ / \ │ / '-._______.-' \ / '-._______.-' \ │ / Capacitor Ripple\ / \ 0V ┼───/─────────────────────v─────────────────────\───────────> Time Current ^ │ ¦ ¦ ¦ ¦ I_pk ┼──────¦───────────────¦─────¦───────────────¦────────────── │ / \ / \ / \ / \ 0A ┼────'───'───────────'───'─'───'───────────'───'────────────> Time │◄-θc-►│ Conduction Angle (Narrow Pulsed Spike!)

2.1 The Physics of Narrow Conduction Angles (\theta_c):

The diodes can only conduct when the input AC voltage exceeds the capacitor voltage: v_{ac(t) > v_{cap(t).

As the capacitor discharges slowly into the load, the diode stays reverse-biased during most of the cycle.

Near the voltage crest, the diode turns ON for a brief interval \theta_c \approx 30^\circ \dots 60^\circ.

Consequence: The entire charge needed to supply the load for a full half-cycle (10\,ms) must be delivered in just 1.5 \dots 3\,ms!

The diode peak current I_{pk spikes to $5 \dots 10 \times$ the average load current, producing:

Severe crest factor: CF = \frac{I_{pk{I_{rms ≥ 3.0

High total harmonic distortion: THD_i ≥ 80\% \dots 120\%

Poor power factor: PF \approx 0.55 \dots 0.65 lagging.

3. Mathematical Formulations & Component Sizing

3.1 Bulk Capacitor Sizing for Specified Ripple (\Delta V_{ripple):

Assuming the capacitor discharges into load P_o during the discharge time t_{dis \approx \frac{1{2 f_{line:

C_{bulk} \ge \frac{P_o}{2 \cdot f_{line} \cdot V_{dc,pk} \cdot \Delta V_{ripple}} = \frac{I_{dc,load}}{2 \cdot f_{line} \cdot \Delta V_{ripple}}

Where:

f_{line = 50\,Hz or 60\,Hz

V_{dc,pk = \sqrt{2 \cdot V_{ac,rms

\Delta V_{ripple is the allowable peak-to-peak DC ripple voltage (typically 5\% \dots 10\% \cdot V_{dc,pk).

3.2 Diode Peak & RMS Current Stresses:

I_{diode,pk} \approx I_{dc} \cdot \frac{\pi}{\theta_c}

I_{diode,rms} \approx I_{dc} \cdot \sqrt{\frac{\pi}{2 \theta_c}}

Component Selection Rule: Specify diodes whose non-repetitive peak forward surge current rating (I_{FSM) is at least 10 \dots 20 × rated DC load current.

4. Inrush Current & Thermal Protection

Inrush Current Limiting: NTC vs. Relay Pre-Charge SCHEME A (Low Cost, < 100W): SCHEME B (High Reliability, > 100W): AC ───[ Fuse ]───[ NTC Thermistor ]─── Bridge AC ───[ Fuse ]───┬───[ Power Resistor R_pre ]───┬─── Bridge │ │ └───[ Bypass Relay Contact ]───┘

Cold Inrush Surge: When first plugged into AC mains at peak voltage (v_{ac = 325\,V), the uncharged bulk capacitor looks like an absolute short circuit (Z_{cap \approx 0\,\Omega). Inrush current is limited only by line impedance, easily exceeding 100\,A \dots 200\,A, which can weld switch contacts and trip circuit breakers.

NTC Thermistor (Scheme A): A Negative Temperature Coefficient thermistor has high resistance when cold (5 \dots 10\,\Omega), limiting inrush to I_{inrush = \frac{325\,V{10\,\Omega = 32.5\,A. Once running, load current heats the NTC, dropping its resistance to < 0.5\,\Omega.

Failure Mode: If power drops out for 1 second and immediately returns, the NTC is still hot and provides zero protection!

Relay-Bypassed Power Resistor (Scheme B): Mandatory for industrial power electronics. A ceramic wirewound resistor (20 \dots 50\,\Omega) limits initial charging. Once C_{bulk reaches 80\% \dots 90\% of peak voltage, a microcontroller or analog timer closes an electromechanical relay across the resistor, eliminating continuous resistor power dissipation.