Isolated Full-Bridge DC-DC Converter: Hard-Switched vs Phase-Shifted ZVS Mechanics

Welcome to the VVDN Engineering Hub Technical Dossier on the Isolated Full-Bridge DC-DC Converter. This document delivers an in-depth hardware engineering analysis of the Full-Bridge topology, focusing on the industrial workhorse: the Phase-Shifted Full-Bridge (PSFB) Zero-Voltage-Switching (ZVS) converter. It covers bipolar transformer utilization, leading-leg vs. lagging-leg commutation physics, duty-cycle loss (\Delta D), secondary diode voltage ringing suppression, and practical design equations for kilowatts-scale power supplies.

1. Operating Principle & Full-Bridge Architecture

The Full-Bridge DC-DC Converter employs four primary semiconductor switches configured in an H-bridge configuration across a center-tapped or full-wave secondary stage:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: PHASE-SHIFTED FULL-BRIDGE (PSFB) ZVS CONVERTER (400V -> 48V/20A, 1kW) ======================================================================================================================== +V_BUS (+400V DC Rail) ───────────────┬───────────────────────────────┬─────────────────────────────┐ │ │ │ │ ┌┴─────────────────┐ Drain (D) Drain (D) ┌┴┐ │ C_BUS_BULK │ ┌────┴──────────┐ ┌────┴──────────┐ │ │ C_CER │ 470µF / 450V │ │ Q1: HS LEADING│ │ Q3: HS LAGGING│ └┬┘ 4x 1µF/630V │ Aluminum Elec. │ +----->│ IPW60R045CP │ +----->│ IPW60R045CP │ │ Poly Film └┬─────────────────┘ ¦ Gate │ (650V, 45mΩ) │ ¦ Gate │ (650V, 45mΩ) │ │ │ ¦ └────┬──────────┘ ¦ └────┬──────────┘ │ │ ¦ Source│ ¦ Source│ │ │ DUAL-LEG DRIVER ¦ │ ¦ │ │ │ HO_A -------------+ +=== NODE MID_A ¦ +=== NODE MID_B │ │ (Leading Leg) ¦ (Leading Leg) ¦ ¦ (Lagging Leg) │ │ ¦ ¦ ¦ │ │ Drain (D) +-----------+ Drain (D) │ │ ┌────┴───────┴──┐ ┌────┴──────────┐ │ │ │ Q2: LS LEADING│ │ Q4: LS LAGGING│ │ │ +----->│ IPW60R045CP │ +----->│ IPW60R045CP │ │ │ ¦ Gate └────┬──────────┘ ¦ Gate └────┬──────────┘ │ │ LO_A -------------+ Source│ LO_B -+ Source│ │ │ HO_B ---------------------------------------------+ │ │ │ │ │ │ GND_PRI ──────────────────────────────┴───────────────────────────────┴─────────────────────────────┴─── GND_PRI │ │ │ RESONANT SHIM INDUCTOR │ ├──[ L_shim: 8.5µH / 15A ]──────┼────────────────────────┐ │ Wurth 7443640850 │ │ │ │ │ │ DC BLOCKING CAPACITOR │ │ ├──[ C_b: 0.22µF / 630V Film ]──┼──[ CT1: Prim Current ]─┤ │ │ Sense Transformer │ │ │ (1:100 Ratio) │ │ │ │ │ │ TRANSFORMER PRIMARY │ │ │ +---[ Np: 16T ]------+ │ │ ¦ * (Dot at Mid_A) │ └───+--------------------+ │ =======================================│===================================== ISOLATION BARRIER ================= │ GND_SEC ──────────────────────────────┼───+─────────────────────────────────────────────────────────┬─── GND_SEC │ │ │ │ │ SECONDARY CENTER-TAP │ │ │ (Ns1: 3T, Ns2: 3T) │ │ │ │ │ │ │ +-------------+-------------+ │ │ │ ¦ ¦ │ │ │ SECONDARY WINDING 1 SECONDARY WINDING 2 │ │ (Ns1: 3T) (Ns2: 3T) │ │ │ * (Dot at Anode D1) ¦ │ │ │ ¦ * (Dot at Center-Tap) │ │ +---[>¦] D1 ¦ │ │ │ IDH12G65C6 +---[>¦] D2 (SiC Schottky) │ │ [Cathode] [Cathode] │ │ │ │ │ │ │ │ +==========+==========+ │ │ │ ¦ │ │ │ +=== NODE SEC_RECT │ │ ¦ (Rectified +75V) │ │ ¦ │ │ │ ├──[ R_snub_sec: 4.7Ω / 3W ] │ │ │ │ │ │ │ │ [ C_snub: 1.5nF/200V ] │ │ │ │ │ │ │ │ GND_SEC │ │ │ │ │ │ │ +---[ Lo: 22µH / 25A Choke ]---+ │ │ Coilcraft AGP4233-223 │ │ │ (DCR = 3.6mΩ, Isat = 28A) │ │ │ │ │ │ +=== +VOUT (+48V/20A) │ │ │ │ │ ┌┴──────────────────┐ │ │ │ COUT_BULK │ │ │ ┌┴┐ 4x 220µF/63V ┌┴┐ COUT_CER │ │ │ │ Poly (ESR=12mΩ) │ │ 6x 10µF/100V │ │ └┬┘ └┬┘ X7R 1210 │ │ │ │ GND_SEC ──────────────────────────────┴───┴──────────────────────────────────────────────────────────────────────┴───────────────────┴─── GND_SEC

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +V_BUS | PFC Pre-Regulator Bus | C_{bus (+), Q_1 Drain, Q_3 Drain | High-voltage 400V DC input rail | Symmetrical high-frequency decoupling film capacitors placed adjacent to each half-bridge leg. |
| MID_A (Leading Leg) | Q_1 Source, Q_2 Drain | Shim Inductor L_{shim Pin 1 | High-speed ZVS leading leg switching node | Driven by 50\% complementary PWM with fixed dead-time (t_{dead \approx 200\,ns). |
| MID_B (Lagging Leg) | Q_3 Source, Q_4 Drain | Transformer Primary N_p Pin 2 | Phase-shifted lagging leg switching node | Phase-shifted by angle \phi relative to Leg A; achieves ZVS using energy stored in L_{shim. |
| PRIM_RESONANT | Shim Inductor L_{shim Pin 2 | Series DC Blocking Cap C_b, Current Transformer CT1 | Primary resonant energy transfer loop | L_{shim supplements transformer leakage inductance to ensure ZVS down to 20\% light load. |
| SEC_RECT | SiC Diodes D_1, D_2 Common Cathodes | Output Filter Choke L_o Pin 1, Snubber | High-frequency rectified DC pulse train | Operating frequency is 2 f_s = 200\,kHz; SiC diodes eliminate reverse-recovery voltage overshoot. |
| +VOUT | Output Choke L_o Pin 2 | C_{out bank (+), Feedback network, Load (+) | High-power regulated +48V DC bus | Power copper plane designed for 20A continuous load current. |
| GND_SEC | Transformer Secondary Center-Tap | C_{out bank (-), Secondary Return | Secondary isolated power ground | Carries total load return current (20\,A). |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| $Q_1 \dots Q_4$ | Primary High-Voltage N-FETs | Infineon IPW60R045CP | V_{DS = 650\,V, I_D = 60\,A, R_{DS(on) = 45\,m\Omega, C_{oss = 160\,pF | Low R_{DS(on) and well-defined output capacitance C_{oss enable complete ZVS discharge during dead-time. |
| $L_{shim}$ | External Resonant Inductor | Würth Elektronik 7443640850 | L = 8.5\,µH, I_{sat = 22\,A, I_{rms = 16\,A, DCR = 3.2\,m\Omega | High-frequency gapped core handles circulating primary reactive energy without thermal runaway. |
| $C_b$ | DC Blocking Film Cap | Vishay MKP1848520704K2 | 0.22\,µF, 700\,V_{DC, Metallized Polypropylene | Prevents net DC volt-second imbalance from walking the transformer core into saturation. |
| $T_1$ | Main PSFB Transformer | Custom ETD49 Core (3C95) | Turns: 16:(3+3), L_m = 2.4\,mH, L_{lk = 1.5\,µH | Triple-insulated wire, interleaved secondary copper foil for low proximity loss and leakage control. |
| $D_1, D_2$ | Secondary SiC Diodes | Infineon IDH12G65C6 | V_{RRM = 650\,V, I_F = 12\,A, Q_c = 17\,nC, V_F = 1.35\,V | SiC Schottky eliminates diode reverse recovery snap-off, allowing smaller snubbers and higher efficiency. |
| $L_o$ | Output Filter Inductor | Coilcraft AGP4233-223ME | L = 22\,µH, I_{sat = 28\,A, I_{rms = 22\,A, DCR = 3.6\,m\Omega | Heavy edge-wound copper ribbon choke maintains CCM across entire active load spectrum. |
| $C_{out,bulk}$ | Output Bulk Capacitor | Panasonic 63SVPF220M | 4 × 220\,µF, 63\,V, OS-CON Polymer, ESR = 12\,m\Omega | Low equivalent impedance (3\,m\Omega) absorbs inductor current ripple with < 25\,mV ripple. |
| $U_1$ (Controller) | Phase-Shifted PWM IC | TI UCC28951-Q1 | Advanced ZVS controller with adaptive delay, sync rect drive | Independent programmable dead-time control for leading and lagging legs. |

1.1 Hard-Switched Full-Bridge vs. Phase-Shifted Full-Bridge (PSFB)

Hard-Switched Full-Bridge: Diagonal switch pairs (Q1-Q4 and Q2-Q3) are driven simultaneously via conventional pulse-width modulation (PWM). Turn-on and turn-off occur at high V_{DS and high I_D, leading to severe switching losses (E_{sw \propto f_s \cdot V_{in^2 \cdot I), preventing operation above 50\,kHz.

Phase-Shifted Full-Bridge (PSFB): Each bridge leg operates at a fixed \approx 50\% duty cycle with a small dead-time. Voltage regulation is achieved by shifting the phase of Leg B (Lagging Leg: Q3/Q4) relative to Leg A (Leading Leg: Q1/Q2). The primary leakage inductance (L_{lk) and MOSFET parasitic output capacitances (C_{oss) are utilized to achieve Zero-Voltage Switching (ZVS) at all switch transitions.

2. PSFB Switching Intervals & Detailed Waveforms

PSFB Key Operating Waveforms Gate Q1 ───┐ ┌───────────────────┐ ┌──────── └────────────────┘ └────────────────┘ Gate Q2 ────────────┐ ┌───────────────────┐ ────────────┘ └───────────────────┘ Gate Q3 ───────┐ ┌───────────────────┐ ┌──── ───────┘ └───────────────────┘ Gate Q4 ────────────────┐ ┌───────────────────┐ ────────────────┘ └───────────────────┘ │◄-φ-►│ Phase Shift Angle V_AB ───┐ ┌──────────┐ ┌──────────┐ │ │ │ │ │ ───┴─────┘ └───────────────────┘ └────────────── │◄────── -Vin ─────►│ I_primary /──────────\ /──────────\ ────────/ \─────────────────/ \───────────── / \ / \ V_rect ┌───┐ ┌───┐ ┌───┐ ┌───┐ ───┘ └────────────┘ └───────────┘ └────────────┘ └──────── │◄ΔD►│ Duty Cycle Loss

2.1 The Six Operating Modes of PSFB:

Power Delivery Interval ($Q_1, Q_4$ ON): V_{AB = +V_{IN. Current flows through Q_1, L_{lk, primary winding, and Q_4. Diode D_1 conducts, powering the output load and charging L_o.

Leading-Leg Transition ($Q_1 \rightarrow Q_2$ Dead-Time): Q_1 turns OFF. The reflected output inductor current (n \cdot I_{Lo) quickly charges C_{oss,Q1 to V_{IN and discharges C_{oss,Q2 to 0\,V. The body diode of Q_2 clamps to ground, allowing Q_2 to turn ON under ZVS.

Freewheeling Primary Interval ($Q_2, Q_4$ ON): V_{AB = 0\,V. Primary current circulates through Q_2 and Q_4. On the secondary, both rectifier diodes D_1 and D_2 conduct simultaneously, shorting the transformer secondary and freewheeling I_{Lo.

Lagging-Leg Transition ($Q_4 \rightarrow Q_3$ Dead-Time): Q_4 turns OFF. Because secondary diodes are shorted, only the energy stored in the primary leakage inductance ($L_{lk}$) is available to discharge C_{oss,Q3 and charge C_{oss,Q4.

Duty Cycle Loss ($\Delta D$) Interval: Q_3 turns ON under ZVS (if energy was sufficient). V_{AB = -V_{IN, but secondary diodes remain shorted until the primary current reverses from +I_{pk to -I_{pk across L_{lk. During this commutation interval, no energy transfers to the output.

Reverse Power Delivery ($Q_2, Q_3$ ON): Primary current reaches -I_{pk, diode D_1 reverse-biases, and D_2 supplies power to the load.

3. Mathematical Formulations & Component Derivations

3.1 Effective Duty Cycle and Voltage Conversion:

Because of the duty cycle loss (\Delta D) during primary current reversal:

D_{eff} = D - \Delta D

\Delta D = \frac{4 \cdot f_s \cdot L_{lk} \cdot I_o}{n \cdot V_{IN}}

Where n = \frac{N_p{N_s is the primary-to-secondary turns ratio.

The DC output voltage is:

V_{out} = 2 \cdot \frac{N_s}{N_p} \cdot V_{IN} \cdot D_{eff} = 2 \cdot \frac{N_s}{N_p} \cdot V_{IN} \cdot \left( D - \frac{4 \cdot f_s \cdot L_{lk} \cdot I_o}{n \cdot V_{IN}} \right)

3.2 ZVS Energy Criterion (Leading vs. Lagging Leg):

Leading Leg Transition:

Assisted by both the leakage inductance and the reflected output filter inductance (L_o):

\frac{1}{2} (L_{lk} + n^2 L_o) I_{pk}^2 > \frac{4}{3} C_{oss} V_{IN}^2

Result: Leading leg achieves ZVS over virtually the entire load range (10\% \dots 100\%).

Lagging Leg Transition:

During the freewheeling state, the secondary is shorted, isolating L_o. ZVS relies purely on primary leakage inductance L_{lk:

\frac{1}{2} L_{lk} I_{pk}^2 > \frac{4}{3} C_{oss} V_{IN}^2

Hardware Consequence: At light load (I_{pk low), the energy \frac{1{2 L_{lk I_{pk^2 is insufficient to discharge C_{oss, causing the lagging leg to lose ZVS and suffer hard switching.

3.3 Dead-Time Calculation:

t_{dead,lead} \ge \frac{2 \cdot C_{oss} \cdot V_{IN}}{I_{pk,lead}}

t_{dead,lag} \approx \frac{\pi}{2} \sqrt{L_{lk} \cdot (2 C_{oss})}

4. Hardware Engineering Challenges & Mitigation

| Engineering Challenge | Physical Mechanism | Hardware Mitigation Solution |
| --- | --- | --- |
| Lagging Leg ZVS Loss at Light Load | L_{lk energy drops with I_{load^2; cannot overcome MOSFET C_{oss. | Add an external shim inductor in series with primary, or introduce auxiliary commutating inductors / saturated inductors. |
| Secondary Diode Voltage Ringing | Resonance between secondary diode junction capacitance C_j and transformer leakage inductance L_{lk. Ringing voltage can exceed 2 \cdot V_{sec. | Install primary auxiliary clamping diodes (clamping to V_{IN rails), active secondary clamps, or RC snubbers across D_1, D_2. |
| Transformer DC Flux Walking | Unequal gate drive delays or asymmetric switch R_{DS(on) create DC voltage offset, saturating the core. | Insert a low-loss polypropylene DC blocking capacitor in series with primary winding, or implement peak current-mode control. |
| Loss of Effective Duty Cycle ($\Delta D$) | Increasing L_{lk expands ZVS range but steals duty cycle, requiring a higher turns ratio and higher secondary voltage stress. | Optimize L_{lk carefully; utilize planar transformers with controlled leakage and low C_{oss GaN/SiC FETs. |

5. Comprehensive Design Example: 3.3 kW Telecom Power Supply

Input Voltage: 380\,V \dots 420\,V DC (PFC Output, Nominal 400\,V)

Output Voltage: 48\,V \dots 54\,V DC (Nominal 50\,V at 66\,A)

Switching Frequency: 100\,kHz

Turns Ratio Selection ($n = N_p / N_s$):

D_{max} = 0.42, \quad \Delta D_{est} = 0.05 \implies D_{eff} = 0.37

n \le \frac{2 \cdot V_{in,min} \cdot D_{eff}}{V_o + V_{drop}} = \frac{2 \cdot 380\,\text{V} \cdot 0.37}{50.5\,\text{V}} \approx 5.56 \implies \text{Select } N_p = 16, N_s = 3 \quad (n = 5.33)

Leakage Inductor Sizing:

L_{lk} \approx \frac{\Delta D \cdot n \cdot V_{IN}}{4 \cdot f_s \cdot I_o} = \frac{0.05 \cdot 5.33 \cdot 400}{4 \cdot 100\,000 \cdot 66} \approx 4.0\,\mu\text{H}

Primary Switch Selection:

650\,V Superjunction MOSFETs with ultra-low C_{oss (e.g., C_{o(er) \approx 120\,pF) or 650\,V GaN HEMTs.