Isolated Half-Bridge DC-DC Converter: Voltage-Divider Mechanics & Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Isolated Half-Bridge DC-DC Converter. This guide delivers a comprehensive hardware engineering analysis of the Half-Bridge topology, exploring capacitive voltage division, transformer flux balancing, switch stress trade-offs, secondary LC rectification, and practical design for 100\,W \dots 500\,W power conversion systems (such as industrial power modules and legacy ATX power supplies).

1. Operating Principle & Half-Bridge Architecture

The Half-Bridge DC-DC Converter replaces two switches of a full bridge with a pair of input divider capacitors (C_1, C_2), applying an alternating square-wave voltage of ± \frac{V_{IN{2 across the transformer primary winding:

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: TWO-SWITCH ASYMMETRICAL/SYMMETRICAL HALF-BRIDGE CONVERTER (400V -> 12V/25A, 300W) ======================================================================================================================== +-----------------[ D_boot: DFLS1100 ]<------+ +VCC_DRV (+12V) ¦ [ 100V / 1A Schottky] ¦ ¦ [R_boot: 2.2Ω] ¦ ¦ ¦ +-----------[ C_boot: 0.1µF/50V X7R ]--+ ¦ ¦ ¦ ¦ ¦ +---[ HIGH-VOLTAGE HALF-BRIDGE DRIVER (e.g. UCC27282) ]---+ ¦ ¦ ¦ ¦ +-------->¦ BOOT [HO] Pin 8 ----[ R_g1: 4.7Ω ]----+ ¦ ¦ ¦ ¦ ¦ +-->¦ PHASE/HB [LO] Pin 5 ----[ R_g2: 4.7Ω ]--+ ¦ ¦ ¦ ¦ ¦ ¦ PWM_IN ---------->¦ IN_HS/IN_LS [VCC] Pin 2 <--- +12V ¦ ¦ ¦ ¦ COM/GND_PRI [COM] Pin 4 ---> GND_PRI ¦ ¦ ¦ +------------------------------------------¦-¦-------------+ ¦ ¦ +V_BUS (+400V DC High-Voltage Rail) ──────┬────────────────────────¦─¦─────────────┬────────────────┐ │ │ │ │ │ │ ┌┴─────────────────┐ Drain (D) │ │ ┌┴┐ C1 │ │ C_BUS_BULK │ ┌────┴──────────┐ │ │ │ │ 2.2µF / 450V ┌┴┐ R_bleed1 │ 220µF / 450V │ │ Q1: HS N-FET │<------------+ │ └┬┘ Poly Film │ │ 100kΩ / 1W │ Electrolytic │ │ IPB60R099C7 │ │ │ (Divides Vin) └┬┘ └┬─────────────────┘ │ (650V, 99mΩ) │ │ │ │ │ └────┬──────────┘ │ +=== VIRTUAL NEUTRAL │ Source│ │ ¦ (Maintains +200V) │ │ │ ¦ │ +=== HALF-BRIDGE MID (HB) │ ¦ │ ¦ (Swings 0V to +400V) │ ¦ │ ¦ │ ¦ │ +----------------+--[ C_b: 0.47µF / 630V ]--+ ¦ │ ¦ ¦ DC BLOCKING CAP ¦ ¦ │ ¦ ¦ (Prevents Xfmr Sat) ¦ ¦ │ ¦ ¦ ¦ ¦ │ ¦ ¦ PRIMARY WINDING ¦ ¦ │ ¦ ¦ +---[ Np: 24T ]--------+ ¦ │ ¦ ¦ ¦ * (Dot at HB) ¦ │ ¦ ¦ +------------------------------------+ │ ¦ ¦ │ │ ¦ Drain (D) ┌┴┐ C2 │ ¦ ┌────┴──────────┐ │ │ 2.2µF / 450V ┌┴┐ R_bleed2 │ ¦ │ Q2: LS N-FET │<--------------+ └┬┘ Poly Film │ │ 100kΩ / 1W │ ¦ │ IPB60R099C7 │ (Gate) │ │ (Divides Vin) └┬┘ │ ¦ └────┬──────────┘ │ │ │ │ ¦ Source│ │ │ │ │ ¦ +---[ CS_PRI ] │ │ │ │ ¦ │ │ │ │ │ ¦ ┌┴┐ R_shunt: 0.10Ω / 3W │ │ │ │ ¦ │ │ 1% Metal Strip │ │ │ │ ¦ └┬┘ (Primary Current Sense) │ │ │ │ ¦ │ │ │ │ GND_PRI ─────────────────┴────────────────┴──────────────────────────┴─────────────┴────────────────┴─── GND_PRI │ ┌┴┐ CY1: Safety Y1 Capacitor (2.2nF / 400VAC Across Galvanic Isolation Barrier) └┬┘ │ ============================================= ISOLATION BARRIER ===================================================== │ GND_SEC ──────────────────────────────────+─────────────────────────────────────────────────────────┬─── GND_SEC │ │ │ │ SECONDARY CENTER-TAP │ │ (Ns1: 2T, Ns2: 2T) │ │ │ │ │ +-------------+-------------+ │ │ ¦ ¦ │ │ SECONDARY WINDING 1 SECONDARY WINDING 2 │ │ (Ns1: 2T) (Ns2: 2T) │ │ * (Dot at Anode D1) ¦ │ │ ¦ * (Dot at Center-Tap) │ │ +---[>¦] D1 ¦ │ │ MBR40200WT +---[>¦] D2 (MBR40200WT Schottky) │ │ [Cathode] [Cathode] │ │ │ │ │ │ +==========+==========+=== NODE SEC_RECT │ │ ¦ (Rectified +20V AC) │ │ ¦ │ │ ├──[ R_snub_sec: 3.3Ω / 2W ] │ │ │ │ │ │ │ [ C_snub_sec: 2.2nF / 100V ] │ │ │ │ │ │ │ GND_SEC │ │ │ │ │ +---[ Lo: 4.7µH / 30A Choke ]───+=== +VOUT (+12V/25A)│ │ Vishay IHLP-6767GZ-11 │ │ │ (DCR = 1.2mΩ, Isat = 36A) ┌┴──────────────────┐ │ │ │ COUT_BULK │ │ │ ┌┴┐ 4x 470µF/25V ┌┴┐│ COUT_CER │ │ │ Poly (ESR=5mΩ) │ ││ 6x 47µF/16V │ └┬┘ └┬┘│ X7R 1210 │ │ │ │ GND_SEC ──────────────────────────────────────────────────────────────────────┴───────────────────┴─┴─ GND_SEC

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +V_BUS | PFC Output (+400V) | C_{bus (+), Q_1 Drain, Divider Cap C_1 Pin 1 | High-voltage stiff DC bus | Trace layout must respect IEC 62368-1 high-voltage creepage rules (≥ 2.5\,mm). |
| HB_MID | Q_1 Source, Q_2 Drain | Driver PHASE (Pin 7), DC Blocking Cap C_b Pin 1 | High-voltage half-bridge switching node | Swings between 0\,V and +400\,V at switching frequency f_s; minimize copper area. |
| VIRTUAL_NEUTRAL | Divider Caps C_1 / C_2 Junction | Transformer Primary N_p Pin 2, Bleeders | Midpoint reference rail (V_{BUS/2 = 200\,V) | Balanced by polypropylene film capacitors C_1, C_2; bleeder resistors prevent DC drift. |
| PRI_XFMR_AC | DC Blocking Cap C_b Pin 2 | Transformer Primary N_p Pin 1 | AC-coupled primary excitation | Capacitor C_b blocks any DC imbalance current, preventing staircase transformer saturation. |
| SEC_RECT | Diodes D_1, D_2 Common Cathodes | Output Choke L_o Pin 1, Secondary Snubber | Full-wave rectified low-voltage AC node | Delivers smooth continuous inductor charging current; double switching frequency ripple. |
| +VOUT | Output Choke L_o Pin 2 | C_{out bank (+), Feedback network, Load (+) | High-current regulated +12V DC rail | Wide power copper plane; handles 25A continuous load current. |
| GND_SEC | Transformer Secondary Center-Tap | C_{out bank (-), Secondary Return | Secondary isolated power ground | Carries total load return current (25\,A). |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| $Q_1, Q_2$ | High-Voltage N-MOSFETs | Infineon IPB60R099C7 | V_{DS = 650\,V, I_D = 24\,A, R_{DS(on) = 99\,m\Omega, Q_g = 32\,nC | CoolMOS C7 technology minimizes switching and gate charge losses at 100\,kHz. |
| $C_1, C_2$ | Voltage Divider Film Caps | KEMET R46KR422000M1M | 2.2\,µF, 450\,V_{DC, Metallized Polypropylene Film | Handles high continuous AC ripple current (I_{rms \approx 2.5\,A); film dielectric avoids capacitance drift. |
| $C_b$ | DC Blocking Capacitor | TDK B32652A6474J000 | 0.47\,µF, 630\,V_{DC, High-Pulse Film | Eliminates any steady-state DC magnetization current through transformer primary winding. |
| $T_1$ | Main Power Transformer | Custom ETD44 Core (3C95) | Turns: 24:(2+2), L_m = 1.8\,m\H, L_{lk < 2.5\,µH | Split primary/secondary sandwich construction to minimize leakage inductance and proximity losses. |
| $D_1, D_2$ | Secondary Dual Schottky | ON Semi MBR40200WT | V_{RRM = 200\,V, I_F = 40\,A, V_F = 0.72\,V | Common-cathode TO-247 Schottky diode mounted on secondary high-performance heatsink. |
| $L_o$ | Output Filter Inductor | Vishay IHLP-6767GZ-11 | L = 4.7\,µH, I_{sat = 36\,A, I_{rms = 30\,A, DCR = 1.2\,m\Omega | Low-loss molded powder choke rated for > 30\,A continuous saturation. |
| $C_{out,bulk}$ | Output Bulk Capacitor | Panasonic 25SVPF470M | 4 × 470\,µF, 25\,V, OS-CON Polymer, ESR = 5\,m\Omega | Paralleled bank yields total ESR of 1.25\,m\Omega for ultra-low output ripple (< 15\,mV). |
| $U_1$ (Driver) | Half-Bridge Gate Driver | TI UCC27282DR | 120\,V / 650\,V bootstrap, 3\,A sink / source, robust dv/dt | Withstands negative transient swings on HB pin down to -5\,V. |

1.1 Core Operating Phases:

Phase 1: High-Side Conduction ($Q_1$ ON, $0 < t \le D \cdot T_s$):

Q_1 turns ON while Q_2 remains OFF.

Primary winding voltage is clamped to V_p = V_{IN - \frac{V_{IN{2 = +\frac{V_{IN{2.

Secondary winding N_{s1 forward-biases diode D_1, driving power into output inductor L_o and capacitor C_o. Secondary voltage is V_s = +\frac{V_{IN{2 \cdot ≤ft(\frac{N_s{N_p\right).

Phase 2: Dead-Time ($Q_1, Q_2$ OFF, $D \cdot T_s < t \le 0.5 T_s$):

Both switches are OFF. Primary current ceases or circulates through parasitic paths.

Secondary diodes D_1 and D_2 both conduct simultaneously, freewheeling output inductor current I_{Lo and clamping secondary voltage to \approx 0\,V.

Phase 3: Low-Side Conduction ($Q_2$ ON, $0.5 T_s < t \le (0.5 + D) \cdot T_s$):

Q_2 turns ON while Q_1 remains OFF.

Primary winding is connected between GND and the virtual neutral \frac{V_{IN{2, applying V_p = -\frac{V_{IN{2.

Secondary winding N_{s2 forward-biases diode D_2, driving load power with reversed transformer polarity.

Phase 4: Dead-Time ($Q_1, Q_2$ OFF, $(0.5 + D) \cdot T_s < t \le T_s$):

Secondary diodes freewheel again until the next cycle begins.

2. Voltage and Current Waveforms

Half-Bridge Key Operating Waveforms Gate Q1 ───┐ ┌──────────────┐ └────────┘ └───────────────────────────── Gate Q2 ───────────────────────────┐ ┌──────────────┐ ───────────────────────────┘ └──────────────┘ │◄─ D ─►│ V_primary +Vin/2 ┌──────────┐ │ │ ───┴──────────┴────────────────────────┐ ┌───────── │ │ └──────────┘ -Vin/2 I_Lo / \ / \ / \ ───/ \────────────────────/ \────────────────────/ \ V_rect ┌──────────┐ ┌──────────┐ ┌────── │ │ │ │ │ ───┴──────────┴─────────────┴──────────┴─────────────┴──────

3. Mathematical Formulations & Component Sizing

3.1 Voltage Conversion Ratio:

Because each half-cycle applies only \frac{V_{IN{2 to the primary winding:

V_{out} = \frac{N_s}{N_p} \cdot V_{IN} \cdot D

Where D = \frac{t_{on{T_s is the duty cycle per switch (0 ≤ D < 0.5, practically D_{max \approx 0.42 \dots 0.45).

3.2 Switch Stress Analysis:

Peak Drain-to-Source Voltage:

V_{DS,max} = V_{IN,max}

(A massive advantage over Push-Pull, which sees $2 \cdot V_{IN}$.)

Primary Current Stress:

Because primary voltage is halved (\frac{V_{IN{2), primary RMS current is doubled compared to full-bridge for identical power:

I_{pri,pk} \approx \frac{N_s}{N_p} \cdot I_o = \frac{V_o \cdot I_o}{\frac{V_{IN}}{2} \cdot 2 D} = \frac{P_{out}}{\eta \cdot V_{IN} \cdot D}

I_{pri,rms} \approx I_{pri,pk} \cdot \sqrt{2 D}

3.3 Capacitive Divider Sizing:

Capacitors C_1 and C_2 carry high AC ripple current. The AC ripple voltage across each capacitor must be restricted to \Delta V_C ≤ 5\% \dots 10\% \cdot \frac{V_{IN{2:

C_1 = C_2 \ge \frac{I_{pri,pk} \cdot D_{max}}{2 \cdot f_s \cdot \Delta V_C}

3.4 Inherent Core Saturation Immunity:

A premier engineering feature of the capacitive half-bridge is automatic DC flux balancing. If a duty cycle asymmetry occurs (\Delta D = D_1 - D_2 \ne 0), a net DC current flows into the midpoint. The capacitor voltages drift (V_{C1 \ne V_{C2) until the volt-second integral across the transformer primary perfectly balances to zero:

\int_0^{T_s} v_p(t) \, dt = V_{C1} \cdot D_1 \cdot T_s - V_{C2} \cdot D_2 \cdot T_s = 0

Thus, the half-bridge core cannot walk into DC saturation, eliminating the need for complex flux-balancing control loops.

4. Hardware Engineering Trade-Offs

| Parameter | Half-Bridge | Full-Bridge | Push-Pull |
| --- | --- | --- | --- |
| Number of Primary Switches | 2 | 4 | 2 |
| Switch Voltage Rating | V_{IN | V_{IN | 2 \cdot V_{IN |
| Primary Current Rating | 2 × | 1 × | 1 × |
| Transformer Utilization | Excellent (Bidirectional ± B) | Excellent (Bidirectional ± B) | Excellent (Bidirectional ± B) |
| Gate Drive Complexity | 1 High-Side + 1 Low-Side Driver | 2 High-Side + 2 Low-Side Drivers | 2 Ground-Referenced Low-Side Drivers |
| DC Core Walking Risk | None (Capacitor Balances) | High (Requires blocking cap) | Severe (Flux walking risk) |
| Typical Power Range | 100\,W \dots 500\,W | 500\,W \dots 10\,kW+ | 50\,W \dots 500\,W (Low V_{IN) |

5. Practical Design Rules & PCB Layout Guidelines

Capacitor Selection: Use low-ESR, high-ripple-current metalized polypropylene (MKP) film capacitors or ceramic arrays in parallel with bulk electrolytics for C_1, C_2. Electrolytic capacitors alone cannot handle high-frequency charging pulses without excessive heating.

High-Side Bootstrapping: Implement a bootstrap diode with ultra-fast reverse recovery (t_{rr < 25\,ns) and a high-voltage gate driver IC (e.g., UCC27712, IR2110) or pulse transformer drive.

Midpoint Parasitic Inductance: Minimize loop area between switch midpoint node, transformer primary, and the capacitor divider junction to limit high-frequency dv/dt radiated EMI.