Synchronous & Asynchronous Buck DC-DC Converter: Theory, Operation & Design

Welcome to the VVDN Engineering Hub Technical Dossier on the Step-Down (Buck) DC-DC Converter. The Buck converter is the ubiquitous foundation of modern power distribution, present in point-of-load (PoL) regulators, microprocessor voltage regulator modules (VRMs), USB Type-C power delivery, and battery chargers.

This guide delivers an exhaustive mathematical and hardware analysis covering CCM/DCM operating modes, synchronous rectification, dead-time loss mechanisms, bootstrap gate-drive physics, output filter design, and multi-phase interleaving.

1. Operating Principle & Circuit Architecture

The Buck Converter steps down a higher DC input voltage V_{IN to a lower, stabilized DC output voltage V_o with high thermodynamic efficiency:

======================================================================================== DETAILED HARDWARE SCHEMATIC: SYNCHRONOUS BUCK DC-DC CONVERTER (12V -> 1.2V/20A) ======================================================================================== +-----------------[ D_boot: DFLS1100 ]<------+ +V_DRV (+5V / +12V) ¦ [ 100V / 1A Schottky] ¦ ¦ [R_boot: 2.2Ω] ¦ ¦ ¦ +-----------[ C_boot: 0.1µF/50V X7R ]--+ ¦ ¦ ¦ ¦ ¦ +---[ GATE DRIVER IC (e.g. UCC27282 / LM5113) ]---+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ [BOOT] Pin 1 ¦ +-------->¦ BOOT [HO] Pin 8 ----[ R_g1: 2.2Ω ]----+ ¦ ¦ ¦ ¦ ¦ +-->¦ PHASE/SW [LO] Pin 5 ----[ R_g2: 1.0Ω ]--+ ¦ ¦ ¦ ¦ ¦ ¦ PWM_IN ---------->¦ IN_HS/IN_LS [VCC] Pin 2 <--- +V_DRV ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ COM/PGND Pin 4 ───+ ¦ ¦ ¦ +-------------------¦----------------------+ ¦ ¦ ¦ ¦ ¦ +VIN (+12V DC) ─────────────────────────────¦───+ ¦ ¦ │ │ │ Drain (D) ¦ ¦ [F1: 25A Fuse] │ ┌┴───────────────┐ ¦ ¦ │ │ │ Q1: HS N-MOSFET│<---+ ¦ ┌┴─────────────────┐ │ │ BSC022N04LS │ (Gate) ¦ │ CIN_BULK │ │ │ (40V, 2.2mΩ) │ ¦ ┌┴┐ 100µF/25V Poly ┌┴┐ CIN_CER │ └┬───────────────┘ ¦ │ │ (Low-ESR 8mΩ) │ │ 2x 22µF/25V X7R │ │ Source (S) ¦ └┬┘ └┬┘ │ │ ¦ │ │ │ +=== SWITCHING NODE (SW) ¦ │ │ │ ¦ (High dv/dt node) ¦ │ │ │ ¦ ¦ │ │ +----------------+ ├──[ R_snub: 2.2Ω ] ¦ │ │ ¦ ¦ │ │ ¦ │ │ ¦ ¦ │ [ C_snub: 1nF/50V ] ¦ │ │ ¦ ¦ │ │ ¦ │ │ ¦ ¦ │ PGND ¦ │ │ ¦ ¦ │ ¦ │ │ ¦ ¦ │ Drain (D) ¦ │ │ ¦ ¦ ┌┴───────────────┐ ¦ │ │ ¦ ¦ │ Q2: LS N-MOSFET│<--------+ │ │ ¦ ¦ │ BSC014N04LS │ (Gate) │ │ ¦ ¦ │ (40V, 1.4mΩ) │ │ │ ¦ ¦ └┬───────────────┘ │ │ ¦ ¦ │ Source (S) │ │ ¦ ¦ │ │ │ ¦ ¦ +---------------------+ │ │ ¦ ¦ │ │ │ │ ¦ ¦ ┌┴┐ R_shunt ┌┴┐ D_FW (Opt) │ │ ¦ │ │ │ 1.0mΩ/2W 1% │>│ Schottky │ │ ¦ │ └┬┘ (Current Sense) └┬┘ (Low Vf) │ │ ¦ │ │ │ PGND┴──────────────────┴────┴────────────────┴───┴─────────────────────┴──── PGND │ +===================+===================+ ¦ POWER INDUCTOR & CURRENT SENSING ¦ +===================+===================+ │ │ SWITCHING NODE (SW) ───[ L1: 0.36µH / 32A ]──────┴─────────+───────> +VOUT (+1.2V/20A) IHLP-5050EZ-01 │ (DCR = 1.3mΩ shielded) ┌┴──────────────────┐ │ COUT_BULK │ ┌┴┐ 470µF/4V Poly ┌┴┐ COUT_CER │ │ (ESR = 5mΩ) │ │ 4x 47µF/6.3V └┬┘ └┬┘ X7R 1210 │ │ │ +VOUT │ │ │ │ │ [ R_fb1: 5.1kΩ, 0.1% ] │ │ │ +---> V_FB (To Controller) │ │ (V_ref = 0.600V) │ [ R_fb2: 5.1kΩ, 0.1% ] │ │ │ AGND (Quiet Ground) │ │ │ (Single Star Point) PGND (Power Ground) ──────────────────────────────────────┴────────+────────> GND (Return)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +VIN_RAW | Input Connector Pin 1 | Fuse F_1 Terminal 1 | Unfiltered +12V DC input bus | Size trace for 25A continuous rating with 2 oz copper minimum. |
| +VIN_FILT | Fuse F_1 Terminal 2 | C_{in,bulk (+), C_{in,cer (+), Q_1 Drain | Decoupled input high-voltage rail | Place C_{in,cer immediately adjacent to Q_1 Drain to minimize loop inductance. |
| SW (Switching Node) | Q_1 Source (Pins 1-3), Q_2 Drain (Pins 5-8) | Inductor L_1 Pin 1, Driver PHASE (Pin 7), C_{boot (-) | High dv/dt rectangular switching node | Minimize copper area to reduce E-field radiated EMI; route Kelvin trace to driver. |
| BOOT | D_{boot Cathode, C_{boot (+) | Driver IC BOOT (Pin 1) | Floating high-side supply rail | Floats at V_{SW + V_{DRV - V_F (\sim 16.7\,V during Q_1 ON state). |
| GATE_HS | Driver IC HO (Pin 8) | Resistor R_{g1 Pin 1 -> Q_1 Gate (Pin 4) | High-side gate drive output | Route differential pair with SW trace to cancel inductive loop pickup. |
| GATE_LS | Driver IC LO (Pin 5) | Resistor R_{g2 Pin 1 -> Q_2 Gate (Pin 4) | Low-side gate drive output | Keep trace width > 20\,mil to provide > 2\,A sink current. |
| CS_P / CS_N | Shunt R_{shunt Kelvin Pads | PWM Controller CS+/CS- Pins | Current sense differential input | Route as tight, shielded differential pair away from noisy SW copper. |
| +VOUT | Inductor L_1 Pin 2 | C_{out bank (+), Feedback R_{fb1, Load (+) | Regulated +1.2V DC output rail | Wide plane pour on top and internal power layers. |
| V_FB | Divider R_{fb1/R_{fb2 Node | Controller FB / Error Amp Inverting Input | Closed-loop voltage sense | Keep node extremely compact and isolated from magnetic flux paths. |
| PGND | C_{in (-), Q_2 Source, C_{out (-) | Input Return / Power Plane | High-current circulating ground | Solid copper pour on Layer 2 directly under components. |
| AGND | Controller Reference, R_{fb2 (-) | Star Ground Tie Point at C_{out GND pad | Analog low-noise reference ground | Single-point connection to PGND; prevents power loop noise from corrupting control. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| $Q_1$ | High-Side N-MOSFET | Infineon BSC022N04LS | V_{DS = 40\,V, I_D = 100\,A, R_{DS(on) = 2.2\,m\Omega, Q_g = 19\,nC | Select for ultra-low gate charge Q_{gd to minimize turn-on switching losses. |
| $Q_2$ | Low-Side Sync MOSFET | Infineon BSC014N04LS | V_{DS = 40\,V, I_D = 125\,A, R_{DS(on) = 1.4\,m\Omega, Q_{rr = 35\,nC | Select for lowest R_{DS(on) to minimize conduction losses at high duty cycles (1-D \approx 90\%). |
| $L_1$ | Shielded Power Inductor | Vishay IHLP-5050EZ-01 | L = 0.36\,µH, I_{sat = 32\,A, I_{rms = 24\,A, DCR = 1.3\,m\Omega | Low core loss powdered iron composite; saturation current must exceed I_o + \frac{\Delta I_L{2 = 23\,A. |
| $C_{in,cer}$ | Input Ceramic MLCC | Murata GRM32ER71H226KE14L | 2 × 22\,µF, 50\,V, X7R, 1210 package | Accommodates high RMS ripple current (I_{Cin,rms \approx 6\,A). |
| $C_{in,bulk}$ | Input Bulk Capacitor | Panasonic 25SVPF100M | 100\,µF, 25\,V, OS-CON Polymer, ESR = 8\,m\Omega | Damps input cable inductance and prevents input rail bounce. |
| $C_{out,cer}$ | Output Ceramic MLCC | TDK C3225X7R0J476M | 4 × 47\,µF, 6.3\,V, X7R, 1210 package | Absorbs high-frequency switching current ripple with negligible ESR loss. |
| $C_{out,bulk}$ | Output Bulk Polymer | Panasonic 4SEPC470M | 470\,µF, 4.0\,V, Polymer Aluminum, ESR = 5\,m\Omega | Handles dynamic step-load transients (0 \rightarrow 15\,A in 1\,µs). |
| $D_{boot}$ | Bootstrap Schottky Diode | Diodes Inc. DFLS1100 | V_R = 100\,V, I_F = 1\,A, V_F = 0.45\,V, t_{rr < 10\,ns | Ultra-fast recovery prevents discharging C_{boot when SW swings to +12\,V. |
| $C_{boot}$ | Bootstrap Capacitor | TDK CGA3E2X7R1H104K | 0.1\,µF, 50\,V, X7R, 0603 package | Value must satisfy C_{boot ≥ 10 \cdot \frac{Q_{g,Q1{V_{DRV - V_F. |
| $R_{snub}$ / $C_{snub}$ | SW RC Snubber Network | Vishay CRCW0805 / TDK C0G | R = 2.2\,\Omega / 0.5\,W, C = 1.0\,nF / 50\,V C0G | Damps 150\,MHz ringing caused by L_{parasitic and MOSFET C_{oss. |

1.1 Switching Intervals:

Interval 1: High-Side Switch ON ($0 < t \le D \cdot T_s$):

Q_1 is ON; Q_2 is OFF.

Node V_{SW = V_{IN. Voltage across inductor is V_L = V_{IN - V_o > 0.

Inductor current rises linearly:

\frac{di_L}{dt} = \frac{V_{IN} - V_o}{L}

Energy is simultaneously transferred to the output load and stored in the inductor's magnetic field.

Interval 2: Dead-Time 1 ($t_{dead1}$):

Both Q_1 and Q_2 are OFF to prevent shoot-through cross-conduction.

The inductor forces V_{SW below GND until the body diode of Q_2 turns ON to freewheel current.

Interval 3: Low-Side Switch ON ($D \cdot T_s < t \le T_s$):

Q_2 turns ON, shorting the body diode and conducting current through its low-R_{DS(on) channel.

Node V_{SW \approx 0\,V. Voltage across inductor is V_L = -V_o < 0.

Inductor current decays linearly:

\frac{di_L}{dt} = -\frac{V_o}{L}

Interval 4: Dead-Time 2 ($t_{dead2}$):

Q_2 turns OFF before Q_1 turns ON to prevent shoot-through.

2. Voltage and Current Waveforms

Continuous Conduction Mode (CCM) Waveforms V_SW ^ Vin ┼──────┐ ┌──────┐ │ │ │ │ 0V ┼──────┴──────────────────────┴──────┴────────────────────> Time │◄─ D*Ts ─►│◄─── (1-D)*Ts ───►│ i_L ^ │ / \ / \ I_max ┼─────────/ \──────────────────/ \──────────────────── I_avg┼─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ ─ ─ ─ /─ ─ ─\─ ─ ─ ─ ─ ─ ─ ─ ─ ─ (I_out) I_min ┼───────/ \──────────────/ \────────────────── 0─────────────────────────────────────────────────────────> Time i_Cin ^ I_out ┼──────┐ ┌──────┐ │ │ │ │ 0A ┼──────┴──────────────────────┴──────┴────────────────────> Time │◄─ Pulsating Input Current (High Input EMI Filter Needed)

3. Mathematical Formulations & Component Sizing

3.1 Voltage Conversion Ratio:

Applying volt-second balance across inductor L in steady state:

\int_0^{T_s} v_L(t) \, dt = (V_{IN} - V_o) \cdot D \cdot T_s + (-V_o) \cdot (1 - D) \cdot T_s = 0

(V_{IN} - V_o) D - V_o (1 - D) = 0 \implies V_o = D \cdot V_{IN}

D = \frac{V_o}{V_{IN}}

3.2 Inductor Current Ripple (\Delta I_L):

\Delta I_L = \frac{(V_{IN} - V_o) \cdot D}{f_s \cdot L} = \frac{V_o \cdot (1 - D)}{f_s \cdot L}

Standard Engineering Rule: Target inductor ripple current ratio r = \frac{\Delta I_L{I_o between 20\% \dots 40\% (nominally r = 0.3).

3.3 Inductor Value Calculation:

L = \frac{V_o \cdot (V_{IN} - V_o)}{V_{IN} \cdot f_s \cdot \Delta I_L} = \frac{V_o \cdot (1 - D)}{f_s \cdot (r \cdot I_o)}

3.4 Boundary Between CCM and DCM:

At the boundary between continuous and discontinuous conduction (I_{min = 0 \implies I_o = \frac{\Delta I_L{2):

I_{o,crit} = \frac{V_o \cdot (1 - D)}{2 \cdot f_s \cdot L}

If load current drops below I_{o,crit, the converter enters DCM, where output voltage becomes load-dependent:

V_{o,DCM} = V_{IN} \cdot \frac{2}{1 + \sqrt{1 + \frac{8 L f_s}{D^2 R_L}}}

3.5 Output Capacitor Sizing & ESR Ripple:

Total output voltage ripple \Delta V_o is the superposition of capacitive charge ripple and capacitor Equivalent Series Resistance (ESR):

\Delta V_o = \Delta V_{C} + \Delta V_{ESR} = \frac{\Delta I_L}{8 \cdot f_s \cdot C_o} + \Delta I_L \cdot R_{ESR}

For ceramic capacitors (where R_{ESR \approx 2 \dots 5\,m\Omega), capacitive term dominates:

C_o \ge \frac{\Delta I_L}{8 \cdot f_s \cdot \Delta V_{o,allowable}}

4. Synchronous Buck Implementation & Gate-Drive Details

Bootstrap Circuit for High-Side N-MOSFET +V_DRV (+5V / +12V) │ ┌┴┐ D_boot (Schottky Diode) └┬┘ │ ├───[ C_boot (0.1uF) ]───┐ │ │ ┌─────┴─────┐ │ │ BOOT │ │ │ │ │ PWM_HS ───────>│ GATE_HS ├───[ R_g ]─── Gate Q1 (High-Side N-FET) │ │ │ │ PHASE/SW ├──────────────────┼─── Switching Node (SW) └─────┬─────┘ │ │ Source Q1 GND

4.1 The Bootstrap Operating Mechanism:

When low-side switch Q_2 turns ON, node SW is pulled to ground (0\,V).

Bootstrap capacitor C_{boot is charged from V_{DRV via diode D_{boot to V_{DRV - V_F \approx 4.7\,V \dots 11.5\,V.

When Q_2 turns OFF and Q_1 turns ON, node SW swings to V_{IN.

The floating bootstrap rail swings to V_{IN + V_{Cboot, maintaining V_{GS,Q1 > V_{th above the drain voltage to keep the high-side N-channel MOSFET fully enhanced.

4.2 Dead-Time Shoot-Through Prevention:

If Q_1 and Q_2 conduct simultaneously for even 10\,ns, the full V_{IN rail shorts directly to GND (shoot-through), generating currents in excess of 100\,A and instantly destroying the MOSFETs.

Adaptive Dead-Time Controllers: Gate drivers monitor the gate voltage of Q_1 and wait until V_{GS,Q1 < 1.0\,V before commanding Q_2 ON, and vice versa.

5. Design Example: 12V to 1.2V, 20A Processor VRM

Input Voltage: V_{IN = 12\,V ± 10\%

Output Voltage: V_o = 1.2\,V, I_o = 20\,A

Switching Frequency: f_s = 500\,kHz

Duty Cycle: D = \frac{1.2\,V{12\,V = 0.10 (10\%)

Inductor Sizing ($r = 0.3 \implies \Delta I_L = 6.0\,\text{A}$):

L = \frac{1.2\,\text{V} \cdot (1 - 0.10)}{500\,000\,\text{Hz} \cdot 6.0\,\text{A}} = \frac{1.08}{3\,000\,000} = 0.36\,\mu\text{H} \implies \text{Select } 0.33\,\mu\text{H} \dots 0.47\,\mu\text{H}

Output Capacitor Sizing ($\Delta V_o \le 15\,\text{mV}$):

C_o \ge \frac{6.0\,\text{A}}{8 \cdot 500\,000\,\text{Hz} \cdot 0.015\,\text{V}} = 100\,\mu\text{F} \implies \text{Implement with } 3 \times 47\,\mu\text{F X7R Ceramic Caps}