Ćuk DC-DC Converter: Capacitive Energy Transfer & Zero-Ripple Physics

Welcome to the VVDN Engineering Hub Technical Dossier on the Ćuk DC-DC Converter. Invented by Dr. Slobodan Ćuk at Caltech, this topology is unique among switched-mode power converters: it uses an intermediate capacitor for energy transfer rather than an inductor's magnetic field, and features continuous current at both input and output ports, resulting in exceptionally low electromagnetic interference (EMI).

This document explores the circuit topology, derives the volt-second and charge-balance equations, examines switch stresses, and details the Coupled-Inductor Zero-Ripple Phenomenon.

1. Operating Principle & Ćuk Topology

The Ćuk Converter comprises two inductors (L_1, L_2), an active switch (Q_1), a diode (D_1), an energy-transfer coupling capacitor (C_1), and an output filter capacitor (C_o):

======================================================================================================================== DETAILED HARDWARE SCHEMATIC: INVERTING ĆUK DC-DC CONVERTER (+12V -> -12V/3A WITH ZERO-RIPPLE CAPABILITY) ======================================================================================================================== +VIN (+12V DC) ─────────────────────────────────────────────────────────────┐ │ │ [F1: 5A Fuse] │ │ │ ┌┴─────────────────┐ │ │ CIN_BULK │ │ ┌┴┐ 150µF/35V Poly ┌┴┐ CIN_CER │ │ │ (Low-ESR 14mΩ) │ │ 2x 10µF/25V X7R │ └┬┘ └┬┘ │ │ │ │ │ │ INPUT CHOKE INDUCTOR │ │ │ +---[ L1: 33µH / 5.5A Shielded ]---+ │ │ │ ¦ Wurth 7443320330 ¦ │ │ │ ¦ (DCR = 16mΩ, Isat = 6.2A) ¦ │ │ │ +----------------------------------+ │ │ │ │ │ │ │ +=== NODE SW1 (Switching Node) │ │ ¦ (0V to +24.5V pulse) │ │ ¦ │ │ ENERGY TRANSFER CAPACITOR¦ │ │ +------[ C1: 10µF / 50V X7R ]---+ │ │ ¦ TDK C3225X7R1H106M │ │ ¦ (Carries high RMS ripple) │ │ ¦ ¦ │ │ ¦ Drain (D) +---[ R_snub: 3.3Ω / 1W ] │ │ ¦ ┌┴──────────────┐ ¦ │ │ │ ¦ │ Q1: N-MOSFET │ ¦ [ C_snub: 470pF ] │ │ ¦ │ BSC040N10NS5 │ ¦ │ │ │ ¦ +->│ (100V, 4.0mΩ) │ ¦ PGND │ │ ¦ ¦ └┬──────────────┘ ¦ │ │ ¦ ¦ │ Source (S) ¦ │ │ ¦ ¦ │ ¦ │ │ ¦ ¦ +---[ CS+ ] ¦ │ │ ¦ ¦ │ ¦ │ │ ¦ ¦ ┌┴┐ R_shunt: 10mΩ / 2W ¦ │ │ ¦ ¦ │ │ Metal Alloy ¦ │ │ ¦ ¦ └┬┘ (Current Sense) ¦ │ │ ¦ ¦ │ ¦ │ │ ¦ ¦ PGND ¦ │ │ ¦ ¦ ¦ │ │ ¦ ¦ GATE DRIVER INTERFACE ¦ │ │ ¦ +--[ R_g: 4.7Ω ]<-- UCC27517 ¦ │ │ ¦ ¦ │ │ +=== NODE SW2 (Diode Node) ¦ │ │ ¦ (-24.5V to 0V pulse) ¦ │ │ ¦ ¦ │ │ +---[ D1: Schottky Diode ] ¦ │ │ ¦ V30100P (100V / 30A) ¦ │ │ ¦ [ Cathode to SW2 ] ¦ │ │ ¦ [ Anode to PGND ] ¦ │ │ ¦ │ ¦ │ │ ¦ PGND ¦ │ │ ¦ ¦ │ │ ¦ OUTPUT CHOKE INDUCTOR ¦ │ │ +---[ L2: 33µH / 5.5A Shielded ]+ │ │ Wurth 7443320330 ¦ │ │ (DCR = 16mΩ) ¦ │ │ ¦ │ │ +=== -VOUT (-12V/3A) │ │ │ │ │ ┌┴──────────────────┐ │ │ │ COUT_BULK │ │ │ ┌┴┐ 2x 220µF/25V ┌┴┐ COUT_CER │ │ │ │ Poly (ESR=12mΩ) │ │ 3x 10µF/25V │ │ └┬┘ (+) to PGND └┬┘ X7R 1210 │ │ │ (-) to -VOUT │ PGND┴──────────────────┴──────────────────────────────────────+───────────────────┴─── PGND (0V Rail)

1.1 Detailed Component Connection Netlist & Terminal Details:

| Net Name | Source (Pin / Terminal) | Destination (Pin / Terminal) | Electrical Function | Hardware Engineering Notes |
| --- | --- | --- | --- | --- |
| +VIN_FILT | Fuse F_1 Output | C_{in bank (+), Inductor L_1 Pin 1 | Filtered positive input DC bus | Carries smooth DC current with minimal ripple due to input inductor L_1. |
| SW1 (Switch Node 1) | Inductor L_1 Pin 2, Q_1 Drain | Transfer Capacitor C_1 Left Plate, Snubber | High-voltage switching node ($0\,\text{V} \dots V_{IN} + \ | V_o\ |
| SW2 (Switch Node 2) | Transfer Capacitor C_1 Right Plate | Diode D_1 Cathode, Inductor L_2 Pin 1 | Negative-swinging switching node | Swings between $-(V_{IN} + \ |
| -VOUT (Negative Rail) | Inductor L_2 Pin 2 | C_{out Negative terminal, Load (-) | Continuous-current regulated negative rail | Because L_2 is in series with output, output ripple current is pure triangular and very low. |
| PGND | C_{in (-), R_{shunt (-), Diode D_1 Anode, C_{out (+) | System power ground return | Common zero-volt reference | Diode D_1 anode connects directly to PGND; carries continuous freewheeling return current. |

1.2 Component Bill of Materials & Parametric Specifications:

| RefDes | Component Description | Manufacturer & Part Number | Key Electrical Specifications | Critical Design Constraint |
| --- | --- | --- | --- | --- |
| $Q_1$ | Low-Side N-MOSFET | Infineon BSC040N10NS5 | V_{DS = 100\,V, I_D = 100\,A, R_{DS(on) = 4.0\,m\Omega, Q_g = 27\,nC | Low ground-referenced driver complexity; V_{DS stress equals $V_{IN} + \ |
| $D_1$ | Catch Schottky Diode | Vishay V30100P | V_{RRM = 100\,V, I_F = 30\,A, V_F = 0.55\,V, t_{rr < 20\,ns | Anode grounded; cathode tied to C_1/L_2. Must handle combined current I_{L1 + I_{L2. |
| $C_1$ | Energy Transfer Capacitor | TDK C3225X7R1H106M | 10\,µF, 50\,V, X7R Ceramic, 1210 package | Crucial Component: Carries full AC load current ripple (I_{C1,rms \approx 4.5\,A); must use low-loss MLCC or film. |
| $L_1, L_2$ | Coupled / Dual Inductors | Würth Elektronik 7443320330 | 2 × 33\,µH, I_{sat = 6.2\,A, DCR = 16\,m\Omega | Can be wound on a single shared core to steer ripple to zero on either input or output side! |
| $C_{out,bulk}$ | Output Bulk Capacitor | Panasonic 25SVPF220M | 2 × 220\,µF, 25\,V, OS-CON Polymer, ESR = 12\,m\Omega | Positive terminal grounded; negative terminal connected to -V_{OUT. |

1.1 Conduction Intervals:

Interval 1: Switch OFF ($D \cdot T_s < t \le T_s$):

Switch Q_1 is OFF. Diode D_1 is forward-biased.

Input inductor L_1 charges coupling capacitor C_1 via diode D_1.

Output inductor L_2 delivers stored energy through diode D_1 into output capacitor C_{out and the load.

The voltage across C_1 in steady state settles to:

V_{C1} = V_{IN} + |V_o|

Interval 2: Switch ON ($0 < t \le D \cdot T_s$):

Switch Q_1 turns ON, pulling the left terminal of C_1 to GND (0\,V).

The right terminal of C_1 drops to -(V_{IN + |V_o|), reverse-biasing diode D_1.

Energy stored in C_1 is transferred directly into output inductor L_2 and the load.

Concurrently, input inductor L_1 draws energy directly from V_{IN to GND.

2. Voltage and Current Waveforms

Ćuk Converter Key Waveforms i_L1 ^ Continuous Input Current │ / \ / \ │──────────/ \──────────────────/ \───────────────────> Time i_L2 ^ Continuous Output Current │ / \ / \ │──────────/ \──────────────────/ \───────────────────> Time V_Q1 ^ Vin+¦Vo¦┼──────────────┐ ┌──────────────┐ │ │ │ │ 0V ┼──────────────┴──────────────────────┴──────────────┴────> Time │◄─── D*Ts ───►│◄──── (1-D)*Ts ──────►│

3. Mathematical Formulations & Transfer Function

3.1 Voltage Conversion Ratio:

Applying volt-second balance to input inductor L_1:

V_{IN} \cdot D + (V_{IN} - V_{C1}) \cdot (1 - D) = 0 \implies V_{C1} = \frac{V_{IN}}{1 - D}

Applying volt-second balance to output inductor L_2:

(V_{C1} - |V_o|) \cdot D + (-|V_o|) \cdot (1 - D) = 0 \implies V_{C1} \cdot D - |V_o| = 0 \implies |V_o| = D \cdot V_{C1}

Substituting V_{C1:

|V_o| = V_{IN} \cdot \frac{D}{1 - D} \implies V_o = -V_{IN} \cdot \frac{D}{1 - D}

3.2 Component Sizing Equations:

Input Inductor ($L_1$):

L_1 = \frac{V_{IN} \cdot D}{f_s \cdot \Delta I_{L1}}

Output Inductor ($L_2$):

L_2 = \frac{|V_o| \cdot (1 - D)}{f_s \cdot \Delta I_{L2}}

Energy Transfer Capacitor ($C_1$):

C_1 carries the full load current during switch ON time. To restrict capacitor ripple to \Delta V_{C1:

C_1 = \frac{I_o \cdot D}{f_s \cdot \Delta V_{C1}}

Switch & Diode Stresses:

Both the MOSFET Q_1 and diode D_1 must withstand the combined rail voltage:

V_{DS,max} = V_{diode,rev} = V_{IN} + |V_o|

4. The Coupled-Inductor Zero-Ripple Phenomenon

Because both inductors L_1 and L_2 experience identical AC voltage waveforms across their terminals ((V_{IN - V_{C1) and -|V_o|), they can be wound onto a single magnetic core:

Coupled-Inductor Ćuk Configuration L1 (Primary) C1 L2 (Secondary) +Vin ─────────────^^^^^^──────┬──────────[ ]─────────┬───^^^^^^─────────┬───> -Vout ││ │ │ ││ │ ││ M │ │ ││ M ┌┴┐ ││ │ │ ││ │ │ Co GND ─────────────────────────┴───────────────────────┴─────────────────┴─── GND

4.1 Ripple Steering Mechanism:

With mutual inductance M = k \sqrt{L_1 L_2 between the two windings:

\frac{di_{L1}}{dt} = \frac{v_1 \cdot (L_2 - M)}{L_1 L_2 - M^2}, \quad \frac{di_{L2}}{dt} = \frac{v_2 \cdot (L_1 - M)}{L_1 L_2 - M^2}

If the turns ratio is selected such that:

M = L_2 \implies k \sqrt{\frac{L_1}{L_2}} = 1 \implies \frac{N_1}{N_2} = \frac{1}{k}

The AC ripple current in inductor L_2 is driven identically to zero ($\Delta i_{L2} = 0$)!

All AC switching ripple is "steered" into the input inductor L_1, yielding completely DC, ripple-free current at the output without requiring a massive electrolytic capacitor!

5. Engineering Trade-Offs & Application Domain

| Advantage | Engineering Challenge |
| --- | --- |
| Continuous currents at both input and output (ultra-low conducted EMI). | Inverted output voltage polarity. |
| Zero output ripple achievable via magnetic coupling. | High component count: 2 inductors + 2 capacitors. |
| Non-pulsating currents extend battery and capacitor lifetime. | High voltage stress ($V_{IN} + |
| Ideal for ultra-low noise instrumentation, audio amplifiers, RF PA bias. | Coupling capacitor C_1 carries high AC RMS ripple current (I_{C1,rms \approx I_o \sqrt{D). |