CAN Hardware Connection & Circuit Schematic Guide

This engineering guide provides detailed, practical hardware schematics and connection blueprints for the CAN (Controller Area Network, ISO 11898) and CAN FD differential automotive bus across high-speed transceivers, split termination networks, common-mode chokes, ESD protection arrays, and galvanically isolated nodes for high-voltage EV/industrial systems.

1. High-Speed CAN / CAN FD Transceiver Circuit (TJA1051 / TCAN1042)

Connecting a microcontroller's integrated CAN controller (e.g., STM32 bxCAN / FDCAN, NXP S32K) to a physical differential bus:

+5.0V (VCC) +3.3V (VIO) │ │ ┌───┴───┐ ┌───┴───┐ │ 0.1µF │ Decoupling│ 0.1µF │ Decoupling └───┬───┘ └───┬───┘ │ │ ┌────┴───────────────────┴──────────────────────────────────────────┐ │ VCC (Pin 3) VIO (Pin 5) │ │ │ MCU │ TXD (Pin 1) CANH (Pin 7) ├─────> CAN_H (To Bus) ────┤ (3.3V Logic) │ │ RXD (Pin 4) CANL (Pin 6) ├─────> CAN_L (To Bus) ◄───┤ (3.3V Logic) │ │ S / STB (Pin 8) ──── GND (Normal) GND (Pin 2) │ │ or MCU GPIO (Chassis Return) │ └───────────────────────────────────────────┬───────────────────────┘ │ GND

Critical Circuit Highlights:

The $V_{IO}$ Logic Translation Pin:

The differential bus drivers require a +5.0\,V supply (V_{CC) to generate the standard +2.5\,V \dots +3.5\,V dominant differential voltages.

However, modern microcontrollers operate on +3.3\,V logic.

Connecting +3.3\,V to the $V_{IO}$ pin (available on TJA1051T/3, TCAN1042V) shifts the TXD and RXD logic thresholds directly to +3.3\,V, eliminating the need for external level shifters!

Silent Mode / Standby Pin (`S` / `STB`):

Tied to Ground for normal active transmission and reception.

Pulling S HIGH disables the transmitter (Silent Mode), preventing the node from sending frames or error flags while keeping the receiver listening (ideal for software diagnostics, babbling-node containment, or listen-only listen modes).

2. Split Termination Network (60\Omega + 60\Omega with Center Capacitor)

While a single 120\,\Omega resistor across CANH and CANL provides basic differential impedance matching, automotive OEMs mandate Split Termination to suppress common-mode electromagnetic radiation:

CAN_H Line ──────────────────────────────────────┬───────────────────────────────── │ [R1] 60.4Ω 1% Metal Film │ ├───[ C_split: 4.7nF ]─── GND (Chassis Ground) │ [R2] 60.4Ω 1% Metal Film │ ──────────────────────────────────────┴───────────────────────────────── CAN_L Line

Why Split Termination is Superior to Single 120\,\Omega:

Differential Resistance: R_{diff = R_1 + R_2 = 60.4\,\Omega + 60.4\,\Omega \approx 120.8\,\Omega (perfect differential impedance matching).

Common-Mode Low-Pass Filter: Common-mode noise appearing equally on CAN_H and CAN_L sees the center node as a virtual AC ground:

f_{cutoff} = \frac{1}{2 \pi \cdot (R/2) \cdot C_{split}} = \frac{1}{2 \pi \cdot (30\,\Omega) \cdot (4.7\,\text{nF})} \approx 1.13\,\text{MHz}

High-frequency common-mode noise and ground bounce are shunted directly to Ground before radiating through the wiring harness.

3. High-Reliability EMC & Surge Clamping Network (CMC + TVS Diode)

For automotive powertrain, chassis, and industrial robotics exposed to severe electromagnetic fields and ESD discharges:

CAN Transceiver Wiring Harness Connector ┌─────────────┐ Common Mode Choke ┌──────────────────────┐ │ CANH (Pin 7)├────────────[ Coil 1 (51µH) ]──────────────────►│ Pin 1 (CAN_H) │ │ │ │ │ │ │ CANL (Pin 6)├────────────[ Coil 2 (51µH) ]────────┼─────────►│ Pin 2 (CAN_L) │ └─────────────┘ │ │ │ ┌┴────┴┐ │ Pin 3 (GND / Shield) │ │NUP2105 └──────────┬───────────┘ │PESD2CAN │ │Dual TVS │ └───┬──┘ │ │ │ Chassis Earth ──────────┘

Component Functions:

Common Mode Choke (CMC, $51\,\mu\text{H} \dots 100\,\mu\text{H}$):

Windings are bifilar wound on a common ferrite core.

Differential signals pass unhindered with near-zero insertion loss.

Unwanted common-mode noise currents induce opposing flux that chokes out the disturbance, providing up to 30\,dB attenuation between 10\,MHz and 100\,MHz.

Automotive Dual TVS Diode (NXP PESD2CAN / onsemi NUP2105L):

SOT-23 package integrating two bidirectional clamping diodes connected to Ground.

Ultra-low capacitance (C_d < 15\,pF) to prevent distorting 2\,Mbps - 5\,Mbps CAN FD bit edges.

Withstands ± 30\,kV ESD contact discharges (IEC 61000-4-2) and ISO 7637-2 electrical transients.

4. Galvanically Isolated CAN Transceiver (TI ISO1042 / ADI ADM3053)

Essential in Electric Vehicles (EV main traction inverters, 400V/800V Battery Management Systems, and EVSE DC Fast Chargers) to bridge between high-voltage battery domains and low-voltage vehicle networks:

Low-Voltage Safe Domain (Microcontroller) High-Voltage Hazardous Domain (400V Inverter / Bus) +3.3V_MCU +5.0V_ISO (From Isolated DC-DC Supply) │ │ ┌───┴───┐ ┌───┴───┐ │ 0.1µF │ Decoupling │ 0.1µF │ Decoupling └───┬───┘ └───┬───┘ │ │ ┌───┴───────────────────────┐ Galvanic Isolation ┌───────────┴────────────────────────────────┐ │ VCC1 (Pin 1) │ Barrier │ VCC2 (Pin 16)│ │ │ ====== │ │ MCU │ TXD (Pin 2) ├──────►¦¦──────►─────┤ Differential Driver CANH (Pin 14)├───> CAN_H ────┤ │ ¦¦ │ │ │ RXD (Pin 3) │◄──────¦¦◄─────┼─────┤ Differential Receiver CANL (Pin 13)├───> CAN_L ◄───┤ │ ¦¦ │ │ │ │ GND1 (Pin 4) │ │ │ GND2 (Pin 9) │ └───────────┬───────────────┘ │ └───────────┬────────────────────────────────┘ │ │ │ GND_MCU (Chassis Ground) │ GND_ISO (Isolated High-Voltage Return) ▼ Capacitive Isolation Barrier (Reinforced Insulation: 5000 Vrms)

5. Multi-Node Linear Bus Wiring Topology

Node #1 (Extreme End) Node #2 (Stub Node) Node #3 (Extreme End) ┌───────────────────┐ ┌───────────────────┐ ┌───────────────────┐ │ Transceiver │ │ Transceiver │ │ Transceiver │ │ [Split Term 120Ω] │ │ [No Termination!] │ │ [Split Term 120Ω] │ └─────────┬─────────┘ └─────────┬─────────┘ └─────────┬─────────┘ │ │ (Stub < 0.3m) │ CAN_H ────────o──────────────────────────────o──────────────────────────────o──────── CAN_L ────────o──────────────────────────────o──────────────────────────────o──────── │ │ [120Ω Split] [120Ω Split]

Fundamental Physical Topology Rules:

Strict Linear Daisy-Chain (No Star / Tree Routing):

CAN is strictly a transmission line bus. Star topologies create impedance discontinuities and severe signal reflections that destroy bit decoding.

Termination Exclusively at Extreme Ends:

Exactly two 120\,\Omega termination networks must exist across the entire network—one at each physical extreme.

Total bus DC resistance measured with an ohmmeter between CAN_H and CAN_L with power OFF must read:

R_{bus,total} = 120\,\Omega \parallel 120\,\Omega \approx 60\,\Omega

Keep Stubs Shorter than $0.3\,\text{m}$:

Connections from the linear backbone trunk to an individual ECU must be kept as short as physically possible (< 0.3\,meters) to prevent transmission line stub reflections.