SPI Hardware Connection & Circuit Schematic Guide

This engineering guide provides detailed, practical hardware schematics and connection blueprints for the SPI (Serial Peripheral Interface) across single-slave interfaces, multi-slave star and daisy-chain networks, high-speed Quad-SPI (QSPI) Flash, galvanically isolated bridges, and high-frequency level shifting.

1. Single-Target Standard 4-Wire SPI Circuit (MCU to Sensor / ADC)

The fundamental interface connecting a microcontroller to a peripheral sensor (e.g., Bosch BMI088 IMU, TI ADS1118 ADC):

Controller (Master MCU) Target Peripheral (IMU / ADC / Flash) ┌─────────────────────────┐ ┌─────────────────────────┐ │ SCLK ├───[ 22Ω ]──────────────────►│ SCLK │ │ │ │ │ │ MOSI / COPI ├───[ 22Ω ]──────────────────►│ MOSI / SDI │ │ │ │ │ │ MISO / CIPO │◄───────────────────[ 22Ω ]──┤ MISO / SDO │ │ │ │ │ │ CS0# ├───[ 22Ω ]──┬───────────────►│ CS# / NSS │ │ │ │ │ │ │ GND ├────────────┼────────────────┤ GND │ └─────────────────────────┘ │ └─────────────────────────┘ [10k] Pull-Up │ +3.3V (VDD)

Critical Hardware Design Rules:

Series Damping Resistors ($R_{series} = 22\,\Omega \dots 33\,\Omega$):

High-speed SPI edges have fast slew rates (t_{rise < 1.5\,ns). At frequencies ≥ 10\,MHz, PCB traces longer than a few centimeters behave as transmission lines.

Placing series damping resistors adjacent to each driver pin (SCLK, MOSI, CS# at the Controller; MISO at the Peripheral) absorbs reflections, eliminates ringing, and prevents double-clocking artifacts.

Chip Select Pull-Up Resistor ($10\,\text{k}\Omega$ to $V_{DD}$):

When the microcontroller powers on or is held in reset, its GPIO pins float in high-impedance (High-Z) mode.

An external pull-up resistor guarantees that \overline{CS remains firmly at Logic HIGH during system boot, keeping the peripheral disabled and preventing spurious commands on the SPI bus.

Decoupling Capacitors:

Place a 0.1\,µF X7R ceramic capacitor directly adjacent to the peripheral's V_{DD pin, with a low-inductance via directly to the ground plane.

2. Multi-Target Dedicated Chip-Select Topology (Star Routing)

When connecting multiple distinct SPI peripherals (e.g., NOR Flash, IMU, and TFT Display) to a single SPI peripheral:

Controller (Master MCU) ┌─────────────────────┐ │ SCLK ├───┬──────────────────────┬──────────────────────┐ │ │ │ │ │ │ MOSI ├───┼──────────┬───────────┼──────────┬───────────┤ │ │ │ │ │ │ │ │ MISO │◄──┼──────┐ │ │ │ │ │ │ │ │ │ │ │ │ │ CS0# ├───┼──────┼───┼───────────┼──────────┼─────┐ │ │ CS1# ├───┼──────┼───┼─────┐ │ │ │ │ │ CS2# ├───┼──────┼───┼─────┼─────┼────┐ │ │ │ └─────────────────────┘ │ │ │ │ │ │ │ │ │ ▼ │ ▼ ▼ ▼ │ ▼ ▼ ▼ ┌───────────┴───────┐ ┌──────────┴─────┐ ┌─────────────┴─────┐ │ SCLK MISO MOSI CS#│ │ SCLK MISO MOSI │ │ SCLK MISO MOSI CS#│ │ │ │ CS# │ │ │ │ Target #1 │ │ Target #2 │ │ Target #3 │ │ (NOR Flash) │ │ (IMU Sensor) │ │ (TFT Display) │ └───────────────────┘ └────────────────┘ └───────────────────┘

Essential Hardware Considerations:

MISO High-Impedance (Tri-State) Verification:

Peripherals must float their MISO pin in high-impedance mode whenever their \overline{CS is HIGH.

Trap: Some cheap display controllers or sensors fail to tri-state MISO when deselected, permanently corrupting the shared MISO line for all other targets. Test with an oscilloscope: verify MISO floats freely when all CS lines are HIGH.

Shared Bus Capacitive Loading:

Each added device adds 5\,pF \dots 10\,pF of pin input capacitance plus trace capacitance (C_L \approx 1\,pF/cm).

Total bus capacitance slows clock rise times: t_r \approx 2.2 × R_{driver × C_{total. Reduce clock frequency if more than 3 to 4 targets share the bus.

3. Multi-Target Daisy-Chain Topology (Shift-Register Architecture)

Used extensively in LED matrix drivers (MAX7219, TLC5940), digital potentiometers, and relay drivers (74HC595) to drive dozens of devices using only 3 MCU pins:

Controller (MCU) ┌─────────────────────┐ │ SCLK ├───┬──────────────────────┬──────────────────────┐ │ │ │ │ │ │ CS# ├───┼──────────┬───────────┼──────────┬───────────┤ │ │ │ │ │ │ │ │ MOSI ├───┼──────────┼─────┐ │ │ │ │ │ │ │ │ │ │ │ │ MISO │◄──┼──────────┼─────┼─────┼──────────┼─────┐ │ └─────────────────────┘ │ │ │ │ │ │ │ ▼ ▼ ▼ ▼ ▼ │ ▼ ┌───────────────────┐ ┌───────────────────┐ ┌────┴──────────────┐ │ SCLK CS# │ │ SCLK CS# │ │ SCLK CS# │ │ │ │ │ │ │ │ DIN DOUT ├─► DIN DOUT ├─► DIN DOUT ├──┘ │ │ │ │ │ │ │ Device #1 │ │ Device #2 │ │ Device #3 │ │ (74HC595 #1) │ │ (74HC595 #2) │ │ (74HC595 #3) │ └───────────────────┘ └───────────────────┘ └───────────────────┘

Working Principle:

Data is clocked in serially as a continuous multi-byte stream.

As new bits enter Device #1 via DIN, overflow bits from Device #1's shift register emerge on DOUT and feed directly into Device #2's DIN.

When the entire payload (e.g., 24 bits for three 8-bit devices) has been shifted, the Controller pulses the shared $\overline{\text{CS}}$ line to latch the data into all devices simultaneously.

4. High-Speed Quad-SPI (QSPI) NOR Flash Circuit (Winbond W25Q128JV)

For high-speed firmware execution (Execute-in-Place / XiP) and graphic assets streaming:

Host Microcontroller (STM32 / NXP i.MX) Quad-SPI NOR Flash (W25Q128JV, 133MHz) ┌─────────────────────────────────────┐ ┌────────────────────────────────────┐ │ QSPI_CLK ├───[ 22Ω ]──────►│ CLK (Pin 6) │ │ │ │ │ │ QSPI_NSS ├───[ 22Ω ]──┬───►│ CS# (Pin 1) │ │ │ │ │ │ │ QSPI_IO0 ├───[ 22Ω ]──┼───►│ DI / IO0 (Pin 5) │ │ QSPI_IO1 ◄───[ 22Ω ]──┼───┤ DO / IO1 (Pin 2) │ │ QSPI_IO2 ◄───[ 22Ω ]──┼───┤ WP# / IO2 (Pin 3) │ │ QSPI_IO3 ◄───[ 22Ω ]──┼───┤ HOLD# / RESET# / IO3 (Pin 7) │ │ │ │ │ │ │ VDD (+3.3V)├────────────┼───►│ VCC (Pin 8) │ │ │ [10k] │ │ │ GND ├────┬───────┼───►│ GND (Pin 4) [ 0.1µF ] [ 4.7µF ] │ └─────────────────────────────────────┘ │ │ └──────────────────┬─────────┬───────┘ GND +3.3V GND GND

PCB Layout Guidelines for QSPI:

Length Matching: Match trace lengths between CLK and all 4 data lines (IO0 through IO3) within ± 1.0\,mm (\approx 6\,ps skew) to prevent data phase misalignment at 104\,MHz - 133\,MHz.

Continuous Ground Return Plane: Route all 6 QSPI lines directly above an uninterrupted, solid ground plane layer. Never cross split plane boundaries.

Controlled Characteristic Impedance: Design traces with 50\,\Omega ± 10\% single-ended characteristic impedance.

5. Galvanically Isolated High-Speed SPI (ADI ADuM3401 / TI ISO7741)

Used in high-voltage automotive EV Battery Management Systems (BMS), motor drives, and solar inverters to cross hazardous voltage barriers (> 1500\,V_{rms):

Low-Voltage Safe Domain (MCU) High-Voltage Domain (Isolated ADC / BMS) +3.3V_MCU +3.3V_ISO (From Isolated DC-DC) │ │ ┌───┴───┐ ┌───┴───┐ │ 0.1µF │ Decoupling │ 0.1µF │ Decoupling └───┬───┘ └───┬───┘ │ │ ┌───┴───────────────────────┐ Galvanic Isolation ┌───────────┴───────────────────────┐ │ VDD1 (Pin 1) │ Barrier │ VDD2 (Pin 16)│ │ │ ====== │ │ MCU │ VIA (Pin 3) SCLK ├──────►¦¦──────►─────┤ VOA (Pin 14) SCLK │ Isolated ────┤ │ ¦¦ │ ├─── BMS │ VIB (Pin 4) MOSI ├──────►¦¦──────►─────┤ VOB (Pin 13) MOSI │ IC / ────┤ │ ¦¦ │ ├─── ADC │ VIC (Pin 5) CS# ├──────►¦¦──────►─────┤ VOC (Pin 12) CS# │ ────┤ │ ¦¦ │ │ │ VOD (Pin 6) MISO │◄──────¦¦◄─────┼─────┤ VID (Pin 11) MISO │ ◄───┤ │ ¦¦ │ │ │ │ GND1 (Pins 2, 8) │ │ │ GND2 (Pins 9, 15) │ └───────────┬───────────────┘ │ └───────────┬───────────────────────┘ │ │ │ GND_MCU (Chassis Ground) │ GND_ISO (Isolated Battery -) ▼ Reverse Channel for MISO

Critical Isolator Engineering Constraints:

Propagation Delay Limit: Digital isolators introduce 8\,ns \dots 15\,ns propagation delay in each direction. Total round-trip delay is doubled (2 × t_{prop \approx 20\,ns - 30\,ns).

Maximum Safe Clock Rate: With a 12\,ns isolator and 10\,ns peripheral clock-to-output time (t_{CO), the maximum safe read frequency is capped at:

f_{max} \le \frac{1}{2 \cdot (t_{prop} + t_{CO} + t_{SU})} \approx \frac{1}{2 \cdot (12\text{ns} + 10\text{ns} + 4\text{ns})} \approx 19.2\,\text{MHz}