UART Hardware Connection & Circuit Schematic Guide

This engineering guide provides detailed, practical hardware schematics and connection blueprints for UART (Universal Asynchronous Receiver-Transmitter) across logic-level MCU connections, USB bridges, RS-232, RS-485 differential buses, and bidirectional voltage level translators.

1. Logic-Level (3.3V / 5V) MCU-to-MCU Cross-Over Connection

When directly interconnecting two microcontrollers on the same PCB or short board-to-board header:

Controller #1 (STM32 / ESP32) Controller #2 (MCU / Sensor) ┌───────────────────────────┐ ┌───────────────────────────┐ │ TXD ├───[ 22Ω ]────────────────────►│ RXD │ │ │ │ │ │ RXD │◄────────────────────[ 22Ω ]───┤ TXD │ │ │ │ │ │ RTS ├───[ 22Ω ]────────────────────►│ CTS │ │ │ │ │ │ CTS │◄────────────────────[ 22Ω ]───┤ RTS │ │ │ │ │ │ GND ├───────────────────────────────┤ GND │ └───────────────────────────┘ Common Ground └───────────────────────────┘

Critical Hardware Design Rules:

The TX/RX Cross-Over Rule:

Controller #1 TXD (Transmit Data, Output) must connect to Controller #2 RXD (Receive Data, Input).

Controller #1 RXD (Receive Data, Input) must connect to Controller #2 TXD (Transmit Data, Output).

Trap: Connecting TX to TX and RX to RX will cause bus contention and complete failure.

Common Ground Connection (Mandatory):

Asynchronous signaling requires a common voltage reference. Without a shared Ground wire, ground potential shifts will shift the logic threshold (V_{IL / V_{IH), causing severe framing errors and corrupted data.

Series Damping Resistors ($R_{series} = 22\,\Omega \dots 47\,\Omega$):

Placed in series on high-speed lines adjacent to the transmitting pin.

Absorbs high-frequency transmission line reflections, damps ringing caused by PCB trace parasitic inductance, and protects against ESD/current spikes.

Hardware Flow Control (RTS/CTS):

RTS (Request to Send / Ready to Receive) is driven by the receiving MCU and connects to the sending MCU's CTS (Clear to Send).

2. USB-to-UART Bridge Circuit (FTDI FT232RL / CP2102N / CH340N)

To interface microcontrollers with a host PC via USB Type-C:

USB-C Receptacle Host Microcontroller ┌──────────────┐ ┌───────────────────────┐ ┌──────────────────┐ │ VBUS (+5V) ├─────────►│ VBUS (Pin 19) │ │ │ │ │ │ │ │ │ │ D- ├─────────►│ USBDM (Pin 15) TXD ├─[ 1kΩ ]───►│ RXD │ │ D+ ├─────────►│ USBDP (Pin 16) RXD │◄─[ 1kΩ ]───┤ TXD │ │ │ │ RTS# ├───────────►│ CTS │ │ CC1 ──[5.1k]─┤ │ CTS# │◄───────────┤ RTS │ │ CC2 ──[5.1k]─┤ │ FT232RL / CP2102N │ │ │ │ GND ├────┬────►│ GND (Pins 7, 18, 21,25│ ┌──────┤ DTR# / NRST │ └──────────────┘ │ └───────────┬───────────┘ │ └──────────────────┘ │ │ 3V3OUT │ GND ├───[ 0.1µF ]─┐ [0.1µF] Cap (Auto-Reset) │ │ │ ▼ To VDDIO GND ▼ To MCU Reset Pin (NRST)

Hardware Circuit Details:

USB-C CC Pull-Down Resistors: Two separate 5.1\,k\Omega resistors from CC1 and CC2 to Ground identify the board as an upstream-facing device (UFP / Sink) to USB-C chargers.

Series Protection Resistors ($1\,\text{k}\Omega$): Placed on TXD and RXD lines between the bridge IC and MCU. If the MCU is unpowered while the USB bridge is connected, this prevents parasitic phantom-powering of the MCU through its internal I/O clamp diodes.

Auto-Reset Circuit for Firmware Upload: Connecting the bridge DTR# pin through a 100\,nF series ceramic capacitor to the microcontroller's active-LOW reset pin (NRST) automatically resets the MCU into the bootloader when flashing firmware.

3. High-Voltage RS-232 Transceiver Circuit (MAX3232 / SP3232)

For interfacing with legacy industrial machinery, PC serial ports, or avionics instruments using ± 12\,V bipolar signaling:

+3.3V / +5.0V VCC │ ┌───────┴───────┐ │ [ 0.1µF MLCC]│ Decoupling └───────┬───────┘ │ ┌───────┴───────────────────────┐ ┌───[ 0.1µF ]──┤ C1+ (Pin 1) │ │ │ MAX3232 Transceiver │ └───[ Cap ]─────┤ C1- (Pin 3) │ │ │ ┌───[ 0.1µF ]──┤ C2+ (Pin 4) │ │ │ │ └───[ Cap ]─────┤ C2- (Pin 5) │ │ │ │ V+ (Pin 2) ───[ 0.1µF ]─── GND │ V- (Pin 6) ───[ 0.1µF ]─── GND │ │ Microcontroller (3.3V Logic) │ │ DB9 Male Connector (DTE) ┌───────────────────────────┐ │ │ ┌──────────────────────┐ │ TXD ├────►│ T1IN (Pin 11) T1OUT (Pin 14)├───►│ Pin 2 (RXD on PC) │ │ │ │ (3.3V Logic) (±6V to ±12V) │ │ │ │ RXD │◄────┤ R1OUT (Pin 12) R1IN (Pin 13) │◄───┤ Pin 3 (TXD on PC) │ │ │ │ (±3V to ±25V) │ │ │ │ GND ├─────┤ GND (Pin 15) │ │ Pin 5 (Signal GND) │ └───────────────────────────┘ └───────────────────────────────┘ └──────────────────────┘

Critical Component Selection:

Charge Pump Flying Capacitors ($C_1, C_2, C_{V+}, C_{V-}$): Four 0.1\,µF X7R ceramic capacitors generate internal +2× V_{CC and -2× V_{CC bipolar supplies.

Inversion Characteristic: RS-232 inverts logic states physically:

Logic 0 (Space) = +5\,V \dots +12\,V

Logic 1 (Mark / Idle) = -5\,V \dots -12\,V

4. Half-Duplex RS-485 Differential Bus Transceiver Circuit (MAX485 / SN65HVD72)

For multi-drop industrial networks, factory automation (Modbus RTU), and long-distance communications up to 1200 meters:

+3.3V / +5.0V VCC │ ┌───┴───┐ │ 0.1µF │ Decoupling └───┬───┘ │ ┌─────┴─────────────────────────────────────────────────────────────────┐ │ VCC (Pin 8) │ │ MAX485 / SN65HVD72 │ │ ┌────────────────────┐ │ │ RO (Pin 1) ◄────────────────┤ Receiver Output │ │ │ │ │ A (Pin 6) │ │ RE# (Pin 2) ──┐ │ Differential ├─────────┬─────────┼──────> Non-Inverting Line (A / +) │ ├──┬──────────┤ Receiver │ │ │ │ DE (Pin 3) ───┘ │ │ │ B (Pin 7) │ │ │ │ Differential ├────┬────┼─────────┼──────> Inverting Line (B / -) │ DI (Pin 4) ──────┼─────────►│ Driver │ │ │ │ │ │ └────────────────────┘ │ │ │ │ GND (Pin 5) │ │ │ │ └──────────┬────────┼────────────────────────────────────┼────┼─────────┘ │ │ │ │ GND ▼ From MCU GPIO [120Ω] Termination (At extreme ends) (DIR: 0=Receive, 1=Transmit) │ │ ┌┴────┴┐ │ SM712│ Bidirectional TVS Diode │ ESD │ (Clamps -7V to +12V surges) └──┬───┘ │ Chassis Earth

Hardware Implementation Rules:

Direction Control (DE and $\overline{\text{RE}}$ Tied Together):

Driver Enable (DE, active HIGH) and Receiver Enable (\overline{RE, active LOW) are shorted together on the PCB and connected to a single MCU GPIO pin (RS485_DIR).

DIR = LOW (0): Driver is disabled (High-Z), receiver is enabled. The node listens to the bus.

DIR = HIGH (1): Driver is enabled, receiver is disabled. The node actively drives the differential pair.

$120\,\Omega$ Bus Termination:

A single 120\,\Omega 1\% metal-film resistor must be placed across A and B at the two extreme physical ends of the transmission line cable. Intermediate nodes must never have termination enabled.

Fail-Safe Biasing Network:

To prevent receiver chatter when all drivers are tri-stated (idle bus where V_A - V_B = 0\,V), install fail-safe biasing:

Pull-up resistor (560\,\Omega \dots 1\,k\Omega) from line A to V_{CC.

Pull-down resistor (560\,\Omega \dots 1\,k\Omega) from line B to Ground.

5. Bidirectional 3.3V to 5V Voltage Level-Shifter (MOSFET BSS138)

To interface a 3.3V microcontroller (STM32, ESP32) with a 5V UART device (5V GPS, Arduino, SIM800L):

+3.3V (LV Rail) +5.0V (HV Rail) │ │ [R1] 10k Pull-Up [R2] 10k Pull-Up │ │ ├──────────────────┐ ┌────────────────────┤ │ │ │ │ 3.3V MCU ─┴─ (LV Node) Source Drain (HV Node) ──────┴─ 5V Device (TXD or RXD) │ ┌─────────┐ │ (RXD or TXD) ┌──┴──┐ │ Body │ ┌──┴──┐ │ ├──┤ Diode ├──┤ │ N-Channel MOSFET (BSS138) │ │ │ ─┤>├── │ │ │ └──┬──┘ └─────────┘ └──┬──┘ │ │ └─── Gate ────────────┘ │ +3.3V (LV Power Rail)

Circuit Operation:

Idle State (Both lines HIGH):

The gate is at +3.3\,V. Source is pulled to +3.3\,V by R_1.

V_{GS = 3.3\,V - 3.3\,V = 0\,V < V_{TH. The MOSFET is OFF.

The 5V side is held at +5.0\,V by pull-up resistor R_2.

3.3V Device Pulls Line LOW (TXD = 0V):

Source is driven to 0\,V.

V_{GS = 3.3\,V - 0\,V = 3.3\,V > V_{TH. The MOSFET turns fully ON.

The conducting channel shorts Drain to Source, pulling the 5V side down to Ground.

5V Device Pulls Line LOW (TXD = 0V):

Drain is driven to 0\,V.

Current flows through the MOSFET's intrinsic body diode from Source to Drain, pulling Source down to \approx 0.6\,V.

Once Source drops, V_{GS rises above V_{TH, fully turning on the channel and pulling the 3.3V node down to pure Ground.