Dossier 1: System Specifications & Topology Selection

Welcome to Dossier 1 of the Step-by-Step 4-Switch Synchronous Buck-Boost Post-Regulator Design Guide. This document establishes the foundational design requirements, boundary operating conditions, topology trade-offs, and multi-mode operational state machine.

Step 1: System Requirements & Parameter Envelopes

The post-regulator operates downstream of the isolated primary Quasi-Resonant (QR) Flyback converter (+24.0\,V ± 1.0\% DC bus). It provides a digitally programmable output voltage from $5.0\,\text{V}$ to $20.0\,\text{V}$ DC at continuous currents up to $3.0\,\text{A}$ (60.0\,W maximum continuous output power).

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ STEP 1: COMPREHENSIVE PARAMETER DESIGN ENVELOPE │ ├────────────────────────────┬─────────────────────────────┬───────────────────────────────────────┤ │ Parameter / Specification │ Value Range / Unit │ Engineering Margin / Condition │ ├────────────────────────────┼─────────────────────────────┼───────────────────────────────────────┤ │ **Input Voltage ($V_{IN}$)**│ $+24.0\,\text{V}$ Nominal │ Delivered by Isolated Primary Flyback │ │ **Input Voltage Tolerance**│ $22.0\,\text{V} \dots 26.0\,\text{V}$│ Dynamic tolerance under line/load sag │ │ **Output Voltage ($V_O$)** │ $5.0\,\text{V} \dots 20.0\,\text{V}$│ Digitally programmable (4:1 span) │ │ **Voltage Programming Step│ $10\,\text{mV}$ Step Size │ Commanded via MCU 12-bit DAC │ │ **Continuous Current ($I_O$)│ $0.1\,\text{A} \dots 3.0\,\text{A}$│ Full rated current down to 5.0V out │ │ **Current Limit Step** │ $10\,\text{mA}$ Step Size │ Hardware Constant Current (CC) clamp │ │ **Maximum Output Power** │ $60.0\,\text{W}$ Continuous │ Full-range load ($20\,\text{V} \times 3\,\text{A}$) │ │ **Switching Frequency** │ $250\,\text{kHz}$ Fixed PWM │ Optimized for inductor size vs loss │ │ **Output Voltage Ripple** │ $< 25\,\text{mV}_{\text{pk-pk}}$│ Full load $20\,\text{V} / 3\,\text{A}$, 20MHz BW │ │ **Output Ripple at 5V/3A** │ $< 15\,\text{mV}_{\text{pk-pk}}$│ Pure buck mode operation │ │ **Peak Conversion Eff.** │ $\ge 96.5\%$ @ 15V / 3A │ Synchronous rectification across FETs │ │ **Average Conversion Eff.**│ $> 94.5\%$ Across Range │ Convection cooled, fanless enclosure │ │ **Load Transient Recovery**│ $< 120\,\mu\text{s}$ to 1% │ $50\% \rightarrow 100\%$ load step ($1.5A \rightarrow 3.0A$)│ │ **Load Transient Deviation**│ $< 250\,\text{mV}$ Peak │ Under $1.5\,\text{A}/\mu\text{s}$ load slew rate │ │ **Ambient Temperature** │ $-20^\circ\text{C} \dots +70^\circ\text{C}$│ Industrial bench environment │ │ **Target Junction Temp** │ $T_j \le 100^\circ\text{C}$ │ Safe derating below $150^\circ\text{C}$ max limit│ └────────────────────────────┴─────────────────────────────┴───────────────────────────────────────┘

Step 2: Topology Selection & Operating Principles

2.1 Why the 4-Switch Non-Inverting Synchronous Buck-Boost (H-Bridge)?

When an application requires an output voltage (5.0\,V \dots 20.0\,V) that can be both lower than and near the nominal input rail (24.0\,V), three classical topologies exist:

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ TOPOLOGY TRADE-OFF COMPARISON │ ├───────────────────────┬───────────────────────────┬──────────────────────────────────────────────┤ │ Topology Candidate │ Key Advantages │ Fatal Disadvantages / Elimination Rationale │ ├───────────────────────┼───────────────────────────┼──────────────────────────────────────────────┤ │ **Traditional Single- │ Simple, only 1 switch and │ **Negative Output Voltage**: Inverts polarity│ │ Switch Inverting BB** │ 1 diode. │ ($-5\text{V} \dots -20\text{V}$). Incompatible│ │ │ │ with common-ground bench test equipment! │ ├───────────────────────┼───────────────────────────┼──────────────────────────────────────────────┤ │ **Single-Ended Primary│ Non-inverting, 1 switch, │ Series AC capacitor ($C_{sep}$) carries full │ │ Inductor (SEPIC)** │ ground-referenced drive. │ RMS load current ($2.74\,\text{A}_{\text{RMS}}$),│ │ │ │ causing high $I^2R$ ESR heating; eff $< 94\%$.│ ├───────────────────────┼───────────────────────────┼──────────────────────────────────────────────┤ │ **4-Switch Synchronous│ **Positive non-inverting**│ Requires two high-side floating gate drivers.│ │ H-Bridge (CHOSEN)** │ **output, peak eff >96.5%,│ *(Easily resolved using modern integrated │ │ │ pure buck mode efficiency,│ controllers such as TI LM5176).* │ │ │ sub-25mV ripple.** │ │ └───────────────────────┴───────────────────────────┴──────────────────────────────────────────────┘

2.2 4-Switch H-Bridge Power Stage Circuit:

The 4-switch topology consists of two half-bridges connected across a single power inductor (L):

Buck Half-Bridge: Switch Q_A (High-Side) and Synchronous Switch Q_B (Low-Side) at node SW1.

Boost Half-Bridge: Switch Q_C (Low-Side) and Synchronous Switch Q_D (High-Side) at node SW2.

4-Switch H-Bridge Topology +24V DC Bus In │ Drain (D) ┌───┴───┐ │ Q_A │ Buck High-Side Switch PWM_A ───┤ │ (BSC034N04LS: 40V, 3.4mΩ) └───┬───┘ │ Source (S) ├────── SW1 Node ──────[ Inductor L: 10µH ]────── SW2 Node ──────┬───> +V_OUT (5V-20V @ 3A) │ │ Drain (D) Source (S) ┌───┴───┐ ┌───┴───┐ PWM_B ───┤ Q_B │ Buck Low-Side Sync PWM_D ───┤ Q_D │ Boost High-Side Sync └───┬───┘ (BSC034N04LS) └───┬───┘ (BSC034N04LS) │ Source (S) │ Drain (D) PGND_SEC Drain (D) ┌───┴───┐ PWM_C ───┤ Q_C │ Boost Low-Side Switch └───┬───┘ (BSC034N04LS) │ Source (S) PGND_SEC

2.3 Multi-Mode Operational State Machine

To maximize efficiency and eliminate unnecessary switching losses, the controller (TI LM5176) operates in three distinct operational modes based on the relationship between V_{IN and V_{OUT:

MULTI-MODE CONTROL REGIME MAP Output Voltage (V_OUT) ^ 20V ┼────────────────────────────────────────────────────────── [ BUCK-BOOST TRANSITION MODE ] │ - Alternating Buck & Boost cycles 18V ┼────────────────────────────────────────────────────────── - All 4 switches active (smooth transition) │ │ │ [ PURE BUCK MODE ] │ - Q_C is OFF (0% duty cycle) │ - Q_D is ON (100% continuous conduction) │ - Only Q_A and Q_B switch at 250 kHz! │ - D_buck = V_OUT / V_IN (20.8% to 75.0%) │ - Slashing switching loss by 50%! │ 5V ┼────────────────────────────────────────────────────────── └─────────────────────────────────────────────────────────> Time

1. Pure Buck Mode (V_{OUT < 0.85 × V_{IN, i.e., V_{OUT = 5.0\,V \dots 18.0\,V):

Switch States:

Q_C is held permanently OFF (0\% duty cycle).

Q_D is held permanently ON (100\% duty cycle, acting as a low-resistance 3.4\,m\Omega pass transistor).

Q_A and Q_B switch synchronously at 250\,kHz:

D_{buck} = \frac{V_{OUT}}{V_{IN}} = \frac{5.0\,\text{V}}{24.0\,\text{V}} = 20.8\% \dots \frac{18.0\,\text{V}}{24.0\,\text{V}} = 75.0\%

Efficiency Impact: Only two switches toggle! Boost switching losses and inductor reverse ringing are zero, enabling peak efficiencies of $96.8\%$.

2. Buck-Boost Transition Mode (0.85 × V_{IN ≤ V_{OUT ≤ 1.15 × V_{IN, i.e., V_{OUT = 18.0\,V \dots 20.0\,V):

When V_{OUT approaches V_{IN (20.0\,V \approx 24.0\,V), pure buck mode requires D \approx 83\%, where narrow off-times cause pulse-skipping or erratic jitter.

The controller engages an interleaved buck-boost switching cycle:

Cycle 1: Buck pulse (induces energy into L).

Cycle 2: Boost pulse (steps up energy from L into C_{OUT).

This ensures seamless, monotonic voltage control with zero subharmonic oscillation or output ripple bursts.

3. Pure Boost Mode (when V_{IN < 0.85 × V_{OUT, e.g., input sag to 18\,V):

Q_A is held permanently ON (100\% duty cycle).

Q_B is held permanently OFF.

Q_C and Q_D switch synchronously at 250\,kHz:

D_{boost} = 1 - \frac{V_{IN}}{V_{OUT}} = 1 - \frac{18.0\,\text{V}}{20.0\,\text{V}} = 10.0\%

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ SWITCH OPERATING STATES ACROSS REGIMES │ ├───────────────────────┬──────────────┬──────────────┬──────────────┬─────────────────────────────┤ │ Switch Identifier │ Pure Buck │ Buck-Boost │ Pure Boost │ Function / Implementation │ ├───────────────────────┼──────────────┼──────────────┼──────────────┼─────────────────────────────┤ │ **Q_A (Buck High)** │ Switching │ Switching │ ON (100%) │ High-side floating drive │ │ **Q_B (Buck Low)** │ Switching │ Switching │ OFF (0%) │ Ground-referenced sync rect │ │ **Q_C (Boost Low)** │ OFF (0%) │ Switching │ Switching │ Ground-referenced switch │ │ **Q_D (Boost High)** │ ON (100%) │ Switching │ Switching │ High-side floating sync rect│ └───────────────────────┴──────────────┴──────────────┴──────────────┴─────────────────────────────┘