Dossier 2: Power Stage Magnetics & Filter Sizing

Welcome to Dossier 2 of the Step-by-Step 4-Switch Synchronous Buck-Boost Post-Regulator Design Guide. This document details the mathematical sizing, saturation margins, component selection, and thermal loss calculations for the power inductor (L), input capacitor bank (C_{IN), and output capacitor bank (C_{OUT).

Step 3: Power Inductor Sizing (L)

The inductor must maintain Continuous Conduction Mode (CCM) across the operational envelope while balancing peak saturation current, physical dimensions, core loss, and transient dynamic slew rate.

INDUCTOR CURRENT RIPPLE WAVEFORM (CCM) Inductor Current (I_L) ^ │ /¦ /¦ │ / ¦ / ¦ I_pk┼─────────────────────────────/──¦─────────────────────────/──¦──── Peak Current = 5.95 A │ / ¦ / ¦ │ / ¦ / ¦ I_avg ─────────────────────────/─────¦──────────────────────/─────¦──── Average Current = 3.0 A │ / ¦ / ¦ │ / ¦ / ¦ I_val ──────────────────────/ ¦ / ¦──── Valley Current = 2.1 A │ / ¦ / ¦ │ / ¦ / ¦ 0A ┴────────────────────/ ¦────────────────/ ¦────> Time <── D*Ts ───><── (1-D)*Ts ──>

3.1 Inductor Sizing Derivations:

Target Ripple Current Ratio ($r$):

A ripple ratio of r = \frac{\Delta I_L{I_{OUT = 30\% at rated current (I_{OUT = 3.0\,A) provides the optimum balance between AC core losses and physical inductor size:

\Delta I_L = 0.30 \times 3.0\,\text{A} = 0.90\,\text{A}

Buck Mode Inductance Formula:

In buck mode, the inductor current ramps up during t_{on = D \cdot T_s:

L = \frac{(V_{IN} - V_{OUT}) \cdot D}{\Delta I_L \cdot f_{sw}} = \frac{V_{OUT} \cdot (V_{IN} - V_{OUT})}{\Delta I_L \cdot f_{sw} \cdot V_{IN}}

The maximum ripple condition occurs at V_{OUT = \frac{V_{IN{2 = 12.0\,V (D = 50\%):

L = \frac{12.0 \cdot (24.0 - 12.0)}{0.90 \cdot 250000 \cdot 24.0} = \frac{144}{5.4 \times 10^6} = 2.67 \times 10^{-5}\,\text{H} \approx 26.7\,\mu\text{H}

Optimized Component Selection: $L = 10.0\,\mu\text{H}$:

While an ideal 30\% ripple at 12\,V dictates 26.7\,µH, practical compact designs select $10.0\,\mu\text{H}$ for the following engineering reasons:

Transient Response: Slew rate is inversely proportional to inductance:

\frac{di}{dt} = \frac{V_L}{L} = \frac{24.0\,\text{V} - 12.0\,\text{V}}{10\,\mu\text{H}} = 1.20\,\text{A}/\mu\text{s} \quad (\text{vs. only } 0.45\,\text{A}/\mu\text{s} \text{ for } 26.7\,\mu\text{H})

A 10\,µH inductor recovers from a 1.5\,A \rightarrow 3.0\,A load step 2.6x faster!

DCR & Footprint: A 10\,µH flat-wire inductor has half the DC resistance (11.8\,m\Omega) of an equivalent 27\,µH inductor (28\,m\Omega), saving 0.15\,W in thermal losses.

Ripple Current with $10\,\mu\text{H}$ at $20\,\text{V} / 3\,\text{A}$ (Buck-Boost regime):

\Delta I_L = \frac{V_{IN} \cdot D}{\Delta L \cdot f_{sw}} \approx 1.20\,\text{A} \quad (\text{Well within continuous conduction limits}).

3.2 Peak & RMS Inductor Current:

Under full-load 20.0\,V / 3.0\,A operation (P_{OUT = 60.0\,W), input current is:

I_{IN} = \frac{P_{OUT}}{\eta \cdot V_{IN}} = \frac{60.0\,\text{W}}{0.96 \times 24.0\,\text{V}} = 2.60\,\text{A}

Peak Inductor Current ($I_{L(pk)}$):

I_{L(pk)} = I_{IN} + I_{OUT} + \frac{\Delta I_L}{2} = 2.60\,\text{A} + 3.00\,\text{A} + 0.60\,\text{A} = \mathbf{5.95\,\text{A}}

RMS Inductor Current ($I_{L(rms)}$):

I_{L(rms)} = \sqrt{I_{L(avg)}^2 + \frac{\Delta I_L^2}{12}} = \sqrt{(3.00)^2 + \frac{(1.20)^2}{12}} = \sqrt{9.0 + 0.12} = \mathbf{3.02\,\text{A}_{\text{RMS}}}

3.3 Inductor Component Selection:

Selected Part: Wurth Elektronik 7443321000 (WE-HCC flat-wire shielded power inductor):

Nominal Inductance: 10.0\,µH ± 20\%

Rated Continuous Current (I_{rms): 8.5\,A (\Delta T = 40^\circC rise)

Saturation Current (I_{sat at 30\% drop): $11.5\,\text{A}$

Saturation Margin:

\text{Margin} = \frac{I_{sat} - I_{L(pk)}}{I_{sat}} = \frac{11.5\,\text{A} - 5.95\,\text{A}}{11.5\,\text{A}} = \mathbf{48.3\% \text{ safety margin over peak!}}

DC Resistance (R_{DC): 11.8\,m\Omega ± 10\%

Copper Conduction Loss:

P_{cu} = I_{L(rms)}^2 \times R_{DC} = (3.02\,\text{A})^2 \times 0.0118\,\Omega = \mathbf{0.108\,\text{W}}

Core Loss (P_{core via Steinmetz model): \approx 0.14\,W at 250\,kHz.

Total Inductor Thermal Dissipation: P_{ind = 0.108 + 0.14 = \mathbf{0.248\,W (runs cool at \Delta T \approx 12^\circC).

Step 4: Input Capacitor Bank Sizing (C_{IN)

In buck and buck-boost operation, the high-side switch Q_A draws chopped, high di/dt rectangular current pulses from the input bus.

INPUT CAPACITOR CURRENT WAVEFORM Input Current (I_in) ^ I_L ┼──────────────┌────────────────┐ ┌────────────────┐ │ │ │ │ │ │ │ │ │ │ 0A ┴──────────────┴────────────────┴─────────────────────────────┴────────────────┴────> Time <── D*Ts ──────><────── (1-D)*Ts ────────────────────────────>

4.1 RMS Ripple Current Stress:

Worst-case input RMS ripple current occurs in buck mode at D = 50\%:

I_{Cin(rms)} = I_{OUT} \cdot \sqrt{D \cdot (1 - D)} = 3.0\,\text{A} \times \sqrt{0.5 \times 0.5} = \mathbf{1.50\,\text{A}_{\text{RMS}}}

4.2 Capacitance Sizing:

Allowing a maximum input ripple of \Delta V_{IN ≤ 40\,mV_{pk-pk:

C_{IN(min)} = \frac{I_{OUT} \cdot D \cdot (1 - D)}{f_{sw} \cdot \Delta V_{IN}} = \frac{3.0 \times 0.25}{250000 \times 0.040} = 7.5 \times 10^{-5}\,\text{F} = \mathbf{75\,\mu\text{F}}

4.3 Capacitor Bank Configuration:

High-Frequency MLCCs: 2× 22\,µF / 35\,V 1210 X7R Ceramic capacitors (TDK C3225X7R1V226M) placed directly across the drain of Q_A and source of Q_B. Absorbs high di/dt edges (< 5\,ns).

Bulk Electrolytic / Polymer: 1× 100\,µF / 35\,V Panasonic OS-CON Aluminum Polymer (25SVPF100M, ESR = 16\,m\Omega, rated ripple current 2.8\,A_{RMS).

Step 5: Output Capacitor Bank Sizing (C_{OUT)

The output capacitor bank filters the inductor ripple current and supplies energy during dynamic load steps (1.5\,A \rightarrow 3.0\,A).

5.1 Ripple Voltage Breakdown:

Total peak-to-peak output ripple is the sum of capacitive charge storage, ESR, and ESL:

\Delta V_{OUT} = \Delta V_C + \Delta V_{ESR} + \Delta V_{ESL}

Capacitive Ripple Component:

\Delta V_C = \frac{\Delta I_L}{8 \cdot f_{sw} \cdot C_{OUT}} = \frac{1.20\,\text{A}}{8 \times 250000 \times 88 \times 10^{-6}} = \mathbf{6.8\,\text{mV}_{\text{pk-pk}}}

ESR Ripple Component:

\Delta V_{ESR} = \Delta I_L \cdot \text{ESR}_{total} = 1.20\,\text{A} \times 2.5\,\text{m}\Omega = \mathbf{3.0\,\text{mV}_{\text{pk-pk}}}

Total Combined Ripple:

\Delta V_{OUT} \approx \sqrt{(\Delta V_C)^2 + (\Delta V_{ESR})^2} = \sqrt{(6.8)^2 + (3.0)^2} = \mathbf{7.4\,\text{mV}_{\text{pk-pk}}}

This easily beats the stringent target limit of < 25\,mV_{pk-pk!

5.2 DC Voltage Bias Derating in Ceramic MLCCs:

[WARNING] Class-II ferroelectric dielectric materials (X5R, X7R) experience drastic capacitance loss when subjected to DC bias voltage. A 22\,µF / 25\,V 1210 ceramic capacitor loses $\approx 55\%$ of its nominal capacitance at 20.0\,V DC bias!

Effective Ceramic Bank Capacitance:

C_{cer(eff)} = 4 \times (22\,\mu\text{F} \times 0.45) = 4 \times 9.9\,\mu\text{F} = \mathbf{39.6\,\mu\text{F}}

Polymer Bulk Capacitor Addition:

To guarantee stability during 1.5\,A \rightarrow 3.0\,A load steps, 1× 100\,µF / 25\,V Panasonic OS-CON Polymer capacitor (ESR = 12\,m\Omega) is placed in parallel.

Combined Output Bank:

Nominal Capacitance: 4× 22\,µF + 100\,µF = \mathbf{188\,µF

Effective Capacitance at 20V DC: \approx \mathbf{140\,µF

Combined Parallel ESR: \mathbf{< 2.5\,m\Omega

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ FILTER COMPONENT SPECIFICATION SUMMARY │ ├─────────────────────┬──────────────┬──────────────┬──────────────┬───────────────────────────────┤ │ Component │ Value / Unit │ Voltage / Pkg│ Manufacturer │ Part Number │ ├─────────────────────┼──────────────┼──────────────┼──────────────┼───────────────────────────────┤ │ **Power Inductor L**│ 10.0 µH / 8.5A│ 10x10mm SMD │ Wurth Elek. │ 7443321000 (DCR=11.8mΩ) │ │ **C_IN Ceramic** │ 2x 22 µF │ 35V / 1210 │ TDK │ C3225X7R1V226M │ │ **C_IN Polymer** │ 1x 100 µF │ 35V / Radial │ Panasonic │ 35SVPF100M (ESR=16mΩ) │ │ **C_OUT Ceramic** │ 4x 22 µF │ 25V / 1210 │ Murata │ GRM32ER71E226KE15L │ │ **C_OUT Polymer** │ 1x 100 µF │ 25V / SMD │ Panasonic │ 25SVPF100M (ESR=12mΩ) │ └─────────────────────┴──────────────┴──────────────┴──────────────┴───────────────────────────────┘