Dossier 3: MOSFET Selection, Gate Drivers & Thermal Analysis

Welcome to Dossier 3 of the Step-by-Step 4-Switch Synchronous Buck-Boost Post-Regulator Design Guide. This document provides mathematical loss modeling, junction temperature derivations, gate driver requirements, and bootstrap network sizing for the four power MOSFETs (Q_A, Q_B, Q_C, Q_D).

Step 6: Power MOSFET Selection & Losses

The 4-switch Buck-Boost converter uses four N-channel power MOSFETs. Selection requires optimizing the trade-off between on-state resistance (R_{DS(on)) to minimize conduction losses, and total gate charge (Q_g) / gate-to-drain charge (Q_{gd) to minimize high-frequency switching losses at 250\,kHz.

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ POWER MOSFET SELECTION BENCHMARK │ ├────────────────────┬────────────────────┬───────────┬─────────────┬─────────────┬────────────────┤ │ Switch Identifier │ Selected Part │ $V_{DS}$ │ $R_{DS(on)}$│ $Q_g$ (Typ) │ Package │ ├────────────────────┼────────────────────┼───────────┼─────────────┼─────────────┼────────────────┤ │ **Q_A (Buck HS)** │ Infineon BSC034N04 │ 40 V │ 3.4 mΩ │ 16 nC │ SuperSO8 / TDSON│ │ **Q_B (Buck LS)** │ Infineon BSC034N04 │ 40 V │ 3.4 mΩ │ 16 nC │ SuperSO8 / TDSON│ │ **Q_C (Boost LS)** │ Infineon BSC034N04 │ 40 V │ 3.4 mΩ │ 16 nC │ SuperSO8 / TDSON│ │ **Q_D (Boost HS)** │ Infineon BSC034N04 │ 40 V │ 3.4 mΩ │ 16 nC │ SuperSO8 / TDSON│ └────────────────────┴────────────────────┴───────────┴─────────────┴─────────────┴────────────────┘

6.1 Breakdown of MOSFET Loss Mechanisms (P_{loss):

Total dissipation per switch is the sum of five distinct loss components:

P_{total} = P_{cond} + P_{sw} + P_{coss} + P_{body} + P_{gate}

1. Conduction Loss (P_{cond):

Accounting for on-resistance thermal degradation at elevated junction temperatures (T_j \approx 100^\circC, temperature coefficient factor \approx 1.4):

R_{DS(on),100^\circ C} = 3.4\,\text{m}\Omega \times 1.4 = \mathbf{4.76\,\text{m}\Omega}

In Buck mode (D \approx 50\%, I_{OUT = 3.0\,A):

I_{Q_A(rms)} = I_{OUT} \sqrt{D} = 3.0 \times \sqrt{0.50} = 2.12\,\text{A}_{\text{RMS}}

P_{cond(QA)} = (2.12\,\text{A})^2 \times 0.00476\,\Omega = \mathbf{0.021\,\text{W}}

In Boost High-Side Sync (Q_D, permanently ON in Buck mode):

P_{cond(QD)} = (3.0\,\text{A})^2 \times 0.00476\,\Omega = \mathbf{0.043\,\text{W}}

2. Switching Loss (P_{sw):

Switching losses occur during the overlapping voltage and current transition intervals:

t_{sw} = t_r + t_f \approx 8.0\,\text{ns} + 6.0\,\text{ns} = 14.0\,\text{ns}

P_{sw(QA)} = \frac{1}{2} \cdot V_{DS} \cdot I_D \cdot t_{sw} \cdot f_{sw} = \frac{1}{2} \times 24.0\,\text{V} \times 3.0\,\text{A} \times (14 \times 10^{-9}\,\text{s}) \times 250000\,\text{Hz} = \mathbf{0.126\,\text{W}}

3. Output Capacitance Dump Loss (P_{coss):

Energy stored in the MOSFET output capacitance (C_{oss = 650\,pF) dissipated on each turn-on transition:

P_{coss} = \frac{1}{2} C_{oss} \cdot V_{DS}^2 \cdot f_{sw} = \frac{1}{2} \times (650 \times 10^{-12}) \times (24)^2 \times 250000 = \mathbf{0.047\,\text{W}}

4. Body Diode Conduction Loss during Dead-Time (P_{body):

During the adaptive dead-time (t_{dead = 25\,ns) between high-side and low-side switching, the low-side body diode conducts load current with forward drop V_F \approx 0.75\,V:

P_{body} = 2 \cdot V_F \cdot I_{OUT} \cdot t_{dead} \cdot f_{sw} = 2 \times 0.75\,\text{V} \times 3.0\,\text{A} \times (25 \times 10^{-9}) \times 250000 = \mathbf{0.028\,\text{W}}

5. Gate Drive Dissipation (P_{gate):

P_{gate} = Q_g \cdot V_{drv} \cdot f_{sw} = (16 \times 10^{-9}\,\text{C}) \times 5.0\,\text{V} \times 250000\,\text{Hz} = \mathbf{0.020\,\text{W}}

Total Dissipation per Active Switch:

P_{total(QA)} = 0.021 + 0.126 + 0.047 + 0.028 + 0.020 = \mathbf{0.242\,\text{W}}

Across all four switches under full load (20\,V / 3\,A), total MOSFET thermal loss is $< 1.10\,\text{W}$!

6.2 Thermal Modeling & Junction Temperature Rise:

The SuperSO8 (PG-TDSON-8) package features an exposed thermal pad soldered directly to the PCB top copper polygon.

Thermal Resistance ($\theta_{JA}$): With a 200\,mm^2 area of 2\,oz copper on Layer 1 and 6 thermal vias to Layer 2 ground plane, effective thermal resistance is:

\theta_{JA} = 35.0^\circ\text{C/W}

Junction Temperature Rise ($\Delta T$):

\Delta T = P_{total} \times \theta_{JA} = 0.242\,\text{W} \times 35^\circ\text{C/W} = \mathbf{8.47^\circ\text{C}}

Worst-Case Junction Temperature ($T_j$):

At maximum specified ambient temperature T_{amb = 60^\circC:

T_j = 60^\circ\text{C} + 8.47^\circ\text{C} = \mathbf{68.5^\circ\text{C}}

This operates with a massive safety margin below the silicon maximum rated limit of 150^\circC!

Step 7: Gate Driver & Bootstrap Circuit Design

Both Q_A (Buck High-Side) and Q_D (Boost High-Side) require floating N-channel gate drives referenced to switching nodes SW1 and SW2 respectively:

BOOTSTRAP GATE DRIVE NETWORK (BUCK HIGH-SIDE Q_A) +5V VCC Bias Rail │ ┌─┴─┐ D_boot1 │ │ Schottky Diode (PMEG6010: 60V, 1A) └─┬─┘ │ ├─── To LM5176 BOOT1 Pin │ ┌─┴─┐ C_boot1 │ │ 0.22µF / 50V X7R Ceramic └─┬─┘ │ ├─── To LM5176 SW1 Pin ────────────────┐ │ │ │ Drain (D) │ │ ┌─────┴─────┐ │ │ GATE_HO1 ────┤ Q_A │ │ │ (via 2.2Ω) │ BSC034N04 │ │ │ └─────┬─────┘ │ │ │ Source (S) │ └─────────────────────┴────────────────┴─── SW1 Node (To Inductor L)

7.1 Bootstrap Capacitor Sizing (C_{boot):

The bootstrap capacitor must supply gate charge Q_g without suffering a voltage sag greater than \Delta V_{boot ≤ 100\,mV:

C_{boot} \ge \frac{Q_g + (I_{leak} \cdot t_{on,max})}{\Delta V_{boot}}

C_{boot} \ge \frac{16\,\text{nC} + (10\,\mu\text{A} \times 3.0\,\mu\text{s})}{0.10\,\text{V}} = \frac{16\,\text{nC} + 0.03\,\text{nC}}{0.10\,\text{V}} = 0.16\,\mu\text{F}

Selected Component: $0.22\,\mu\text{F} / 50\,\text{V}$ X7R $0805$ Ceramic Capacitor (Murata GRM21BR71H224KA01L). Provides > 35\% margin against voltage droop.

7.2 Bootstrap Diode Selection (D_{boot1, D_{boot2):

Component: Nexperia PMEG6010CEH (60\,V, 1.0\,A planar Schottky).

Fast recovery (t_{rr \approx 0) prevents reverse charge loss back into the +5\,V internal bias rail when SW1 swings up to +24\,V.

7.3 Gate Resistor Dampening (R_g):

A small 2.2\,\Omega 0603 resistor in series with each gate lead damps high-frequency LC oscillations formed by the gate trace parasitic inductance (L_{trace \approx 5\,nH) and MOSFET input capacitance (C_{iss = 2200\,pF), preventing gate oxide puncture and false turn-on without sluggish switching delays.