Dossier 5: Schematics, PCB Layout & Bill of Materials

Welcome to Dossier 5 of the Step-by-Step 4-Switch Synchronous Buck-Boost Post-Regulator Design Guide. This document provides the pin-level ASCII schematic of the complete power stage, critical high-frequency PCB layout rules, and the complete engineering Bill of Materials (BOM).

Step 11: Complete Component-Level Hardware Schematic

Below is the complete component-level hardware schematic of the 4-switch synchronous Buck-Boost post-regulator based on the Texas Instruments LM5176 multi-mode controller:

==================================================================================================================================================== COMPLETE 4-SWITCH SYNCHRONOUS BUCK-BOOST POST-REGULATOR (24V IN -> 5-20V / 3A PROGRAMMABLE OUT) ==================================================================================================================================================== +24V Intermediate DC Bus ───┬─────────────────────────────────────────────────────────────────────────┐ (From Isolated Flyback) │ │ ┌─┴─┐ C_IN_CER ┌─┴─┐ C_IN_POLY │ │ 2x 22µF / 35V MLCC │ │ 100µF / 35V Polymer └─┬─┘ (TDK C3225X7R1V226M) └─┬─┘ (Panasonic 35SVPF100M) │ │ PGND_SEC PGND_SEC │ Drain (D) ┌─────┴─────┐ │ Q_A │ BSC034N04LS (40V, 3.4mΩ) BUCK_HO1 ───────┤ │ Buck High-Side Switch └─────┬─────┘ │ Source (S) ├────────────── SW1 Switching Node ─────────────┐ │ │ Drain (D) ┌─┴─┐ ┌─────┴─────┐ │ │ Power Inductor L │ Q_B │ BSC034N04LS │ │ 10µH / 8.5A Flat-Wire BUCK_LO1 ───────┤ │ Buck Low-Side Sync Switch └─┬─┘ (Wurth 7443321000) └─────┬─────┘ │ │ Source (S) │ PGND_SEC ├────────────── SW2 Switching Node ─────────────┐ │ │ Source (S) Drain (D) ┌─────┴─────┐ ┌─────┴─────┐ │ Q_D │ BSC034N04LS │ Q_C │ BSC034N04LS BOOST_HO2 ───────────┤ │ Boost High-Side Sync │ │ Boost Low-Side Sw └─────┬─────┘ BOOST_LO2 ───┤ │ │ Drain (D) └─────┬─────┘ │ │ Source (S) ├──────────────────────────┐ PGND_SEC │ │ ┌─┴─┐ ┌─┴─┐ Kelvin Force Lead 1 │ │ R_shunt │ │ C_OUT_POLY └─┬─┘ 10mΩ / 0.1% / 3W │ │ 100µF / 25V Polymer Kelvin Force Lead 2 ├───┐ └─┬─┘ (ESR = 12mΩ) │ │ │ │ ┌─┴─┐ C_OUT_CER PGND_SEC │ │ │ 4x 22µF / 25V MLCC │ └─┬─┘ (Murata 1210) │ │ │ PGND_SEC │ +V_OUT Programmable Rail ───────────────────────────────────────────────────┴───────────────────────────────────────────────────────> +V_OUT (5V-20V/3A) (To Load & Dual Metering) │ ├──────────────────────────┐ │ │ ┌─┴─┐ R_top ┌─┴─┐ C_FF │ │ 49.9kΩ (0.1%) │ │ 47pF └─┬─┘ └─┬─┘ │ │ ├─── FB Summing Node ──────┘ │ (Held at V_REF = 1.20V) ┌───────────────┼───────────────────────────────┐ │ │ │ ┌─┴─┐ ┌─┴─┐ ┌─┴─┐ R_DAC │ │ 11.0kΩ │ │ R_bot │ │ (0.1%)└─┬─┘ └─┬─┘ 3.65kΩ (0.1%) │ │ LM5176PWPR │ │ │ F │ Error Amp Pin MCU_DAC1 (0.0V - 3.3V) ─────────────────────────────────────┘ AGND_SEC │ B │ (Voltage Setpoint: 20V down to 5V) └───┘ │ ┌─┴─┐ │ C │ From CC Analog Clamp Diode (BAT54) ───────────────────────────────────────────────────────────────────────┤ O │ COMP Pin │ M │ │ P │ └─┬─┘ │ ├───[ R_comp: 10.0kΩ ]───┬───[ C_comp1: 2.2nF ]──┐ │ │ │ ├───[ C_comp2: 47pF ]────┴───────────────────────┤ │ │ └───────────────────────────────────────────── AGND_SEC ====================================================================================================================================================

Step 12: High-Frequency PCB Layout Guidelines

The PCB is architected as a high-density 4-layer board ($1.6\,\text{mm}$ FR-4, $2\,\text{oz}$ copper outer layers, $1\,\text{oz}$ inner layers):

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ 4-LAYER PCB STACKUP ALLOCATION │ ├───────┬──────────────────────────┬────────┬──────────────────────────────────────────────────────┤ │ Layer │ Layer Name │ Copper │ Signal & Power Plane Allocation │ ├───────┼──────────────────────────┼────────┼──────────────────────────────────────────────────────┤ │ **L1**│ Top Layer (High Power) │ 2.0 oz │ Power MOSFETs, Inductor, C_IN, C_OUT, high-current │ │ **L2**│ Inner Layer 1 (Ground) │ 1.0 oz │ Continuous, solid ground return (PGND plane) │ │ **L3**│ Inner Layer 2 (Power) │ 1.0 oz │ +5V controller VCC, +3.3V MCU, gate drive power │ │ **L4**│ Bottom Layer (Signals) │ 2.0 oz │ Sensitive FB traces, Kelvin shunt lines, I2C buses │ └───────┴──────────────────────────┴────────┴──────────────────────────────────────────────────────┘

MINIMIZING HIGH di/dt POWER LOOPS ON LAYER 1 BUCK INPUT POWER LOOP BOOST OUTPUT POWER LOOP (Area < 0.8 cm²!) (Area < 0.8 cm²!) +VIN ──────────────┐ Inductor L ─────────┐ │ │ │ │ ┌─┴─┐ C_IN ┌─┴─┐ Q_A ┌─┴─┐ Q_D ┌─┴─┐ C_OUT │ │ MLCC │ │ Buck HS │ │ Boost HS │ │ MLCC └─┬─┘ └─┬─┘ └─┬─┘ └─┬─┘ │ │ │ │ │ ┌─┴─┐ Q_B │ ┌─┴─┐ Q_C │ │ │ Buck LS │ │ │ Boost LS │ └─┬─┘ │ └─┬─┘ │ │ │ │ PGND ──────────────┴─── PGND PGND ─────────────┴─── PGND

Critical Layout Rules:

Loop Area Minimization: The Buck input loop (C_{IN \rightarrow Q_A \rightarrow Q_B \rightarrow PGND) and Boost output loop (Q_D \rightarrow C_{OUT \rightarrow Q_C \rightarrow PGND) carry pulsed currents with di/dt > 1.5\,A/ns. These loops must be placed directly adjacent with total enclosed surface area < 0.8\,cm^2 to minimize magnetic radiated EMI.

Switch Nodes (SW1, SW2): The copper polygons connecting Q_A/Q_B to the inductor (SW1) and the inductor to Q_C/Q_D (SW2) experience rapid dV/dt voltage transitions (> 15\,V/ns). Keep these polygons wide enough to carry 3.5\,A without overheating, but compact in surface area to avoid forming RF radiating patch antennas.

4-Wire Kelvin Shunt Routing: The differential sense traces from R_{shunt to the INA240A2 current sense amplifier must exit directly from the inner Kelvin pads of the 2512 package, running parallel on Layer 4 shielded by the Layer 2 ground plane.

Single-Point Ground Star: Analog Ground (AGND) for sensitive error amplifier resistors and controller references must be kept completely separated from Power Ground (PGND), joining together at exactly one star connection directly beneath the output capacitor bank.

Step 13: Complete Bill of Materials (BOM)

┌──────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ BILL OF MATERIALS (BOM): 4-SWITCH BUCK-BOOST STAGE │ ├──────┬─────┬───────────────────────────┬─────────────────────────┬──────────────┬────────────────────────────────────────┤ │ Item │ Qty │ Reference Designator │ Description │ Package │ Manufacturer & Part Number │ ├──────┼─────┼───────────────────────────┼─────────────────────────┼──────────────┼────────────────────────────────────────┤ │ 1 │ 1 │ U1 (Controller) │ 4-Switch Sync BB IC │ HTSSOP-28 │ Texas Instruments LM5176PWPR │ │ 2 │ 4 │ Q_A, Q_B, Q_C, Q_D │ Power MOSFET 40V 3.4mΩ │ SuperSO8 │ Infineon BSC034N04LS │ │ 3 │ 1 │ L1 │ Flat-Wire Inductor 10uH │ 10x10mm SMD │ Wurth Elektronik 7443321000 │ │ 4 │ 1 │ R_shunt │ Kelvin Shunt 10mΩ 0.1% │ 2512 SMD │ Bourns CSS2H-2512R-L010F │ │ 5 │ 1 │ U2 (Current Sense Amp) │ High-Side Current Sense │ SOIC-8 │ Texas Instruments INA240A2D │ │ 6 │ 1 │ U3 (Fast Comparator) │ 4.5ns High-Speed Comp │ SOT-23-6 │ Texas Instruments TLV3501AIDBVR │ │ 7 │ 1 │ D_clamp │ Schottky Diode 30V 200mA│ SOT-23 │ Diodes Inc. BAT54 │ │ 8 │ 2 │ D_boot1, D_boot2 │ Schottky Diode 60V 1.0A │ SOD-123F │ Nexperia PMEG6010CEH │ │ 9 │ 2 │ C_boot1, C_boot2 │ Ceramic Cap 0.22µF 50V │ 0805 X7R │ Murata GRM21BR71H224KA01L │ │ 10 │ 2 │ C_IN_CER │ Ceramic Cap 22µF 35V │ 1210 X7R │ TDK C3225X7R1V226M │ │ 11 │ 1 │ C_IN_POLY │ Polymer Cap 100µF 35V │ Radial 8x10 │ Panasonic 35SVPF100M │ │ 12 │ 4 │ C_OUT_CER │ Ceramic Cap 22µF 25V │ 1210 X7R │ Murata GRM32ER71E226KE15L │ │ 13 │ 1 │ C_OUT_POLY │ Polymer Cap 100µF 25V │ SMD 8x10.5 │ Panasonic 25SVPF100M │ │ 14 │ 1 │ R_top │ Resistor 49.9kΩ 0.1% │ 0805 SMD │ Vishay TNPW080549K9BEEA │ │ 15 │ 1 │ R_DAC │ Resistor 11.0kΩ 0.1% │ 0805 SMD │ Vishay TNPW080511K0BEEA │ │ 16 │ 1 │ R_bot │ Resistor 3.65kΩ 0.1% │ 0805 SMD │ Vishay TNPW08053K65BEEA │ │ 17 │ 1 │ C_FF │ Ceramic Cap 47pF 50V │ 0603 C0G/NP0 │ KEMET C0603C470J5GACTU │ │ 18 │ 1 │ R_comp │ Resistor 10.0kΩ 1.0% │ 0603 SMD │ Panasonic ERA-3AEB103V │ │ 19 │ 1 │ C_comp1 │ Ceramic Cap 2.2nF 50V │ 0603 X7R │ Murata GRM188R71H222KA01D │ │ 20 │ 1 │ C_comp2 │ Ceramic Cap 47pF 50V │ 0603 C0G/NP0 │ KEMET C0603C470J5GACTU │ │ 21 │ 4 │ R_gate1..R_gate4 │ Resistor 2.2Ω 5% │ 0603 SMD │ Panasonic ERJ-3GEYJ2R2V │ └──────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┘