Dossier 6: Bring-Up, Calibration & Bench Testing

Welcome to Dossier 6 of the Step-by-Step 4-Switch Synchronous Buck-Boost Post-Regulator Design Guide. This document provides the hardware bring-up checklist, digital calibration algorithms, dynamic load step testing procedures, and the bench troubleshooting matrix.

Step 14: Step-by-Step Bring-Up Procedure

Follow this rigorous multi-stage procedure when powering up and verifying prototype hardware for the first time:

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ HARDWARE BRING-UP CHECKLIST │ ├──────┬──────────────────────┬────────────────────────────────────────────────────────────────────┤ │ Step │ Verification Phase │ Test Description & Expected Pass Criterion │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **1**│ Cold Impedance Check │ Before applying power, verify with a calibrated DMM: │ │ │ │ - Resistance across $+V_{IN}$ to PGND: $> 100\,\text{k}\Omega$. │ │ │ │ - Resistance across $+V_{OUT}$ to PGND: $> 50\,\text{k}\Omega$. │ │ │ │ - Gate-to-Source resistance on all 4 FETs: $> 100\,\text{k}\Omega$.│ │ │ │ - Ensure no solder bridges across SuperSO8 MOSFET pins. │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **2**│ Low-Voltage Auxiliary│ Connect current-limited $+12.0\,\text{V}$ bench supply to $V_{IN}$ │ │ │ Verification │ with current compliance limit set to $150\,\text{mA}$. │ │ │ │ - Verify internal VCC regulator outputs $+5.0\,\text{V} \pm 2\%$. │ │ │ │ - Measure oscillator frequency: $250\,\text{kHz} \pm 5\%$. │ │ │ │ - Check quiescent standby current: $< 25\,\text{mA}$. │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **3**│ Gate Drive & Deadtime│ Probe high-side and low-side gate pins with high-speed scope: │ │ │ Inspection │ - Confirm clean square waves with rise/fall times $< 15\,\text{ns}$.│ │ │ │ - Verify non-overlapping dead-time: $t_{dead} \approx 25\,\text{ns}$│ │ │ │ - Confirm zero shoot-through cross-conduction current spikes. │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **4**│ Nominal $+24.0V$ Ramp│ Increase bench supply to $+24.0\,\text{V}$ nominal. │ │ │ │ - Verify default output voltage stabilizes at $20.0\,\text{V}$. │ │ │ │ - Measure no-load output ripple: $< 10\,\text{mV}_{\text{pk-pk}}$. │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **5**│ DAC Voltage Sweep │ Execute MCU DAC test firmware sweep: │ │ │ │ - Command $V_{DAC} = 0.00\,\text{V} \longrightarrow V_O = 20.00\,\text{V}$ │ │ │ │ - Command $V_{DAC} = 1.65\,\text{V} \longrightarrow V_O = 12.50\,\text{V}$ │ │ │ │ - Command $V_{DAC} = 3.30\,\text{V} \longrightarrow V_O = 5.00\,\text{V}$ │ │ │ │ - Verify monotonic transition across the entire 4:1 voltage span. │ ├──────┼──────────────────────┼────────────────────────────────────────────────────────────────────┤ │ **6**│ Full Load Soak │ Connect programmable electronic load. Gradually increment current │ │ │ │ from $0.5\,\text{A} \rightarrow 1.5\,\text{A} \rightarrow 3.0\,\text{A}$.│ │ │ │ - Verify output ripple remains $< 25\,\text{mV}_{\text{pk-pk}}$. │ │ │ │ - Confirm conversion efficiency $\ge 96.5\%$ at $15\,\text{V}/3A$. │ └──────┴──────────────────────┴────────────────────────────────────────────────────────────────────┘

Step 15: Calibration & Accuracy Verification

Due to component tolerances in R_{top, R_{DAC, R_{bot and the MCU DAC reference, a 2-point linear software calibration guarantees sub-10\,mV absolute accuracy:

2-POINT LINEAR CALIBRATION MATRIX Measured Voltage (V_OUT) ^ 20V ┼───────────────────────────────────────────────────────────── [ Cal Point 2: DAC = 0.0V ] │ - Target: 20.000 V │ - Measured: V_meas_high │ │ Linear Fit: V_meas = m * V_prog + b │ 5V ┼────────────────────────────── [ Cal Point 1: DAC = 3.3V ] │ - Target: 5.000 V │ - Measured: V_meas_low └────────────────────────────────────────────────────────────> Commanded Voltage

Calibration Formulation in Firmware:

Calibration Slope ($m$):

m = \frac{V_{meas,high} - V_{meas,low}}{V_{target,high} - V_{target,low}} = \frac{V_{meas}(0\text{V}) - V_{meas}(3.3\text{V})}{20.000 - 5.000}

Calibration Offset ($b$):

b = V_{meas,low} - (m \times 5.000)

Calibrated DAC Command:

V_{DAC,cal} = \frac{V_{OUT,target} - b}{m}

Stored in the MCU internal Flash EEPROM emulation sector, this eliminates resistor initial tolerance error and guarantees < 0.1\% absolute voltage accuracy!

Step 16: Dynamic Load Transient & Thermal Stress Testing

DYNAMIC LOAD STEP RESPONSE (1.5A -> 3.0A -> 1.5A) Output Current (I_OUT) 3.0A ┼ ┌───────────────────────────────┐ │ │ │ 1.5A ┼───────────────┘ └─────────────────────────────── │ Output Voltage (V_OUT) │ Steady-State 20.0V┼───. .───────────────────────. .─────────────────── │ \ / \ / │ \ / < 120µs Recovery \ / Overshoot < 180mV │ '─────────' '─────────' │ Droop < 220mV └──────────────────────────────────────────────────────────────────────────────> Time

16.1 Oscilloscope Probe Technique for Ripple Measurement:

[IMPORTANT] Never use standard 6-inch alligator ground leads to measure switching ripple! The loop inductance picks up radiated magnetic noise from the inductor, showing artificial 150\,mV spikes. Always use the tip-and-barrel or coaxial pig-tail* probe connection directly across output capacitor C_{OUT. Set oscilloscope channel bandwidth limit to $20\,\text{MHz}$*.

Step 17: Bench Root Cause Analysis (RCA) Troubleshooting Matrix

┌──────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ BENCH ROOT CAUSE ANALYSIS (RCA) MATRIX │ ├────────────────────────────┬───────────────────────────────────────────┬─────────────────────────────────────────────────┤ │ Observed Fault Symptom │ Probable Root Cause │ Diagnostic & Corrective Action │ ├────────────────────────────┼───────────────────────────────────────────┼─────────────────────────────────────────────────┤ │ **Output oscillates or │ 1. Phase margin degraded by insufficient │ 1. Increase feedforward capacitor $C_{FF}$ from │ │ hunts at 5 kHz – 10 kHz** │ lead compensation across $R_{top}$. │ $47\,\text{pF}$ to $100\,\text{pF}$. │ │ │ 2. Excessive ESR on output capacitor bank.│ 2. Verify ceramic MLCCs are soldered cleanly; │ │ │ 3. Digital noise injected via DAC trace. │ add $100\,\text{pF}$ bypass cap at DAC pin. │ ├────────────────────────────┼───────────────────────────────────────────┼─────────────────────────────────────────────────┤ │ **Excessive output ripple │ 1. Ceramic MLCCs suffering extreme DC │ 1. Ensure rated voltage of MLCCs is $\ge 25V$; │ │ voltage (> 50 mV pk-pk)** │ voltage bias derating (> 70% loss). │ verify $100\,\mu\text{F}$ polymer cap is populated│ │ │ 2. Measurement artifact from probe loop. │ 2. Use tip-and-barrel ground spring probe. │ ├────────────────────────────┼───────────────────────────────────────────┼─────────────────────────────────────────────────┤ │ **MOSFET runs excessively │ 1. Insufficient gate drive voltage │ 1. Measure BOOT-to-SW voltage; ensure it stays │ │ hot (> 90°C) at light load│ causing MOSFET to operate in linear │ above $4.5\,\text{V}$. Replace bootstrap cap │ │ │ resistive region. │ with $0.22\,\mu\text{F} / 50\,\text{V}$ X7R. │ │ │ 2. Cross-conduction shoot-through. │ 2. Increase series gate resistor to $4.7\,\Omega$.│ ├────────────────────────────┼───────────────────────────────────────────┼─────────────────────────────────────────────────┤ │ **Constant Current (CC) │ 1. Kelvin sense traces picking up switch │ 1. Reroute Kelvin traces as a tight differential│ │ mode chatters or falsely │ node SW1/SW2 capacitive displacement │ pair on Layer 4 shielded by ground plane. │ │ trips below limit** │ currents ($dV/dt$). │ 2. Add $100\,\Omega + 1\,\text{nF}$ low-pass │ │ │ 2. INA240 input filter missing. │ RC differential filter at INA240 inputs. │ ├────────────────────────────┼───────────────────────────────────────────┼─────────────────────────────────────────────────┤ │ **Output voltage collapses │ 1. Hardware CC clamp comparator threshold │ 1. Verify MCU DAC2 output voltage $V_{ISET}$; │ │ prematurely under load** │ set too low. │ ensure $V_{ISET} \ge 1.50\,\text{V}$ for 3A. │ │ │ 2. Controller inductor peak current limit │ 2. Verify current sense resistor $R_S$ between │ │ │ resistor ($R_{S}$) tripping early. │ `CS` and `CSG` pins matches datasheet spec. │ └────────────────────────────┴───────────────────────────────────────────┴─────────────────────────────────────────────────┘