Complete Step-by-Step Engineering Design Guide: 4-Switch Synchronous Buck-Boost Post-Regulator (5–20 V, 3 A DC Output)

Welcome to the VVDN Engineering Hub Definitive Step-by-Step Design Guide for the 4-Switch Synchronous Buck-Boost Converter. This document delivers an exhaustive, pedagogical, practical engineering reference detailing every calculation, derivation, component selection, closed-loop model, schematic, layout rule, and bench bring-up procedure required to develop the post-regulator stage of the Smart Programmable Power Supply (SPPS).

1. Executive Summary & Design Scope

The post-regulator operates downstream of the isolated primary Quasi-Resonant (QR) Flyback converter (+24.0\,V ± 1.0\% DC bus). It provides a digitally programmable output voltage from $5.0\,\text{V}$ to $20.0\,\text{V}$ DC at continuous currents up to $3.0\,\text{A}$ (60.0\,W maximum continuous output power).

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ 4-SWITCH SYNCHRONOUS BUCK-BOOST ENGINEERING SPECIFICATIONS │ ├────────────────────────────┬─────────────────────────────┬───────────────────────────────────────┤ │ Parameter / Specification │ Target Value │ Engineering Condition / Margin │ ├────────────────────────────┼─────────────────────────────┼───────────────────────────────────────┤ │ **Input Voltage ($V_{IN}$)**│ $+24.0\,\text{V} \pm 1.0\%$ │ From Primary Flyback Stage │ │ **Input Voltage Range** │ $22.0\,\text{V} \dots 26.0\,\text{V}$│ Dynamic tolerance during transients │ │ **Programmable Output ($V_O$)│ $5.0\,\text{V} \dots 20.0\,\text{V}$ DC│ Digitally set via 12-bit MCU DAC │ │ **Voltage Step Resolution**│ $\le 10\,\text{mV}$ Step │ Calculated: $3.65\,\text{mV / LSB}$ │ │ **Continuous Output ($I_O$)│ $0.1\,\text{A} \dots 3.0\,\text{A}$ │ Programmable hardware CC clamp limit │ │ **Current Step Resolution**│ $\le 10\,\text{mA}$ Step │ Secondary DAC current limit setpoint │ │ **Maximum Continuous Power**│ $60.0\,\text{W}$ │ Natural convection, fanless enclosure │ │ **Switching Frequency** │ $250\,\text{kHz}$ Fixed PWM │ Optimized for inductor size vs loss │ │ **Peak Power Efficiency** │ $\ge 96.5\%$ @ 15V / 3A │ All-NMOS synchronous rectification │ │ **Output Voltage Ripple** │ $< 25\,\text{mV}_{\text{pk-pk}}$│ Full load $20\,\text{V} / 3\,\text{A}$, 20MHz BW │ │ **Load Transient Recovery**│ $< 120\,\mu\text{s}$ to 1% │ $50\% \rightarrow 100\%$ load step ($1.5A \rightarrow 3.0A$)│ │ **Operating Regimes** │ Pure Buck, Buck-Boost, Boost│ Automatic seamless 3-mode transitions │ └────────────────────────────┴─────────────────────────────┴───────────────────────────────────────┘

2. Step 1: System Requirements & Parameter Envelopes

The post-regulator operates downstream of the isolated primary Quasi-Resonant (QR) Flyback converter (+24.0\,V ± 1.0\% DC bus). It provides a digitally programmable output voltage from $5.0\,\text{V}$ to $20.0\,\text{V}$ DC at continuous currents up to $3.0\,\text{A}$ (60.0\,W maximum continuous output power).

┌──────────────────────────────────────────────────────────────────────────────────────────────────┐ │ STEP 1: COMPREHENSIVE PARAMETER DESIGN ENVELOPE │ ├────────────────────────────┬─────────────────────────────┬───────────────────────────────────────┤ │ Parameter / Specification │ Value Range / Unit │ Engineering Margin / Condition │ ├────────────────────────────┼─────────────────────────────┼───────────────────────────────────────┤ │ **Input Voltage ($V_{IN}$)**│ $+24.0\,\text{V}$ Nominal │ Delivered by Isolated Primary Flyback │ │ **Input Voltage Tolerance**│ $22.0\,\text{V} \dots 26.0\,\text{V}$│ Dynamic tolerance under line/load sag │ │ **Output Voltage ($V_O$)** │ $5.0\,\text{V} \dots 20.0\,\text{V}$│ Digitally programmable (4:1 span) │ │ **Voltage Programming Step│ $10\,\text{mV}$ Step Size │ Commanded via MCU 12-bit DAC │ │ **Continuous Current ($I_O$)│ $0.1\,\text{A} \dots 3.0\,\text{A}$│ Full rated current down to 5.0V out │ │ **Current Limit Step** │ $10\,\text{mA}$ Step Size │ Hardware Constant Current (CC) clamp │ │ **Maximum Output Power** │ $60.0\,\text{W}$ Continuous │ Full-range load ($20\,\text{V} \times 3\,\text{A}$) │ │ **Switching Frequency** │ $250\,\text{kHz}$ Fixed PWM │ Optimized for inductor size vs loss │ │ **Output Voltage Ripple** │ $< 25\,\text{mV}_{\text{pk-pk}}$│ Full load $20\,\text{V} / 3\,\text{A}$, 20MHz BW │ │ **Output Ripple at 5V/3A** │ $< 15\,\text{mV}_{\text{pk-pk}}$│ Pure buck mode operation │ │ **Peak Conversion Eff.** │ $\ge 96.5\%$ @ 15V / 3A │ Synchronous rectification across FETs │ │ **Average Conversion Eff.**│ $> 94.5\%$ Across Range │ Convection cooled, fanless enclosure │ │ **Load Transient Recovery**│ $< 120\,\mu\text{s}$ to 1% │ $50\% \rightarrow 100\%$ load step ($1.5A \rightarrow 3.0A$)│ │ **Load Transient Deviation**│ $< 250\,\text{mV}$ Peak │ Under $1.5\,\text{A}/\mu\text{s}$ load slew rate │ │ **Ambient Temperature** │ $-20^\circ\text{C} \dots +70^\circ\text{C}$│ Industrial bench environment │ │ **Target Junction Temp** │ $T_j \le 100^\circ\text{C}$ │ Safe derating below $150^\circ\text{C}$ max limit│ └────────────────────────────┴─────────────────────────────┴───────────────────────────────────────┘

3. Step 2: Topology Selection & Multi-Mode Operating Principles

The 4-Switch Synchronous Buck-Boost Topology uses an H-bridge configuration around a single power inductor (L):

Buck Half-Bridge: Q_A (High-Side) and Q_B (Low-Side Synchronous) at node SW1.

Boost Half-Bridge: Q_C (Low-Side) and Q_D (High-Side Synchronous) at node SW2.

4-Switch H-Bridge Topology +24V DC Bus In │ Drain (D) ┌───┴───┐ │ Q_A │ Buck High-Side Switch PWM_A ───┤ │ (BSC034N04LS: 40V, 3.4mΩ) └───┬───┘ │ Source (S) ├────── SW1 Node ──────[ Inductor L: 10µH ]────── SW2 Node ──────┬───> +V_OUT (5V-20V @ 3A) │ │ Drain (D) Source (S) ┌───┴───┐ ┌───┴───┐ PWM_B ───┤ Q_B │ Buck Low-Side Sync PWM_D ───┤ Q_D │ Boost High-Side Sync └───┬───┘ (BSC034N04LS) └───┬───┘ (BSC034N04LS) │ Source (S) │ Drain (D) PGND_SEC Drain (D) ┌───┴───┐ PWM_C ───┤ Q_C │ Boost Low-Side Switch └───┬───┘ (BSC034N04LS) │ Source (S) PGND_SEC

Multi-Mode Control Strategy:

Pure Buck Mode ($V_{OUT} < 0.85 \times V_{IN}$, i.e., $5.0\,\text{V} \dots 18.0\,\text{V}$):

Q_C is held OFF (0\%).

Q_D is held ON (100\%, continuous low-resistance conduction).

Q_A and Q_B switch at 250\,kHz with duty cycle D_{buck = \frac{V_{OUT{V_{IN.

Loss benefit: Only 2 switches toggle, achieving > 96.5\% efficiency!

Buck-Boost Transition Mode ($18.0\,\text{V} \dots 20.0\,\text{V}$):

The controller operates with interleaved buck and boost cycles, preventing narrow pulse skipping and eliminate subharmonic jitter.

Pure Boost Mode (when $V_{IN} < 0.85 \times V_{OUT}$):

Q_A is held ON (100\%), Q_B is held OFF (0\%).

Q_C and Q_D switch at 250\,kHz with duty cycle D_{boost = 1 - \frac{V_{IN{V_{OUT.

4. Step 3: Power Inductor Sizing (L = 10.0\,µH)

4.1 Ripple & Inductance Derivations:

Target Ripple Current Ratio: r = 30\% \implies \Delta I_L = 0.30 × 3.0\,A = 0.90\,A.

Under worst-case buck condition (V_{IN = 24.0\,V, V_{OUT = 12.0\,V):

L = \frac{V_{OUT} \cdot (V_{IN} - V_{OUT})}{\Delta I_L \cdot f_{sw} \cdot V_{IN}} = \frac{12.0 \cdot (24.0 - 12.0)}{0.90 \cdot 250000 \cdot 24.0} = 26.7\,\mu\text{H}

Why $L = 10.0\,\mu\text{H}$ is Selected:

Transient Slew Rate: \frac{di{dt = \frac{V_L{L = \frac{12.0\,V{10\,µH = 1.20\,A/µs (delivers 2.6× faster recovery than 27\,µH).

Ultra-Low DCR: 11.8\,m\Omega dissipates only 0.108\,W at full load!

Peak & RMS Inductor Current:

I_{IN} = \frac{60.0\,\text{W}}{0.96 \times 24.0\,\text{V}} = 2.60\,\text{A}

I_{L(pk)} = I_{IN} + I_{OUT} + \frac{\Delta I_L}{2} = 2.60\,\text{A} + 3.00\,\text{A} + 0.60\,\text{A} = \mathbf{5.95\,\text{A}}

I_{L(rms)} = \sqrt{(3.00)^2 + \frac{(1.20)^2}{12}} = \mathbf{3.02\,\text{A}_{\text{RMS}}}

Selected Inductor: Wurth Elektronik 7443321000 (Flat-wire shielded, 10.0\,µH, I_{rms = 8.5\,A, I_{sat = 11.5\,A).

Saturation margin is \mathbf{48.3\% above maximum peak current!

5. Step 4 & Step 5: Input & Output Capacitor Bank Sizing

5.1 Input Capacitor Bank (C_{IN):

RMS ripple current in buck mode (D = 50\%):

I_{Cin(rms)} = I_{OUT} \cdot \sqrt{D \cdot (1 - D)} = 3.0 \times \sqrt{0.25} = \mathbf{1.50\,\text{A}_{\text{RMS}}}

Sizing for \Delta V_{IN ≤ 40\,mV:

C_{IN(min)} = \frac{3.0 \times 0.25}{250000 \times 0.040} = 75\,\mu\text{F}

Bank Selection: 2× 22\,µF / 35\,V X7R 1210 Ceramic MLCCs in parallel with 1× 100\,µF / 35\,V Panasonic OS-CON Polymer capacitor (ESR = 16\,m\Omega).

5.2 Output Capacitor Bank (C_{OUT):

Sizing for < 25\,mV_{pk-pk ripple:

\Delta V_C = \frac{\Delta I_L}{8 \cdot f_{sw} \cdot C_{OUT}} = \frac{1.20\,\text{A}}{8 \times 250000 \times 88 \times 10^{-6}} = 6.8\,\text{mV}

\Delta V_{ESR} = \Delta I_L \cdot \text{ESR} = 1.20\,\text{A} \times 2.5\,\text{m}\Omega = 3.0\,\text{mV}

\Delta V_{OUT} = \sqrt{(6.8)^2 + (3.0)^2} = \mathbf{7.4\,\text{mV}_{\text{pk-pk}} \ll 25\,\text{mV}}

DC Voltage Bias Derating Alert:

Ceramic MLCCs lose \approx 55\% capacitance at 20\,V DC bias.

Bank Selection: 4× 22\,µF / 25\,V X7R 1210 MLCCs (C_{eff \approx 40\,µF) in parallel with 1× 100\,µF / 25\,V Panasonic OS-CON Polymer capacitor (ESR = 12\,m\Omega). Total effective capacitance \approx 140\,µF, combined ESR < 2.5\,m\Omega.

6. Step 6 & Step 7: Power MOSFETs, Gate Drivers & Bootstrap Sizing

6.1 Power MOSFET Selection (Q_A, Q_B, Q_C, Q_D):

Selected Component: Infineon BSC034N04LS (40\,V, 3.4\,m\Omega, SuperSO8 package, Q_g = 16\,nC).

Loss Breakdown per Switch:

Conduction loss (T_j = 100^\circC): P_{cond = (2.12\,A)^2 × 0.00476\,\Omega = 0.021\,W.

Switching loss: P_{sw = 0.5 × 24\,V × 3\,A × (14\,ns) × 250\,kHz = 0.126\,W.

Capacitive dump loss: P_{coss = 0.5 × (650\,pF) × (24)^2 × 250\,kHz = 0.047\,W.

Dead-time body diode loss: P_{body = 2 × 0.75\,V × 3\,A × (25\,ns) × 250\,kHz = 0.028\,W.

Gate charge loss: P_{gate = 16\,nC × 5\,V × 250\,kHz = 0.020\,W.

Total Loss per Switch: P_{total = \mathbf{0.242\,W.

Junction Temperature:

With \theta_{JA = 35^\circC/W and T_{amb = 60^\circC:

T_j = 60^\circ\text{C} + (0.242\,\text{W} \times 35^\circ\text{C/W}) = \mathbf{68.5^\circ\text{C}} \ll 150^\circ\text{C}

6.2 Bootstrap Circuit Sizing:

High-side switches Q_A and Q_D are driven by bootstrap capacitors (C_{boot1, C_{boot2):

C_{boot} \ge \frac{Q_g + I_{leak} \cdot t_{on}}{\Delta V_{boot}} \ge \frac{16\,\text{nC} + (10\,\mu\text{A} \times 3.0\,\mu\text{s})}{0.10\,\text{V}} = 0.16\,\mu\text{F} \longrightarrow \mathbf{0.22\,\mu\text{F} / 50\,\text{V X7R}}

Bootstrap Diodes: Nexperia PMEG6010CEH (60\,V, 1.0\,A Schottky).

Series Gate Resistors: R_g = 2.2\,\Omega dampens parasitic LC ringing.

7. Step 8: Digital Voltage Programming via DAC Feedback Injection

DAC CURRENT-INJECTION FEEDBACK SUMMING NODE +V_OUT (5.0V - 20.0V) ────────┬───────────────────────────────────────┐ │ │ ┌─┴─┐ R_top ┌─┴─┐ C_FF │ │ 49.9kΩ (0.1% Precision) │ │ 47pF └─┬─┘ └─┬─┘ │ │ ├─── Summing Node (V_FB) ───────────────┘ │ Clamped by Error Amp to V_REF = 1.20V │ ┌───────────────┼───────────────────────────────┐ │ │ │ ┌─┴─┐ ┌─┴─┐ ┌─┴─┐ R_DAC │ │ 11.0kΩ │ │ R_bot │ │ (0.1%)└─┬─┘ └─┬─┘ 3.65kΩ (0.1%) │ │ LM5176 Controller │ │ │ F │ Error Amplifier │ AGND_SEC │ B │ Inverting Pin From MCU ─────┴─────────────────────────────────────────────┤ │ 12-Bit DAC1 Output (0.0V - 3.3V) └───┘

7.1 Mathematical Derivation:

Applying KCL at the V_{FB summing junction (V_{FB = V_{REF = 1.20\,V):

\frac{V_{OUT} - V_{REF}}{R_{top}} + \frac{V_{DAC} - V_{REF}}{R_{DAC}} = \frac{V_{REF}}{R_{bot}}

V_{OUT} = V_{REF} \cdot \left[ 1 + \frac{R_{top}}{R_{bot}} + \frac{R_{top}}{R_{DAC}} \right] - V_{DAC} \cdot \left( \frac{R_{top}}{R_{DAC}} \right)

7.2 Selected Precision Values:

R_{top = 49.9\,k\Omega (0.1\%)

R_{DAC = 11.0\,k\Omega (0.1\%)

R_{bot = 3.65\,k\Omega (0.1\%)

At V_{DAC = 0.00\,V: V_{OUT = \mathbf{20.05\,V

At V_{DAC = 3.30\,V: V_{OUT = \mathbf{5.08\,V

Quantization Resolution:

\Delta V_{OUT(LSB)} = \frac{20.05\,\text{V} - 5.08\,\text{V}}{4096} = \mathbf{3.65\,\text{mV / LSB}}

8. Step 9: Precision Hardware Constant-Current (CC) Clamp Loop

ANALOG HARDWARE CONSTANT-CURRENT (CC) CLAMP From Inductor L ───┬───[ R_shunt: 10mΩ / 0.1% ]───┬───> To V_OUT Terminal │ │ ├─── IN+ (INA240A2) ├─── IN- (INA240A2) │ │ ┌─┴──────────────────────────────┴─┐ │ INA240A2 High-Side Current Sense │ (Fixed Gain = 50 V/V) │ Enhanced PWM Common-Mode (-4V..80V│ └────────────────┬─────────────────┘ │ V_ISENSE = 50 * (I_load * 0.010Ω) = 0.50 V/A │ (e.g., 0.50V at 1.0A; 1.50V at 3.0A) │ ┌─┴─┐ │ - │ Inverting Input │ ├────────────────────[ D_clamp: BAT54 Schottky ]───┐ │ │ │ From MCU ────────┤ + │ Ultra-Fast Analog Op-Amp │ DAC2 (0.1A - 3A) │ │ (TLV3501: 4.5ns propagation delay) │ V_ISET (0.05-1.5V└───┘ ▼ Pulls down COMP pin (Overrides CV Error Amp)

Current Shunt: 10\,m\Omega 0.1\% 3\,W Kelvin Shunt (Bourns CSS2H-2512R-L010F).

Current Sense Amp: TI INA240A2 (50\,V/V fixed gain) scales current linearly to V_{ISENSE = 0.50\,V/A (1.50\,V at 3.0\,A).

Hardware Clamp: Fast comparator (TI TLV3501, 4.5\,ns) drives BAT54 Schottky diode pulling down the LM5176 COMP pin in < 5\,µs during short circuits.

9. Step 10: Control Loop Modeling & Compensation Design

Right-Half-Plane (RHP) Zero Constraint:

f_{RHPZ} = \frac{R_L \cdot (1 - D)^2}{2\pi \cdot L} = \frac{6.67 \times (0.80)^2}{2\pi \times 10\,\mu\text{H}} \approx \mathbf{68.0\,\text{kHz}}

Target Loop Crossover Frequency: f_c = 15.0\,kHz (≤ f_{RHPZ / 4).

Type-II Compensator Components:

R_{comp = 10.0\,k\Omega (1\%)

C_{comp1 = 2.2\,nF (50\,V X7R)

C_{comp2 = 47\,pF (50\,V C0G)

Stability Margins: Phase Margin \Phi_m = \mathbf{64.5^\circ, Gain Margin G_m = \mathbf{-16.2\,dB (guarantees zero overshoot).

10. Step 11: Complete Component-Level Hardware Schematic

================================================================================================================================== 4-SWITCH SYNCHRONOUS BUCK-BOOST POST-REGULATOR (24V IN -> 5-20V / 3A PROGRAMMABLE OUT) ================================================================================================================================== +24V Intermediate DC Bus In ────┬─────────────────────────────────────────────────────────────┐ (From Isolated Primary Flyback) │ │ ┌─┴─┐ C_IN1 ┌─┴─┐ C_IN2 │ │ 2x 22µF / 35V MLCC │ │ 100µF / 35V Low-ESR Polymer └─┬─┘ └─┬─┘ │ │ PGND_SEC PGND_SEC │ Drain (D) ┌─────┴─────┐ │ Q_A │ Buck High-Side Switch BUCK_HO ─────────────┤ │ Infineon BSC034N04LS (40V, 3.4mΩ) └─────┬─────┘ │ Source (S) ├────────────── SW1 Switching Node ────┐ │ │ Drain (D) ┌─┴─┐ ┌─────┴─────┐ │ │ Inductor L │ Q_B │ Buck Low-Side Synchronous │ │ 10µH / 8.5A Flat-Wire BUCK_LO ─────────────┤ │ BSC034N04LS (40V, 3.4mΩ) └─┬─┘ (Wurth 7443321000) └─────┬─────┘ │ │ Source (S) │ PGND_SEC ├────────────── SW2 Switching Node ────┐ │ │ Source (S) Drain (D) ┌─────┴─────┐ ┌─────┴─────┐ │ Q_D │ Boost High-Side Sync │ Q_C │ Boost Low-Side Sw BOOST_HO ────────────┤ │ BSC034N04LS │ │ BSC034N04LS └─────┬─────┘ BOOST_LO ────┤ │ │ Drain (D) └─────┬─────┘ │ │ Source (S) ├──────────────────────────┐ PGND_SEC │ │ ┌─┴─┐ ┌─┴─┐ Kelvin Sense Lead 1 │ │ R_shunt │ │ C_OUT_POLY └─┬─┘ 10mΩ / 0.1% / 3W │ │ 100µF / 25V Polymer Kelvin Sense Lead 2 ├───┐ └─┬─┘ (ESR = 12mΩ) │ │ │ │ ┌─┴─┐ C_OUT_CER PGND_SEC │ │ │ 4x 22µF / 25V MLCC │ └─┬─┘ │ │ │ PGND_SEC │ +V_OUT Programmable Terminal ──────────────────────────────────────────┴────────────────────────────────────────────────> +V_OUT (5.0V - 20.0V @ 3A) (To Load & Dual-Domain Sensing) │ ├───────────────────┐ │ │ ┌─┴─┐ R_top ┌─┴─┐ C_FF │ │ 49.9kΩ │ │ 47pF └─┬─┘ (0.1%) └─┬─┘ │ │ ├─── V_FB Node ─────┘ │ (Held at V_REF = 1.20V) ┌───────────────┼───────────────────────────────┐ │ │ │ ┌─┴─┐ ┌─┴─┐ ┌─┴─┐ R_DAC │ │ 11.0kΩ │ │ R_bot │ │ (0.1%)└─┬─┘ (0.1%) └─┬─┘ 3.65kΩ (0.1%) │ │ │ │ │ C │ LM5176 MCU_DAC1 (0.0V - 3.3V) ────────────────────────────────┘ AGND_SEC │ O │ Error Amp (Voltage Setpoint: 20V down to 5V) │ M │ Pin │ P │ From CC Op-Amp Clamp Diode (BAT54) ──────────────────────────────────────────────────────────────────┤ │ └───┘ ==================================================================================================================================

11. Step 12 & Step 13: PCB Layout Guidelines & Bill of Materials

PCB Stackup: 4-Layer (1.6\,mm FR-4, L1 Top Power 2oz, L2 PGND Plane 1oz, L3 Logic 1oz, L4 Signals 2oz).

Power Loop Minimization: Buck input loop and Boost output loop are both constrained to < 0.8\,cm^2.

Kelvin Shunt Routing: 4-wire differential sense traces on Layer 4 shielded by Layer 2 ground plane.

Ground Partitioning: PGND and AGND joined at a single star point directly beneath C_{OUT.

┌──────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ BILL OF MATERIALS (BOM): 4-SWITCH BUCK-BOOST STAGE │ ├──────┬─────┬───────────────────────────┬─────────────────────────┬──────────────┬────────────────────────────────────────┤ │ Item │ Qty │ Reference Designator │ Description │ Package │ Manufacturer & Part Number │ ├──────┼─────┼───────────────────────────┼─────────────────────────┼──────────────┼────────────────────────────────────────┤ │ 1 │ 1 │ U1 (Controller) │ 4-Switch Sync BB IC │ HTSSOP-28 │ Texas Instruments LM5176PWPR │ │ 2 │ 4 │ Q_A, Q_B, Q_C, Q_D │ Power MOSFET 40V 3.4mΩ │ SuperSO8 │ Infineon BSC034N04LS │ │ 3 │ 1 │ L1 │ Flat-Wire Inductor 10uH │ 10x10mm SMD │ Wurth Elektronik 7443321000 │ │ 4 │ 1 │ R_shunt │ Kelvin Shunt 10mΩ 0.1% │ 2512 SMD │ Bourns CSS2H-2512R-L010F │ │ 5 │ 1 │ U2 (Current Sense Amp) │ High-Side Current Sense │ SOIC-8 │ Texas Instruments INA240A2D │ │ 6 │ 1 │ U3 (Fast Comparator) │ 4.5ns High-Speed Comp │ SOT-23-6 │ Texas Instruments TLV3501AIDBVR │ │ 7 │ 1 │ D_clamp │ Schottky Diode 30V 200mA│ SOT-23 │ Diodes Inc. BAT54 │ │ 8 │ 2 │ D_boot1, D_boot2 │ Schottky Diode 60V 1.0A │ SOD-123F │ Nexperia PMEG6010CEH │ │ 9 │ 2 │ C_boot1, C_boot2 │ Ceramic Cap 0.22µF 50V │ 0805 X7R │ Murata GRM21BR71H224KA01L │ │ 10 │ 2 │ C_IN_CER │ Ceramic Cap 22µF 35V │ 1210 X7R │ TDK C3225X7R1V226M │ │ 11 │ 1 │ C_IN_POLY │ Polymer Cap 100µF 35V │ Radial 8x10 │ Panasonic 35SVPF100M │ │ 12 │ 4 │ C_OUT_CER │ Ceramic Cap 22µF 25V │ 1210 X7R │ Murata GRM32ER71E226KE15L │ │ 13 │ 1 │ C_OUT_POLY │ Polymer Cap 100µF 25V │ SMD 8x10.5 │ Panasonic 25SVPF100M │ │ 14 │ 1 │ R_top │ Resistor 49.9kΩ 0.1% │ 0805 SMD │ Vishay TNPW080549K9BEEA │ │ 15 │ 1 │ R_DAC │ Resistor 11.0kΩ 0.1% │ 0805 SMD │ Vishay TNPW080511K0BEEA │ │ 16 │ 1 │ R_bot │ Resistor 3.65kΩ 0.1% │ 0805 SMD │ Vishay TNPW08053K65BEEA │ │ 17 │ 1 │ C_FF │ Ceramic Cap 47pF 50V │ 0603 C0G/NP0 │ KEMET C0603C470J5GACTU │ │ 18 │ 1 │ R_comp │ Resistor 10.0kΩ 1.0% │ 0603 SMD │ Panasonic ERA-3AEB103V │ │ 19 │ 1 │ C_comp1 │ Ceramic Cap 2.2nF 50V │ 0603 X7R │ Murata GRM188R71H222KA01D │ │ 20 │ 1 │ C_comp2 │ Ceramic Cap 47pF 50V │ 0603 C0G/NP0 │ KEMET C0603C470J5GACTU │ │ 21 │ 4 │ R_gate1..R_gate4 │ Resistor 2.2Ω 5% │ 0603 SMD │ Panasonic ERJ-3GEYJ2R2V │ └──────────────────────────────────────────────────────────────────────────────────────────────────────────────────────────┘

12. Step 14–17: Bring-Up, Calibration & RCA Matrix

Step 14: Cold Impedance Verification: Measure > 100\,k\Omega across +V_{IN and +V_{OUT before applying DC bench power.

Step 15: 2-Point Linear Software Calibration:

V_{meas} = m \cdot V_{command} + b

Eliminates passive tolerance offsets and guarantees < 0.1\% absolute voltage accuracy.

Step 16: Dynamic Load Step Testing:

Apply 1.5\,A \rightarrow 3.0\,A step (1.5\,A/µs slew rate). Verify transient droop < 250\,mV and recovery < 120\,µs.

Step 17: Bench Troubleshooting Matrix:

5-10kHz Output Oscillation: Increase C_{FF from 47\,pF to 100\,pF; add 100\,pF bypass at DAC pin.

Ripple $> 50\,\text{mV}$: Use tip-and-barrel oscilloscope probe; verify MLCCs are rated for ≥ 25\,V.

MOSFET runs hot: Ensure bootstrap voltage exceeds 4.5\,V; increase R_g to 4.7\,\Omega if shoot-through is present.

13. Sizing Verification CLI Commands

You can verify all mathematical parameters using the CLI calculation tools:

# 1. Size 4-switch Buck-Boost inductor, ripple, and capacitors: python3 tools/psu_calc.py --buckboost --vin 24 --vout-min 5.0 --vout-max 20.0 --iout 3.0 --fsw 250000 # 2. Calculate DAC summing resistor network: python3 tools/psu_calc.py --feedback --vout-min 5.0 --vout-max 20.0 --vdac-max 3.3 --vref 1.20 # 3. Compare Buck-Boost losses against alternative post-regulators: python3 smart_programmable_power_supply_alternatives/tools/alternative_converter_calc.py --compare-losses --vout 20.0 --iout 3.0