3-Floor Dual Lift Controller: Gate-Level Logic Design

This document details the complete gate-level digital logic design for an intelligent Dual Lift System serving a 3-Floor Building (Floors 0, 1, and 2, corresponding to Ground, First, and Second floors). When a passenger initiates a hall call at any floor, the combinational logic dispatches the nearest lift car.

1. System Specifications & Signal Definitions

Floor 2 ─── [ R2 Call ] ────────── [ Lift A: A2 ] ────────── [ Lift B: B2 ] Floor 1 ─── [ R1 Call ] ────────── [ Lift A: A1 ] ────────── [ Lift B: B1 ] Floor 0 ─── [ R0 Call ] ────────── [ Lift A: A0 ] ────────── [ Lift B: B0 ]

1.1 Input Signals

Requested Floor ($R$):

1-Hot Inputs: R_0 (Floor 0 Call), R_1 (Floor 1 Call), R_2 (Floor 2 Call).

Only one request is active at an instant (R_0 + R_1 + R_2 = 1).

2-Bit Binary Equivalent: (R_1, R_0) \in \{00_2, 01_2, 10_2\.

Lift A Position ($A$):

1-Hot Position Sensors: A_0, A_1, A_2 (Limit switches on Floor 0, 1, 2).

2-Bit Binary Equivalent: (A_1, A_0) \in \{00_2, 01_2, 10_2\.

Lift B Position ($B$):

1-Hot Position Sensors: B_0, B_1, B_2 (Limit switches on Floor 0, 1, 2).

2-Bit Binary Equivalent: (B_1, B_0) \in \{00_2, 01_2, 10_2\.

1.2 Output Signals

Arbitration / Lift Selection:

SEL_A: Logic 1 if Lift A is dispatched to answer the call.

SEL_B: Logic 1 if Lift B is dispatched to answer the call.

Motor Control Signals for Lift A:

UP_A: Energizes hoisting motor in UP direction.

DOWN_A: Energizes hoisting motor in DOWN direction.

STOP_A: Cuts motor power, engages mechanical brake, triggers door opening.

Motor Control Signals for Lift B:

UP_B, DOWN_B, STOP_B (Identical control for Lift B).

2. Exhaustive 27-State Truth Table for Nearest Lift Selection

Because each of the 3 variables (R, A, B) can take 3 valid values (0, 1, 2), the entire state space consists of exactly 3 × 3 × 3 = \mathbf{27  valid combinations:

Distance of Lift A: d_A = |R - A|

Distance of Lift B: d_B = |R - B|

Tie-Breaker Rule: When d_A = d_B, Lift A is selected by default (SEL_A = 1, SEL_B = 0).

| State | Req (R) | Lift A (A) | Lift B (B) | d_A | d_B | Winner (SEL_A, SEL_B) | Lift A Motor | Lift B Motor | Operational Rationale |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | Lift A (1, 0) | STOP_A | STOP_B | Both at floor 0; Lift A opens door. |
| 1 | 0 | 0 | 1 | 0 | 1 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 0 (d_A=0). |
| 2 | 0 | 0 | 2 | 0 | 2 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 0 (d_A=0). |
| 3 | 0 | 1 | 0 | 1 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 0 (d_B=0). |
| 4 | 0 | 1 | 1 | 1 | 1 | Lift A (1, 0) | DOWN_A | STOP_B | Equidistant (d=1); Lift A dispatched DOWN. |
| 5 | 0 | 1 | 2 | 1 | 2 | Lift A (1, 0) | DOWN_A | STOP_B | Lift A closer (d_A=1 < d_B=2). |
| 6 | 0 | 2 | 0 | 2 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 0 (d_B=0). |
| 7 | 0 | 2 | 1 | 2 | 1 | Lift B (0, 1) | STOP_A | DOWN_B | Lift B closer (d_B=1 < d_A=2). |
| 8 | 0 | 2 | 2 | 2 | 2 | Lift A (1, 0) | DOWN_A | STOP_B | Equidistant (d=2); Lift A dispatched DOWN. |
| 9 | 1 | 0 | 0 | 1 | 1 | Lift A (1, 0) | UP_A | STOP_B | Equidistant (d=1); Lift A dispatched UP. |
| 10 | 1 | 0 | 1 | 1 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 1 (d_B=0). |
| 11 | 1 | 0 | 2 | 1 | 1 | Lift A (1, 0) | UP_A | STOP_B | Equidistant (d=1); Lift A dispatched UP. |
| 12 | 1 | 1 | 0 | 0 | 1 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 1 (d_A=0). |
| 13 | 1 | 1 | 1 | 0 | 0 | Lift A (1, 0) | STOP_A | STOP_B | Both at floor 1; Lift A opens door. |
| 14 | 1 | 1 | 2 | 0 | 1 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 1 (d_A=0). |
| 15 | 1 | 2 | 0 | 1 | 1 | Lift A (1, 0) | DOWN_A | STOP_B | Equidistant (d=1); Lift A dispatched DOWN. |
| 16 | 1 | 2 | 1 | 1 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 1 (d_B=0). |
| 17 | 1 | 2 | 2 | 1 | 1 | Lift A (1, 0) | DOWN_A | STOP_B | Equidistant (d=1); Lift A dispatched DOWN. |
| 18 | 2 | 0 | 0 | 2 | 2 | Lift A (1, 0) | UP_A | STOP_B | Equidistant (d=2); Lift A dispatched UP. |
| 19 | 2 | 0 | 1 | 2 | 1 | Lift B (0, 1) | STOP_A | UP_B | Lift B closer (d_B=1 < d_A=2). |
| 20 | 2 | 0 | 2 | 2 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 2 (d_B=0). |
| 21 | 2 | 1 | 0 | 1 | 2 | Lift A (1, 0) | UP_A | STOP_B | Lift A closer (d_A=1 < d_B=2). |
| 22 | 2 | 1 | 1 | 1 | 1 | Lift A (1, 0) | UP_A | STOP_B | Equidistant (d=1); Lift A dispatched UP. |
| 23 | 2 | 1 | 2 | 1 | 0 | Lift B (0, 1) | STOP_A | STOP_B | Lift B already at floor 2 (d_B=0). |
| 24 | 2 | 2 | 0 | 0 | 2 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 2 (d_A=0). |
| 25 | 2 | 2 | 1 | 0 | 1 | Lift A (1, 0) | STOP_A | STOP_B | Lift A already at floor 2 (d_A=0). |
| 26 | 2 | 2 | 2 | 0 | 0 | Lift A (1, 0) | STOP_A | STOP_B | Both at floor 2; Lift A opens door. |

3. Direct 1-Hot Gate Logic Design

In relay and discrete digital logic controllers, 1-hot encoding is widely preferred because it eliminates encoders and decoders, reducing gate propagation delay to a bare minimum.

3.1 Distance Generation Gates

For a 3-floor building, the distance between any floor and a lift can only be 0, 1, or 2 floors.

For Lift A:

Distance = 0 ($d_{A0}$): Lift A is at the requested floor.

d_{A0} = R_0 A_0 + R_1 A_1 + R_2 A_2

Distance = 1 ($d_{A1}$): Lift A is 1 floor away.

d_{A1} = R_0 A_1 + R_1 A_0 + R_1 A_2 + R_2 A_1

Distance = 2 ($d_{A2}$): Lift A is 2 floors away (one at Floor 0, other at Floor 2).

d_{A2} = R_0 A_2 + R_2 A_0

For Lift B:

Distance = 0 ($d_{B0}$):

d_{B0} = R_0 B_0 + R_1 B_1 + R_2 B_2

Distance = 1 ($d_{B1}$):

d_{B1} = R_0 B_1 + R_1 B_0 + R_1 B_2 + R_2 B_1

Distance = 2 ($d_{B2}$):

d_{B2} = R_0 B_2 + R_2 B_0

3.2 Distance Comparator & Dispatch Arbitration Gates

Lift B is selected if and only if it is strictly closer than Lift A (d_B < d_A):

SEL_B = (d_{B0} \cdot \overline{d_{A0}}) + (d_{B1} \cdot d_{A2})

Expanding with the distance definitions:

When d_B = 0 and d_A \neq 0: Lift B is already at the requested floor, but Lift A is not:

SEL_B = d_{B0} \cdot (d_{A1} + d_{A2}) + d_{B1} \cdot d_{A2}

Lift A is selected for all remaining cases (when d_A ≤ d_B):

SEL_A = \overline{SEL_B}

LIFT ARBITRATION GATES (3-FLOOR) dA1 ──┐ ├──[ OR1 ]───┐ dA2 ──┘ │ ├──[ AND1 ]───┐ dB0 ───────────────┘ │ ├──[ OR3 ]────────────────► SEL_B (Dispatch Lift B) dB1 ───────────────┐ │ │ ├──[ AND2 ]───┘ ┌─┴─┐ dA2 ───────────────┘ │NOT│ └─┬─┘ │ └─► SEL_A (Dispatch Lift A)

3.3 Direction Control Gates

Once a lift is selected (SEL_A = 1 or SEL_B = 1), its motor direction is determined by comparing the request with its current floor:

Lift A Direction Logic:

Move UP ($UP_A$): Requested floor is higher than current floor.

Request Floor 1 while Lift A is at Floor 0 (R_1 A_0)

Request Floor 2 while Lift A is at Floor 0 (R_2 A_0)

Request Floor 2 while Lift A is at Floor 1 (R_2 A_1)

\mathbf{UP_A = SEL_A \cdot [A_0(R_1 + R_2) + A_1 R_2]}

Move DOWN ($DOWN_A$): Requested floor is lower than current floor.

Request Floor 0 while Lift A is at Floor 1 (R_0 A_1)

Request Floor 0 while Lift A is at Floor 2 (R_0 A_2)

Request Floor 1 while Lift A is at Floor 2 (R_1 A_2)

\mathbf{DOWN_A = SEL_A \cdot [A_2(R_0 + R_1) + A_1 R_0]}

STOP ($STOP_A$): Lift A is at the requested floor OR Lift A is not selected.

\mathbf{STOP_A = (R_0 A_0 + R_1 A_1 + R_2 A_2) + \overline{SEL_A} = d_{A0} + \overline{SEL_A}}

Lift B Direction Logic:

\mathbf{UP_B = SEL_B \cdot [B_0(R_1 + R_2) + B_1 R_2]}

\mathbf{DOWN_B = SEL_B \cdot [B_2(R_0 + R_1) + B_1 R_0]}

\mathbf{STOP_B = (R_0 B_0 + R_1 B_1 + R_2 B_2) + \overline{SEL_B} = d_{B0} + \overline{SEL_B}}

LIFT A DIRECTION CONTROL GATES R1 ──┐ ├──[ OR ]───┐ R2 ──┘ ├──[ AND ]───┐ A0 ──────────────┘ │ ├──[ OR ]───┬──[ AND ]───► UP_A R2 ──┐ │ │ ├──[ AND ]───────────────┘ │ A1 ──┘ │ │ SEL_A ────────────────────────────────────┴────────────► (Enable)

4. Door Control & Passenger Boarding Logic

When a lift stops at the requested floor (STOP = 1 and d_0 = 1), the controller triggers the automatic door mechanism:

Arrival Pulse 5-Second Monostable Timer (555 / Counter) d_A0 ──┐ ┌───────────────┐ ├──[ AND ]───► Trigger (Active-Low)│ ├──► DOOR_OPEN_A (Motor) SEL_A ─┘ │ Gate Timer │ │ (t = 5.0 s) │ DOOR_OPEN_SENSOR (Photocell / Bump) ─────►│ RESET / PAUSE │ └───────────────┘

Door Open Condition: DOOR\_OPEN_A = SEL_A \cdot d_{A0 \cdot \overline{UP_A \cdot \overline{DOWN_A.

Door Safety Interlock: An optical light curtain or edge bumper sensor detects passengers in the doorway, resetting the 5-second timer to hold the door open until the obstruction clears.

Motion Interlock:

MOTOR\_ENABLE_A = DOOR\_CLOSED\_SWITCH_A

The hoisting motor power relays are hard-interlocked through the physical door-closed limit switch. Power cannot flow to the motor while the door is open.