Dual Lift Controller: Gate-Level Control Logic Design for 3 & 4 Floor Buildings

Welcome to the Dual Lift Controller Digital Logic Design Suite. This project provides an exhaustive, industry-grade gate-level implementation of an intelligent elevator dispatch system for 3-floor and 4-floor residential and commercial buildings.

1. Problem Statement

[NOTE] Design Prompt: "Design a control logic using gates for two lifts which need to be designed for 3 (or) 4 floor building. When a lift request is initiated from any floor, nearest lift should be started."

Core Functional Objectives:

Two Independent Lifts: Lift A (L_A) and Lift B (L_B) serving common floors.

Floor Coverage: Configurable for 3-Floor Buildings (Floors 0, 1, 2) or 4-Floor Buildings (Floors 0, 1, 2, 3).

Nearest Lift Dispatch Rule: When a passenger initiates a hall call at floor R, the system calculates the absolute spatial distance from both lifts to the requested floor:

d_A = |R - A|

d_B = |R - B|

If d_A < d_B \implies Lift A is dispatched (SEL_A = 1, SEL_B = 0).

If d_B < d_A \implies Lift B is dispatched (SEL_A = 0, SEL_B = 1).

If d_A = d_B \implies Equidistant Tie: Deterministic priority rule selects Lift A (or idle lift).

Autonomous Direction Control: The selected lift automatically determines its motion:

If R > Position \implies \mathbf{UP motor energized.

If R < Position \implies \mathbf{DOWN motor energized.

If R = Position \implies \mathbf{STOP motor, open doors, trigger boarding timer.

Pure Gate-Level Realization: Built entirely using fundamental combinational and sequential digital logic gates (AND, OR, NOT, XOR, XNOR, Multiplexers, Adders/Subtractors, Magnitude Comparators, and D/JK Flip-Flops).

2. Top-Level System Architecture

The complete dual lift controller is partitioned into five distinct modular functional units:

┌────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ DUAL LIFT SYSTEM BLOCK DIAGRAM │ └────────────────────────────────────────────────────────────────────────────────────────────────────────┘ Floor Call Buttons (R0..R3) ──► [Priority Encoder] ──► 2-Bit Target Floor (R1, R0) │ ┌───────────────────────────┴───────────────────────────┐ │ │ Lift A Sensors (Limit Sw) ──► [Reg A] (A1, A0) Lift B Sensors (Limit Sw) ──► [Reg B] (B1, B0) │ │ │ │ │ ▼ │ ▼ │ ┌──────────────────────┐ │ ┌──────────────────────┐ │ │ Absolute Difference │ │ │ Absolute Difference │ │ │ Subtractor ¦R - A¦ │ │ │ Subtractor ¦R - B¦ │ │ └──────────┬───────────┘ │ └──────────┬───────────┘ │ │ Distance dA (dA1, dA0) │ │ Distance dB (dB1, dB0) │ │ │ │ │ └───────────────────┬───────────────────┘ │ │ │ │ │ ▼ │ │ ┌─────────────────────┐ │ │ │ 2-Bit Magnitude │ │ │ │ Comparator │ │ │ │ (dA vs dB) │ │ │ └──────────┬──────────┘ │ │ │ │ │ ┌──────────────────┴──────────────────┐ │ │ │ Nearest Lift Arbiter & Tie-Breaker │ │ │ └──────────┬──────────────────┬───────┘ │ │ │ │ │ │ SEL_A = 1│ │SEL_B = 1 │ │ ▼ ▼ │ │ ┌─────────────────────┐ ┌─────────────────────┐ │ └─────────────────────►│ Lift A Direction │ │ Lift B Direction │◄────────┘ │ Controller │ │ Controller │ │ (R vs A Comparator) │ │ (R vs B Comparator) │ └──────────┬──────────┘ └──────────┬──────────┘ │ │ ┌───────────┼───────────┐ ┌───────────┼───────────┐ ▼ ▼ ▼ ▼ ▼ ▼ UP_A DOWN_A STOP UP_B DOWN_B STOP

3. Subsystem Breakdown

1. Position & Request Encoding Stage

Sensors: Hall-effect or mechanical limit switches at each floor detect lift car presence (A_0, A_1, A_2, A_3 and B_0, B_1, B_2, B_3).

Request Buttons: Hall call buttons at each floor latch an incoming passenger request.

Encoding: Converted to 2-bit binary (00 = Ground/Floor 0, 01 = Floor 1, 10 = Floor 2, 11 = Floor 3) via a standard 74LS148 priority encoder or minimal OR gates.

2. Distance Computation Engine (|R - A| and |R - B|)

Implements gate-level absolute difference:

d_0 = R_0 \oplus A_0

d_1 = R_1 \overline{A_1}(\overline{A_0} + R_0) + \overline{R_1} A_1 (\overline{R_0} + A_0)

Generates distance values (0, 1, 2, 3 floors away) in < 15\,ns propagation delay.

3. Nearest Lift Arbiter

Compares d_A and d_B using a 2-bit magnitude comparator.

Asserts SEL_A = 1 when d_A ≤ d_B, and SEL_B = 1 when d_B < d_A.

4. Direction & Motor Drive Gates

Compares requested floor R with selected lift's position:

UP = SEL \cdot (R > Position)

DOWN = SEL \cdot (R < Position)

STOP = (R == Position) + \overline{SEL

5. Door Interlock & Arrival Logic

When a lift reaches the target floor (R == Position), motor power is cut, and a 555-timer / counter gate sequence opens the door for 5\,seconds.

Safety limit: Motor cannot be energized unless Door Closed Limit Switch is asserted.

4. Documentation Suite Roadmap

vvdn/dual_lift_controller/ ├── README.md (.docx) # Executive overview, architecture & directory roadmap ├── 3-floor-dual-lift-logic-design.md (.docx) # Gate design for 3-floor building (27-state truth table, K-maps) ├── 4-floor-dual-lift-logic-design.md (.docx) # Gate design for 4-floor building (64-state truth table, subtractors) ├── gate-level-schematics-and-simulation.md (.docx)# TTL 7400 IC schematics, gate counts & simulation waveforms ├── dual_lift_controller.v # Synthesizable gate-level Verilog hardware description module └── dual_lift_controller_tb.v # Verilog testbench validating all 64 state combinations

1. 3-Floor Dual Lift Logic Design

Complete design for 3-Floor Buildings (Floors 0, 1, 2).

1-Hot input encoding (R_0, R_1, R_2) and 2-bit binary encoding.

Exhaustive 27-state truth table for distance and nearest lift selection.

Karnaugh Maps (K-maps) and Boolean minimal equations for SEL_A and SEL_B.

Gate-level schematics using fundamental AND, OR, NOT, XOR gates.

2. 4-Floor Dual Lift Logic Design

Complete design for 4-Floor Buildings (Floors 0, 1, 2, 3).

2-bit binary arithmetic: Gate-level absolute subtractor and 2-bit comparator.

64-state truth table and algebraic reduction.

Direction control logic (UP, DOWN, STOP) with motor interlocking.

Lift Busy and direction-affinity arbitration.

3. Gate-Level Schematics & Simulation

Component-level pinout schematics using standard 7400-series TTL ICs (74LS04, 74LS08, 74LS32, 74LS86, 74LS85, 74LS74).

Total IC and gate budget calculation.

Verilog simulation waveforms and timing analysis.

5. Verification Tools: tools/lift_sim.py

A gate-level verification simulator is provided to model and verify all logic combinations:

# 1. Run exhaustive verification across all 64 states for 4-floor building: python3 tools/lift_sim.py --floors 4 --verify # 2. Run exhaustive verification across all 27 states for 3-floor building: python3 tools/lift_sim.py --floors 3 --verify # 3. Simulate specific operational scenario: # Request at Floor 2, Lift A at Floor 0, Lift B at Floor 3 -> Nearest is Lift A (dA=2, dB=1 -> Lift B wins!) python3 tools/lift_sim.py --floors 4 --req 2 --lift-a 0 --lift-b 3