Gate-Level Schematics, TTL IC Implementation & Simulation Guide

This document provides the hardware implementation details for the Dual Lift Controller: standard 7400-series TTL Integrated Circuit (IC) Bill of Materials, detailed pin-to-pin wiring schematics, propagation delay analysis, and complete simulation verification.

1. Standard TTL 7400-Series IC Implementation & Bill of Materials

The entire dual lift controller can be constructed using readily available off-the-shelf standard 7400-series TTL or 74HCT CMOS digital logic integrated circuits:

| IC Part Number | Function Description | Package | Quantity Required | Subsystem Usage |
| --- | --- | --- | --- | --- |
| 74LS148 | 8-to-3 Line Priority Encoder | 16-pin DIP | 1 | Encodes 1-hot floor call pushbuttons (R_0..R_3) into 2-bit binary (R_1, R_0). |
| 74LS74 | Dual D-Type Positive-Edge-Triggered Flip-Flops | 14-pin DIP | 2 | 2-bit floor position registers for Lift A (A_1, A_0) and Lift B (B_1, B_0). |
| 74LS86 | Quad 2-Input XOR Gates | 14-pin DIP | 2 | Distance calculation LSBs (d_0 = X_0 \oplus Y_0) and bit equivalence logic. |
| 74LS04 | Hex Inverters (NOT Gates) | 14-pin DIP | 2 | Signal inversions (\overline{R, \overline{A, \overline{B, \overline{SEL). |
| 74LS08 | Quad 2-Input AND Gates | 14-pin DIP | 3 | Distance MSB terms, magnitude comparators, and motor enable gates. |
| 74LS32 | Quad 2-Input OR Gates | 14-pin DIP | 2 | Sum-of-Products distance combine and nearest lift arbitration. |
| 74LS85 | 4-Bit Magnitude Comparator | 16-pin DIP | 3 | Optional single-chip replacement for Distance Compare and Direction Comparators. |
| NE555 / 74LS123 | Precision Timer / Dual Monostable Multivibrator | 8/16-pin DIP | 2 | 5.0-second Door Open monostable delay timer for Lift A and Lift B. |
| ULN2003A | High-Voltage High-Current Darlington Transistor Array | 16-pin DIP | 1 | Interfacing 5V logic signals to 12V/24V motor contactor and door solenoid coils. |

2. Gate-Level Pin-to-Pin Schematic Architecture

2.1 Complete 2-Bit Absolute Distance Subtractor (|R - A|)

74LS86 (XOR) 74LS04 (NOT) 74LS08 (AND) / 74LS32 (OR) ┌─────────────────────┐ ┌─────────────────────┐ ┌──────────────────────────────────┐ R0 ────►│ Pin 1 (1A) │ │ │ │ │ │ Pin 3 (1Y) ├──────────┼─────────────────────┼─────┼──────────────────────────────────┼────► dA0 (LSB) A0 ────►│ Pin 2 (1B) │ │ │ │ │ └─────────────────────┘ │ │ │ │ R1 ────┬─────────────────────────────────┼─────────────────────┼────►│ Pin 1 (1A) │ │ │ │ │ Pin 3 (1Y) ──┐ │ A1 ────┼─────────────────────────────────┼►Pin 1 (1A) │ │ Pin 2 (1B) │ │ │ │ Pin 2 (1Y: ~A1)───┼────►│ (74LS08) ├──[74LS32]─┼────► dA1 (MSB) │ │ │ │ │ OR Gate │ │ │ │ │ │ Pin 3 (1Y) │ │ │ │ Pin 4 (2A) ──┐ │ │ └─────────────────────────────────┴─────────────────────┴────►│ Pin 5 (2B) ──┴───────┘ │ └──────────────────────────────────┘

2.2 Nearest Lift Arbitration Unit (dA vs. dB)

74LS85 4-Bit Magnitude Comparator ┌───────────────────────┐ dA0 ─────────►│ Pin 10 (A0) │ dA1 ─────────►│ Pin 12 (A1) │ GND ─────────►│ Pin 13 (A2) │ GND ─────────►│ Pin 15 (A3) │ │ │ dB0 ─────────►│ Pin 9 (B0) │ dB1 ─────────►│ Pin 11 (B1) │ GND ─────────►│ Pin 14 (B2) │ GND ─────────►│ Pin 1 (B3) │ │ │ +5V ─────────►│ Pin 3 (A=B In) │ GND ─────────►│ Pin 2 (A<B In) │ GND ─────────►│ Pin 4 (A>B In) │ │ │ │ Outputs: │ │ Pin 7 (A<B Out: LT) ├──────────┐ (74LS32 OR) │ Pin 6 (A=B Out: EQ) ├──────────┴──[ OR Gate ]───► SEL_A (Lift A Selected) │ Pin 5 (A>B Out: GT) ├────────────────────────────► SEL_B (Lift B Selected) └───────────────────────┘

Lift A Dispatch ($SEL_A$): Fired when d_A < d_B OR when d_A = d_B (Equidistant tie-break).

Lift B Dispatch ($SEL_B$): Fired when d_A > d_B (d_B is strictly closer).

2.3 Motor Power Contactor Driver (ULN2003A Interfacing)

Logic gates operate at 5\,V TTL levels and cannot drive the heavy 12\,V or 24\,V DC coils of the industrial reversing contactors. A ULN2003A Darlington Transistor Array bridges the logic to the motor power stage:

TTL Logic Level (5V) ULN2003A Driver High-Voltage Contactor (24V) ┌──────────────────────┐ ┌─────────────────────┐ ┌────────────────────────────┐ │ │ │ │ │ +24V Supply Rail │ │ UP_A Gate Output ├────────────►│ Pin 1 (1B) │ │ │ │ │ │ │ Pin 16 (1C) ├─────────────────┼─────┴──[ UP Contactor Coil ] │ DOWN_A Gate Output ├────────────►│ Pin 2 (2B) │ │ │ │ │ │ │ Pin 15 (2C) ├─────────────────┼─────┴──[ DOWN Contactor ] │ │ DOOR_A Gate Output ├────────────►│ Pin 3 (3B) │ │ │ │ │ │ │ Pin 14 (3C) ├─────────────────┼─────┴──[ Door Motor Relay ] │ │ │ │ Pin 8 (GND) │ │ │ │ │ │ Pin 9 (COM Clamp) ├─────────────────┼────► Tie to +24V Rail │ └──────────────────────┘ └─────────────────────┘ └────────────────────────────┘

3. Propagation Delay & Timing Analysis

In a pure combinational dispatch architecture, speed is governed by the total cumulative propagation delay (t_{pd) through the longest path (critical path):

Critical Path: Floor Call (R) ──► Priority Encoder ──► Abs Difference ──► Magnitude Comp ──► Arbiter ──► Direction Logic ──► Motor Out (74LS148) (74LS86/08) (74LS85) (74LS32) (74LS85/08) (ULN2003) Delay: ~ 14 ns ~ 18 ns ~ 16 ns ~ 10 ns ~ 16 ns ~ 1.5 µs

Delay Budget Calculation:

\text{Total Combinational Gate Delay} = 14\,\text{ns} + 18\,\text{ns} + 16\,\text{ns} + 10\,\text{ns} + 16\,\text{ns} \approx \mathbf{74\,\text{ns}}

The logic gates calculate the nearest lift, resolve ties, and select motor direction in under $80\,\text{nanoseconds}$.

This instantaneous response eliminates human-perceptible latency and prevents contactor race conditions.

4. Verilog Simulation Waveforms & Verification Scenarios

The Verilog testbench `dual_lift_controller_tb.v` was executed to verify all operational permutations. Below are three canonical test vector walkthroughs:

Scenario 1: Request Closer to Lift B

State: Request at Floor 2 (R = 10_2). Lift A is at Floor 0 (A = 00_2). Lift B is at Floor 3 (B = 11_2).

Calculated Distances:

d_A = |2 - 0| = 2 \ (10_2)

d_B = |2 - 3| = 1 \ (01_2)

Comparator Output: d_A > d_B (GT = 1, LT = 0, EQ = 0).

Arbiter Output: SEL_A = 0, \mathbf{SEL_B = 1.

Direction Output: Lift B is at Floor 3, Target is Floor 2 (R < B \implies \mathbf{DOWN_B = 1, UP_B = 0, STOP_B = 0).

Result: Lift B immediately descends to Floor 2 while Lift A remains idle at Floor 0.

Time (ns) Req Lift_A Lift_B dA dB Winner UP_A DOWN_A STOP_A UP_B DOWN_B STOP_B 120.0 2 0 3 2 1 LIFT B 0 0 1 0 1 0

Scenario 2: Equidistant Request (Tie-Breaking Rule)

State: Request at Floor 1 (R = 01_2). Lift A is at Floor 0 (A = 00_2). Lift B is at Floor 2 (B = 10_2).

Calculated Distances:

d_A = |1 - 0| = 1 \ (01_2)

d_B = |1 - 2| = 1 \ (01_2)

Comparator Output: d_A == d_B (EQ = 1, LT = 0, GT = 0).

Arbiter Output: Tie detected; priority goes to Lift A (\mathbf{SEL_A = 1, SEL_B = 0).

Direction Output: Lift A is at Floor 0, Target is Floor 1 (R > A \implies \mathbf{UP_A = 1).

Result: Lift A moves UP to Floor 1 while Lift B stays stationed at Floor 2.

Time (ns) Req Lift_A Lift_B dA dB Winner UP_A DOWN_A STOP_A UP_B DOWN_B STOP_B 250.0 1 0 2 1 1 LIFT A 1 0 0 0 0 1

Scenario 3: Request at Floor Where Lift A is Already Parked

State: Request at Floor 3 (R = 11_2). Lift A is at Floor 3 (A = 11_2). Lift B is at Floor 1 (B = 01_2).

Calculated Distances:

d_A = |3 - 3| = 0 \ (00_2)

d_B = |3 - 1| = 2 \ (10_2)

Comparator Output: d_A < d_B (LT = 1).

Arbiter Output: Lift A (\mathbf{SEL_A = 1, SEL_B = 0).

Direction Output: Target matches current position (R == A \implies \mathbf{STOP_A = 1, UP_A = 0, DOWN_A = 0).

Door Mechanism: DOOR\_OPEN_A triggers, opening the cabin doors immediately for waiting passengers.

Time (ns) Req Lift_A Lift_B dA dB Winner UP_A DOWN_A STOP_A UP_B DOWN_B STOP_B 380.0 3 3 1 0 2 LIFT A 0 0 1 0 0 1

5. Summary Verification Checklist

[x] Nearest Lift Rule Verified: Confirmed across all 64 4-floor combinations and all 27 3-floor combinations via Python simulation (tools/lift_sim.py) and Verilog testbench (dual_lift_controller_tb.v).

[x] Zero Conflict: Interlocking gates ensure UP and DOWN signals can never be asserted concurrently.

[x] Fail-Safe Stopping: Any unselected lift automatically receives STOP = 1 to maintain mechanical brakes.

[x] Hardware Feasibility: Designed strictly using standard TTL logic ICs with low component count and sub-100\,ns latency.