EVSE Hardware Schematics, Component Selection & Electrical Interfacing

This document provides a detailed circuit-level reference for the Low Voltage Controller Board Schematics, Component Selection, Galvanic Isolation Barriers, EMC Protection, and Hardware Bring-Up.

1. Schematic Sub-Circuits & Analog Signal Chains

Below are the detailed component-level schematic implementations for the core analog and digital circuits on the low-voltage controller board:

1.1 Control Pilot (CP) Bipolar ± 12\,V Driver & Peak Sampler

+12V Supply Rail │ ┌──┴──┐ MCU 1 kHz PWM ──[ R_in ]──┤ + │ (0 to 3.3V) │ ├───────[ R1: 1.0 kΩ, 1%, 0.5W ]──────┬────► CP Terminal MCU Bias DAC ──[ R_fb ]──┤ - │ │ (To Vehicle) └──┬──┘ ──┴── │ ▲ ▼ Dual Bi-directional TVS -12V Supply Rail ──┬── (PESD12VS1UL - ±15 kV ESD) │ GND │ ┌─────────────────────────────────────────┘ │ ├──[ R_div1: 10 kΩ, 0.1% ]──┐ │ │ │ ┌─┴─┐ │ │ │ (ADC Buffer / Peak Detector) │ └─┬─┘ │ │ └──[ R_div2: 2.2 kΩ, 0.1% ]─┴─────► To MCU Precision ADC Channel

Circuit Theory & Component Selection:

High-Voltage Rail-to-Rail Op-Amp (e.g., TI OPA2171 / Microchip MCP6022):

Operating across +12\,V and -12\,V supply rails (24\,V total span).

High slew rate (> 5\,V/µs) to preserve sharp < 2\,µs rise/fall edges on the 1.0\,kHz square wave.

High output current drive (> 50\,mA) to charge the parasitic capacitance of long (> 5--7\,meter) vehicle charging cables without rounding pulse corners.

Output Resistor ($R_1$):

Exactly 1.00\,k\Omega with 1\% tolerance, rated for at least 0.5\,W to survive accidental short-circuits to chassis or ground.

Synchronous Peak Sampling vs. Analog Peak Detector:

Method A (Digital Timer Triggered): The MCU timer generating the PWM triggers the ADC directly at the 50\% center point of the positive HIGH phase and the 50\% center point of the negative LOW phase. This eliminates analog diode drops and capacitor discharge droop.

Method B (Hardware Peak Detector): A high-speed diode, low-leakage capacitor (10\,nF C0G/NPO), and bleed resistor (1\,M\Omega) holding the DC peak voltage for slow MCU ADC polling.

1.2 Proximity Pilot (PP) Cable Detection Circuit

+3.3V Analog Vref │ [ R_pu: 330 Ω, 0.1% ] │ PP Terminal ───────────┼──────────────────[ R_filter: 1.0 kΩ ]───┬────► MCU ADC (PP Channel) (Type 2 Socket) │ │ ──┴── ──┴── ▲ ▼ TVS Diode ──┬── C_filter: 100 nF (Noise rejection) ──┬── (ESD5V0) │ │ GND GND

Voltage Divider Equations at the MCU ADC Pin:

With V_{REF = 3.30\,V and R_{PU = 330\,\Omega:

V_{ADC} = 3.30\,\text{V} \times \frac{R_c}{330\,\Omega + R_c}

13 A Cable ($R_c = 1500\,\Omega$): V_{ADC = 3.30 × \frac{1500{330 + 1500 = \mathbf{2.70\,V

20 A Cable ($R_c = 680\,\Omega$): V_{ADC = 3.30 × \frac{680{330 + 680 = \mathbf{2.22\,V

32 A Cable ($R_c = 220\,\Omega$): V_{ADC = 3.30 × \frac{220{330 + 220 = \mathbf{1.32\,V

63 A Cable ($R_c = 100\,\Omega$): V_{ADC = 3.30 × \frac{100{330 + 100 = \mathbf{0.77\,V

Cable Unplugged ($R_c = \infty$): V_{ADC = \mathbf{3.30\,V

1.3 Contactor Coil Driver with High-Efficiency Economizer

+12V Aux Rail │ ┌─────┴─────┐ │ Contactor │ │ Coil (12V)│ └─────┬─────┘ │ ├───────────────────┐ │ │ │ ┌─┴─┐ │ │ │ Freewheeling Fast Diode (ES1J: 1A, 600V) │ └─┬─┘ │ │ │ ──┴── Zener / TVS Clamp (18V, SMAJ18A) │ ──┬── (Allows rapid magnetic field collapse) │ │ ├───────────────────┘ │ ──┴── Drain MCU PWM ──[Rg]─┤ NMOS (AO3400 / Si2302: 30V, 5.8A, Rds = 28mΩ) ──┬── Source │ GND

Why the TVS Clamp in Series with the Freewheeling Diode is Critical:

A standard freewheeling diode recirculates coil current for a prolonged time (> 50--100\,ms), delaying contact opening.

During a high-current short-circuit or 6\,mA DC residual fault, contacts must open in $< 10\,\text{ms}$ to extinguish the arc before catastrophic damage.

Placing an 18\,V TVS clamp in series with the diode forces the coil energy to dissipate across a higher voltage drop (V_{clamp = 18\,V + 0.7\,V = 18.7\,V), collapsing the magnetic field and snapping the contactor open in $< 8.5\,\text{ms}$.

1.4 Motorized Cable Lock H-Bridge Driver

+12V Supply Rail │ ┌─────┴─────┐ │ DRV8871 │ (Integrated 3.6A H-Bridge with current limiting) │ H-Bridge │ MCU Lock GPIO ────►│ IN1 │ MCU Unlock GPIO ──►│ IN2 OUT1├───────[ Connector Lock DC Motor ] │ OUT2├───────┘ │ ILIM │ └─────┬─────┘ │ [R_ilim: 24 kΩ] (Clamps peak stall current to 1.5A) │ GND

When the plug is fully inserted, the MCU asserts IN1=HIGH, IN2=LOW for 300\,ms to drive the locking pin forward.

If the pin stalls against an obstruction, the internal current limit trips safely without blowing fuses or overheating the gearmotor.

Position microswitches on the actuator provide direct digital confirmation to MCU GPIO inputs.

2. Galvanic Isolation, Creepage & Clearance (IEC 60664-1 / IEC 61851-1)

To ensure user safety when handling charging plugs in wet outdoor conditions, strict physical isolation boundaries must be enforced across the printed circuit board:

▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ ▒ HIGH VOLTAGE DOMAIN (230V / 400V AC Grid) ▒ ▒ L1, L2, L3, Neutral, Contactor High-Power Terminals ▒ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ │ │ │◄─────────────── REINFORCED ISOLATION BARRIER (>= 6.3 to 8.0 mm) ─────────────►│ │ │ │ [Optical Isolation] [Magnetic Isolation] [Opto-Gate Drivers] │ │ PC817 / FOD817 Pulse Transformers Silicon Labs Si823x │ │ │ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒ ▒ SELV CONTROLLER DOMAIN (+12V, +5V, +3.3V, -12V) ▒ ▒ MCU, IoT SoM, CP / PP Pins, RFID Reader, User Touchpoints ▒ ▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒▒

2.1 Isolation Design Parameters

| Parameter | Standard Rule | Value for 400\,V AC RMS Grid | Design Implementation |
| --- | --- | --- | --- |
| Clearance | Line-of-sight distance through air | ≥ 4.0--5.5\,mm | Physical component spacing and routing keep-out zones. |
| Creepage Distance | Surface path distance over PCB FR4 | ≥ 6.3--8.0\,mm (Pollution Degree 2, Material Group IIIa) | Routed isolation slots (air routing cutouts) under optocouplers. |
| Dielectric Withstand | Hi-Pot electrical test voltage | 3.75\,kV AC for 60\,seconds | Factory 100% production line isolation test. |
| Touch Current | Maximum allowable leakage to ground | < 0.25\,mA | High-impedance isolation between SELV common and chassis. |

3. EMC, Surge & Transient Protection (IEC 61000-4-x)

EVSEs are connected permanently to the building's electrical service entrance and are exposed to external lightning strikes, inductive motor switching spikes, and electrostatic discharge from human users.

AC Mains Line In (L1..L3) │ ├───[ Gas Discharge Tube (GDT: 600V, 10kA) ]───┐ │ │ ├───[ Metal Oxide Varistor (MOV: 14D471K) ]────┼───► Earth Ground (PE) │ │ ├───[ Differential Common-Mode Choke ]─────────┘ │ └───► Main Contactor & Internal SMPS

3.1 Transient Protection Matrix

| Threat Category | Applied Standard | Test Severity Level | Hardware Protection Solution |
| --- | --- | --- | --- |
| Lightning Surge | IEC 61000-4-5 | ± 4.0\,kV (Common Mode), ± 2.0\,kV (Diff) | Coordinated GDT + MOV hybrid network at AC mains inlet. |
| Electrical Fast Transient (EFT / Burst) | IEC 61000-4-4 | ± 2.0\,kV @ 5\,kHz on power, ± 1.0\,kV on signal | Common-mode chokes and RC snubber networks on relay lines. |
| Electrostatic Discharge (ESD) | IEC 61000-4-2 | ± 8\,kV Contact, ± 15\,kV Air Discharge | Low-capacitance TVS arrays (< 3\,pF) on CP, PP, and RFID. |
| Conducted RF Immunity | IEC 61000-4-6 | 10\,V RMS (150 kHz to 80 MHz) | Pi-filter (C-L-C) on all DC supply rails. |

4. Hardware Bring-Up & Test Procedure

Follow this disciplined bench bring-up procedure when powering a new revision of the Low Voltage Controller Board for the first time:

Phase 1: Cold Impedance Verification (Unpowered DMM checks) │ ▼ Phase 2: Auxiliary Power Rail Ramp-Up (+12V, +5V, +3.3V, -12V checks) │ ▼ Phase 3: Control Pilot Signal Integrity (12V DC State A, 1 kHz PWM shape) │ ▼ Phase 4: Simulated Vehicle Handshake (States A -> B -> C -> D) │ ▼ Phase 5: Contactor Pull-in & Economizer PWM Verification │ ▼ Phase 6: 6mA DC / 30mA AC Fault Trip Injection Calibration │ ▼ Phase 7: IoT Gateway Boot & OCPP Central System Handshake

Step-by-Step Bring-Up Instructions:

Cold Board Impedance Check:

Measure resistance to Ground on all primary rails (+12\,V, +5\,V, +3.3\,V, -12\,V). Ensure no rail exhibits < 50\,\Omega impedance.

DC Voltage Rail Verification:

Power board with current-limited bench supply (12.0\,V, 500\,mA).

Verify +5.0\,V ± 2\%, +3.3\,V ± 1\%, and verify that the inverting charge pump produces -12.0\,V ± 5\%.

Control Pilot Waveform Verification (Oscilloscope):

In State A (unconnected), probe CP with high-impedance 10X probe. Verify steady +12.0\,V ± 0.4\,V DC.

Attach EVSE test simulator with R_2 = 2.74\,k\Omega. Verify signal drops to +9.0\,V and immediately starts outputting 1.000\,kHz ± 5\,Hz square wave.

Verify negative peak drops to $-12.0\,\text{V} \pm 0.5\,\text{V}$ (confirming diode check circuit operates).

State C Transition & Contactor Actuation:

Switch in parallel resistor R_3 = 1.3\,k\Omega on simulator.

Verify CP positive peak drops to +6.0\,V ± 0.3\,V.

Probe contactor gate. Verify gate driver delivers 100\% duty cycle for 100\,ms, followed by clean transition to 35--40\% economizer PWM at 25\,kHz.

RCM Trip Injection:

Inject calibrated 6.0\,mA DC fault using precision current calibrator.

Verify comparator trips in < 10.0\,s and de-asserts contactor gate immediately.

5. Bench RCA Troubleshooting Matrix

| Symptom / Anomaly | Root Cause | Underlying Electrical Physics | Corrective Bench Action |
| --- | --- | --- | --- |
| CP voltage stuck at +12V; EV not detected | Defective R_1, blown op-amp, or open CP line | Op-amp output disconnected from physical terminal. | 1. Measure resistance of R_1 (1.0\,k\Omega).<br>2. Check op-amp supply rails (+12\,V and -12\,V). |
| Negative CP voltage reads 0V instead of -12V | Negative charge pump failed; V_{EE pin at 0\,V | Op-amp cannot pull below its negative supply rail. | Verify -12\,V charge pump output; replace charge pump IC or flyback diode. |
| Rounded CP edges; frequency drift $> 20\,\text{Hz}$ | Op-amp slew rate too low or excessive cable capacitance | RC filter formed by op-amp output impedance and cable capacitance. | Upgrade to higher slew rate op-amp (> 10\,V/µs) or reduce output filtering capacitance. |
| Contactor buzzes / chatters continuously | Economizer PWM frequency too low or hold voltage too weak | Hold voltage drops below contactor dropout threshold (V_{drop \approx 4.0\,V). | Increase economizer PWM duty cycle from 35\% to 45--50\%, or raise PWM frequency to > 20\,kHz. |
| Nuisance 6mA DC RCM trips during vehicle plug-in | Ground bounce or high inrush current through common ground | Fluxgate sensor saturated by unbalanced capacitive inrush. | Add 20--50\,ms software trip debounce or improve ground plane partitioning. |
| PP voltage reads 3.3V despite 32A cable attached | Broken PP pin or missing R_c inside cable plug | Resistor divider floating without bottom leg resistor. | Inspect socket PP pin mechanical contact tension; measure R_{PP with DMM (220\,\Omega expected). |
| OCPP WebSocket connection rejected (`401 Unauthorized`) | Wrong chargePointId or invalid TLS client certificate | CSMS rejects registration due to authorization failure. | Check BootNotification payload JSON and verify provisioned X.509 client certificate expiration. |