EVSE IoT & Communication Subsystem: Gateway Architecture, OCPP 1.6J/2.0.1 & Cloud Connectivity

This document provides a comprehensive technical guide to the IoT Gateway, Networking Interfaces, Cloud Protocols, and Edge Communication of the EVSE Low Voltage Controller Board. It details the dual-processor safety topology, wireless/wired network interfaces, OCPP 1.6-J and 2.0.1 message flows over Secure WebSockets, Dynamic Load Management (DLM) over RS-485 Modbus, and secure dual-bank Over-the-Air (OTA) firmware updates.

1. Dual-Processor System Topology (Safety vs. Connectivity)

Industrial and commercial EVSE controller boards split operations between two specialized computing domains:

┌────────────────────────────────────────────────────────┐ ┌────────────────────────────────────────────────────────┐ │ REAL-TIME SAFETY MICROCONTROLLER (MCU) │ │ IoT APPLICATION PROCESSOR / SoM │ │ (e.g., STM32G4 / STM32F4 / LPC5500) │ │ (e.g., ESP32-S3 / NXP i.MX6ULL / i.MX8M) │ │ │ │ │ │ • IEC 61851-1 Control Pilot PWM generation (1 kHz) │ │ • Embedded Linux / FreeRTOS Network Stack │ │ • Diode check & ADC Pilot Voltage Peak Sampling │ │ • Wi-Fi 802.11 b/g/n & BLE 5.0 Stack │ │ • Proximity Pilot (PP) cable resistor decoding │ │ • 4G LTE Cat-1 / NB-IoT PPP Cellular Engine │ │ • 6mA DC / 30mA AC RCM hardware trip loop (<10 ms) │ │ • 10/100 Mbps Ethernet MAC/PHY │ │ • Main Contactor PWM Economizer Drive │ │ • OCPP 1.6-J / OCPP 2.0.1 JSON-over-WebSockets Engine │ │ • Motorized Connector Lock H-Bridge Control │ │ • TLS 1.3 / X.509 Cryptographic Security Layer │ │ • Precision Energy Metering DSP Sampling │ │ • Local Web Server & Bluetooth BLE Commissioning App │ │ • Hardware Watchdog & Fail-Safe E-Stop Loop │ │ • Dual-Bank A/B Secure OTA Firmware Manager │ └───────────────────────────┬────────────────────────────┘ └───────────────────────────┬────────────────────────────┘ │ │ └────────────── High-Speed Isolated SPI / UART ────────────────┘ (Protected CRC-16 Command Framing)

Why a Dual-Processor Architecture is Mandatory:

Safety Isolation (IEC 61851-1): Fault trips (such as 6\,mA DC residual current or pilot disconnect) require contactor de-energization within $< 10\text{--}40\,\text{ms}$. A general-purpose OS (such as Linux) or a Wi-Fi/cellular stack undergoing TCP retransmissions or garbage collection can introduce non-deterministic latencies exceeding hundreds of milliseconds.

Cybersecurity Containment: If the IoT network interface is subjected to an external DDoS attack, buffer overflow, or unauthorized network intrusion, the real-time MCU continues to run its hardware safety state machine independently, preventing hazardous electrical conditions.

Low-Power Standby: During idle nighttime hours, the high-power application processor can enter deep sleep, leaving the low-power MCU consuming < 10\,mW while monitoring for a vehicle plug-in event.

2. Multi-Bearer Physical Connectivity Subsystem

To guarantee seamless connectivity in diverse deployment environments (residential garages, underground parking basements, highway charging hubs), the controller board incorporates four primary network bearers:

COMMUNICATIONS INTERFACE MATRIX ┌───────────────────────┐ ┌───────────────────────┐ ┌───────────────────────┐ ┌───────────────────────┐ │ Wi-Fi & BLE │ │ 4G LTE Cellular │ │ Wired Ethernet │ │ RS-485 (Modbus) │ │ • 802.11 b/g/n (2.4G) │ │ • Cat-1 / NB-IoT │ │ • 10/100 Mbps RJ45 │ │ • Half-Duplex Balanced│ │ • BLE 5.0 Provisioning│ │ • Nano-SIM / eSIM │ │ • RMII PHY + Magnetics│ │ • Dynamic Load Balance│ └───────────┬───────────┘ └───────────┬───────────┘ └───────────┬───────────┘ └───────────┬───────────┘ │ │ │ │ ▼ ▼ ▼ ▼ ┌────────────────────────────────────────────────────────────────────────────────────────────────────────┐ │ IoT APPLICATION PROCESSOR NETWORK ROUTING LAYER │ └────────────────────────────────────────────────────────────────────────────────────────────────────────┘

2.1 Wi-Fi & Bluetooth Low Energy (BLE 5.0)

Wi-Fi Station Mode (STA): Connects to the site's local 2.4\,GHz WLAN for internet access and OCPP transport.

Wi-Fi Soft-AP Mode: Broadcasts a temporary setup SSID (e.g., VVDN-EVSE-Setup-9A4F) for initial unboxing configuration without physical access.

BLE 5.0 Commissioning GATT Service: Allows an electrician or installer to connect via a mobile app to:

Configure grid parameters (Maximum panel current limit, Single vs. Three-phase).

Push local Wi-Fi SSID and WPA2/WPA3 credentials.

Set the Central System URL (wss://ocpp.operator.com/ocpp16/CP1234).

Read active hardware diagnostic logs and test contactor actuation.

2.2 4G LTE Cellular Modem (Cat-1 / NB-IoT / LTE-M)

Modem Hardware: Quectel EC200U / BG95, SIMCom SIM7600, or Telit ME910.

Interfacing: Connected to the application processor via high-speed UART (921,600\,bps) or USB 2.0 High-Speed.

Software Integration: Uses Point-to-Point Protocol (PPP) daemon or internal TCP/IP stack driven via 3GPP AT commands.

Fallback Hierarchy: Automatic failover from Ethernet \to Wi-Fi \to Cellular to maintain constant billing connectivity.

2.3 Wired Industrial Ethernet (10/100 Mbps)

Hardware: Dedicated RMII PHY (e.g., Microchip LAN8720A or KSZ8081) coupled to an integrated RJ45 jack with built-in isolation magnetics (1.5\,kV dielectric isolation).

Ideal for commercial fleets, multi-story carparks with no cellular reception, and heavy EMI industrial environments.

2.4 RS-485 Modbus RTU (Dynamic Load Management)

Physical Layer: Half-duplex differential RS-485 transceiver (e.g., TI SN65HVD72 or MAX485) with TVS surge protection.

Purpose: Communicates with external energy meters (e.g., Eastron SDM630, Schneider iEM3000) installed at the building's main electrical service panel.

Dynamic Load Balancing (DLB): If total household or building power exceeds the utility supply threshold (e.g., 63\,A), the controller polls the smart meter via Modbus RTU every 1.0\,second and throttles the EVSE charging current proportionally to prevent tripping the main utility fuse.

Solar Diversion (Zero Grid Export): Reads rooftop solar inverter generation and adjusts charging current so the vehicle charges exclusively on surplus PV power.

3. OCPP (Open Charge Point Protocol) Architecture

OCPP is the global de facto open standard for EVSE-to-Cloud communication managed by the Open Charge Alliance (OCA). The LV controller board implements both OCPP 1.6-J (JSON) and OCPP 2.0.1.

3.1 Transport & Security Layer

Transport: JSON framing over WebSockets (ws:// unencrypted or wss:// encrypted).

Security Profile (OCPP Security Whitepaper):

Security Profile 1: Unencrypted HTTP/WebSocket with Basic HTTP Authentication (Username/Password).

Security Profile 2: TLS 1.2 / TLS 1.3 encryption with HTTP Basic Authentication.

Security Profile 3 (Enterprise): Mutual TLS (mTLS) with client and server X.509 cryptographic certificates stored in an onboard secure element (e.g., ATECC608A).

3.2 Canonical OCPP Message Framing

OCPP messages over WebSockets are formatted as JSON arrays with four fundamental message types:

CALL (`MessageTypeId = 2`): [2, "<UniqueMessageId>", "<ActionName>", {<PayloadJson>}]

CALLRESULT (`MessageTypeId = 3`): [3, "<UniqueMessageId>", {<PayloadJson>}]

CALLERROR (`MessageTypeId = 4`): [4, "<UniqueMessageId>", "<ErrorCode>", "<ErrorDescription>", {<Details>}]

3.3 End-to-End Charging Session Message Lifecycle

The sequence below illustrates a complete user-authenticated charging transaction:

VEHICLE (OBC) EVSE CONTROLLER BOARD CLOUD CSMS (BACKEND) │ │ │ │ ├───── [2, "msg01", "BootNotification"] ─►│ │ │◄──── [3, "msg01", {status: Accepted}] ──┤ │ │ │ │ ├───── [2, "msg02", "StatusNotification", │ │ │ {status: "Available"}] ──────────►│ │ │◄──── [3, "msg02", {}] ──────────────────┤ │ │ │ │ User taps RFID Card │ │ │───────────────────────────►│ │ │ ├───── [2, "msg03", "Authorize", │ │ │ {idTag: "B4A198F0"}] ────────────►│ │ │◄──── [3, "msg03", {status: Accepted}] ──┤ │ │ │ │ Cable plugged into vehicle │ │ │ (CP transitions to State B)│ │ │ Motorized lock engages │ │ │ Vehicle asserts State C │ │ │ (Charging requested) │ │ │ Contactor snaps CLOSED │ │ │ ├───── [2, "msg04", "StartTransaction", │ │ │ {connectorId: 1, idTag: "...", │ │ │ meterStart: 12450, ...}] ───────►│ │ │◄──── [3, "msg04", {transactionId: 402}]─┤ │ │ │ │◄════ POWER TRANSFER ══════►│ │ │ (32A Active) │ │ │ │ (Every 30s Periodic Meter Stream) │ │ ├───── [2, "msg05", "MeterValues", │ │ │ {transactionId: 402, values: [ │ │ │ {Power: 21950, Current: 32.0, │ │ │ Energy: 13120}]}] ─────────────►│ │ │◄──── [3, "msg05", {}] ──────────────────┤ │ │ │ │ Vehicle completes charge │ │ │ (CP transitions to State B)│ │ │ Contactor opens │ │ │ Lock disengages │ │ │ ├───── [2, "msg06", "StopTransaction", │ │ │ {transactionId: 402, │ │ │ meterStop: 14820, │ │ │ reason: "EVDisconnected"}] ─────►│ │ │◄──── [3, "msg06", {}] ──────────────────┤ │ │ │ │ ├───── [2, "msg07", "StatusNotification", │ │ │ {status: "Available"}] ──────────►│ │ │◄──── [3, "msg07", {}] ──────────────────┤

3.4 Key OCPP Actions & Payload Anatomy

1. BootNotification

Sent on initial power-up or reboot to register station identity with the cloud:

[2, "10001", "BootNotification", { "chargePointVendor": "VVDN Technologies", "chargePointModel": "VVDN-AC-22KW-PRO", "chargePointSerialNumber": "VVDN-EVSE-2026-0042", "firmwareVersion": "v2.4.12-secure", "iccid": "89014103211118510720", "imsi": "310410123456789", "meterType": "ADE7953-MID-Class1" }]

2. MeterValues

Streams real-time power, energy, voltage, and SoC telemetry for billing and smart charging algorithms:

[2, "10005", "MeterValues", { "connectorId": 1, "transactionId": 402, "meterValue": [{ "timestamp": "2026-09-25T12:55:00Z", "sampledValue": [ {"value": "21950", "context": "Sample.Periodic", "measurand": "Power.Active.Import", "unit": "W"}, {"value": "32.1", "context": "Sample.Periodic", "measurand": "Current.Import", "unit": "A", "phase": "L1"}, {"value": "31.9", "context": "Sample.Periodic", "measurand": "Current.Import", "unit": "A", "phase": "L2"}, {"value": "32.0", "context": "Sample.Periodic", "measurand": "Current.Import", "unit": "A", "phase": "L3"}, {"value": "230.4", "context": "Sample.Periodic", "measurand": "Voltage", "unit": "V", "phase": "L1-N"}, {"value": "13120", "context": "Sample.Periodic", "measurand": "Energy.Active.Import.Register", "unit": "Wh"} ] }] }]

4. User Authentication: RFID / NFC Subsystem

The controller board features an on-board or external 13.56 MHz RFID / NFC Transceiver (e.g., NXP PN532, MFRC522, or STMicroelectronics ST25R3916):

Supported Standards: ISO/IEC 14443 Type A/B, MIFARE Classic/Ultralight, MIFARE DESFire EV1/EV2/EV3, and ISO 15693.

Host Interface: Interfaced to the controller over SPI or UART.

Card Tap Flow:

Card presence detected via low-power inductive card detection (LPCD) polling every 200\,ms.

Transceiver reads the card's 4-byte or 7-byte Unique Identifier (UID / CSN).

Controller formats the hex UID as an OCPP idTag (e.g. 04A2B3C4D5E6F7).

Controller queries the CSMS via Authorize(idTag) or checks the local offline whitelist (Local Authorization List).

If valid, an audible confirmation beep sounds, the RGB LED changes to Cyan, and the session begins.

5. High-Level Communication: ISO 15118 & Plug & Charge (PnC)

For next-generation smart charging and DC fast chargers, the simple analog PWM pilot is augmented with digital communication via ISO 15118:

┌────────────────────────────────────────────────────────┐ │ ISO 15118 GREENPHY PLC MODEM │ │ (Qualcomm QCA7000 / QCA7005) │ └───────────────────────────┬────────────────────────────┘ │ (Tx/Rx HF Signals 2-30 MHz) ▼ ┌────────────────────────────────────────────────────────┐ │ Passive High-Pass Analog Matching Network │ │ (Blocking 1 kHz PWM & ±12V DC) │ └───────────────────────────┬────────────────────────────┘ │ ▼ ─────────────────────────[ CP Wire ]───────────────────── (Coupled to Control Pilot)

HomePlug GreenPHY PLC: Modulates digital IP traffic (2–30 MHz OFDM) directly onto the existing Control Pilot wire.

Plug & Charge (PnC):

Eliminates RFID cards and smartphone apps.

The vehicle and EVSE establish an automated TLS 1.3 cryptographic session.

The vehicle transmits its digitally signed contract certificate.

The EVSE validates the certificate with the mobility operator clearing house and automatically begins billing.

Smart Grid V2G (Vehicle-to-Grid): Coordinates dynamic bidirectional power flow, allowing the EV battery to feed energy back into the grid during peak demand.

6. Secure Dual-Bank Over-The-Air (OTA) Firmware Updates

To prevent field bricking during firmware upgrades across thousands of deployed chargers, the controller board implements a robust A/B Dual-Bank Partitioning Architecture:

Internal / External SPI Flash Memory (32 MB / 64 MB) ┌───────────────────────┬───────────────────────┬───────────────────────┬───────────────────────┐ │ Secure Bootloader │ Bank A (Active) │ Bank B (Candidate) │ Non-Volatile Storage│ │ (Immutable ROM) │ Firmware v2.4.12 │ New Firmware v2.5.0 │ • Meter Calibration │ │ • ECDSA Signature │ • Verified OK │ • Pending Validation│ • OCPP Offline Cache│ │ • Rollback Watchdog │ • Running Now │ • Written via OTA │ • Certs & Keys │ └───────────────────────┴───────────────────────┴───────────────────────┴───────────────────────┘

OTA Update State Machine:

Download Phase: The CSMS issues an OCPP UpdateFirmware(location: "https://...") command. The controller downloads the cryptographically signed binary image to the inactive Bank (Bank B).

Signature Verification: The bootloader checks the image against the manufacturer's public key using ECDSA SHA-256. If corrupted or tampered with, the update is rejected.

Execution & Trial Boot: The bootloader sets the boot vector to Bank B and starts a hardware watchdog timer (180\,seconds).

Self-Test & Confirmation: The new firmware must successfully:

Initialize the safety MCU and CP op-amp circuits.

Establish network connectivity (Wi-Fi/Cellular).

Reconnect to the CSMS and receive an accepted Heartbeat.

Commit or Rollback:

If self-tests pass within 180\,s, Bank B is permanently marked as Active.

If the firmware crashes, freezes, or fails to connect, the hardware watchdog resets the processor. The bootloader detects the failed boot counter and automatically reverts to Bank A, restoring full operational status.