NMOS LDO: Working Mechanism, Gate Drive & Architecture

1. Architectural Topology & Circuit Schematic

In an NMOS-based Low-Dropout Regulator, the series pass element is an N-channel enhancement-mode MOSFET configured in a Source-Follower (Common-Drain) topology.

+---------------------------+ ¦ Auxiliary High-Voltage ¦ ¦ Rail (V_BIAS = 3.3V/5V) ¦ +-------------┬-------------+ │ Unregulated Input (Vin = 1.05V) │ │ │ ├─── Drain (D) │ │ ┌───────────────┐ │ │ │ NMOS Pass │ │ └───┤ Transistor │ │ │ (Source-Follower) │ Gate (G) <───┤ (High gm) │ │ └───────┬───────┘ │ │ Source (S) │ ├─────────────────┼────────────┬───────────────> Regulated Vout (0.90V) │ │ │ │ │ [R1] Feedback │ │ │ Resistors ┌─────────────┴─────────────┐ │ ├─── (V_FB) │ High-Voltage Error Amp ├───┘ │ │ (Powered by V_BIAS) │ [R2] V_REF ────>│ (+) Non-inverting Input │ │ V_FB ────>│ (-) Inverting Input │ GND │ │ │ Output drives Gate HIGH ┼─── Gate Drive (V_GATE = 2.1V) └───────────────────────────┘

1.1 Pin and Terminal Polarities

Drain (D): Connected directly to the low-voltage power supply rail (V_{IN, e.g. 1.05\,V \dots 1.5\,V). The heavy load current flows through Drain to Source.

Source (S): Connected directly to the output voltage rail (V_{OUT, e.g. 0.8\,V \dots 1.2\,V).

Gate (G): Driven by the Error Amplifier. Must be biased to a voltage higher than both V_{OUT and V_{IN.

Auxiliary Bias Pin ($V_{BIAS}$): Provides the elevated headroom voltage (3.3\,V \dots 5.5\,V) required by the error amplifier to drive the NMOS gate above V_{IN.

2. Operating Mechanism & The Gate Headroom Challenge

2.1 Source-Follower Physics & Local Negative Feedback

The NMOS pass transistor operates in Common-Drain (Source-Follower) configuration:

The input to the transistor is applied to the Gate, and the output is taken from the Source.

Intrinsic Local Feedback: In this configuration, the effective gate-to-source overdrive is:

V_{GS} = V_{GATE} - V_{OUT}

Instantaneous Reaction: If a sudden load surge causes V_{OUT to droop by 50\,mV, V_{GS instantly increases by $50\,\text{mV}$ without waiting for the outer error amplifier loop to respond!

The increased V_{GS immediately forces the NMOS channel to conduct more current, actively clamping the output droop. This confers NMOS LDOs with vastly superior load transient response compared to PMOS regulators.

2.2 The Gate Headroom Constraint (V_{GATE > V_{IN)

To turn an NMOS transistor fully ON into its linear triode region:

V_{GATE} \ge V_{OUT} + V_{th} + V_{ov}

Where:

V_{th is the NMOS threshold voltage (typically 0.6\,V \dots 1.2\,V).

V_{ov is the overdrive voltage required for low R_{DS(on) (typically 0.3\,V \dots 0.6\,V).

The Fundamental Headroom Dilemma:

Consider an embedded core supply requiring V_{OUT = 0.90\,V from a V_{IN = 1.05\,V input rail (headroom = 150\,mV):

V_{GATE} = V_{OUT} + V_{GS} = 0.90\,\text{V} + 1.20\,\text{V} = 2.10\,\text{V}

Notice that the required Gate voltage (2.10\,V) is more than $1.0\,\text{V}$ higher than the input supply rail ($1.05\,\text{V}$):

V_{GATE} (2.10\,\text{V}) > V_{IN} (1.05\,\text{V})

[CAUTION] If an NMOS LDO attempts to operate from a single supply rail without an elevated voltage source, the maximum gate voltage cannot exceed V_{IN. The resulting dropout voltage would be: V_{DO} = V_{IN} - V_{OUT} \ge V_{GS} \approx 1.0\,\text{V} \dots 1.5\,\text{V} This completely destroys the "low-dropout" capability of the regulator!

3. Architectural Solutions to the Gate Headroom Constraint

Modern IC designers utilize two distinct architectural approaches to supply the elevated gate voltage:

Dual-Supply Architecture (V_BIAS Pin) Single-Supply with Internal Charge Pump ┌────────────────────────────────────┐ ┌────────────────────────────────────┐ │ LDO IC │ │ LDO IC │ │ Vin (1.05V) ───────> Drain │ │ Vin (1.05V) ──┬────> Drain │ │ │ │ │ │ │ V_BIAS (3.3V) ────> Error Amp VDD │ │ ▼ │ │ │ │ │ [Charge Pump] │ │ ▼ Gate Drive │ │ │ │ │ (2.1V) │ │ ▼ V_BOOST (2.5V) │ │ │ │ [Error Amp VDD] │ └────────────────────────────────────┘ └────────────────────────────────────┘

3.1 Dual-Supply Architecture (V_{BIAS Pin)

High-Current Power Path ($V_{IN}$): Connects to a low-voltage, high-current switching rail (e.g. 1.05\,V \dots 1.5\,V from an efficient intermediate buck converter).

Control Path ($V_{BIAS}$): Connects to an available system auxiliary rail (typically 3.3\,V or 5.0\,V). The V_{BIAS pin draws only a tiny quiescent current (1\,mA \dots 5\,mA) to power the internal reference and error amplifier.

Engineering Advantages:

Achieves the world's lowest dropout voltages: $30\,\text{mV} \dots 60\,\text{mV}$ at $2.0\,\text{A} \dots 4.0\,\text{A}$.

Extremely high overall efficiency:

\eta \approx \frac{V_{OUT} \times I_{LOAD}}{V_{IN} \times I_{LOAD}} = \frac{0.90\,\text{V}}{1.05\,\text{V}} \approx 85.7\%

Pristine, noise-free DC gate drive with zero switching ripple.

3.2 Single-Supply Architecture with Internal Charge Pump

If an auxiliary V_{BIAS supply is not available on the PCB, the LDO incorporates an on-chip switched-capacitor charge pump (voltage doubler):

Operates at high frequency (1\,MHz \dots 5\,MHz) driven by an internal ring oscillator.

Generates an internal boosted voltage:

V_{BOOST} \approx 2 \times V_{IN} - V_{diode}

Powers the error amplifier, allowing single-supply operation down to V_{IN = 0.8\,V.

Design Trade-Off:

The charge pump introduces a slight quiescent current overhead (1\,mA \dots 3\,mA) and switching clock harmonics that require rigorous internal low-pass filtering.

4. Small-Signal Loop Dynamics & Inherent Wideband Stability

While PMOS LDOs struggle with two competing low-frequency poles, the NMOS Source-Follower architecture is inherently stable.

4.1 Ultra-Low Source-Follower Output Impedance (R_{OUT)

In a Source-Follower, the small-signal output resistance looking into the Source terminal is governed by the transistor's transconductance (g_m):

R_{OUT} = \frac{1}{g_m} \parallel r_{ds} \parallel R_{LOAD} \approx \frac{1}{g_m}

For a power NMOS pass element delivering 1\,A \dots 4\,A, g_m is large (typically 2\,S \dots 10\,S):

R_{OUT} \approx \frac{1}{2\,\text{S} \dots 10\,\text{S}} = 0.10\,\Omega \dots 0.50\,\Omega

4.2 Pushing the Output Pole Out of Band

Because R_{OUT is tens to hundreds of times smaller than a PMOS Common-Source output impedance, the Output Pole ($p_{out}$) is pushed to extremely high frequencies:

p_{out} = \frac{1}{2\pi \cdot R_{OUT} \cdot C_{OUT}} = \frac{g_m}{2\pi \cdot C_{OUT}}

Worked Numerical Example:

With g_m = 5\,S and C_{OUT = 10\,µF:

p_{out} = \frac{5}{2\pi \times 10 \times 10^{-6}} \approx 79.6\,\text{kHz}

At light load or with smaller capacitors, p_{out pushes past 500\,kHz \dots 2\,MHz!

Loop Gain (dB) 60 ¦──────┐ (Internal Dominant Pole: p_int) ¦ \ -20 dB/dec 40 ¦ \ ¦ \ 20 ¦ \ ¦ \ 0 ¦────────────\─────────────────────────── 0 dB Crossover (UGBW ~ 100 kHz) ¦ \ -20 ¦ \────────┐ (Output Pole: p_out sits well OUTSIDE the loop bandwidth!) +─────────────+──────────+──────────> Frequency p_int p_out

4.3 Immunity to the "ESR Stability Tunnel"

Because the loop contains only one dominant low-frequency pole ($p_{int}$) within the unity-gain bandwidth, the loop gain rolls off at a stable -20\,dB/decade slope.

Total phase lag is only -90^\circ, providing an inherent Phase Margin of $+70^\circ \dots +90^\circ$!

Zero-ESR MLCC Ceramic Stability: NMOS LDOs do not require an ESR zero to achieve stability. They are unconditionally stable with pure, ultra-low ESR multi-layer ceramic capacitors (R_{ESR = 1\,m\Omega \dots 50\,m\Omega).

5. Body Effect & Triple-Well CMOS Fabrication

In standard bulk CMOS processes, the P-type substrate is tied to the most negative potential in the circuit (Ground).

In an NMOS pass transistor, the Source terminal sits at the output voltage (V_{OUT = 0.9\,V \dots 1.8\,V).

This creates a reverse source-to-body bias:

V_{SB} = V_{OUT} - 0\,\text{V} > 0\,\text{V}

NMOS Pass Transistor Drain (Vin) Source (Vout) │ │ ┌┴┐ ┌┴┐ ┌┘ └───────────┘ └┐ N+ │ │ N+ ────────┴─────────────────┴──────── Isolated P-Well (Body) ──┐ ─────────────────────────────────── │ Tie Body directly to Source! Deep N-Well │ (V_SB = 0V: Eliminates Body Effect!) ─────────────────────────────────── │ P-Substrate (GND) │ ─────────────────────────────────── │ ▲

5.1 The Body Effect Problem

When V_{SB > 0, the depletion region widens, increasing the threshold voltage:

V_{th} = V_{th0} + \gamma \left( \sqrt{2\phi_F + V_{SB}} - \sqrt{2\phi_F} \right)

An increase of \Delta V_{th \approx 200\,mV \dots 400\,mV demands an even higher gate drive voltage to achieve low on-resistance.

5.2 The Triple-Well Silicon Solution

High-performance NMOS LDO ICs are fabricated on Triple-Well CMOS processes:

The NMOS pass transistor is placed inside an isolated P-well enclosed by a Deep N-well barrier.

This allows the transistor's P-well Body to be tied directly to its Source ($V_{OUT}$).

Because V_{SB = 0\,V, the body effect is completely eliminated, minimizing threshold voltage and maximizing channel transconductance.

6. Real-World NMOS LDO ICs & Application Schematics

Texas Instruments TPS7A84 (NMOS High-Current LDO) 3A, Dual-Rail, 100mV Dropout ┌───────────────────────────┐ Vin (1.1V - 1.4V) ──┤ IN OUT ├─────┬───────> Clean Vout (0.9V, 3A) │ │ │ ─┴─ │ SENSE├─────┤ ─┴─ Output MLCC C_IN (10uF) ─┬─ │ │ │ ─┬─ (47uF, X7R, 6.3V) GND │ BIAS GND ├─┐ │ GND V_BIAS (3.3V) ──────┤ │ │ │ │ EN │ │ │ ─┴─ │ │ │ │ C_BIAS (1uF) ─┬─ │ SS/NR ANY ├─┤ │ GND └────┬───────────────────┬──┘ │ │ │ │ │ │ ─┴─ C_SS (10nF) GND GND │ ─┬─ (Soft-Start) │ GND │ ┌─────────────────────────────────┴┐ │ ANY-OUT Pin Strapping Network │ │ (Configures precise Vout without │ │ external resistor dividers) │ └──────────────────────────────────┘

Representative Production NMOS LDOs:

TI `TPS7A84` / `TPS7A85`: High-current 3.0\,A / 4.0\,A dual-rail NMOS LDO with 100\,µV dropout at 3\,A, 4.4\,µV_{RMS ultra-low noise, designed for FPGA SerDes and ASIC core rails.

TI `TPS7A54`: 4.0\,A single-supply NMOS regulator with an internal charge pump, allowing single-rail operation down to 0.8\,V without an external V_{BIAS pin.

ADI `ADP1740`: 2.0\,A low-dropout NMOS regulator operating from input rails down to 1.6\,V.