MOSFET (Metal-Oxide-Semiconductor Field-Effect Transistor): Comprehensive Engineering & Study Guide

Welcome to the VVDN Engineering Hub MOSFET Knowledge Base. This repository serves as an exhaustive, mathematically rigorous, and practical hardware engineering reference for studying, selecting, designing, and troubleshooting MOSFETs across small-signal, power switching, automotive, high-frequency RF, and nanoscale VLSI applications.

1. Executive Summary & Physics Foundations

A MOSFET (Metal-Oxide-Semiconductor Field-Effect Transistor) is a four-terminal (Drain, Source, Gate, Body/Substrate), voltage-controlled semiconductor device. Unlike current-controlled Bipolar Junction Transistors (BJTs) where a base current I_B controls collector current I_C (I_C = \beta \cdot I_B), a MOSFET controls drain-to-source current (I_{DS) by modulating the electric field across an insulating dielectric gate oxide layer (SiO_2 or High-\kappa dielectric):

Gate Terminal (G) │ ┌────┴────┐ (Metal / Poly-Si Gate) │ GATE │ ┌─────┴─────────┴─────┐ │ Gate Oxide (SiO2) │ <-- Dielectric Insulator (tox ~ 5-100nm) ┌───────────────────┴─────────────────────┴───────────────────┐ │ Source (n+) Drain (n+) │ ───┤ ┌────────┐ Induced Channel [e-] ┌────────┐ ├─── (S) │ │ n+ │ ............................... │ n+ │ │ (D) └──┴────────┴───────────────────────────────────┴────────┴───┘ │ │ │ p-type Substrate (Body) │ │ │ └──────────────────────────────┬──────────────────────────────┘ │ Body Terminal (B) (Usually shorted to Source)

Core Semiconductor Working Mechanisms:

Electrostatic Gate Accumulation, Depletion, and Inversion:

Applying a voltage V_{GS establishes a perpendicular electric field across the gate insulator.

For an N-channel enhancement device, when V_{GS = 0\,V, two back-to-back p-n junctions (n^+-p and p-n^+) block current flow between Drain and Source (I_{DS \approx 0).

As V_{GS increases positively, mobile holes (h^+) are repelled from the semiconductor surface beneath the gate, forming a depletion region.

When V_{GS exceeds the Threshold Voltage ($V_{TH}$), the surface potential bends sufficiently such that minority electrons are attracted to the oxide interface, creating a conducting inversion layer (the n-channel) directly connecting the Source and Drain n^+ diffusions.

Channel Modulation by $V_{DS}$ (Linear vs. Saturation):

For low V_{DS, the channel behaves as an ohmic, resistive path whose conductance is modulated by the overdrive voltage (V_{GS - V_{TH).

As V_{DS increases, the potential difference across the gate oxide at the drain end drops to (V_{GS - V_{DS). When V_{DS ≥ V_{GS - V_{TH, the inversion layer thickness at the drain end collapses to zero—a phenomenon known as channel pinch-off.

Beyond pinch-off, the drain current saturates, entering the constant-current (saturation) regime.

2. Universal MOSFET Taxonomy & Classification Tree

MOSFETs are categorized along several fundamental engineering dimensions:

MOSFET Classification │ ┌───────────────────────────────────┴───────────────────────────────────┐ ▼ ▼ Enhancement Mode (Normally OFF) Depletion Mode (Normally ON) Channel formed only when ¦Vgs¦ > ¦Vth¦ Channel exists naturally at Vgs = 0V ├── N-Channel (NMOS: Vgs_th > 0) ├── N-Channel (Vgs_off < 0) └── P-Channel (PMOS: Vgs_th < 0) └── P-Channel (Vgs_off > 0) │ │ └───────────────────────────────────┬───────────────────────────────────┘ │ ┌────────────────────────────┼────────────────────────────┐ ▼ ▼ ▼ Power Topologies Specialized Drive / RF Advanced Multi-Gate ├── Trench-Gate (UMOS) ├── Logic-Level MOSFETs ├── FinFET (3D tri-gate) │ (Low Vds: 20V-100V) │ (Vgs_th: 1V-2V, direct MCU) ├── GAAFET (Nanosheets, <=3nm) ├── VDMOS (Planar Power) ├── LDMOS (Lateral Power RF) └── FD-SOI (Fully Depleted) │ (Med Vds: 100V-600V) │ (High fT, GHz base stations) ├── Superjunction (CoolMOS) └── Dual-Gate MOSFET │ (High Vds: 500V-900V) (Tetrode, AGC, RF Mixers) └── Wide-Bandgap (SiC) (Ultra Vds: 650V-3.3kV)

3. Directory Navigation & Study Modules

This repository is structured into dedicated, modular technical dossiers for every major MOSFET category:

| Directory | Core Focus | Voltage Range | Typical Application Domains |
| --- | --- | --- | --- |
| **`n-channel-enhancement/`** | Standard e-NMOS, electron conduction, low-side switching, conduction losses | 20\,V \dots 600\,V+ | Low-side switches, synchronous buck low-side, motor H-bridge |
| **`p-channel-enhancement/`** | Standard e-PMOS, hole mobility, high-side switching, reverse battery protection | 20\,V \dots 200\,V | High-side power distribution, reverse polarity protection, CMOS |
| **`depletion-mode/`** | Normally-ON d-MOSFET, channel pinch-off, negative gate cutoff | 50\,V \dots 800\,V | SMPS high-voltage startup circuits, constant-current sources, fail-safe disconnects |
| **`power-trench-mosfet/`** | Vertical trench gate, ultra-dense cell pitch, minimum R_{DS(on) | 20\,V \dots 100\,V | Automotive 12\,V/48\,V systems, BMS battery protection, PoL converters |
| **`power-vdmos/`** | Vertical double-diffused planar power, avalanche ruggedness (E_{AS) | 100\,V \dots 600\,V | Inductive solenoid drivers, industrial relays, linear power amplifiers |
| **`superjunction-mosfet/`** | Multi-epi charge balance, breaking the Silicon limit, CoolMOS physics | 500\,V \dots 900\,V | Offline AC-DC flyback/forward, PFC stages, server PSUs, EV Onboard Chargers |
| **`sic-mosfet/`** | Wide-bandgap 3.26\,eV SiC, zero reverse recovery, extreme temp | 650\,V \dots 1700\,V+ | EV main traction inverters, 800V DC fast chargers (EVSE), solar inverters |
| **`logic-level-mosfet/`** | Thin oxide low-V_{TH (1-2\,V), direct 3.3\,V/5\,V MCU GPIO drive | 20\,V \dots 60\,V | Microcontroller peripherals, LED dimming, relay drives, direct logic gates |
| **`ldmos-rf/`** | Lateral DMOS, high-frequency power, low C_{rss, ground flange | 28\,V \dots 100\,V | 4G/5G cellular base stations, RF broadcast, radar transmitters, ISM generators |
| **`finfet-and-gaafet/`** | 3D multi-gate, FinFET, GAA nanosheets, FD-SOI, short-channel mitigation | < 1.2\,V | Advanced VLSI processors, mobile SoCs, GPUs, AI tensor cores |
| **`dual-gate-mosfet/`** | Integrated series cascode tetrode, two control gates, low Miller cap | 10\,V \dots 25\,V | RF mixers, low-noise VHF/UHF amplifiers, automatic gain control (AGC) |

4. Fundamental Mathematical Model & Operating Regions

For an N-Channel Enhancement MOSFET with channel width W, channel length L, oxide capacitance per unit area C_{ox = \frac{\varepsilon_{ox{t_{ox, electron mobility µ_n, and threshold voltage V_{TH:

Parameter Grouping:

\mu_n C_{ox} = k_n' \quad [\text{Process transconductance parameter, } \mu\text{A/V}^2]

K_n = \frac{1}{2} \mu_n C_{ox} \left(\frac{W}{L}\right) \quad [\text{Device conduction parameter}]

1. Cutoff Region:

V_{GS} < V_{TH}

I_D \approx 0 \quad (\text{Subthreshold leakage } I_{D,sub} \propto \exp\left(\frac{V_{GS} - V_{TH}}{n V_t}\right))

2. Linear (Triode / Ohmic) Region:

Condition: V_{GS > V_{TH and V_{DS < V_{GS - V_{TH (or V_{DS < V_{DS(sat)):

I_D = \mu_n C_{ox} \left(\frac{W}{L}\right) \left[ (V_{GS} - V_{TH}) V_{DS} - \frac{1}{2} V_{DS}^2 \right]

For very small drain-to-source voltages (V_{DS \ll 2(V_{GS - V_{TH)), the quadratic term drops out, and the channel behaves as an ideal linear resistor (R_{DS(on)):

R_{DS(on)} = \left( \frac{\partial I_D}{\partial V_{DS}} \right)^{-1} \approx \frac{1}{\mu_n C_{ox} \left(\frac{W}{L}\right) (V_{GS} - V_{TH})}

3. Saturation (Active) Region:

Condition: V_{GS > V_{TH and V_{DS ≥ V_{GS - V_{TH:

I_D = \frac{1}{2} \mu_n C_{ox} \left(\frac{W}{L}\right) (V_{GS} - V_{TH})^2 (1 + \lambda V_{DS})

where \lambda represents the Channel Length Modulation parameter (analogous to the Early voltage V_A = 1/\lambda in BJTs).

Small-signal transconductance (g_m):

g_m = \left. \frac{\partial I_D}{\partial V_{GS}} \right|_{V_{DS}} = \mu_n C_{ox} \left(\frac{W}{L}\right) (V_{GS} - V_{TH}) = \sqrt{2 \mu_n C_{ox} \left(\frac{W}{L}\right) I_D} = \frac{2 I_D}{V_{GS} - V_{TH}}

5. Dynamic Switching Characteristics & Parasitics

In high-speed power electronics and converter design, steady-state DC resistance is only half the equation. The switching transient is governed by internal terminal parasitic capacitances and total gate charge:

Drain (D) │ ├──────────────┐ │ │ ┌─┴─┐ ┌─┴─┐ Cgd │ │ │ │ Coss = Cds + Cgd (Crss)│ │ │ │ └─┬─┘ └─┬─┘ │ Body │ Gate (G) ──┬─────┼── Diode ───┼ │ │ /│\ │ ┌─┴─┐ │ ──┼── │ Cgs │ │ │ │ │ │ │ └───┬─┴──────┘ └─┬─┘ │ │ │ └─────────┴────────── Source (S) Ciss = Cgs + Cgd

1. Terminal Capacitance Relationships:

Datasheets report terminal capacitances measured at specific DC bias voltages (normally with V_{GS = 0\,V):

Input Capacitance: C_{iss = C_{gs + C_{gd

Output Capacitance: C_{oss = C_{ds + C_{gd

Reverse Transfer (Miller) Capacitance: C_{rss = C_{gd

2. The Gate Charge (Q_g) Curve & Miller Plateau:

When a gate driver supplies current to turn on a MOSFET, the gate voltage waveform exhibits distinct phases:

Vgs ^ │ Phase 3: Vgs rises to Vdrive │ (Channel heavily enhanced, Rds_on reaches minimum) │ ┌─────────────────────── V_drive │ Phase 2: │ Miller Plateau (V_plateau) │ Vds falls│ [Vgs held constant while Cgd is discharged] │ ┌────────┘ │ / │ / Phase 1: Charging Cgs from 0V up to Vth │ / (Device in cutoff, Id = 0) └──┴────────────┴───────────────────────> Gate Charge (Qg) [nC] 0 Qth Qgs Qgd Qtotal

Phase 1 ($0 \rightarrow Q_{th}$): Gate driver charges C_{gs. Gate voltage rises exponentially. No drain current flows until V_{gs = V_{TH.

Phase 2 ($Q_{gs} \rightarrow Q_{gd}$): The Miller Plateau. Drain current reaches maximum load current, and drain voltage V_{DS collapses from high bus voltage to near zero. All gate drive current is diverted to discharging the highly non-linear C_{gd capacitance. The gate voltage remains stuck at V_{plateau = V_{TH + \frac{I_{LOAD{g_m.

Phase 3 ($Q_{gd} \rightarrow Q_{total}$): V_{DS has fallen to I_D \cdot R_{DS(on). The gate voltage rises up to the full gate driver output rail (V_{drive), driving the channel into deep ohmic conduction to minimize R_{DS(on).

6. Power Loss Breakdown & Thermal Sizing

The total power dissipated by a MOSFET in a switching converter or power stage is given by:

P_{TOTAL} = P_{COND} + P_{SW} + P_{GATE} + P_{COSS} + P_{DIODE}

1. Conduction Loss (P_{COND):

P_{COND} = I_{D,rms}^2 \times R_{DS(on)}(T_j)

Caution: R_{DS(on) has a strong positive temperature coefficient in Silicon (typically increasing by a factor of 1.5× to 2.0× between +25^\circC and +150^\circC):

R_{DS(on)}(T_j) = R_{DS(on)}(25^\circ\text{C}) \times \left(1 + \frac{\alpha}{100}(T_j - 25^\circ\text{C})\right) \quad (\alpha \approx 0.4\% \dots 0.7\% / ^\circ\text{C})

2. Switching Losses (P_{SW):

During each turn-on and turn-off transition, V_{DS and I_D overlap, dissipating instantaneous energy:

P_{SW} = \frac{1}{2} V_{DS,bus} \cdot I_{D,load} \cdot (t_{rise} + t_{fall}) \cdot f_{sw}

where t_{rise and t_{fall are governed by the gate driver source/sink current capability and gate charge (Q_{gs2 + Q_{gd).

3. Gate Drive Power Loss (P_{GATE):

Power delivered by the gate driver supply rail to charge and discharge the input capacitance:

P_{GATE} = Q_g \times V_{GS,drive} \times f_{sw}

(Note: Most of this loss is dissipated inside the gate resistor $R_G$ and gate driver output stage).

4. Output Capacitive Stored Energy Loss (P_{COSS):

During hard-switching turn-on, the energy stored in C_{oss is dissipated directly into the MOSFET channel:

P_{COSS} = \frac{1}{2} C_{oss(er)} \cdot V_{DS}^2 \cdot f_{sw} = f_{sw} \int_0^{V_{DS}} v \cdot C_{oss}(v) \, dv

5. Body Diode Conduction & Reverse Recovery Loss (P_{DIODE):

In half-bridge and synchronous converters during dead-time:

P_{DIODE} = (V_{SD} \cdot I_{LOAD} \cdot t_{dead} \cdot f_{sw}) + (Q_{rr} \cdot V_{DS,bus} \cdot f_{sw})

6. Thermal Network Junction Temperature Calculation:

T_j = T_A + P_{TOTAL} \cdot (R_{\theta JC} + R_{\theta CS} + R_{\theta SA})

R_{\theta JC: Junction-to-case thermal resistance (^\circC/W, from datasheet)

R_{\theta CS: Case-to-heatsink thermal interface resistance (thermal paste/sil-pad)

R_{\theta SA: Heatsink-to-ambient thermal resistance

7. Comparative Benchmark Matrix: Major MOSFET Types

| Parameter / Feature | N-Channel Enhancement | P-Channel Enhancement | Power Trench MOSFET | Superjunction (CoolMOS) | SiC Power MOSFET | Logic-Level MOSFET |
| --- | --- | --- | --- | --- | --- | --- |
| Charge Carrier | Electrons (µ_n \approx 1400) | Holes (µ_p \approx 450) | Electrons | Electrons | Electrons | Electrons |
| Relative Die Area for Same $R_{DS(on)}$ | 1.0× (Baseline) | 2.5× \dots 3.0× | 0.4× \dots 0.6× | 0.2× (High V) | 0.1× (High V) | 1.1× |
| Nominal Voltage Spectrum | 20\,V \dots 600\,V | 20\,V \dots 150\,V | 20\,V \dots 100\,V | 500\,V \dots 900\,V | 650\,V \dots 3300\,V | 20\,V \dots 60\,V |
| Typical $V_{GS(th)}$ | 2.0\,V \dots 4.0\,V | -2.0\,V \dots -4.0\,V | 1.5\,V \dots 3.0\,V | 3.0\,V \dots 4.5\,V | 2.0\,V \dots 3.5\,V | $0.8\,\text{V} \dots 2.0\,\text{V}$ |
| Recommended Gate Drive Voltage | +10\,V \dots +12\,V | -10\,V \dots -12\,V | +10\,V | +10\,V \dots +12\,V | $+18\,\text{V} / -4\,\text{V}$ | $+3.3\,\text{V} \dots +5.0\,\text{V}$ |
| Reverse Recovery Charge ($Q_{rr}$) | High | Moderate | Moderate to High | High (Low in CFD) | Virtually Zero | Low to Moderate |
| Temperature Capability ($T_{j,max}$) | 150^\circC \dots 175^\circC | 150^\circC | 175^\circC | 150^\circC | $175^\circ\text{C} \dots 200^\circ\text{C}$ | 150^\circC |
| Switching Speed ($dv/dt$) | Moderate (10-30\,V/ns) | Moderate | Fast | Extremely Fast (>50\,V/ns) | Ultra-Fast (>100\,V/ns) | Moderate |
| Key Circuit Topologies | Low-side, H-bridge | High-side disconnect | PoL Buck, BMS | PFC, LLC, Flyback | EV Inverter, OBC, EVSE | Direct MCU switching |

8. CLI Engineering Tool: mosfet_calc.py

To assist hardware designers in calculating switching and conduction budgets, sizing gate resistors, and verifying junction thermal margins, the repository includes a dedicated CLI calculation utility located at **`tools/mosfet_calc.py`**.

Quick Usage Examples:

# Calculate power dissipation, switching losses, and thermal junction temperature: python3 tools/mosfet_calc.py --loss --vds 48 --id 15 --rdson 0.005 --trise 25e-9 --tfall 20e-9 --fsw 150e3 --vgs 10 --qg 35e-9 --theta-ja 35 --ta 50 # Size gate resistor for peak driver current and target switching speed: python3 tools/mosfet_calc.py --gate --vdrv 12 --vplat 4.2 --qgd 12e-9 --target-dt 30e-9 --idriver-max 2.0 # Evaluate body diode conduction and reverse recovery losses: python3 tools/mosfet_calc.py --diode --vds 400 --iload 20 --vsd 1.2 --tdead 100e-9 --qrr 450e-9 --fsw 100e3