Depletion-Mode MOSFETs (d-MOSFET): Physics, Architecture & Applications

A Depletion-Mode MOSFET (d-MOSFET) is a unique class of field-effect transistor that is normally-on at zero gate-to-source bias (V_{GS = 0\,V). Unlike enhancement-mode devices where a channel must be electrostatically induced, a depletion MOSFET features a physically doped semiconductor conduction channel formed during fabrication between the Source and Drain.

1. Semiconductor Physics & Device Structure

Gate (G) │ ┌─────┴─────┐ Poly-Si / Metal Gate │ GATE │ ┌────┴───────────┴────┐ │ Gate Oxide (SiO2) │ ┌───────────────┴─────────────────────┴───────────────┐ │ Source (n+) Drain (n+) │ ───┤ ┌───────┐ Physically Implanted ┌───────┐ ├─── (S) │ │ n+ │ == n-Channel Layer (Doped) =│ n+ │ │ (D) └──┴───────┴──────────────────────────────┴───────┴───┘ │ Depletion Layer (Wdep) │ │ - - - - - - - - - - - - - - - - - - - - - - - - - │ │ p-type Substrate │ │ │ └──────────────────────────┬──────────────────────────┘ │ Body (B)

Modes of Operation (Dual-Mode Flexibility):

Depletion Mode ($V_{GS} < 0\,\text{V}$ for N-Channel):

Applying a negative voltage to the gate repels electrons away from the oxide interface, squeezing and depleting the pre-existing n-channel.

When V_{GS ≤ V_{GS(off) (the Pinch-Off Voltage, typically -1.5\,V \dots -5.0\,V), the channel is completely depleted of free electrons, choking off drain current (I_D \approx 0).

Zero Bias ($V_{GS} = 0\,\text{V}$):

The device conducts naturally with drain saturation current I_{DSS.

Enhancement Mode ($V_{GS} > 0\,\text{V}$ for N-Channel):

If the gate is driven positively, additional electrons are attracted into the already conductive channel, further decreasing R_{DS(on) and increasing current capacity beyond I_{DSS.

2. Terminal Characteristic Curves & Mathematical Model

Drain Current (Id) vs Gate-to-Source Voltage (Vgs) Id ^ │ (Enhancement Mode: Vgs > 0) │ / │ / Idss ├─────────────────────────* (Vgs = 0V: Id = Idss) │ / │ / (Depletion Mode: Vgs < 0) │ / │ / │ / 0 └────────────────*──────────────────────────────────> Vgs Vgs(off) (Pinch-off)

Mathematical Equation (Shockley / Depletion Model):

In the saturation region (V_{DS ≥ V_{GS - V_{GS(off)):

I_D = I_{DSS} \left( 1 - \frac{V_{GS}}{V_{GS(off)}} \right)^2

Where:

I_{DSS is the drain current at V_{GS = 0\,V.

V_{GS(off) (or V_P) is the gate-to-source cutoff threshold (negative for N-channel, positive for P-channel).

3. High-Value Industrial Circuit Applications

1. Ultra-Low Standby Loss Offline SMPS High-Voltage Startup:

In universal AC-DC power converters (85\,V_{AC \dots 265\,V_{AC rectified to 400\,V_{DC), high-value resistive bleeders (100\,k\Omega) waste continuous power during normal operation. A high-voltage depletion MOSFET (such as Supertex/Microchip DN2540) achieves zero-standby startup:

+400V DC High-Voltage Bus (Rectified Mains) │ Drain (D) │ ┌──┴──┐ │ │ Depletion NMOS (DN2540: 400V rated) │ │ [Normally ON at Vgs = 0V] └──┬──┘ │ Source (S) ├──────────────────┬──────────────────> VCC Rail to PWM Controller IC │ │ [R_bias] ┌─┴─┐ │ │ │ Bulk Cap (47µF, 25V) │ └─┬─┘ Gate (G) │ │ GND ├── Drain of Auxiliary N-FET (Controlled by PWM IC) │ GND

Sequence of Events:

Mains Applied: At initial power-on, the PWM IC is dead (V_{CC = 0\,V). The depletion MOSFET gate is at 0\,V (V_{GS = 0\,V), so it conducts immediately, acting as a current source charging the bulk V_{CC capacitor.

IC Boot: Once V_{CC charges to the Under-Voltage Lockout threshold (V_{UVLO \approx 12\,V), the PWM controller starts switching and powers itself from an auxiliary transformer winding.

Lossless Disconnect: The controller turns ON an auxiliary small-signal FET connected to the depletion MOSFET's gate, pulling the gate to ground while the source is at +12\,V. Thus:

V_{GS} = V_G - V_S = 0\,\text{V} - 12\,\text{V} = -12\,\text{V} < V_{GS(off)}

The depletion MOSFET is driven into total cutoff, drawing negligible leakage (< 1\,µA) and reducing standby power to < 5\,mW to comply with Energy Star / EU CoC Tier 2 regulations.

2. High-Precision Two-Terminal Constant-Current Source:

A depletion MOSFET combined with a single self-biasing source resistor forms a robust, floating two-terminal constant current regulator (CCR) requiring no external power supply:

(+) Input Terminal (up to 400V) │ Drain (D) │ ┌──┴──┐ │ │ Depletion NMOS (e.g. LND150) └──┬──┘ │ Source (S) ├───┐ │ [R_SET] (Sets current: R_SET = ¦Vgs¦ / I_target) │ │ Gate (G) │ │ └───┴─────────────────────────────> (-) Output Terminal

Self-Biasing Negative Feedback:

Current flowing through R_{SET generates a voltage drop:

V_S = I_D \times R_{SET}

Since the gate is tied to the lower end of R_{SET, V_G = 0\,V relative to that node:

V_{GS} = V_G - V_S = -I_D \times R_{SET}

If I_D attempts to surge due to an input voltage transient, V_{GS becomes more negative, constricting the channel and forcing the current back down to the target setpoint:

I_D = \frac{|V_{GS(off)}| \cdot \left(1 - \sqrt{I_D / I_{DSS}}\right)}{R_{SET}}

4. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | V_{DS(max) | I_{DSS(min) | V_{GS(off) Range | Typical Use-Case |
| --- | --- | --- | --- | --- | --- | --- |
| BSS126 | Infineon | SOT-23 | 600\,V | 30\,mA | -1.4\,V \dots -2.1\,V | Offline SMPS startup, high-voltage active clamps |
| BSP135 | Infineon | SOT-223 | 600\,V | 120\,mA | -1.0\,V \dots -2.2\,V | Telecom line feeds, surge limiters |
| DN2540 | Microchip (Supertex) | TO-220 / TO-92 | 400\,V | 150\,mA | -1.5\,V \dots -3.5\,V | High-fidelity audio CCS, piezo drivers |
| LND150 | Microchip (Supertex) | SOT-89 / TO-92 | 500\,V | 1.0\,mA | -1.0\,V \dots -3.0\,V | Ultra-low power current limiters, smoke detectors |
| IXTY02N120P | IXYS / Littelfuse | TO-252 (DPAK) | 1200\,V | 200\,mA | -2.0\,V \dots -4.5\,V | Industrial solar inverter startup (1000V bus) |