Lateral Double-Diffused MOSFETs (LDMOS): RF Power & High-Frequency Amplifiers

A Lateral Double-Diffused MOSFET (LDMOS) is a specialized power field-effect transistor engineered specifically for high-power, high-gain radio frequency (RF) and microwave amplification across the 1\,MHz \dots 3.8\,GHz frequency spectrum. By utilizing a lateral conduction profile coupled with an integrated backside grounded substrate flange, LDMOS delivers extreme breakdown voltage (> 100\,V), negligible feedback capacitance (C_{rss), and unmatched VSWR mismatch ruggedness across 4G/5G cellular base stations, broadcast transmitters, avionics radar, and industrial ISM generators.

1. Semiconductor Physics & Device Cross-Section

Source Metal Gate (Poly-Si) Drain Metal ┌──────────┐ ┌──────────┐ ┌──────────┐ │ Al │ │ GATE │ │ DRAIN │ ├───┬──────┴──────────────────┴────┬─────┴─────────────┴───┬──────┤ │n+ │ p-Body (Channel) SiO2 Oxide│ n- Drift Extension │ n+ │ └───┴───────────────┐ │ (Drift Region) │──────┘ p+ Sink │ └───────────────────────┘ (Low-Z Plug) │ ────────────────────┴────────────────────────────────────────────── │ p+ Highly Doped Substrate │ ────────────────────────────────────────────────────────────────── │ Backside Flange (Bolted Directly to RF Ground) │

Key Structural Innovations:

Backside Grounded Flange (Zero Common-Lead Inductance):

In RF power amplification, common-source parasitic inductance (L_S) is the primary destroyer of RF power gain:

G_{max} \propto \frac{1}{\omega^2 \cdot L_S}

LDMOS connects the top n^+ source to the p^+ substrate through a low-resistance vertical diffusion plug (the p+ sink). The metallic package flange serves simultaneously as the RF ground, DC ground return, and physical heat dissipation mounting surface. Parasitic source inductance is effectively reduced to near zero.

Extended Lateral $n^-$ Drift Region:

To withstand high drain supply voltages (28\,V \dots 65\,V DC bus with RF peak swings > 150\,V), an extended lateral n^- drift region is placed between the channel and the drain contact.

Stepped field oxide plates over the drift region spread the electric field evenly, preventing localized dielectric breakdown.

Ultra-Low Feedback Miller Capacitance ($C_{rss} = C_{gd}$):

The lateral separation between the gate and the drain contact slashes C_{rss to less than 1\,pF even in multi-hundred-watt transistors. Low C_{rss prevents parasitic self-oscillation and yields immense power gain (18\,dB \dots 26\,dB) in a single stage.

2. RF Power Amplifier Topologies: The Doherty Architecture

In modern cellular base stations (4G LTE / 5G NR), signals have extreme Peak-to-Average Power Ratios (PAPR \approx 7 - 10\,dB). Standard Class-AB amplifiers suffer abysmal efficiency at back-off. LDMOS is the core technology powering Doherty Power Amplifiers:

┌───[ Main Carrier Amp (Class AB) ]───[ λ/4 Line ]───┐ │ (Biased to conduct continuously) │ RF Input ──[ 90° Hybrid ]──┤ ├──> RF Out (50Ω) Splitter │ │ └───[ Peak / Peaking Amp (Class C) ]─────────────────┘ (Turns ON only during high power crests)

Working Principle:

Average Signal Power (Back-off): The Main (Carrier) amplifier operates close to saturation (peak efficiency), while the Peaking amplifier is biased in Class C and remains completely OFF.

Peak Signal Power (Crest): As the RF envelope exceeds average power, the Peaking amplifier turns ON and delivers current, actively pulling down the apparent load impedance seen by the Main amplifier via active load modulation (\lambda/4 transmission line).

Result: Peak power efficiency of > 55\% is maintained across the entire modulation envelope.

3. Ruggedness & Extreme VSWR Survival (> 65:1)

In industrial RF applications (CO2 laser excitation, RF plasma chambers, magnetic resonance imaging), antenna disconnection or load arcing creates a total reflection of RF power back into the transistor (|\Gamma| = 1).

Modern "XR" (Extreme Ruggedness) LDMOS:

Devices such as NXP BLF188XR and NXP MRFX1K80H integrate monolithic protection diodes and internal capacitive ballasting.

They withstand a $65:1$ Voltage Standing Wave Ratio (VSWR) at all phase angles with zero degradation, surviving conditions that would instantly vaporize standard power MOSFETs.

4. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | Supply (V_{DD) | Frequency Range | Pout (P1dB / Sat) | Power Gain | Primary Application |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AFT09MS007N | NXP Semiconductor | SOT-89 | 7.5\,V - 13.6\,V | 136 - 941\,MHz | 7\,W | 20\,dB | Handheld two-way radio, L-band telemetry |
| BLF188XR | Ampleon | SOT539A | 50\,V | 10 - 600\,MHz | 1400\,W | 28\,dB | FM broadcast (88-108 MHz), CO2 Laser, MRI |
| MRFX1K80H | NXP Semiconductor | NI-1230-4 | 65\,V | 1.8 - 400\,MHz | 1800\,W | 26\,dB | 65:1 VSWR Industrial heating, Avionics radar |
| MRFE6VP61K25H | Freescale / NXP | NI-1230 | 50\,V | 1.8 - 600\,MHz | 1250\,W | 24\,dB | High-power pulsed radar, particle accelerators |