P-Channel Enhancement-Mode MOSFET (e-PMOS): Physics, Design & Applications

A P-Channel Enhancement-Mode MOSFET (e-PMOS) is a normally-off field-effect transistor where current conduction occurs through the flow of holes (h^+) across an induced p-type inversion layer within an n-type substrate. Because its source terminal is naturally connected to the most positive potential in a circuit, the e-PMOS is predominantly deployed in high-side power switching, reverse battery polarity protection, and complementary CMOS logic stages.

1. Semiconductor Physics & Device Structure

Gate (G) │ ┌─────┴─────┐ Poly-Si / Metal Gate │ GATE │ ┌────┴───────────┴────┐ │ Gate Oxide (SiO2) │ (tox: 10nm - 50nm) ┌───────────────┴─────────────────────┴───────────────┐ │ Source (p+) Drain (p+) │ ───┤ ┌───────┐ P-Inversion Channel ┌───────┐ ├─── (S) │ │ p+ │ ========================= │ p+ │ │ (D) └──┴───────┴─────────────────────────────┴───────┴────┘ │ Depletion Layer (Wdep) │ │ - - - - - - - - - - - - - - - - - - - - - - - - - │ │ n-type Substrate (or N-Well) │ │ │ └──────────────────────────┬──────────────────────────┘ │ Body (B) (Internally tied to Source)

Physical Operation & Mobility Physics:

Gate Drive Polarity ($V_{GS} < 0\,\text{V}$):

For an enhancement PMOS, the threshold voltage is negative: V_{TH(p) \approx -1.0\,V \dots -3.5\,V.

To turn the transistor ON, the Gate potential must be driven more negative than the Source (V_{GS ≤ V_{TH(p), or V_G < V_S - |V_{TH|).

The "Hole Mobility Penalty":

In crystalline Silicon, hole mobility (µ_p \approx 450\,cm^2/V\cdots) is approximately $2.5\times$ to $3\times$ lower than electron mobility (µ_n \approx 1350\,cm^2/V\cdots).

Because R_{DS(on) \propto \frac{1{µ C_{ox (W/L), achieving the same on-resistance in a PMOS requires roughly 2.5× to 3.0× larger die silicon area (and consequently proportional increases in gate charge Q_g and cost) compared to an NMOS of identical voltage and current ratings.

2. Circuit Topologies: High-Side Power Switching

Unlike an NMOS which requires a complex bootstrap circuit or charge pump to drive its gate above the positive supply rail in high-side configurations, a PMOS can be turned ON simply by pulling its gate down toward ground:

1. Direct Low-Voltage High-Side Switch (V_{IN ≤ 12\,V):

+V_IN (+5V / +12V Rail) │ ├───┬───────────────────────────────┐ │ │ │ [R1] │ Source (S) │ 100k └───┐ │ │ │ │ ├─── Gate (G) PMOS │ │ │ [e.g. AO3401] │ [R2] ────┘ │ 10k └─── Drain (D) │ │ │ │ Collector ▼ To Load │ │ │ │ ┌────┴────┐ [ LOAD ] │ MCU ────┤ NPN BJT │ │ │ GPIO │ (2N3904)│ GND │ via 1k └────┬────┘ │ │ │ GND GND

2. High-Voltage High-Side Switch with Zener Gate Clamp (V_{IN > 20\,V):

If V_{IN = 24\,V or 48\,V, pulling the gate to ground would subject the gate oxide to V_{GS = -24\,V or -48\,V, instantly destroying the oxide (since V_{GS(max) = ± 20\,V). A Zener clamp is mandatory:

+V_IN (+24V / +48V Bus) │ ├───┬─────────────────────────┐ │ │ │ [R1] │ Source (S) │ 10k └───┐ │ │ │ │ ├──┬─ Gate (G) PMOS │ │ │ │ │ [12V] │ │ │ Zener ▲ │ │ │ Diode ──┘ └─── Drain (D) │ │ │ │ [R2] ▼ To Load │ 22k │ │ │ [ LOAD ] │ Drain (D) │ │ ┌───┴───┐ GND │ MCU ──[100Ω]──┤ NMOS │ │ GPIO └───┬───┘ │ │ Source (S) │ GND GND

3. Reverse Polarity (Reverse Battery) Protection

In automotive and industrial electronics, connecting battery leads in reverse will destroy downstream capacitors and ICs. A PMOS provides ideal reverse polarity protection with millivolt-level insertion loss compared to a classical series diode:

Battery Input (+) Protected System Rail ──────────────────┬──────────────┬──────────────────────────────> (+V_SYS) │ │ │ Drain (D) [R1] │ 100k ┌──┴──┐ │ │ │ PMOS ├── Gate (G)┤ │ [Body diode cathode points to Vin!] │ │ │ [15V] └──┬──┘ Zener │ Diode ▲ Source (S) ├───┘ │ │ │ Battery Input (-) ┴──────────────┴──────────────────────────────> (GND_SYS) ────────────────────────────────────────────────────────────────>

Working Principle:

Normal Connection (Correct Polarity):

Initial power application: Current flows through the PMOS body diode from Drain to Source, establishing a positive potential at the Source terminal.

The Gate is tied through R_1 to Ground. Therefore, V_{GS = V_G - V_S \approx 0\,V - V_{IN = -V_{IN.

Once |V_{GS| > |V_{TH|, the PMOS channel turns fully ON. The low R_{DS(on) shunts the body diode, reducing voltage drop across the switch to mere millivolts (V_{drop = I_{LOAD \cdot R_{DS(on)) with zero forward diode power dissipation.

Reverse Connection (Battery Inverted):

The Source terminal is connected to the negative battery terminal, while the Gate is connected to the positive terminal through R_1.

Consequently, V_G > V_S, meaning V_{GS > 0\,V. The PMOS channel remains completely OFF.

The body diode is reverse-biased, completely blocking any reverse current flow and safeguarding all downstream circuits.

4. Key Datasheet Parameters & Traps

| Parameter | Symbol | Critical Significance | Rule of Thumb |
| --- | --- | --- | --- |
| Drain-to-Source Breakdown | V_{(BR)DSS | Negative breakdown voltage between D and S | Must handle max positive rail plus inductive overshoot |
| Gate-to-Source Limit | V_{GS(max) | Maximum gate dielectric stress (typically ± 20\,V) | Clamp with 12\,V - 15\,V Zener in systems where V_{IN > 12\,V |
| Threshold Voltage | V_{GS(th) | Gate voltage where conduction starts (typically -1.0\,V \dots -3.0\,V) | Ensure drive voltage pulls gate well below V_{TH (ideally -10\,V) |
| Static On-Resistance | R_{DS(on) | Channel on-resistance | Evaluate at specified V_{GS (e.g. -4.5\,V vs -10\,V) |

5. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | -V_{DS(max) | -I_{D(max) | R_{DS(on) (@-10V) | Typical Use-Case |
| --- | --- | --- | --- | --- | --- | --- |
| AO3401A | Alpha & Omega | SOT-23 | 30\,V | 4.0\,A | 44\,m\Omega | Compact 3.3V/5V/12V load switches, battery cutoffs |
| FDN340P | onsemi / Fairchild | SOT-23 | 20\,V | 2.0\,A | 70\,m\Omega | Handheld power path management |
| DMP3098L | Diodes Inc | SOT-23 | 30\,V | 3.8\,A | 65\,m\Omega | Reverse battery protection in low-power microcontrollers |
| IRF9540N | Infineon (IR) | TO-220 | 100\,V | 23\,A | 117\,m\Omega | Industrial high-side actuator/relay switching |
| SQJ407EP | Vishay Siliconix | PowerPAK SO-8 | 40\,V | 60\,A | 7.2\,m\Omega | AEC-Q101 Automotive high-side power distribution |