Superjunction MOSFETs (CoolMOS & Charge Balance): Physics & Applications

A Superjunction MOSFET (SJ-MOSFET, commercially recognized under brand names such as Infineon CoolMOS™, STMicroelectronics MDmesh™, and onsemi SUPERFET®) represents a revolutionary breakthrough in high-voltage (500\,V \dots 900\,V) Silicon power semiconductors. By leveraging the principle of charge compensation across alternating vertical p- and n-doped semiconductor columns, superjunction technology fundamentally shattered the theoretical "1D Silicon Limit," slashing specific on-resistance (R_{DS(on) × A) by up to 80\%.

1. The Classical 1D Silicon Limit vs. Superjunction Physics

In conventional planar or VDMOS power transistors, blocking a high reverse breakdown voltage (V_{(BR)DSS) requires an epitaxial drift layer that is both physically thick (W_{drift) and lightly doped (N_D). The resulting electric field profile is triangular:

Conventional Planar Drift Region Superjunction Column Architecture ┌────────────────────────────────────┐ ┌───────┬───────┬───────┬───────┐ │ │ │ p-Col │ n-Col │ p-Col │ n-Col │ │ Lightly Doped n- Drift │ │ (Boron│(Phos) │ (Boron│(Phos) │ │ (Low Doping) │ │ ) │ ) │ ) │ ) │ │ │ │ │ │ │ │ └────────────────────────────────────┘ └───────┴───────┴───────┴───────┘ Triangular Electric Field Rectangular Flat Electric Field Rds(on)*A ∝ V_BR^(2.5) [Severe!] Rds(on)*A ∝ V_BR^(1.0 - 1.3) [Linear!]

Mathematical Scaling Laws:

Classical 1D Silicon Limit:

R_{DS(on)} \times A \approx 5.93 \times 10^{-9} \cdot (V_{(BR)DSS})^{2.5} \quad [\Omega \cdot \text{cm}^2]

At 600\,V, this power law dictates that Silicon chips had to become enormous and costly, with crippling gate charge and output capacitance.

Superjunction Scaling:

R_{DS(on)} \times A \propto (V_{(BR)DSS})^{1.0 \dots 1.32}

By balancing charges precisely:

N_A \cdot W_p = N_D \cdot W_n

When reverse voltage is applied across the Drain, the horizontal electric field between adjacent p- and n-columns completely sweeps away mobile carriers at very low voltage (V_{DS \approx 50\,V). The entire drift region becomes fully depleted, creating a uniform, rectangular electric field distribution.

Because the field is rectangular rather than triangular, the drift region thickness can be halved, and the n-columns can be doped an order of magnitude more heavily, reducing conduction losses dramatically.

2. Dynamic Switching Behavior: Non-Linear C_{oss & Extreme dv/dt

The deep column depletion creates a unique electrical signature: a massively non-linear output capacitance (C_{oss):

Coss vs Drain-to-Source Voltage (Vds) Coss ^ │ 10,000 pF (Un-depleted columns at low Vds) │ █ │ █ │ █ │ █ │ █ Colossal drop at ~50V column pinch-off! │ ████████████████████████████ 50 pF (High Vds) └───────────────────────────────────────────────> Vds 0V 20V 50V 400V

High-Speed Switching Traps:

Ultra-Fast Voltage Slewing ($dv/dt > 50\,\text{V/ns}$ to $100\,\text{V/ns}$):

Because C_{oss collapses to mere picofarads at high voltage, the drain node snaps upward with ferocious speed during turn-off.

Parasitic Package Oscillations:

This rapid dv/dt interacts with trace and bondwire parasitic inductances (L_{source, L_{gate), generating high-frequency ringing (50\,MHz - 200\,MHz) and electromagnetic interference (EMI).

Remediation Techniques:

Source Kelvin Pin: Use 4-pin packages (e.g. TO-247-4 or PG-HDSOP) where the gate driver return has an independent source connection that bypasses high load di/dt.

Ferrite Beads: Place an SMD ferrite bead (impedance 30\,\Omega - 60\,\Omega at 100\,MHz) directly adjacent to the gate pin.

Tuned Turn-On Gate Resistor ($R_{G,on}$): Avoid making R_G overly small; damp the turn-on transition to control peak diode recovery current.

3. Fast-Recovery Body Diode Variants (CFD / DM2)

In hard-switched bridge topologies (such as Phase-Shifted Full-Bridge or resonant LLC converters during abnormal transients/start-up), the MOSFET's intrinsic body diode must conduct freewheeling load current:

Topology Compatibility Comparison ┌─────────────────────────────────────────────────────────────┐ │ Standard Superjunction FET │ Fast-Recovery Body Diode (CFD)│ ├─────────────────────────────┼───────────────────────────────┤ │ High Qrr (> 4000 nC) │ Low Qrr (< 400 nC) │ │ Long trr (> 400 ns) │ Ultra-Fast trr (< 100 ns) │ │ Prone to body-diode dv/dt │ Immune to diode latch-up │ │ latch-up in half-bridges │ │ ├─────────────────────────────┼───────────────────────────────┤ │ Ideal for: Single-switch PFC│ Ideal for: Resonant LLC, │ │ Boost, Single-Ended Flyback │ Full-Bridge ZVS, EV Chargers │ └─────────────────────────────┴───────────────────────────────┘

By introducing heavy metal lifetime killers (Platinum/Gold doping or Electron Irradiation) into the p-n column junction, manufacturers like Infineon (CoolMOS™ CFD series) crush minority carrier lifetime, preventing disastrous bridge shoot-through.

4. Commercial Part Catalog & Selection

| Part Number | Manufacturer | Package | V_{DS(max) | I_{D(max) | R_{DS(on) | Diode Class | Target Application |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IPW60R099CP | Infineon (CoolMOS CP) | TO-247 | 650\,V | 31\,A | 99\,m\Omega | Standard | High-efficiency CCM Power Factor Correction (PFC) |
| IPW60R045CFD7 | Infineon (CoolMOS CFD7) | TO-247 | 600\,V | 50\,A | 45\,m\Omega | Fast CFD | Resonant LLC converter, Server Power, EV On-Board Charger |
| STW48N60M2 | STMicroelectronics | TO-247 | 600\,V | 42\,A | 76\,m\Omega | Standard | Telecom rectifiers, Solar micro-inverters |
| TK20A60U | Toshiba (DTMOS) | TO-220SIS | 600\,V | 20\,A | 165\,m\Omega | Standard | Consumer flat-panel TV power supplies |