CAN Protocol: Frame Data & Protocol Analysis

1. Bit-Level CAN Frame Anatomy

CAN frames are tightly packed serial bitstreams designed for zero overhead and maximum fault detection. The CAN standard defines four fundamental frame types:

Data Frame: Carries real-time application data payload from transmitter to receivers.

Remote Frame: Transmitted by a node to solicit data from another node without transmitting data itself.

Error Frame: Driven by any node upon detecting an electrical, framing, or protocol violation.

Overload Frame: Used to request an internal delay between frames (rarely required in modern MCUs).

1.1 Classical CAN 2.0A Standard Data Frame (11-bit Identifier)

The Standard Data Frame consists of seven primary logical fields occupying between 44 and 108 bits (excluding bit stuffing):

┌───┬─────────────┬─────┬─────┬────┬─────┬──────────────┬────────┬───┬─────┬───┬───────┬─────┐ │SOF│ Identifier │ RTR │ IDE │ r0 │ DLC │ Data Field │ CRC-15 │CD │ ACK │AD │ EOF │ IFS │ ├───┼─────────────┼─────┼─────┼────┼─────┼──────────────┼────────┼───┼─────┼───┼───────┼─────┤ │ 1 │ 11 bits │ 1 │ 1 │ 1 │ 4 │ 0 - 8 Bytes │15 bits │ 1 │ 1 │ 1 │7 bits │3 bit│ └───┴─────────────┴─────┴─────┴────┴─────┴──────────────┴────────┴───┴─────┴───┴───────┴─────┘ ¦<---------------- Bit Stuffed Fields ---------------->¦固定¦<-- Unstuffed Fields -->¦

Field-by-Field Breakdown:

Start of Frame (SOF) — 1 bit:

Single Dominant ('0') bit.

Falling edge synchronizes the internal baud rate clocks of all connected bus receivers (Hard Synchronization).

Identifier (Arbitration Field) — 11 bits:

Transmitted MSB first (ID10 down to ID0).

Dictates frame content and bus access priority.

Remote Transmission Request (RTR) — 1 bit:

Dominant ('0'): Indicates a Data Frame carrying a payload.

Recessive ('1'): Indicates a Remote Frame requesting data.

Identifier Extension (IDE) — 1 bit:

Dominant ('0'): Standard CAN 2.0A (11-bit ID).

Recessive ('1'): Extended CAN 2.0B (29-bit ID).

Reserved Bit (r0) — 1 bit:

Always driven Dominant ('0') by the transmitter.

Data Length Code (DLC) — 4 bits:

Encodes payload length in bytes (0 to 8 bytes, 0000b to 1000b).

Data Field — 0 to 64 bits (0 to 8 Bytes):

Payload data, transmitted byte-by-byte, MSB first within each byte.

Cyclic Redundancy Check (CRC Field) — 16 bits:

CRC Sequence (15 bits): Generated via generator polynomial:

P(x) = x^{15} + x^{14} + x^{10} + x^8 + x^7 + x^4 + x^3 + 1

CRC Delimiter (1 bit): Always Recessive ('1'). Marks end of bit stuffing.

Acknowledgment Field (ACK Field) — 2 bits:

ACK Slot (1 bit): Transmitter transmits a Recessive ('1') bit. Every node on the bus that received the frame without error forces this bit to Dominant ('0') on the wire.

ACK Delimiter (1 bit): Always Recessive ('1').

End of Frame (EOF) — 7 bits:

7 consecutive Recessive ('1') bits.

Interframe Space (IFS) — 3 bits minimum:

Consists of Intermission (ITM): 3 consecutive Recessive bits before a node may transmit again.

1.2 Extended CAN 2.0B Data Frame (29-bit Identifier)

To accommodate complex networks (such as commercial vehicles adhering to SAE J1939), CAN 2.0B expands the arbitration identifier from 11 bits to 29 bits.

┌───┬─────────┬─────┬─────┬──────────┬─────┬────┬────┬─────┬──────────┬────────┬───┬─────┬───┬─────┐ │SOF│Base ID │ SRR │ IDE │ Ext ID │ RTR │ r1 │ r0 │ DLC │ Data │ CRC-15 │CD │ ACK │AD │ EOF │ ├───┼─────────┼─────┼─────┼──────────┼─────┼────┼────┼─────┼──────────┼────────┼───┼─────┼───┼─────┤ │ 1 │ 11 bits │ 1 │ 1 │ 18 bits │ 1 │ 1 │ 1 │ 4 │ 0 - 8 B │15 bits │ 1 │ 1 │ 1 │ 7 │ └───┴─────────┴─────┴─────┴──────────┴─────┴────┴────┴─────┴──────────┴────────┴───┴─────┴───┴─────┘

Base Identifier (11 bits): Corresponds to standard ID bits ID28 down to ID18.

Substitute Remote Request (SRR) — 1 bit: Always driven Recessive ('1'). Ensures that if a Standard frame and an Extended frame share the same first 11 bits, the Standard frame (RTR=0) wins arbitration over the Extended frame (SRR=1).

Identifier Extension (IDE) — 1 bit: Driven Recessive ('1') to signal an extended 29-bit format.

Extended Identifier (18 bits): Bits ID17 down to ID0.

Total Identifier Space: 2^{29 = 536,870,912 unique IDs.

1.3 CAN FD (Flexible Data-rate) Frame Architecture

Standardized under ISO 11898-1:2015, CAN FD introduces two revolutionary enhancements:

Payload Expansion: Increases maximum payload from 8 bytes to 64 bytes.

Dual Bitrate (Bit Rate Switch - BRS): Switches to a higher clock rate (e.g., 2 Mbps to 5 Mbps) during the transmission of the data payload and CRC fields, reverting to the nominal bitrate (e.g., 500 kbps) for arbitration and ACK.

Arbitration Phase Data Phase (Fast Clock) Arbitration Phase ┌───┬────────────┬─────┬─────┬─────┬─────┬─────┬────┬─────────────────────┬──────────────┬────┬───┬───┬─────┐ │SOF│ Identifier │ RRS │ IDE │ FDF │ res │ BRS │ESI │ DLC (0-15 = 0-64 B) │ Data (0-64B) │CRC │CD │ACK│AD │ EOF │ └───┴────────────┴─────┴─────┴─────┴─────┴─────┴────┴─────────────────────┴──────────────┴────┴───┴───┴─────┘ ¦<---- Nominal Bitrate (e.g., 500 kbps) ------->¦<---- Fast Bitrate (e.g., 2.0 - 5.0 Mbps) ------>¦<-- Nom. -->¦

New CAN FD Control Bits:

FDF / EDL (Extended Data Length) — 1 bit: Driven Recessive ('1') to designate a CAN FD frame (Classical CAN nodes see this as dominant reserve bit r0 and flag a form error; hence, classical nodes cannot coexist on a CAN FD bus without FD-tolerant transceivers).

BRS (Bit Rate Switch) — 1 bit:

Recessive ('1'): The transmission clock switches to the high-speed data phase bitrate at the BRS sample point.

Dominant ('0'): The frame remains at the nominal arbitration bitrate throughout.

ESI (Error State Indicator) — 1 bit:

Driven Dominant ('0') if the transmitting node is in Error Active mode.

Driven Recessive ('1') if the node is in Error Passive mode.

CAN FD DLC Encoding Table:

Unlike Classical CAN where DLC directly matches byte count, CAN FD uses non-linear mapping for values above 8:

| DLC Code (Binary) | Classical CAN Payload | CAN FD Payload |
| --- | --- | --- |
| 0000b – 1000b (0 - 8) | 0 - 8 Bytes | 0 - 8 Bytes |
| 1001b (9) | 8 Bytes | 12 Bytes |
| 1010b (10) | 8 Bytes | 16 Bytes |
| 1011b (11) | 8 Bytes | 20 Bytes |
| 1100b (12) | 8 Bytes | 24 Bytes |
| 1101b (13) | 8 Bytes | 32 Bytes |
| 1110b (14) | 8 Bytes | 48 Bytes |
| 1111b (15) | 8 Bytes | 64 Bytes |

2. Bit Stuffing Algorithm & Verification

To ensure continuous synchronization of all receiver bit clocks across long sequences of static bits (such as consecutive zeros in an empty payload or identifier), CAN enforces Bit Stuffing.

2.1 The Bit Stuffing Rule

Whenever the transmitter hardware detects 5 consecutive bits of identical polarity (five '0's or five '1's) within the bit-stuffed zone, it automatically inserts a complementary stuff bit of the opposite polarity.

The receiver hardware detects the 5 identical bits, extracts and discards the 6th stuff bit, and reconstructs the original un-stuffed payload.

Stuffed Zone: Starts at the Start of Frame (SOF) and ends at the last bit of the CRC Sequence.

Fixed/Un-stuffed Zone: The CRC Delimiter, ACK Slot, ACK Delimiter, and EOF fields are fixed form and never stuffed.

Transmitted Raw Data: 1 1 1 1 1 0 0 0 0 0 1 Line After Stuffing: 1 1 1 1 1 [0] 0 0 0 0 [1] 0 1 ^^^ ^^^ Stuff Bit Stuff Bit

2.2 Worst-Case Frame Length Formula

In Classical CAN 2.0A with an 8-byte payload:

Nominal bits = 1\,(SOF) + 11\,(ID) + 1\,(RTR) + 1\,(IDE) + 1\,(r0) + 4\,(DLC) + 64\,(Data) + 15\,(CRC) + 1\,(CD) + 2\,(ACK) + 7\,(EOF) + 3\,(IFS) = 111\,bits.

Bit-stuffed portion = 1 + 11 + 1 + 1 + 1 + 4 + 64 + 15 = 97\,bits.

Theoretical maximum stuff bits:

\text{Max Stuff Bits} = \left\lfloor \frac{97 - 1}{4} \right\rfloor = 24\,\text{bits}

Absolute Worst-Case Frame Length:

\text{Total Bits}_{max} = 111 + 24 = 135\,\text{bits}

3. Higher-Layer Automotive Protocol Frameworks

Classical CAN provides only Data Link Layer framing (OSI Layers 1 & 2). Real-world automotive and industrial systems build on standardized higher-layer protocols:

┌────────────────────────────────────────────────────────┐ │ OSI Layer 7: Application Layer │ │ SAE J1939 (Heavy Duty) │ CANopen │ UDS (ISO 14229) │ ├────────────────────────────────────────────────────────┤ │ OSI Layer 3/4: Network & Transport Layer │ │ J1939-21 (TP BAM/CM) │ ISO 15765-2 (DoCAN TP) │ ├────────────────────────────────────────────────────────┤ │ OSI Layer 2: Data Link Layer │ │ CAN 2.0A/B & CAN FD (ISO 11898-1) │ ├────────────────────────────────────────────────────────┤ │ OSI Layer 1: Physical Layer │ │ Differential Transceiver (ISO 11898-2) │ └────────────────────────────────────────────────────────┘

3.1 SAE J1939 (Commercial Vehicles & Heavy Duty)

Used across trucks, buses, diesel generators, and marine engines, SAE J1939 utilizes the 29-bit CAN Identifier to structure a complete network addressing framework:

┌──────────┬─────┬───────────┬────────────┬────────────────────────┬────────────────┐ │ Priority │ Res │ Data Page │ PDU Format │ PDU Specific (DA or GE)│ Source Address │ ├──────────┼─────┼───────────┼────────────┼────────────────────────┼────────────────┤ │ 3 bits │ 1 │ 1 bit │ 8 bits │ 8 bits │ 8 bits │ └──────────┴─────┴───────────┴────────────┴────────────────────────┴────────────────┘ ¦<---------------------- Parameter Group Number (PGN, 18 bits) ------------------------>¦

PGN (Parameter Group Number): 18-bit identifier indexing standard vehicle messages (e.g., PGN 61444 / 0xF004 = Electronic Engine Controller 1, containing Engine Speed).

SPN (Suspect Parameter Number): Specific sensor or actuator variable embedded within a PGN (e.g., SPN 190 = Engine Speed, resolution 0.125\,rpm/bit).

Transport Protocol (TP): Breaks payloads larger than 8 bytes (up to 1,785 bytes) into multiple frames using BAM (Broadcast Announce Message) or RTS/CTS connection-mode.

3.2 UDS (ISO 14229) over CAN (ISO 15765-2 / DoCAN)

Unified Diagnostic Services (UDS) is the universal automotive standard for flashing firmware, reading diagnostic trouble codes (DTCs), and calibrating ECU sensors. Because diagnostic payloads often exceed 8 bytes, ISO 15765-2 (CAN-TP) segments messages:

| Frame Type | PCI Nibble | Description |
| --- | --- | --- |
| Single Frame (SF) | 0x0_ | Payload fits in 1 to 7 bytes within a single CAN frame. |
| First Frame (FF) | 0x1_ | Initiates multi-frame transfer; encodes total 12-bit payload length (L ≤ 4095 bytes). |
| Consecutive Frame (CF) | 0x2_ | Carries subsequent data chunks with a 4-bit rolling Sequence Number (0 \dots 15). |
| Flow Control (FC) | 0x3_ | Sent by receiver to regulate pacing; sets Block Size (BS) and Separation Time ($ST_{min}$). |

4. Logic Analyzer Trace Walkthrough & Linux SocketCAN

4.1 Oscilloscope / Logic Analyzer Decoded Waveform

The following timing diagram illustrates a logic analyzer decode of an 11-bit Standard Data Frame transmitting ID 0x123, DLC 2, and payload 0xAA 0x55:

Line State: SOF [ ID = 0x123 ] RTR IDE r0 DLC [ Byte 0 ] [ Byte 1 ] [ CRC15 ] CD ACK AD [ EOF ] CAN_H: ───┐ ┌──┐ ┌──┐ ┌──┐ ┌──┐ ┌──┬──┬──┐ ┌──┬──┐ ┌──┐ ┌─┐ ┌─┐ ─────── │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ CAN_L: ───┴────┴──┴──┴──┴──┴──┴──────┴──┴────────┴──┴──┴──┴────┴──┴──┴───────┴──┴─────┴─┴─────┴─┴─────── Values: 0 0 0 1 0 0 1 0 0 0 0 0 0010 10101010 01010101 0x3A21 1 0 1 1111111 Bit Type: DOM <--- Arbitration ---> DOM DOM DOM 2Byte 0xAA 0x55 CRC REC DOM REC End-of-frame

4.2 Linux SocketCAN Application Architecture

Linux integrates CAN directly into the networking subsystem (AF_CAN). CAN interfaces are treated like network interfaces (eth0, can0), allowing standard POSIX socket APIs:

/* socketcan_tx.c - Production SocketCAN Transmit Example */ #include <stdio.h> #include <stdlib.h> #include <string.h> #include <unistd.h> #include <net/if.h> #include <sys/ioctl.h> #include <sys/socket.h> #include <linux/can.h> #include <linux/can/raw.h> int main(void) { int s; struct sockaddr_can addr; struct ifreq ifr; struct can_frame frame; /* 1. Open Raw CAN Socket */ if ((s = socket(PF_CAN, SOCK_RAW, CAN_RAW)) < 0) { perror("Socket creation failed"); return 1; } /* 2. Bind to can0 interface */ strcpy(ifr.ifr_name, "can0"); ioctl(s, SIOCGIFINDEX, &ifr); memset(&addr, 0, sizeof(addr)); addr.can_family = AF_CAN; addr.can_ifindex = ifr.ifr_ifindex; bind(s, (struct sockaddr *)&addr, sizeof(addr)); /* 3. Construct Standard 11-bit Frame */ frame.can_id = 0x123; frame.can_dlc = 4; frame.data[0] = 0xDE; frame.data[1] = 0xAD; frame.data[2] = 0xBE; frame.data[3] = 0xEF; /* 4. Transmit frame over physical bus */ if (write(s, &frame, sizeof(struct can_frame)) != sizeof(struct can_frame)) { perror("CAN write error"); close(s); return 1; } printf("CAN frame 0x123 sent successfully on can0.\n"); close(s); return 0; }

# Bring up can0 with 500 kbps nominal and 2 Mbps data bitrate on Linux sudo ip link set can0 type can bitrate 500000 dbitrate 2000000 fd on sudo ip link set can0 up # Monitor traffic in real time with timestamping candump -tz can0 # Inject diagnostic frame cansend can0 7DF#0201050000000000